3GPP TSG RAN Meeting #85		 RP-192319

September 16th – 20th, 2019

Newport Beach, CA, U.S.A.

Agenda item:	14

Source: 	Huawei

Title: 	Signalling of power class (offline drafting minutes)

Document for:	Discussion

Introduction

The contributions on EN-DC power class were submitted and discussed in RAN#85. This contribution captures the offline drafting minutes and conclusions.

Discussion

RAN4 agreement and specification for high UE power class

The following EN-DC 26dBm power classes are defined in TS38.101-3.

Table 6.2B.1.1-1: Maximum output power for EN-DC (continuous sub-blocks)

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA |  |  | 23 | +2/-3 |
| DC_(n)41AA | 26 | +2/-21 | 23 | +2/-21 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. |  |  |  |  |

Table 6.2B.1.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_3A_n3A2 |  |  | 23 | +2/-3 |
| DC_41A_n41A | 26 | +2/-21 | 23 | +2/-21 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: Power Class 3 is the default power class unless otherwise stated. |  |  |  |  |

The agreed power classes are defined.

| Option | EN-DC power class | LTE power class | NR Power class |
| --- | --- | --- | --- |
| A (intra-band/inter-band) | PC3 | PC3 | PC3 |
| F (intra-band) | PC2 | PC2 | PC2 |
| I (inter-band) | PC2 | PC3 | PC3 |

Question is that UE can report supporting of either PC2 or PC3 in standalone mode.

| Option | EN-DC power class | LTE power class | NR Power class | Reported NR power class in standalone mode |
| --- | --- | --- | --- | --- |
| A (intra-band/inter-band) | PC3 | PC3 | PC3 | PC2 |
| F (intra-band) | PC2 | PC2 | PC2 | PC2 2Tx 23dBm per Tx |
| I (inter-band) | PC2 | PC3 | PC3 | PC2 2Tx one 23dBm and one 26dBm |

Nokia: PC2 UE can fall back to PC3. There is no issue.

Sprint: Agree with Nokia. 23+23 was not discussed before.

Vivo: for SA, 23 + 23 should be PC2 UE.

RAN2 signalling

There is no signalling to indicate NR power class when UE is in EN-DC mode.

BandNR ::= SEQUENCE { bandNR FreqBandIndicatorNR, modifiedMPR-Behaviour BIT STRING (SIZE (8)) OPTIONAL, mimo-ParametersPerBand MIMO-ParametersPerBand OPTIONAL, extendedCP ENUMERATED {supported} OPTIONAL, multipleTCI ENUMERATED {supported} OPTIONAL, bwp-WithoutRestriction ENUMERATED {supported} OPTIONAL, bwp-SameNumerology ENUMERATED {upto2, upto4} OPTIONAL, bwp-DiffNumerology ENUMERATED {upto4} OPTIONAL, crossCarrierScheduling-SameSCS ENUMERATED {supported} OPTIONAL, pdsch-256QAM-FR2 ENUMERATED {supported} OPTIONAL, pusch-256QAM ENUMERATED {supported} OPTIONAL, ue-PowerClass ENUMERATED {pc1, pc2, pc3, pc4} OPTIONAL, ... BandCombinationList-v1540 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540 BandCombination ::= SEQUENCE { bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters, featureSetCombination FeatureSetCombinationId, ca-ParametersEUTRA CA-ParametersEUTRA OPTIONAL, ca-ParametersNR CA-ParametersNR OPTIONAL, mrdc-Parameters MRDC-Parameters OPTIONAL, supportedBandwidthCombinationSet BIT STRING (SIZE (1..32)) OPTIONAL, powerClass-v1530 ENUMERATED {pc2} OPTIONAL CA-ParametersEUTRA ::= SEQUENCE { multipleTimingAdvance ENUMERATED {supported} OPTIONAL, simultaneousRx-Tx ENUMERATED {supported} OPTIONAL, supportedNAICS-2CRS-AP BIT STRING (SIZE (1..8)) OPTIONAL, additionalRx-Tx-PerformanceReq ENUMERATED {supported} OPTIONAL, ue-CA-PowerClass-N ENUMERATED {class2} OPTIONAL, supportedBandwidthCombinationSetEUTRA-v1530 BIT STRING (SIZE (1..32)) OPTIONAL, ... } powerClass Power class that the UE supports when operating according to this band combination. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClass in BandNR), the latter determines maximum TX power available in each band. The UE sets the new power class parameter only in band combinations with two FR1 uplink serving cells.

Summary

No consensus was reached.

Reference

[1] RP‑192103, Huawei, HiSilicon, On EN-DC power class, 3GPP TSG RAN meeting #85, September 16th – 20th, 2019.

[2] RP‑192153, Vivo, Discussion on clear indication of power classes for EN-DC, 3GPP TSG RAN meeting #85, September 16th -20th, 2019.