3GPP TSG RAN WG1 Meeting #93  		R1-1807723

Busan, Korea, May 21st – April 25th, 2018

Title:	Reply LS on removal of CSI-RS-for-tracking from TCI-State

Response to:	R2-1806495

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	TSG RAN WG2

CC:

Contact Person:

#### Name:	Nan Zhang

E-mail Address:	zhang.nan152@zte.com.cn

1. Overall Description:

RAN1 would like to thank RAN2 for the question whether the csi-RS-for-tracking IE should actually be part of the TCI-State IE (and in case provide justification and/or reference to a section of their specification) or whether it can be removed and placed somewhere else, if needed.

From RAN1 perspective, including the resource set ID in the configuration of TCI is not aligned with the description of TCI in existing RAN1 spec. RAN1 has agreed that CSI-RS for tracking is configured as a set of CSI-RS resource with the same antenna port, periodicity, and bandwidth and subcarrier location. As the reference RS, any of the NZP CSI-RS resource IDs within the resource set configured with “TRS-Info” can be used to represent the CSI-RS for tracking in TCI configuration. The indication of corresponding CSI-RS resource ID is already supported by following IE in TCI-State IE:

{csi-rs						NZP-CSI-RS-ResourceId};

Hence functionality for QCL indication via corresponding configuration of TCI with the removal of the IE csi-RS-for-tracking is still complete.

According to the above analysis, for the IE csi-RS-for-tracking, RAN1 would like to confirm that it can be directly removed and there is no need to be placed somewhere else.

2. Actions:

To RAN2 group

ACTION:

RAN1 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next RAN1 Meetings:

RAN1#94	20th – 24th August 2018	Gothenburg, Sweden

RAN1#94bis	8th – 12th October 2018	Chengdu, China