3GPP TSG-RAN WG1 Meeting #94	R1-1809780

Gothenburg, Stockholm, August 20th – 24th, 2018

Source:	Ericsson

Title:	IMT-2020 self evaluation: mMTC non-full buffer connection density for LTE-MTC and NB-IoT

Agenda Item:	7.2.7.2

Document for:	Discussion

# Introduction

3GPP has as part of the New Study Item on Self Evaluation towards IMT-2020 Submission [1] agreed to “Provide self evaluation results against technical performance requirements for mMTC as per defined in Report ITU-R M.[IMT-2020. TECH PERF REQ]  [RAN1, RAN2], including Connection density”.

A submission time plan has also been agreed [2] where the 3GPP meetings for submission of description and compliance templates according to ITU-R report M.2411 [3] are set. The Compliance template for technical performance contains the minimum technical performance requirement item Connection density. To fulfil this requirement 3GPP must show that it’s candidate Radio Interface Technologies (RITs) submitted to ITU-R supports a connection density of 1 000 000 devices per km2. The evaluation is to be performed in accordance to test environment Urban Macro-mMTC as described in report ITU-R M.2412 [4].

In this contribution, we present results on Connection Density for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, and for NB-IoT. The LTE BL/CE mode operation is hereafter referred to as LTE-M operation. It is shown that LTE-M and NB-IoT meets the IMT-2020 requirement and should be part of the 3GPP submission to ITU-R. It is proposed to capture these results in the self-evaluation SI TR 37.919 [7].

The contribution also introduces the connection density requirement definition, the herein followed non-full buffer system simulation procedure and some of the most relevant simulation assumptions defining the test environment Urban Macro-mMTC.

This contribution is a revision of R1-1809263. A complete L2S model is now implemented including modelling of (N)PRACH, NPUSCH F2 and PUCCH link level performance.

# Introduction to the Connection density requirement

## Requirement definition

The connection density requirement requires a RIT to provide a certain Quality of Service (QoS) to 1 000 000 devices per km2 at a grade of service (GoS) of 99 percent. Service is considered provided when a message latency of less than 10 seconds is supported for a user attempting to send an uplink data packet of 32 bytes defined at layer 2. Besides the supported connection density, it is encouraged to report the connection efficiency which is defined as the connection density normalized by the required system bandwidth.

The connection density requirement requires 99% grade of service where acceptable quality of service is defined by a message latency of 10 seconds or less.

## System simulation procedure

Report IMT-2020.EVAL [3] outlines two system simulator procedures for evaluating connection density. The first is a non-full buffer system level simulation that requires a state of the art system simulator to perform the evaluations. In this paper, we have followed this setup. The second approach is for a full buffer system simulation that allows input based on a more rudimentary system simulator combined with post processing supported by link level simulations. Both approaches have their merits and we provide input for the full buffer methodology in [6].

Table 1 Non-full buffer system level simulation procedure [3].

## Test environment

Report IMT-2020.EVAL specifies the test environment to be used in the evaluations according to Table 2. The simulations presented in this contribution where to a large extent following this set of assumptions. Further detailed assumptions are outlined in section 3.

Table 2: Urban Macro-mMTC test environment definition [3].

| Parameters | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss Note: Applies only to Channel model B. | 20% high loss, 80% low loss Note: Applies only to Channel model B. |
| Inter-site distance | 500 m | 1732 m |
| Number of antenna elements per TRxP | Up to 64 Tx/Rx | Up to 64 Tx/Rx |
| Number of UE antenna elements | Up to 2 Tx Up to 2 Rx | Up to 2 Tx Up to 2 Rx |
| Device deployment | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| UE speeds of interest | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Only 1 message/2 hours/device studied herein. | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Only 1 message/2 hours/device studied herein. |
| Simulation bandwidth | Up to 10 MHz | Up to 50 MHz |
| UE antenna height | 1.5 m | 1.5 m |

# Simulation configuration

## Simulation assumptions

Table 3 presents the herein investigated LTE-M and NB-IoT system level configurations, that overrides or comes in addition to those presented in Table 2.

Table 3: Simulation configuration.

| Parameter | LTE-M | NB-IoT |
| --- | --- | --- |
| Mode of operation | Inband | Inband |
| DL carrier configuration | NB not carrying SSS, PSS, PBCH or PDSCH containing SIB-BR transmission. | Non-anchor carrier not carrying NSSS, NPSS, NPBCH or NPDSCH containing SIB1-NB transmission. |
| UL carrier configuration | PRACH configured on 1 subframe in each radio frame corresponding to 10% overhead. | NPRACH configured on 7% of all UL resources. |
| (N)PRACH configuration | CE 0: 1 rep, 10 ms periodicity CE 1: 4 rep, 40 ms periodicity, -122 dBm CE threshold CE 2: 16 rep, 160 ms periodicity, -130 dBm CE threshold CE 3: 64 rep, 640 ms periodicity, -138 dBm CE threshold | CE 0: 2 rep, 160 ms periodicity CE 1: 8 rep, 640 ms periodicity, -126 dBm CE threshold CE 2: 32 rep, 2560 ms periodicity, -136 dBm CE threshold |
| Simulated system bandwidth | 1 NB (6 PRB) | 1 PRB |
| eNB total transmit power per TRxP | 29 dBm/PRB | 29 dBm/PRB Note: Power boosting not applied. |
| eNB number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] |
| eNB number of TXRU per TRxP | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] |
| UE transmit power | 23 dBm | 23 dBm |
| UE number of TXRU | 1 TXRU [5] | 1 TXRU [5] |
| UE number of antenna elements | 1Tx/Rx 0° polarization [5] | 1Tx/Rx 0° polarization [5] |
| UE antenna element gain | 0 dBi [5] | 0 dBi [5] |
| UE antenna element pattern | Omni-directional [5] | Omni-directional [5] |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | Conf A: 93° in GCS Conf B: 99° in GCS Note: Selected to optimize UL/DL SINR. | Conf A: 93° in GCS Conf B: 99° in GCS Note: Selected to optimize UL/DL SINR. |
| Handover margin (dB) | 2 dB (handover modelled explicitly) | 2 dB (handover modelled explicitly) |
| TRxP boresight | 30 / 150 / 270 degrees [5] | 30 / 150 / 270 degrees [5] |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] |
| Wrapping around method | Radio distance-based wrapping | Radio distance-based wrapping |
| Minimum distance of TRxP and UE | 10 m [5] | 10 m [5] |
| Polarized antenna model | Model-2 in TR36.873 [5] | Model-2 in TR36.873 [5] |
| System layout | 7 sites, 3 sectors per site | 7 sites, 3 sectors per site |
| TRxP number per site | 3 [5] | 3 [5] |
| Scheduling strategy | Round robin | Round robin |
| (N)PDCCH, (N)PDSCH, (N)PUSCH (F1) Link to system (L2S) model | Based on Mutual information Effective SINR mapping. A doubling in the number of repetitions is modelled with a 3 dB Effective SINR gain. | Based on Mutual information Effective SINR mapping. A doubling in the number of repetitions is modelled with a 3 dB Effective SINR gain. |
| PUCCH, NPUSCH F2 | PUCCH is transmitted at the edges of the LTE system BW, i.e. outside of the NB. The PUCCH BLEP is therefore based on an average SNR to BLEP mapping derived from TDL-iii link level simulations. (See Annex.) | NPUSCH F2 is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. (See Annex.) |
| (N)PRACH L2S model | PRACH BLEP is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. Access is failed in case of intra-cell collision. (See Annex.) | NPRACH BLEP is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. Access is failed in case of intra-cell collision. (See Annex.) |
| (N)PBCH L2S model | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations. (See Annex.) | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations (See Annex.) |
| (N)PSS/(N)SSS L2S model | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations (See Annex.) | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations (See Annex.) |
| Packet drop timer | 20 sec | 20 sec |
| RRC Connection setup procedure | CIoT UP Optimization (RRC Resume) | CIoT UP Optimization (RRC Resume) |
| RLC Mode | AM | AM |

The RCC Resume procedure modelled is aligned with the message flow presented in Figure 1.

Figure 1: Data and signalling flow used to model RRC Resume latency performance.

The message sizes used in the RRC Resume procedure are summarised in Table 4.

Table 4: RRC resume message sizes.

| Message | LTE-M | NB-IoT |
| --- | --- | --- |
| Msg2: RAR | 7 bytes | 7 bytes |
| Msg3: RRC Connection Resume Request | 7 bytes | 11 bytes |
| Msg4: RRC Connection Resume | 19 bytes | 19 bytes |
| Msg5: RRC Connection Resume Comp + RLC Ack for Msg4 | 17 bytes | 16 bytes |
| RRC Connection Release | 18 bytes | 17 bytes |

## Channel model

The coupling gain for the different configurations and channel models was recorded by the simulator and are shown in Figure 2 below. These include antenna gains, Urban Macro A and Urban Macro B SCM channel models and outdoor to indoor losses. The large cell size of Configuration B leads to a higher coupling loss than what is seen for Configuration A. Channel model UMA A contains aggressive assumptions for the indoor to outdoor loss which is also visible in the below figure.

Figure 2: Coupling gain distribution for test environment Urban Macro mMTC based on the settings of Table 1.

# Connection density evaluation

The achieved latencies for LTE-M and NB-IoT are presented in Figure 3. The results are reported per cell and per resource block (RB). To translate the x-axis to arrival intensity per km2 the Configuration A results should be scaled by a factor 1/0.072 and the Configuration B results by a factor 1/0.866. To get the LTE-M results per narrowband a scaling factor of 6 should be applied.

LTE-M offers a superior latency performance until the system reaches the point of congestion. This thanks to its support of higher order modulation and bandwidths up to 6 PRBs. NB-IoT offers a higher connection density partly thanks to its robust performance under low SINR. These observations are supported by Figure 4 in the Annex: It is shown that LTE-M has a higher DL resource utilization than NB-IoT which is believed to be partly due to the use of higher bandwidth to facilitate low latency and high DL coverage. Figure 5 indicate that the higher resource utilization leads to higher DL interference levels, which eventually limits the performance of the systems. Figure 6 shows the UL interference statistics for the devices in worst radio conditions. For Config B a significant offset between LTE-M and NB-IoT UL interference levels are seen. This is explained by the minimal scheduling unit, which for NB-IoT is one subcarrier while for LTE-M it is 1 PRB. The LTE-M Release 15 feature for sub-PRB allocation is hence not supported in these simulations.

In absolute numbers the highest connection density is supported for Configuration A. This since the cell area is 12 times smaller for the 500 m ISD, compared to the 1732 m ISD in Configuration B. This supports a densification of the cell grid which supports higher access loads.

It should be noted that the reported latency includes the idle mode time to synchronize to the cell and read the master information block.

Figure 3: LTE-M and NB-IoT service latency at the 99th percentile.

In Table 5 the connection density is reported based on the results presented in Figure 3. Given the assumed cell size of 0.07 km2 in Configuration A and 0.86 km2 in Configuration B and the traffic model 1 message/2 hours/device the arrival intensity can be translated into number of supported devices per km2.

Table 5 also presents the needed bandwidth to support the connection density target. In addition to the reported bandwidth a NB is needed for LTE-M to carry synchronization and master/system information signaling. For NB-IoT an additional anchor PRB is needed to carry synchronization and master/system information signalling. For completeness the connection efficiency and spectral efficiency are presented.

Table 5 LTE-M and NB-IoT performance metrics.

|  | Connection density @ 99 percent GoS [devices/NB or PRB] | Bandwidth to support 1 000 000 devices per km2 [NB or PRB] | Connection efficiency [devices/Hz] | Spectral efficiency [bits/s/Hz] |
| --- | --- | --- | --- | --- |
| LTE-M: UMA A, Conf A | 5.680.683 devices/NB | 1 NB | 5.26 devices/Hz | 0.19 bits/s/Hz |
| LTE-M: UMA B, Conf A | 5.680.683 devices/NB | 1 NB | 5.26 devices/Hz | 0.19 bits/s/Hz |
| LTE-M: UMA A, Conf B | 342.218 devices /NB | 3 NB | 0.32 devices/Hz | 0.01 bits/s/Hz |
| LTE-M: UMA B, Conf B | 444.983 devices /NB | 3 NB | 0.41 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA A, Conf A | 1.233.109 devices /1 PRB | 1 PRBs | 6.85 devices/Hz | 0.24 bits/s/Hz |
| NB-IoT: UMA B, Conf A | 1.225.128 devices/1 PRB | 1 PRBs | 6.81 devices/Hz | 0.24 bits/s/Hz |
| NB-IoT: UMA A, Conf B | 67.928 devices/1 PRB | 15 PRBs | 0.38 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA B, Conf B | 94.035 devices/1 PRB | 11 PRBs | 0.52 devices/Hz | 0.02 bits/s/Hz |

# Text proposal to TR 37.910 v0.1.0

To capture the results presented in this paper we below make a text proposal for TR 37.910 v0.1.0 [7]. Text highlighted in yellow should be verified, and if needed updated before inserted in TR 37.910.

-----------------------------------------------------------Text start: TR 37.910 ---------------------------------------------------

# 7	Self evaluation of mMTC technical performance

## 7.1	Connection density

### 7.1.1	Full buffer evaluation

…

### 7.1.2	Non-full buffer evaluation

The IMT-2020 submission specifies two types of evaluation methods for the mMTC Connection Density performance metric. Here results are presented for the Non-full buffer methodology where a dynamic system level simulator is configured to evaluate the supported mMTC Connection density at 99% grade of service. The evaluation framework followed for these evaluations is presented in [Report ITU-R M.2412].

Two 3GPP entities provided input for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, and for NB-IoT. The results are summarized in section 7.1.2.1.

#### 7.1.2.1	System level configurations

Table T1 presents the LTE and NB-IoT system level configurations, used by one of the contributing 3GPP entities, that complements the IMT-2020 evaluation framework.

Table T1: Simulation configuration.

| Parameter | LTE | NB-IoT |
| --- | --- | --- |
| Mode of operation | Inband | Inband |
| Simulated system bandwidth | 1 NB (6 PRB) | 1 PRB |
| DL carrier configuration | NB not carrying SSS, PSS, PBCH or PDSCH containing SIB-BR transmission. | Non-anchor carrier not carrying NSSS, NPSS, NPBCH or NPDSCH containing SIB1-NB transmission. |
| UL carrier configuration | PRACH configured on 1 subframe in each radio frame corresponding to 10% overhead. | NPRACH configured on 7% of all UL resources. |
| (N)PRACH configuration | CE 0: 1 rep, 10 ms periodicity CE 1: 4 rep, 40 ms periodicity, -122 dBm CE threshold CE 2: 16 rep, 160 ms periodicity, -130 dBm CE threshold CE 3: 64 rep, 640 ms periodicity, -138 dBm CE threshold | CE 0: 2 rep, 160 ms periodicity CE 1: 8 rep, 640 ms periodicity, -126 dBm CE threshold CE 2: 32 rep, 2560 ms periodicity, -136 dBm CE threshold |
| eNB total transmit power per TRxP | 29 dBm/PRB | 29 dBm/PRB Note: Power boosting not applied. |
| eNB number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| eNB number of TXRU per TRxP | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] |
| UE transmit power | 23 dBm | 23 dBm |
| UE number of TXRU | 1 TXRU | 1 TXRU |
| UE number of antenna elements | 1Tx/Rx 0° polarization | 1Tx/Rx 0° polarization |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | Conf A: 93° in GCS Conf B: 99° in GCS Note: Selected to optimize UL/DL SINR. | Conf A: 93° in GCS Conf B: 99° in GCS Note: Selected to optimize UL/DL SINR. |
| Handover margin (dB) | 2 dB (handover modelled explicitly) | 2 dB (handover modelled explicitly) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Radio distance-based wrapping | Radio distance-based wrapping |
| Minimum distance of TRxP and UE | 10 m | 10 m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| System layout | 7 sites, 3 sectors per site | 7 sites, 3 sectors per site |
| TRxP number per site | 3 | 3 |
| Scheduling strategy | Round robin | Round robin |
| (N)PDCCH, (N)PDSCH, (N)PUSCH (F1) Link to system (L2S) model | Based on Mutual information Effective SINR mapping. A doubling in the number of repetitions is modelled with a 3 dB Effective SINR gain. | Based on Mutual information Effective SINR mapping. A doubling in the number of repetitions is modelled with a 3 dB Effective SINR gain. |
| PUCCH, NPUSCH F2 | PUCCH is transmitted at the edges of the LTE system BW, i.e. outside of the NB. The PUCCH BLEP is therefore based on an average SNR to BLEP mapping derived from TDL-iii link level simulations. | The NPUSCH F2 L2S is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. |
| (N)PRACH L2S model | PRACH L2S is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. Access is failed in case of intra-cell collision. | NPRACH L2S is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. Access is failed in case of intra-cell collision. |
| (N)PBCH L2S model | Average SINR to acquisition delay mapping. | Average SINR to acquisition delay mapping |
| (N)PSS/(N)SSS L2S model | Average SINR to acquisition delay mapping | Average SINR to acquisition delay mapping |
| Packet drop timer | 20 sec | 20 sec |
| RRC Connection setup procedure | CIoT UP Optimization (RRC Resume) | CIoT UP Optimization (RRC Resume) |
| RLC Mode | AM | AM |

The RCC Resume procedure model is aligned with the message flow presented in Figure F1.

Figure F1. Data and signalling flow used to model RRC Resume latency performance.

The message sizes used in the RRC Resume procedure are summarised in Table T2.

Table T2.  RRC resume message sizes.

| Message | LTE-M | NB-IoT |
| --- | --- | --- |
| Msg2: RAR | 7 bytes | 7 bytes |
| Msg3: RRC Connection Resume Request | 7 bytes | 11 bytes |
| Msg4: RRC Connection Resume | 19 bytes | 19 bytes |
| Msg5: RRC Connection Resume Comp + RLC Ack for Msg4 | 17 bytes | 16 bytes |
| RRC Connection Release | 18 bytes | 17 bytes |

#### 7.1.2.2	Non-full buffer connection density performance

Figure F3 shows the simulated load, in terms of user arrival intensity, versus achieved message latency for LTE and NB-IoT presented by one of the contributing 3GPP entities.

Figure F3. LTE and NB-IoT 99th percentile message latency.

Table T3 reports the connection density based on the results presented in Figure F3. Given the assumed cell size of 0.07 km2 in Configuration A and 0.86 km2 in Configuration B and the assumed traffic model of 1 message/2 hours/device the arrival intensity can be translated into number of supported devices per km2.

Table T3 also presents the needed bandwidth to support the connection density target. In addition to the reported bandwidth a NB is needed for LTE to carry synchronization and master/system information signalling. For NB-IoT an additional anchor PRB is needed to carry synchronization and master/system information signalling. For completeness the connection efficiency and spectral efficiency are presented.

Table T3. LTE and NB-IoT performance.

|  | Connection density @ 99 percent GoS [devices/NB or PRB] | Bandwidth to support 1 000 000 devices per km2 [NB or PRB] | Connection efficiency [devices/Hz] | Spectral efficiency [bits/s/Hz] |
| --- | --- | --- | --- | --- |
| LTE-M: UMA A, Conf A | 5.680.683 devices/NB | 1 NB | 5.26 devices/Hz | 0.19 bits/s/Hz |
| LTE-M: UMA B, Conf A | 5.680.683 devices/NB | 1 NB | 5.26 devices/Hz | 0.19 bits/s/Hz |
| LTE-M: UMA A, Conf B | 342.218 devices /NB | 3 NB | 0.32 devices/Hz | 0.01 bits/s/Hz |
| LTE-M: UMA B, Conf B | 444.983 devices /NB | 3 NB | 0.41 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA A, Conf A | 1.233.109 devices /1 PRB | 1 PRBs | 6.85 devices/Hz | 0.24 bits/s/Hz |
| NB-IoT: UMA B, Conf A | 1.225.128 devices/1 PRB | 1 PRBs | 6.81 devices/Hz | 0.24 bits/s/Hz |
| NB-IoT: UMA A, Conf B | 67.928 devices/1 PRB | 15 PRBs | 0.38 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA B, Conf B | 94.035 devices/1 PRB | 11 PRBs | 0.52 devices/Hz | 0.02 bits/s/Hz |

# Conclusions

In this contribution, we have discussed the IMT-2020 connection density requirement and provided input to the 3GPP Study Item on Self Evaluation towards IMT-2020 Submission. It has been shown that LTE-M and NB-IoT fulfils the IMT-2020 connection density requirement and should be part of the 3GPP IMT-2020 submission. A text proposal to the TR 37.919 is made to capture the presented results.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

RP-172098, “3GPP submission towards IMT-2020”, ITU-R Ad-Hoc Contact Person

Report ITU-R M.2411-0 Requirements, evaluation criteria and submission templates for development of IMT-2020, ITU-R

Report ITU-R M.2412-0 Guidelines for evaluation of radio interface technologies for IMT-2020, ITU-R

RP-172728, LS on Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”, RAN#78, Huawei

R1-1809262, “IMT-2020 self evaluation: mMTC full buffer connection density”, RAN1#93, Ericsson

RP-180523, TR 37.919 v0.1.0 Study on Self Evaluation towards IMT-2020 Submission (Rel-15), RAN#79

# Annex: Simulation results

Figure 4 Average DL resource utilization.

Figure 5 1st percentile DL SINR.

Figure 6 1st percentile UL SINR.

Figure 7 PBCH and NPBCH acquisition delay link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective channel.

Figure 8 PSS/SSS and NPSS/NSSS acquisition delay link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective signal.

Figure 9 PRACH and NPRACH misdetection link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective signal.

Figure 10 PUCCH and NPUSCH F2 misdetection link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective signal.