3GPP TSG-RAN WG2 #101bis	Tdoc R2-180 05427

# Sanya, P. R China, 16 – 20 April 2018						Revision of R2-1803153

Agenda Item:	10.2.2.1

Source:	Ericsson

Title:	Notification control

Document for:	Discussion, Decision

# Introduction

RAN2 received LS from SA2 related to notification control:

S2-182856:

SA2 thanks RAN2 for the LS on QoS. SA2 has discussed the LS from RAN2 and agreed to the following for notification control as in agreed CR (attached):

“If, for a given GBR QoS Flow, notification control is enabled and the NG-RAN determines that the GFBR cannot be fulfilled, RAN shall send a notification towards SMF and keep the QoS Flow (i.e. while the NG-RAN is not delivering the requested GFBR for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow , e.g. due to RLF or internal-RAN congestion.”

Notification Control feature is part of NR QoS framework. TS 23.501 states following about the notification control:

5.7.2.4	Notification control

The Notification control indicates whether notifications are requested from the RAN when the GFBR can no longer (or again) be fulfilled for a QoS Flow during the lifetime of the QoS Flow. If, for a given GBR QoS Flow, notification control is enabled and the NG-RAN determines that the GFBR cannot be fulfilled, RAN shall send a notification towards SMF. The RAN shall keep the QoS Flow, and should try to fulfil the GFBR. Upon receiving a notification from the RAN that the GFBR cannot be fulfilled, the 5GC may initiate N2 signalling to modify or remove the QoS Flow. When applicable, NG-RAN sends a new notification, informing SMF that the GFBR can be fulfilled again. After a configured time, the NG-RAN may send a subsequent notification that the GFBR cannot be fulfilled.

# Discussion

Notification control may be configured for GBR Flows. When configured, RAN has possibility to send indication to the CN if requirements set for the QoS Flow cannot be fulfilled. The decision is made based on parameters associated with the QoS Flows and it is up to the implementation to decide exactly when the notification is sent.

In the RAN#100 meeting, some companies proposed that there is need to associate additional information to the notification control message. For example, information about the scheduler state. This, however, is problematic approach as the scheduling information is dynamic may already be outdated when notification reaches the destination. Therefore, the notification control should be kept simple from RAN2 perspective.

Table 1	3GPP TS 23.502, 4.3 Session Management procedures

4.3.2 PDU Session Establishment

14.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, AN Tunnel Info, List of accepted/rejected QFI(s))).

4.3.3	PDU Session Modification

6.	The (R)AN may acknowledge N2 PDU Session Request by sending a N2 PDU Session Ack (N2 SM information (List of accepted/rejected QFI(s), AN Tunnel Info, PDU Session ID), User location Information)

The Notification Control only applies in the case when QoS Flow has already been established and RAN has not made decision on removing a Flow. It is clear from the Session management procedures above that RAN has option to reject QoS Flow on QoS Flow establishment procedure. Therefore, RAN has possibility to reject the QoS Flow if it cannot fulfil the requirements set by the QoS parameters.  Additionally, if RAN decides to release QoS Flow, there is no need to send notification, as the QoS Flow release message is sent to the CN. It hardly makes sense to send notification of congestion if QoS Flow is removed. Therefore, Notification Control should only apply between QoS Flow establishment and QoS Flow release.

It is clear that RAN has option to reject QoS Flows that cannot be fulfilled. Therefore, Notification Control only applies to the case where QoS Flows have already been accepted.

Notification control only applies during the lifetime of QoS Flow, i.e. after RAN has accepted the QoS Flow and before RAN has decided to release QoS Flow.

RAN2#AH January sent LS to SA2 with the following content.

In RAN2’s understanding the gNB is allowed to release a bearer/QoS flow for which a notification control to the CN is sent, and for which a response or action have not been received from the CN. This might be needed e.g. in overload conditions or in case of RLF. Concerns were expressed that not allowing the gNB to release a flow when notification control is enabled would leave radio resources under the control of 5GC.

While RAN2 is waiting for the SA2 input, the RAN view could be clarified. In general, failure to fulfil the GFBR of a GBR QoS Flow with Notification Control enabled is not a sufficient condition to trigger release of the RAN resources onto which that GBR QoS Flow is mapped. No other RAN specific aspect/condition is considered by the Notification Control functionality. In other words, any other RAN aspect (e.g. RLF, internal resource failure etc.) may be used by RAN as a trigger to release the RAN resources onto which a GBR QoS Flow is mapped, regardless of whether the GFBR can be fulfilled or not and regardless whether Notification control is enabled or not.

Regardless of Notification Control, RAN is responsible of RAN resources.

Notification Control is expected to include behaviour that 5GC executes upon receiving the notification from RAN. The discussion of Notification Control should be left for RAN3 and SA2.

Notification Control has no impact on RAN2 specifications

Discussion of Notification Control is left for RAN3 and SA2.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is clear that RAN has option to reject QoS Flows that cannot be fulfilled. Therefore, Notification Control only applies to the case where QoS Flows have already been accepted.

Observation 2	Notification control only applies during the lifetime of QoS Flow, i.e. after RAN has accepted the QoS Flow and before RAN has decided to release QoS Flow.

Observation 3	Regardless of Notification Control, RAN is responsible of RAN resources.

Observation 4	Notification Control has no impact on RAN2 specifications

Based on the discussion in section 2 we propose the following:

Proposal 1	Discussion of Notification Control is left for RAN3 and SA2.

# References

3GPP TS 23.501 V1.5.0 (2017-11). System Architecture for the 5G System

R2-1801619. LS on QoS. RAN2. Vancouver, Canada, 22 – 26 January 2018.