3GPP TSG RAN WG2 Meeting #102	R2-1808869

Busan, South Korea, May 21-25, 2018

Title:	Security related agreements on Early Data Transmission

# Release:	Rel-15

Work Item:	LTE_eMTC4-Core, NB_IOTenh2-Core

Attachment:

Source:	RAN2

To:	SA3

CC:

# Contact Person:

# Name:	Umesh Phuyal

E-mail Address:	uphuyal@qti.qualcomm.com

# 1. Overall Description:

Regarding the Rel-15 WIs on ReMTC and NB-IoT enhancements [1] [2], RAN2 discussed further on details of the Early Data Transmission and reached the following agreements:

Agreements: - RAN2 understands that target eNB checking the shortResumeMAC-I may be an implementation possibility, but there is no specification impact in RAN2. - UL EDT data is deciphered by the target eNB. - The UE may use EDT in the next resume procedure if it receives RRCConnectionReject message with suspend indication in response to RRCConnectionResumeRequest message for EDT. - No additional parameters are included in the calculation of shortResumeMAC-I for Msg3 in EDT considering that the issue, i.e. replay attack and MiM attack, exists today in legacy resume procedure. - 16-bit version of shortResumeMAC-I, i.e. same in legacy, is used in Msg3 for EDT. - UL data in Msg3 is ciphered using newly derived key. - RRCConnectionRelease message in Msg4 is integrity protected and ciphered. - RRCConnectionReject message is sent in CCCH (SRB0), i.e. unprotected, after the security has been activated for a UE that has initiated EDT. - The user DL data, if any, in Msg4, i.e. RRC Release message for EDT, is de-ciphered using the same keys used for ciphering the UL data in Msg3 for the same EDT session. - RRCConnectionResume in Msg4 is integrity protected and ciphered. - NCC in RRCConnectionResume message is ignored if the RRCConnectionResume message was received in response to RRCConnectionResumeRequest message for EDT. - RRCConnectionSetup message is sent unprotected in CCCH (SRB0). - The UE shall continue using the activated AS security and ignore NCC in Msg4 in case of fall back before receiving Msg4, i.e. due to legacy grant received in Msg2, legacy RRC connection resume procedure. - PDCP checks integrity protection of RRCConnectionResume message and RRCConnectionRelease message in Msg4 for EDT. - Old integrity key (used in the last RRC connection as in legacy) is used to generate shortResumeMAC-I. - If RRCConnectionReject message with suspend indication is received in response to RRCConnectionResumeRequest message for EDT, same key, used for the calculation of shortResumeMAC-I for the connection attempt that was rejected, is used for the calculation of shortResumeMAC-I in the next resume procedure. - If the UE decides to fall back after EDT procedure has been initiated, but no RAR message has been received yet, i.e. UE uses non-EDT preamble for the next attempt, the UE reverts to legacy resume procedure.

RAN2 kindly requests SA3 to take the above information into account and implement necessary changes in SA3 specifications, if any.

# 2. Actions to SA3:

RAN2 kindly requests SA3 to take the above information into account and implement necessary changes in SA3 specifications, if any.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #103	20 - 24 Aug 2018	 	Gothenburg, Sweden

TSG RAN WG2 Meeting #103bis	08 - 12 Oct 2018		Chengdu, China

References:

- RP-170732, New WID on Even further enhanced MTC for LTE, Ericsson, Qualcomm

- RP-170852, New WID on Further NB-IoT enhancements, Huawei, HiSilicon, Neul