3GPP TSG-RAN WG2 Meeting #120	 R2-2213064

Toulouse, France, 14 – 18 November, 2022

# Title:	Reply LS on reference SSB for s-MeasureConfig checking

# Response to:	R2-2212985/R4-2218138

# Release:	Release 17

Work Item:	       NR_redcap-Core

Source:	RAN2

To:	RAN4

Cc:	-

Contact Person:

Name:	Naveen Palle

E-mail Address:	naveen dot palle at apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 would like to thank RAN4 for their LS on reference SSB for s-MeasureConfig checking.

RAN2 confirms the RAN4 understanding that the reference SSB used for threshold for s-MeasureConfig is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP, and if the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig.

RAN2 has the following field descriptions in TS38.331 for the usage of NCD-SSB for serving cell measurements and the corresponding note in TS38.300 which reflects the RAN4 understanding. RAN2 thinks that no changes are needed in specification.

TS38.331 (sec 6.2.3):

nonCellDefiningSSB

If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB.

The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

servingCellMO

measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and nonCellDefiningSSB in BWP-DownlinkDedicated of the associated downlink BWP: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB in the nonCellDefiningSSB. If the field is present in a downlink BWP and the BWP is activated, the RedCap UE uses this measurement object for serving cell measurements, otherwise, the RedCap UE uses the servingCellMO in ServingCellConfig IE.

TS38.300 (sec 9.2.4):

NOTE 2a:	If a RedCap UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above.

2. Actions:

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to take above information into consideration.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #121                   27th February – 3rd March 2023		Athens, GR

TSG-RAN WG2 Meeting #121bis-e	17th - 26th April 2023 		e-Meeting