3GPP TSG-RAN WG4 Meeting #85																R4-1712469

Reno, US, 27th Nov – 1st Dec 2017

Source: 	Nokia, U.S. Cellular

Title: 					TP to TR 36.715-03-01: CA_3DL_12A-46C_1UL_BCS0

Agenda Item: 			8.3.2

Document for:	Approval

# Introduction

This TP introduces 3DL/1UL CA configuration CA_12A-46C in to the TR 36.715-04-01

**************************** Start of changes ********************************************

## 5.X	CA_3DL_12A-46C-1UL_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_12A-46C | - | 12 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.X.2	UE Co-existence studies

Table 5.X.3-3 summarizes frequency ranges where harmonics occur due to Band 12 and Band 46 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 1398 | 1432 | 2097 | 2148 |
| 46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |

It can be seen from Table 5.X.2-1 that there is no harmonic interference for CA_12A-46Ctowards own receive bands.

### 5.X.3	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of band 12 and Band 46 for 1UL/3DL CA_12A-46C, the TIB,c and RIB,c  values are shown in table 5.X.3-1 and  table 5.X.3-2, respectively.

dTib values for bands 5 and 12 are taken from existing CA_13A-46A.

Table 5.X.3-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_12A-46C | 12 | 0 |
|  | 46 | 0 |

dRib values for bands 5 and 12 are taken from existing CA_13A-46A.

Table 5.X.0-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_12A-46C | 12 | 0 |
|  | 46 | 0 |

### 5.X.4	REFSENS requirements

REFSENS requriement for CA_12A-46C is presented in Table 5.X-4-1

Table 5.X.4-1: REFSENS requriements for CA_12A-46C

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_12A-46C | 12 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |

**************************** End of changes ********************************************