3GPP TSG-SA WG3 LI Meeting #72	S3i190058

Sophia Antipolis (FRA), Jan 22nd - Jan 25th 2019

Source:	NTAC

Title:	pCR to resolve Editor’s Notes in 33.128 Clause 4

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	FS_15LIS

Abstract of the contribution: This pCR proposes text to resolve the Editor’s Notes in 33.128 at the start of clause 4

FIRST CHANGE (Clause 4)

# 4	General

## 4.1	Introduction

The present document provides details of the internal and external interfaces required for a network operator, access provider and/or service provider to provide the necessary information to a Law Enforcement Agency (LEA) required to meet LI requirements. LI requirements for 3GPP networks and services are given in TS 33.126 [3].

The high-level architecture that defines the necessary interfaces is specified in TS 33.127 [5]. The generic high-level architecture is as follows:

The specification of the interfaces is split into two parts.

- Internal interfaces used between an operator’s network functions are described in clause 4.2.

- External interfaces used in communicating with a LEA are described in section 4.3.

## 4.2	Basic Principles for Internal Interfaces

This clause lists the internal interfaces shown in clause 4.1, indicates the protocol used to realise each interface, and gives a reference to the relevant clauses of the present document that specify how the protocol is to be used for the given interface.

Table 4.2-1: Internal interfaces and related protocols

| Interface | Description | Protocol used to realise interface | Usage |
| --- | --- | --- | --- |
| LI_SI | Used to provide system information to the LIPF from the SIRF | Out of scope of the present document |  |
| LI_X1 | Used to configure and audit Directly-provisioned POIs, TFs and MDFs. | ETSI TS 103 221-1 [7] | See section 5.2.2 |
| LI_X1 (Management) | Used to audit Triggered POIs | ETSI TS 103 221-1 [7] | See section 5.2.3 |
| LI_X2 | Used to pass xIRI information from IRI-POIs to the MDF2 | ETSI TS 103 221-2 [7] | See section 5.3.2 |
| LI_X3 | Used to pass xCC information from CC-POIs to the MDF3 | ETSI TS 103 221-2 [7] | See section 5.3.3 |
| LI_T2 | Used to pass triggering information from the IRI-TF to a Triggered IRI-POI | ETSI TS 103 221-1 [7] | See section 5.2.4 |
| LI_T3 | Used to pass triggering information from a CC-TF to a Triggered CC-POI | ETSI TS 103 221-1 [7] | See section 5.2.4 |
| LI_ADMF | Used to pass intercept provisioning information form the LICF to the LIPF | Out of scope of the present document |  |
| LI_MDF | Used by MDF2 and MDF3 in interactions necessary to correctly generate CC and IRI from xCC and xIRI. | Out of scope of the present document |  |

## 4.3	Basic Principles for Handover Interfaces

This clause lists the external and handover interfaces shown in clause 4.1, indicates the protocol used to realise each interface, and gives a reference to the relevant clauses of the present document that specify how the protocol is to be used for the given interface.

Table 4.3-1: Internal interfaces and related protocols

| Interface | Description | Protocol used to realise interface | Usage |
| --- | --- | --- | --- |
| LI_HI1 | Used to send warrant and other interception request information from LEA to operator. | ETSI TS 103 120 [6] shall be supported Other methods (e.g manual exchange) may be used depending on national regulatory requirements. | See section 5.3.4 |
| LI_HI2 | Used to send IRI from the MDF2 to the LEMF | ETSI TS 102 232 - 1 [9] and ETSI TS 102 232 – 7 [10] shall be supported | See section 5.3.5 |
| LI_HI3 | Used to send CC from the MDF3 to the LEMF | ETSI TS 102 232 -1 [9] and ETSI TS 102 232 – 7 [10] shall be supported | See section 5.3.5 |
| LI_HI4 | Used to send notification information from MDF2/3 to LEMF | ETSI TS 102 232 - 1 [9] and ETSI TS 102 232 – 7 [10] shall be supported | See section 5.3.6 |

END OF CHANGES