3GPP TSG-SA4 Meeting #101 	S4-181288

# Busan, Korea, Nov 19 - 23, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.929 | CR | - | rev | - | Current version: | 0.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client reference architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_QoE_VR |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Client architecture does not match the defined 3GPP one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update architecture based on client architecture in 26.118. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unaligned client architectures in 3GPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 6	VR QoE reference model under consideration

## 6.1	General description

A reference model for VR QoE measurement is illustrated in Figure 6.1.1, defining a number of observation points where specific metric-related information can be made available to the Metrics Collection and Computation (MCC) function. The MCC can use and combine information from the different observation points to calculate more complex metrics.

Note that these observation points are only defined conceptually, and might not always directly interface to the MCC. For instance, an implementation might relay information from the actual observation points to the MCC via the VR application. It is also possible that the MCC is not separately implemented, but simply included as an integral part of the VR application.

Figure 6.1.1: Client reference model for VR QoE measurement

## 6.2	Observation point 1

The access engine fetches the MPD, constructs and issues segment requests for relevant adaptation sets or preselections as ordered by the VR application, and receives segments or parts of segments. It may also adapt between different representations due to changes in available bitrate. The access engine provides a conforming 3GPP VR track to the file decoder.

The interface from the access engine towards MCC is referred to as observation point 1 (OP1) and

is defined to monitor:

- A sequence of transmitted network requests, each defined by its transmission time, contents, and the TCP connection on which it is sent

- For each network response, the reception time and contents of the response header and the reception time of each byte of the response body

- The projection/orientation metadata carried in network manifest file

- The reception time and intended playout time for each received segment

## 6.3	Observation point 2

The file decoder processes the 3GPP VR Track and typically includes a file parser and a media decoder. The file parser processes the file or segments, extracts elementary streams, and parses the metadata, if present. The processing may be supported by dynamic information provided by the VR application, for example which tracks to choose based on static and dynamic configurations. The media decoder decodes media streams of the selected tracks into the decoded signals. The file decoder outputs the decoded signals and metadata which is used for rendering.

The interface from the file decoder towards MCC is referred to as observation point 2 (OP2) and

is defined to monitor:

- Media resolution

- Media codec

- Media frame rate

- Media projection

- Media decoding time

## 6.4	Observation point 3

The sensor extracts the current pose according to the user's head and/or eye movement and provides it to the renderer for viewport generation. The current pose may also be used by the VR application to control the access engine on which adaptation sets or preselections to fetch.

The interface from the sensor towards MCC is referred to as observation point 3 (OP3) and

is defined to monitor:

- Head pose

- Eye pose

- Pose timestamp

- Depth

## 6.5	Observation point 4

The VR Renderer uses the decoded signals and rendering metadata, together with the pose and the knowledge of the horizontal/vertical field of view, to determine a viewport and render the appropriate part of the video and audio signals.

The interface from the media presentation towards MCC is referred to as observation point 4 (OP4) and

is defined to monitor:

- The media type

- The media sample presentation timestamp

- Wall clock counter

- Actual presentation viewport

- Actual presentation time

- Actual playout frame rate

- Audio-to-video synchronization

- Video-to-motion latency

- Audio-to-motion latency

## 6.6	Observation point 5

The VR application manages the complete device, and controls the access engine, the file decoder and the rendering based on media control information, the dynamic user pose, and the display and device capabilities.

The interface from the VR application towards MCC is referred to as observation point 5 (OP5) and

is defined to monitor:

- Display resolution

- Max display framerate

- Field of view, horizontal and vertical

- Eye to screen distance

- Lens separation distance

- OS support, e.g. OS type, OS version