3GPP TSG-SA4 Meeting #101	S4-181447

Busan, Korea, 19 - 23 Nov 2018	revision of S4-181242

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0445 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On ANBR-Triggered Adaptation Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_MEDIA_MTSI_ext |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of the 5G_MEDIA_MTSI_ext work item aims to specify the following ANBR functionality, as aligned with the agreed conclusions in TR 26.919: - Recommend support of ANBR as an adaptation trigger, as described by clause 10.7 of TS 26.114 - Support for negotiation of radio capabilities on ANBR signalling as described in clause 5.5.6 of TR 26.919. No signaling is currently present to provide end-to-end coordination between the UEs in regards to ANBR-triggered adaptation capabilities. As observed in Tdoc S4-181240, ANBR procedures in TS 26.114 in the absence of end to end coordination may suffer from unfavourable consequences such as high PLR and poor quality. Enabling such end-to-end coordination would be useful for a UE to learn the ANBR-triggered adaptation capabilities of its remote UE and may accordingly determine the most suitable media adaptation on its end, e.g., in response to ANBR information received from its local eNB/gNB. Furthermore, from a conformance point of view, the LTE_VoLTE_ViLTE_enh test case included in TS 36.523-1 assumes that if a UE is capable of receiving ANBR information then it is also capable of using this ANBR information as an adaptation trigger. Such a conformance point is currently not supported by the existing media handling requirements on ANBR in TS 26.114, where ANBR is an optional/recommended feature, i.e., a UE is free to ignore ANBR information as an adaptation trigger. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SDP-based indication of ANBR-triggered adaptation capabilities. If UE signals this SDP capability, then it is capable of receiving ANBR information from its access network and its MTSI client ‘shall’ use ANBR as an adaptation trigger and it will not ignore ANBR signaling from its eNB / gNB to the UE that gets passed onto its MTSI client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No end-to-end coordination between the UEs on their ANBR-triggered adaptation capabilities. Conformance test case in TS 36.523-1 cannot be satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.X (new), Annex M.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change (NEW)

### 6.2.X	ANBR-Triggered Adaptation attribute "anbr_adapt"

Access network bitrate recommendation (ANBR) as an adaptation trigger is described in clause 10.7. Use of ANBR with dynamic bitrate adaptation is described in clause 10.7.3 and related adaptation of sent and received media is described in clauses 10.7.3.2 and 10.7.3.3, respectively. At the radio signalling level, ANBR signaling capability, also known as RAN-assisted codec adaptation as specified in TS 36.321 [157] for LTE access and TS 38.321 [Y] for NR access.

If the MTSI client in terminal supports ANBR-triggered adaptation and its UE is able to obtain ANBR information from the access network, it shall signal this capability in the SDP offer-answer exchanges through the media-level SDP attribute "anbr_adapt". If the MTSI client in terminal signals this capability in the SDP, then it shall use ANBR as an adaptation trigger and it will not ignore ANBR signaling signalling over E-UTRA or NR access from its local eNB / gNB to its UE that gets passed onto its MTSI client in terminal.

For LTE access, inclusion of "anbr_adapt" in the SDP indicates whether the UE is able to query and receive ANBR information (for both downlink and uplink ANBR) from its eNB and whether its eNB is able to send ANBR information to the UE. Likewise, for NR access inclusion of this attribute indicates whether the UE is able to query and receive ANBR information and its gNB is able to send ANBR information to the UE.

Informing the remote MTSI client via the SDP about the ANBR-triggered adaptation capabilities using "anbr_adapt" helps the remote MTSI client to determine the most suitable media adaptation on its end, this is especially valid if the remote MTSI client itself is also capable of performing ANBR-triggered adaptation, e.g., in response to ANBR information received from its local eNB/gNB. As an example, for ANBR-triggered rate up-switching, an MTSI client may adapt its sent bitrate and also perform CMR / RTCP-APP / TMMBR / TMMBN signalling more responsively based on the ANBR information it receives from its local eNB / gNB, if it knows that the remote MTSI client also supports ANBR-triggered adaptation. In the absence of the knowledge of the ANBR-triggered adaptation capabilities of the remote UE, an MTSI client may choose not to perform media adaptation triggered by ANBR, especially for rate up-switching, since the new rate suggested by ANBR may be too high for the RAN of the remote UE resulting in packet losses and poor quality.

The ABNF for the "anbr_adapt" attribute is described as follows:

Synchronization-Info	= "a" "=" "anbr_adapt" ":" adapt-value

adapt-value	= "Yes" / "No"

Here is an example use of the "a=anbr_adapt" attribute relative to a media line:

a=anbr_adapt:Yes

The IANA registration information for the "a=anbr_adapt" SDP attribute is provided in Annex M.X.

For MTSI clients supporting speech, signalling "a=anbr_adapt:Yes" in the SDP indicates support for the following adaptation behaviors:

-	For an MTSI client in terminal receiving speech, if DL ANBR information is received from eNB / gNB that is lower than the received bitrate, then the MTSI receiver shall send a CMR / RTCP-APP message to the MTSI sender based on the ANBR value.

-	For an MTSI client in terminal receiving speech, if DL ANBR information is received from eNB / gNB that is higher than the received bitrate, then the MTSI receiver shall send a CMR / RTCP-APP message to the MTSI sender based on the ANBR value (to a new value that is the lowest bitrate of all relevant adaptation triggers).

-	For an MTSI client in terminal sending speech, if UL ANBR information is received from the eNB / gNB that is lower than the sent bitrate, then the MTSI sender shall reduce the sent bitrate based on the ANBR value.

-	For an MTSI client in terminal sending speech, if UL ANBR information is received from the eNB / gNB that is higher than the sent bitrate, then the MTSI sender shall increase the sent bitrate based on the ANBR value (to a new value that is the lowest bitrate of all relevant adaptation triggers).

For MTSI clients supporting video, signalling "a=anbr_adapt:Yes" in the SDP indicates support for the following adaptation behaviors:

-	For an MTSI client in terminal receiving video, if DL ANBR information is received from eNB / gNB that is lower than the received bitrate, then the MTSI receiver shall send a RTCP TMMBR message to the MTSI sender based on the ANBR value.

-	For an MTSI client in terminal receiving video, if DL ANBR information is received from eNB / gNB that is higher than the received bitrate, then the MTSI receiveer shall send a RTCP TMMBR message to the MTSI sender based on the ANBR value (to a new value that is the lowest bitrate of all relevant adaptation triggers).

-	For an MTSI client in terminal sending video, if UL ANBR information is received from the eNB / gNB that is lower than the sent bitrate, then the MTSI sender shall reduce the sent bitrate based on the ANBR value.

-	For an MTSI client in terminal sending video, if UL ANBR information is received from the eNB / gNB that is higher than the sent bitrate, then the MTSI sender shall increase the sent bitrate based on the ANBR value (to a new value that is the lowest bitrate of all relevant adaptation triggers). Alternatively, before increasing the sent bitrate the MTSI sender shall send a RTCP TMMBN message to the MTSI receiver based on the ANBR value (to a new value that is the lowest bitrate of all relevant adaptation triggers). Then the MTSI receiver shall send an ANBR query to its eNB / gNB and obtain the latest ANBR information, to check whether the bitrate value indicated in the TMMBN message can be supported. If it receives a lower ANBR value from its eNB / gNB, then the MTSI receiver send a TMMBR message to the MTSI sender with this ANBR information.

Second Change (NEW)

# M.X	anbr_adapt

Contact name, email address, and telephone number:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200

Attribute Name (as it will appear in SDP)

anbr_adapt

Long-form Attribute Name in English:

3GPP access network bitrate recommendation (ANBR) based adaptation attribute

Type of Attribute

Media level

Is Attribute Value subject to the Charset Attribute?

This Attribute is not dependent on charset.

Purpose of the attribute:

This attribute is used to indicate the UE’s ability to use ANBR as an adaptation trigger

Appropriate Attribute Values for this Attribute:

The attribute is a value attribute. The defined values are "Yes" and "No". See clause 6.2.X

End of document