Source:	5GMSA Rapporteur (Ericsson LM)

Title:	5GMSA Permanent Document on list of functionalities – v0.1

## Document for:	Agreement

## Agenda Item:	8.8/15.12

# 1	Introduction

The 5GMSA (5G Media streaming architecture) Work Item was approved at SA#82 as TD SP-181250.

The present “Permanent Document” purpose is to maintain a list of the functionalities – in particular those functions that are already present in SA4 Rel-15 specifications - required or not required for 5GMSA.

# 2	5GMSA (5G Media streaming architecture) Work Item objectives

Create a new 5G Media Streaming (5GMSA) architecture specification which supports:

- MNO and 3rd party Media Downlink Streaming Services with relevant functions and interfaces to support:

  - Different collaboration scenarios between third party-providers and mobile network operators for media distribution over 5G;

  - Appropriate service and session definitions in the context of 5G Media Distribution, especially for third-party media services and corresponding network interfaces to establish, announce and discover those;

  - A distribution-independent service establishment and content ingest interface;

  - Relevant functions for operators and third-party service providers in different collaboration scenarios, including but not limited to aspects such as session management, QoS framework, network assistance, QoE reporting, accessibility, content replacement, notification, content rights management, etc.

  - The delivery of 3GPP-defined media formats and profiles as well as third-party formats based on commonly defined packaging formats.

Note:  Support of 5G Media streaming over MBMS with 5GC is not considered in this work item.

- MNO and 3rd party Media Uplink Streaming Services based on the non-IMS FLUS architecture:

  - Specify the non-IMS FLUS entities and interfaces as part of the 5GMSA where the FLUS sink is not in the UE;

  - Enable different collaboration scenarios between third party-providers and mobile network operators for media over 5G.

- Corresponding UE functions and APIs;

- Backwards compatibility for EUTRAN deployments (with and without MBMS)

- Usage of 5G specific features such as network slicing and edge computing.

The new 5G Media Streaming architecture should be functionally decomposed into independent components enabling different deployments with various degrees of integration between 5G MNOs and Content Providers. It should be specified as a set of extensions to TS 23.501 “System Architecture for the 5G System”.

The work should consider input from key industry players on their requirements and usage scenarios for media streaming over 5G. This includes broadcasters and content providers/emerging media service providers, as well as mobile network operators running their own media services.

# 3	5GMSA (5G Media streaming architecture) functionalities

Table 1 Proposed functions for 5GMSA

| Function/Protocol | Proposed Status in 5GMSA | Comment |
| --- | --- | --- |
| Video Decoder | Maintain – focus on TV Video Profiles and FF/CMAF/DASH encapsulation | 5GMSA should have mandatory and recommended video codecs |
| Image Decoder | Maintain | Foreseen to be linked to the File Format or the presentation format, or with another function e.g. thumbnail for a streaming app. |
| Vector Graphics Decoder | Not maintained | This is mostly defined apps. Note: check TS 26.307 Graphics API. |
| Text | Not maintained/Part of Presentation format | This is mostly defined by apps and does not address relevant deployment cases |
| Timed Text | Maintain and update to modern Subtitle formats | Subtitles are key elements in TV and media distribution |
| Audio Decoder | Maintain and update – possibly define TV Audio Profiles and FF/CMAF/DASH encapsulation | 5GMSA should have mandatory and recommended audio codecs |
| Speech Decoder | Maintain. Potentially merged with Audio codecs specifications. |  |
| Synthetic audio Decoder | Not maintained | This seems not be of relevance today |
| Scene Description | Maintain and revise to presentation format | This update was done in PSS, but not reflected in the diagram of TS 26.234. |
| Session Control | Maintain and update | TBD on which level this is required |
| Session Establishment | Maintain and update | The concept of a session on media level should be maintained, but may be much more lightweight with modern stateless streaming protocols |
| Capability Exchange | Maintain and extend | Rely on typical Internet methods, but make sure that this is well documented |
| Encapsulation and Container Format | Maintain and extend | Rely on CMAF and file format Do 3GPP relevant media profiles |
| DASH access client | Maintain and update | Rely on TS 26.247, but update to address latest findings referring to MPEG-DASH. Align more with MPEG-DASH and other industry profiles |
| Decryption | Add | Rely on industry practice |
| HTTP/TCP/IP with specific variants and configurations | Maintain | With Optional extensions as they are considered reasonable and practical and Security related matters |
| PSS RTP/UDP/IP and RTSP | TBD | To be decided at SA4#103 |
| Media Download | Maintain and update |  |
| Media Progressive Download | Maintain and update |  |
| Media On Demand Streaming | Maintain and update |  |
| Media Live Streaming | Maintain and update |  |
| File Format | Maintain and update | MPEG CMAF is mentioned as convergence point |
| QoE/Metrics reporting and collection | Maintain and update | Convergence to e.g. MPEG and/or CTA metrics is mentioned. |
| [Support SAND and the modes as specified in TS 26.247, thus maintaining the SAND modes “Network Assistance”, “Proxy Caching”, and “Consistent QoE/QoS”] | TBD | Considered in the context of the 5GMSA Network Assistance function. |