3GPP TS 38.141 V0.2.0 (2018-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group RAN;

NR;

Base Station (BS) conformance testing

Part 2: Radiated conformance testing

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, NR

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	11

3.3	Abbreviations	12

4	General radiated test conditions and declarations	13

4.1	Measurement uncertainties and test requirements	13

4.1.1	General	13

4.1.2	Acceptable uncertainty of OTA Test System	13

4.1.2.1	General	13

4.1.2.2	Measurement of transmitter	14

4.1.2.3	Measurement of receiver	14

4.1.2.4	Measurement of performance requirement	14

4.1.3	Interpretation of measurement results	15

4.2	Radiated requirement reference points	15

4.3	Base station classes	16

4.4	Regional requirements	17

4.5	BS configurations	17

4.5.1	Transmit configurations	17

4.5.2	Receive configurations	17

4.5.3	Power supply options	17

4.6	Manufacturer’s declarations	18

4.7	Applicability of requirements	18

4.8.1	General	18

4.8	Test configurations	19

4.8.1	General	19

4.8.2	Test signal configurations	19

4.9	 RF channels and test models	19

4.9.1	RF channels	19

4.9.3	Test models	20

4.10	Requirements for contiguous and non-contiguous spectrum	21

4.11	Requirements for BS capable of multi-band operation	21

4.12	OTA co-location reference antenna	21

5	Operating bands and channel arrangement	23

6	Radiated transmitter characteristics	24

6.1	General	24

6.2	Radiated transmit power	24

6.2.1	Definition and applicability	24

6.2.2	Minimum requirement	24

6.2.3	Test purpose	24

6.2.4	Method of test	25

6.2.4.1	Initial conditions	25

6.2.4.2	Procedure	25

6.2.5	Test requirement	25

6.3	OTA base station output power	26

6.3.1	Definition and applicability	26

6.3.2	Minimum requirement	26

6.3.3	Test purpose	26

6.3.4	Method of test	26

6.3.4.1	Initial conditions	26

6.3.4.2	Procedure	27

6.3.5	Test requirement	27

6.3.5.1	BS type 1-O	27

6.3.5.2	BS type 2-O	28

6.4	OTA output power dynamics	28

6.4.1	General	28

6.4.2	OTA RE power control dynamic range	28

6.4.2.1	Definition and applicability	28

6.4.2.2	Minimum requirement	28

6.4.2.4	Method of test	28

6.4.3	OTA total power dynamic range	28

6.4.3.1	Definition and applicability	28

6.4.3.2	Minimum requirement	29

6.4.3.3	Test purpose	29

6.4.3.4	Method of test	29

6.4.3.4.1	Initial conditions	29

6.4.3.4.2	Procedure	29

6.4.3.5	Test requirement	30

6.4.3.5.1	BS type 1-O	30

6.4.3.5.2	BS type 2-O	30

6.5	OTA transmit ON/OFF power	31

6.6	OTA transmitted signal quality	31

6.7	OTA unwanted emissions	31

6.7.1	General	31

6.7.2	OTA occupied bandwidth	31

6.7.2.1	Definition and applicability	31

6.7.2.2	Minimum requirement	31

6.7.2.3	Test purpose	31

6.7.2.4	Method of test	32

6.7.2.4.1	Initial conditions	32

6.7.2.4.2	Procedure	32

6.7.2.5 	Test requirement	34

6.7.2.5.1	BS type 1-O	34

6.7.2.5.2	BS type 2-O	34

6.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)	34

6.7.4	OTA operating band unwanted emissions	34

6.7.5	OTA transmitter spurious emissions	34

6.8	OTA transmitter intermodulation	34

7	Radiated receiver characteristics	35

7.1	General	35

7.2	OTA sensitivity	35

7.3	OTA reference sensitivity level	37

7.4	OTA dynamic range	41

7.5	OTA in-band selectivity and blocking	51

7.6	OTA out-of-band blocking	61

7.7	OTA receiver spurious emissions	61

7.8	OTA receiver intermodulation	61

7.8.1	Definition and applicability	61

7.8.2	Minimum requirement	61

7.8.3	Test purpose	61

7.8.4	Method of test	61

7.8.4.1	Initial conditions	61

7.8.4.2	Procedure	62

7.8.5	Test requirement	62

7.8.5.1	BS type 1-O	62

7.8.5.2	BS type 2-O	66

7.9	OTA in-channel selectivity	66

7.9.1	Definition and applicability	66

7.9.2	Minimum requirement	66

7.9.3	Test purpose	67

7.9.4	Method of test	67

7.9.4.1	Initial conditions	67

7.9.4.2	Procedure	67

7.9.5	Test requirement	68

7.9.5.1	BS type 1-O	68

7.9.5.2	BS type 2-O	71

8	Radiated performance requirements	73

Annex A (normative): Reference measurement channels	74

Annex B (normative): Environmental requirements for the BS equipment	75

B.1	General	75

B.2	Normal test environment	75

B.3	Extreme test environment	75

B.3.1	Extreme temperature	75

B.4	Vibration	76

B.5	Power supply	76

B.6	Measurement of test environments	76

Annex C (informative): Test tolerances and derivation of test requirements	77

C.1	Measurement of transmitter	77

C.2	Measurement of receiver	77

Annex D (normative): Calibration	78

Annex E (informative): OTA test system set-up	79

E.1	Transmitter	79

E1.1	Radiated transmit power	79

E.2	Receiver	80

E.2.1	OTA sensitivity	80

Annex F (normative): Estimation of Measurement Uncertainty	81

Annex G (informative): Transmitter Spatial emissions Declaration	82

G.1	General	82

G.2	Declarations	82

Annex H (informative): Format and interpretation of tests	84

Annex I (informative): Change history	85

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Base Station (BS). These have been derived from, and are consistent with the NR BS specification defined in 3GPP TS 38.104 [2]. The technical specification 3GPP TS 38.141 is in two parts:

- 3GPP TS 38.141-1 [3] covers conducted test requirements,

- 3GPP TS 38.141-2 (the present document) covers radiated test requirements.

A BS type 1-C requires only conducted requirements so requires compliance to part 1 of the specification only.

A BS type 1-H has both conducted and radiated requirements so requires compliance to the applicable requirements of part 1 and part 2 of the specification.

BS type 1-O and BS type 2-O have only radiated requirements so require compliance to part 2 of the specification only.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[3]	3GPP TS 38.141-1: “NR, Base Station (BS) conformance testing, Part 1: Conducted conformance testing”

[4]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[6]	3GPP TR 37.842: "E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[7]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[8]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations"

[9] 		IEC 60 721: "Classification of environmental conditions"

[10]		IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[11]		IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[12]		IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[13]	Recommendation ITU-R M.328: "Spectra and bandwidth of emissions"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

antenna connector: connector at the conducted interface of the BS type 1-C

aggregated BS channel bandwidth: the RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated BS channel bandwidth is measured in MHz

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [5] used for the formulation of unwanted emission requirements for FR1

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

channel edge: lowest or highest frequency of the NR carrier, separated by the BS channel bandwidth

carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

carrier aggregation configuration: a set of one or more operating bands across which the BS aggregates carriers with a specific set of technical requirements

co-location reference antenna: a passive antenna used as reference for base station to base station co-location requirements

contiguous carriers: set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

directional requirement: requirement which is applied in a specific direction within the OTA coverage range for the Tx and when the AoA of the incident wave of a received signal is within the OTA REFSENS RoAoA or the minSENS RoAoA as appropriate for the receiver

Editor’s note: This definition is aligned with the definition of Single directional requirement for AAS BS. The naming should be further aligned between NR and AAS specifications.

highest carrier: the carrier with the highest carrier frequency transmitted/received in a specified frequency band

inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band

intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band

[Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands]

lowest Carrier: the carrier with the lowest carrier frequency transmitted/received in a specified frequency band

lower sub-block edge: frequency at the lower edge of one sub-block

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

maximum carrier TRP output power: mean power level measured per RIB during the transmitter ON period for a specific carrier in a specified reference condition and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

maximum total TRP output power: mean power level measured per RIB during the transmitter ON period in a specified reference condition and corresponding to the declared rated total TRP output power (Prated,t,TRP)

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement.

minSENS RoAoA: The reference RoAoA associated with the OSDD with the lowest declared EIS

multi-band RIB: operating band specific RIB which is paired with one or more additional operating band specific RIBs where the multiple bands are supported through common active electronic component(s)

multi-carrier transmission configuration: set of one or more contiguous or non-contiguous carriers that a NR BS is able to transmit simultaneously according to the manufacturer’s specification

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

NR BS receiver: composite receiver function of a NR BS receiving in an operating band

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in TS 38.104 [2].

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:     The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: Is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction

NOTE:	This contour will be related to the average element/sub-array radiation pattern 3dB beam width.

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one set of declared minimum EIS values (with related RAT and BS channel bandwidth), and related directions over which the EIS applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radiated interface boundary: operating band specific reference point for radiated requirements, defined in the near field or in the far field depending on the radiated requirement’s testability feasibility

rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period

rated carrier TRP output power: mean power level declared by the manufacturer per carrier, for BS operating in single carrier, multi-carrier, or carrier aggregation configurations that the manufacturer has declared to be available at the RIB during the transmitter ON period

rated total TRP output power: mean power level declared by the manufacturer, that the manufacturer has declared to be available at the RIB during the transmitter ON period

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

receiver target: AoA in which reception is performed by BS types 1-H, 1-O and BS types 2-O

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD

requirement set:	one of the NR base station requirement’s set as defined for BS type 1-C, BS type 1-H, BS type 1-O, and BS type 2-O

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific BS direction setting

single-band RIB: operating band specific RIB without any common active electronic component(s) shared with other operating bands

sub-block: one contiguous allocated block of spectrum for transmission and reception by the same base station

NOTE:	There may be multiple instances of sub-blocks within a Base Station RF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within a Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: RF Bandwidth of an instantaneous transmission from a UE or BS, measured in resource block units

total radiated power: the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.  Total radiated power is defined in both the near-field region and the far-field region.

upper sub-block edge: frequency at the upper edge of one sub-block

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ,REFSENS 	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees

BWChannel	BS channel bandwidth

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12

f 	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax 	f_offsetmax minus half of the bandwidth of the measuring filter

ΔFGlobal 	Global frequency raster granularity

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔminSENS 	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS 	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster 	Channel raster granularity

ΔSUL	Channel raster offset for SUL

EISminsens		The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC_low 	The Fc of the lowest carrier, expressed in MHz

FC_high 	The Fc of the highest carrier, expressed in MHz

Fedge_low 	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low

Fedge_high 	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset_low

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset_high

Foffset_high 	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from F C,block, high to the upper sub-block edge

Foffset_low 	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF,SUL		RF reference frequency for Supplementary Uplink (SUL) bands

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

Pmax,c,EIRP 	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (PRated,c,TRP)

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

PREFSENS	Conducted reference Sensitivity power levelSSREF	SS block reference frequency position

TTOTA	Test tolerance for OTA requirements

Wgap	Sub-block gap or Inter RF Bandwidth gap size

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Antenna Array

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

CA	Carrier Aggregation

CACLR	Cumulative ACLR

DUT	Device Under Test

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

NR	New Radio

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

RDN	Radio Distribution Network

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RoAoA	Range of Angles of Arrival

SCS	Sub-Carrier Spacing

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TRP	Total Radiated Power

TT	Test Tolerance

# 4	General radiated test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in 3GPP TS 38.141-2 (the present document), i.e. to all radiated tests defined for BS type 1-H, BS type 1-O and BS type 2-O.

The minimum requirements are given in TS 38.104 [2]. Test Tolerances for the radiated test requirements (TTOTA) explicitly stated in the present document are given in annex C.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

### 4.1.2	Acceptable uncertainty of OTA Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the OTA Test System is specified below for each radiated test defined explicitly in the present specification, where appropriate.

The OTA Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the DUT to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95% is the measurement uncertainty tolerance interval for a specific measurement that contains 95% of the performance of a population of test equipment.

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum OTA Test System uncertainty for OTA transmitter tests

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power | FR1, Normal: ±1.0 dB, f ≤ 3.0 GHz ±1.2 dB, 3.0 GHz < f ≤ 4.2 GHz TBD, 4.2 GHz < f ≤ 6.0 GHz FR1, Extreme: FFS | For FR1 OTA Test System uncertainty derivation for Normal conditions, see 3GPP TR 37.842 [6], subclause 10.3.2.2. |
|  | FR2: FFS | FFS |
| 6.3 OTA base station output power |  |  |
| 6.4.x OTA RE power control dynamic range |  |  |
| 6.4.x OTA total power dynamic range |  |  |
| 6.5.2 OTA transmitter OFF power |  |  |
| 6.5.3 OTA transmitter transient period |  |  |
| 6.6.x OTA frequency error |  |  |
| 6.6.x OTA modulation quality |  |  |
| 6.6.x OTA time alignment error |  |  |
| 6.7.2 OTA occupied bandwidth |  |  |
| 6.7.3 OTA ACLR |  |  |
| 6.7.4 OTA operating band unwanted emissions |  |  |
| 6.7.5.x OTA transmitter spurious emissions, mandatory requirements |  |  |
| 6.7.5.x OTA transmitter spurious emissions, protection of BS receiver |  |  |
| 6.7.5.x OTA transmitter spurious emissions, additional spurious emissions requirements |  |  |
| 6.7.5.x OTA transmitter spurious emissions, co-location |  |  |
| 6.8 OTA transmitter intermodulation |  |  |

#### 4.1.2.3	Measurement of receiver

Table 4.1.2.3-1: Maximum OTA Test System uncertainty for OTA receiver tests

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 7.2 OTA sensitivity | FR1: ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz TBD, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.842 [6], subclause 10.3.2.2. |
| 7.3 OTA reference sensitivity level |  |  |
| 7.4 OTA dynamic range |  |  |
| 7.5 OTA in-band selectivity and blocking |  |  |
| 7.6 OTA out-of-band blocking |  |  |
| 7.7 OTA receiver spurious emissions |  |  |
| 7.8 OTA receiver intermodulation |  |  |
| 7.9 OTA in-channel selectivity |  |  |

#### 4.1.2.4	Measurement of performance requirement

Table 4.1.2.4-1: Maximum OTA Test System uncertainty for performance requirements

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| TBD |  |  |

### 4.1.3	Interpretation of measurement results

Editor’s note: Square brackets on the Shared Risk text to be removed or the text to be changed once the OTA test procedures and MU for BS type 1-O and BS type 2-O are stable enough.

[The measurement results returned by the OTA Test System are compared - without any modification - against the test requirements as defined by the Shared Risk principle in Recommendation ITU-R M.1545 [4].]

The actual measurement uncertainty of the OTA Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the OTA Test System uncertainty shall be, for each OTA measurement, equal to or lower than the appropriate figure in subclause 4.1.2 of this specification.

If the OTA Test System for an OTA test is known to have a measurement uncertainty greater than that specified in subclause 4.1.2, it is still permitted to use this apparatus provided that an adjustment is made as follows:

Any additional uncertainty in the OTA Test System over and above that specified in subclause 4.1.2 shall be used to tighten the OTA test requirement, making the test harder to pass. For some tests e.g. receiver tests, this may require modification of stimulus signals. This procedure will ensure that an OTA Test System not compliant with subclause 4.1.2 does not increase the chance of passing a DUT where that device would otherwise have failed the test if an OTA Test System compliant with subclause 4.1.2 had been used.

## 4.2	Radiated requirement reference points

Radiated characteristics for BS type 1-H, BS type 1-O and BS type 2-O are defined over the air (OTA) where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. For BS type 1-H the requirements are defined for two points of reference, signified by radiated requirements at the RIB and the conducted requirements at transceiver array boundary (TAB). The OTA requirements of BS type 1-H are tested in the far field (Fraunhofer) region.

General architecture and reference points of BS type 1-H, BS type 1-O and BS type 2-O are presented on the following figures 4.2-1 – 4.2-2.

Figure 4.2-1: General architecture of BS type 1-H

This specification details only radiated test requirements and hence only requires the radiated reference points.

Figure 4.2-2: General architecture of BS type 1-O and BS type 2-O

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to receive/send parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network that distributes the RF power between the transceiver array boundary and the antenna array, in an implementation specific way.

## 4.3	Base station classes

The requirements in this specification apply to Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations unless otherwise stated. The associated deployment scenarios and definitions of BS classes are exactly the same for BS with and without connectors.

BS classes for BS type 1-H are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to minimum coupling loss equal to 45 dB.

BS classes for BS type 1-O and BS type 2-O are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

The manufacturer shall declare the intended class of the BS under test.

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Subclause | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.6.2, 9.7.2 | Occupied bandwidth, OTA occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in present specification. |
| 6.6.3.4, 9.7.3.2 | Absolute ACLR, OTA absolute ACLR, | The emission limits specified for OTA absolute ACLR as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |
| 6.6.4.2.5.1 | Limits in FCC Title 47 | The BS may have to comply with the additional requirements, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.4.4, 9.7.4.2 | Operating band unwanted emissions, OTA out-of-band emissions | The emission limits specified for OTA out-of-band emissions as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |
| 6.6.5.2.1, 9.7.5.2 | Tx spurious emissions, OTA Tx spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [2], may apply regionally. The emission limits specified as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |
| 6.6.5.2.3, 9.7.5.3.3 | Tx spurious emissions: additional requirements, OTA Tx spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the BS operating band. |
| 7.6.4, 10.7.2 | Rx spurious emissions, OTA Rx spurious emissions | The emission limits specified as the basic limit + X [dB] are applicable, unless stated differently in regional regulation. |

## 4.5	BS configurations

### 4.5.1	Transmit configurations

Unless otherwise stated, the radiated transmitter characteristics in clause 6 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor’s note: to be aligned with the figures for the RIB interfaces and co-location concept.

### 4.5.2	Receive configurations

Unless otherwise stated, the radiated receiver characteristics in clause 7 are specified at RIB, with a full complement of transceiver units for the configuration in normal operating conditions.

Editor’s note: to be aligned with the figures for the RIB interfaces and co-location concept.

### 4.5.3	Power supply options

If the BS is supplied with a number of different power supply configurations, it may not be necessary to test RF parameters for each of the power supply options, provided that it can be demonstrated that the range of conditions over which the equipment is tested is at least as great as the range of conditions due to any of the power supply configurations.

## 4.6	Manufacturer’s declarations

The following manufacturer’s declarations are required for radiated requirements for BS type 1-H, BS type 1-O and BS type 2-O.

Table 4.6-1 Manufacturers declarations common for BS type 1-H, BS type 1-O and BS type 2-O

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| TDB |  |  |

Table 4.6-2 Manufacturers declarations specific for BS type 1-H

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| TDB |  |  |

Table 4.6-3 Manufacturers declarations specific for BS type 1-O

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| TDB |  |  |

Table 4.6-4 Manufacturers declarations specific for BS type 2-O

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| TDB |  |  |

## 4.7	Applicability of requirements

Editor’s note: NR BS aspects related to the TS 37.145-2 radiated MSR conformance will be addressed after single RAT specification aspects in the present specification will be settled.

### 4.8.1	General

### 4.7.2	Requirement set applicability

In table 4.7.2-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement subclause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.7.2-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | 1-H | 1-O | 2-O |
| Radiated transmit power | 6.2 | 6.2 | 6.2 |
| OTA base station output power | NA | 6.3 | 6.3 |
| OTA output power dynamics |  | 6.4 | 6.4 |
| OTA transmit ON/OFF power |  | 6.5 | 6.5 |
| OTA transmitted signal quality |  | 6.6 | 6.6 |
| OTA occupied bandwidth |  | 6.7.2 | 6.7.2 |
| OTA ACLR |  | 6.7.3 | 6.7.3 |
| OTA out-of-band emission |  | 6.7.4 | 6.7.4 |
| OTA transmitter spurious emission |  | 6.7.5 | 6.7.5 |
| OTA transmitter intermodulation |  | 6.8 | NA |
| OTA sensitivity | 7.2 | 7.2 | NA |
| OTA reference sensitivity level | NA | 7.3 | 7.3 |
| OTA dynamic range |  | 7.4 | NA |
| OTA in-band selectivity and blocking |  | 7.5 | 7.5 |
| OTA out-of-band blocking |  | 7.6 | 7.6 |
| OTA receiver spurious emission |  | 7.7 | 7.7 |
| OTA receiver intermodulation |  | 7.8 | 7.8 |
| OTA in-channel selectivity |  | 7.9 | 7.9 |
| Radiated performance requirements |  | 8 | 8 |

## 4.8	Test configurations

Editor’s note: to capture multi-carrier/CA operation, contiguous/non-contiguous operation. Test models to be captured in this clause. Test models other than those already listed below, are not precluded.

### 4.8.1	General

Editor’s note: test configuration aspects to be confirmed once the test models and test configurations discussion is concluded.

The test configurations shall be constructed using the methods defined below subject to the parameters declared by the manufacturer as listed in subclause 4.6.

[For test contiguous spectrum operation configurations used in receiver tests only the carriers in the outermost frequency positions in the Base Station RF Bandwidth need to be generated by the test equipment. For non-contiguous spectrum operation test configurations used in receiver tests, outermost carriers for each sub-block need to be generated by the test equipment.]

The applicable test models for generation of the carrier transmit test signal are defined in subclause [x].

### 4.8.2	Test signal configurations

## 4.9		RF channels and test models

Editor’s note: to capture multi-carrier/CA operation, contiguous/non-contiguous operation

Editor’s note: RF channels to be confirmed once the test models and test configurations discussion is concluded.

### 4.9.1	RF channels

For single carrier tests unless otherwise stated the tests shall be performed with a single carrier at each of the RF channels are TBD.

Many tests in this TS are performed with the maximum Base Station RF Bandwidth located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BRFBW (bottom), MRFBW (middle) and TRFBW (top).

Unless otherwise stated, the test shall be performed at BRFBW, MRFBW and TRFBW defined as following:

-	BRFBW: maximum Base Station RF Bandwidth located at the bottom of the supported frequency range in the operating band.

-	MRFBW: maximum Base Station RF Bandwidth located in the middle of the supported frequency range in the operating band. MRFBW may be shifted maximum 100 kHz towards lower frequencies to align carriers with the channel raster.

-	TRFBW: maximum Base Station RF Bandwidth located at the top of the supported frequency range in the operating band.

For a BS capable of multi-band operation and capable of dual-band operation, unless otherwise stated, the test shall be performed at BRFBW_T'RFBW and B'RFBW_TRFBW defined as following:

-	BRFBW_ T'RFBW: the Base Station RF Bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the upper operating band.

-	B'RFBW_TRFBW: the Base Station RF Bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the maximum Radio Bandwidth, in the lower operating band.

NOTE:	BRFBW_T'RFBW = B'RFBW_TRFBW = BRFBW_TRFBW when the declared maximum Radio Bandwidth (see subclause 4.6) spans both operating bands. BRFBW_TRFBW means the Base Station RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band.

When a test is performed by a test laboratory, the position of BRFBW, MRFBW and TRFBW in each supported operating band, the position of BRFBW_T'RFBW and B'RFBW_TRFBW in the supported operating band combinations shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

Occupied bandwidth test in this TS are performed with the Aggregated Channel Bandwidth and sub-block bandwidths located at the bottom, middle and top of the supported frequency range in the operating band. These are denoted as BBW Channel CA(bottom), MBW Channel CA (middle) and TBW Channel CA (top) for contiguous spectrum operation.

Unless otherwise stated, the test for contiguous spectrum operation shall be performed at BBW Channel CA, MBW Channel CA and TBW Channel CA defined as following:

-	BBW Channel CA: Aggregated Channel Bandwidth located at the bottom of the supported frequency range in each operating band;

-	MBW Channel CA: Aggregated Channel Bandwidth located close in the middle of the supported frequency range in each operating band, with the center frequency of each component carrier aligned to the channel raster;

-	TBW Channel CA: Aggregated Channel Bandwidth located at the top of the supported frequency range in each operating band.

When a test is performed by a test laboratory, the position of BBW Channel CA, MBW Channel CA and TBW Channel CA for contiguous spectrum operation in the operating band shall be specified by the laboratory. The laboratory may consult with operators, the manufacturer or other bodies.

### 4.9.3	Test models

Editor’s note: test models to be added here – suggest prefix N-TM-x.x.

## 4.10	Requirements for contiguous and non-contiguous spectrum

A spectrum allocation where a BS operates can either be contiguous or non-contiguous. Unless otherwise stated, the requirements in the present specification apply for BS configured for both contiguous spectrum operation and non-contiguous spectrum operation.

For BS operation in non-contiguous spectrum, some requirements apply both at the Base Station RF Bandwidth edges and inside the sub-block gaps. For each such requirement, it is stated how the limits apply relative to the Base Station RF Bandwidth edges and the sub-block edges respectively.

## 4.11	Requirements for BS capable of multi-band operation

For multi-band RIB, the radiated test requirements in clause 6 and 7 apply separately to each supported operating band, unless otherwise stated. For some radiated test requirements, it is explicitly stated that specific additions or exclusions to the test requirement apply at multi-band RIB(s) as detailed in the requirement subclause.

BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

For multi-band RIBs supporting the bands for TDD, the radiated test requirements in the present specification assume no simultaneous uplink and downlink occur between the bands.

The radiated test requirements for multi-band RIBs supporting bands for both FDD and TDD are FFS and are not covered by the present release of this specification.

## 4.12	OTA co-location reference antenna

Co-location requirements are requirements which are based on assuming the NR BS is co-located with another BS of the same base station class, they ensure that both co-located systems can operate with minimal degradation to each other.

Unwanted emission and out of band blocking co-location requirements are optional requirements based on declaration. TX OFF power and Transmitter intermodulation are mandatory requirements and is the form of a co-location requirement as it represents the worst-case scenario of all the interference cases.

The co-location reference antenna, shall be a single column passive antenna which has the same vertical radiating dimension (h), frequency range, polarization, as the composite antenna of NR AAS BS and nominal 65 degrees horizontal half-power beam width suitable for 3-sector deployments at a distance d from the edge of the NR BS, as shown in figure 4.12-1.

Figure 4.12-1 Placement of NR BS and co-location reference antenna

Edge-to-edge separation d, between the NR BS and the co-location reference antenna shall be set to 0.1 m.

The alignment of the NR BS and the co-location reference antenna shall be aligned in a common plane perpendicular to the mechanical bore-sight direction, as shown in figure 4.12-1.

The co-location reference antenna and the NR BS can have different width.

The vertical radiating regions of the co-location reference antenna and the NR BS composite antenna shall be aligned.

For co-location requirements where the frequency range of the signal at the co-location reference antenna is different from the NR BS, a co-location reference antenna suitable for the frequency stated in the requirement is assumed.

OTA co-location requirements are based on power at the conducted interface of a co-location reference antenna, depending on the requirement this interface is either an input or an output. For NR BS with dual polarization the co-location reference antenna has two conducted interfaces each representing one polarization.

# 5	Operating bands and channel arrangement

Editor’s note: to be copied from the TS 38.104 specification which is still evolving, the latest version from the core should be implemented in the final version. Alternatively, refer to the TS 38.104 instead of copy-pasting.

# 6	Radiated transmitter characteristics

## 6.1	General

This subclause describes any general aspects of radiated transmitter characteristics and relations between requirements.

General test conditions for transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in subclause 4.5.

If beams have been declared equivalent and parallel (see table 4.6-1, Dx.yy, Dx.zz), only a representative beam is necessary to demonstrate conformance.

## 6.2	Radiated transmit power

### 6.2.1	Definition and applicability

Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each declared beam, the requirement is based on declarations captured in [4.6-1] for a beam identifier ([declaration ID]), reference beam direction pair ([declaration ID]), rated beam EIRP at the beam's reference direction pair ([declaration ID]), EIRP accuracy directions set ([declaration ID]), the beam direction pairs at the maximum steering directions ([declaration ID]) and their associated rated beam EIRP ([declaration ID]) and beamwidth(s) for reference beam direction pair and maximum steering directions ([declaration ID]).

For a declared beam identifier and beam direction pair, the rated beam EIRP level is the maximum power that the BS is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the EIRP accuracy directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in subclause 6.2.4.1.

NOTE 1:	The EIRP accuracy directions set for a beam is the complete continuous or discrete set of all beam direction for which the EIRP accuracy is intended to be achieved for the beam.

NOTE 2:	A beam direction pair consists of a beam centre direction and an associated beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

Radiated transmit power is directional requirement applicable to BS type 1-H, BS type 1-O and BS type 2-O.

### 6.2.2	Minimum requirement

Radiated transmit power minimum requirement for BS type 1-H and BS type 1-O is defined in 3GPP TS 38.104 [2], subclause 9.2.2.

Radiated transmit power minimum requirement for BS type 2-O is defined in 3GPP TS 38.104 [2], subclause 9.2.3.

### 6.2.3	Test purpose

The test purpose is to verify the ability to accurately generate and direct radiated power per beam, across the frequency range and under normal conditions, for all declared beams of the BS type 1-H, BS type 1-O and BS type 2-O.

### 6.2.4	Method of test

#### 6.2.4.1	Initial conditions

Test environment:

- Normal, see annex B.2,

- Extreme, see annex B.3.

RF bandwidth positions to be tested:

- BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.9.1,

- BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

- Reference beam direction pair (Dx.y), and

- Maximum steering directions (Dx.y).

#### 6.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2. For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (Dx.y) of the BS with the test system.

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.3.

In addition, for a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.8.2.

6)	Measure EIRP by either a) or b) below:

a)	If the OTA test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS.

b)	If the OTA test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Test steps 3 to 6 are repeated for all declared beams (Dx.y) and their reference beam direction pairs and maximum steering directions (Dx.y and Dx.y).

For multi-band capable BS and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carriers activated in the other band.

Test procedure for Extreme test environment is FFS.

Test procedure to be verified for FR2 operating bands.

### 6.2.5	Test requirement

For each declared conformance beam direction pair, the EIRP measurement results in subclause 6.2.4.2 shall remain within the values provided in table 6.2.5-1, relative to the manufacturer's declared rated beam EIRP (Dx.y) value:

Table 6.2.5-1: Test requirement for radiated transmit power

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| BS type 1-H, BS type 1-O | f ≤ 3.0 GHz: ± 3.2 dB | f ≤ 3.0 GHz: ± (2.7 + TTOTA_EXTREME_1) dB |
|  | 3.0 GHz < f ≤ 4.2 GHz: ± 3.4 dB | 3.0 GHz < f ≤ 4.2 GHz: ± (2.7 + TTOTA_EXTREME_2) dB |
|  | 4.2 GHz < f ≤ 6.0 GHz: TBD | 4.2 GHz < f ≤ 6.0 GHz: ± (2.7 + TTOTA_EXTREME_3) dB |
| BS type 2-O | fTBD1 < f ≤ fTBD2: ± (3.4 + TTOTA_FR2) dB … | fTBD1 < f ≤ fTBD2: ± (EIRPacc_EXTREME_FR2+ TTOTA_FR2) dB … |

## 6.3	OTA base station output power

### 6.3.1	Definition and applicability

OTA BS output power is declared as the TRP radiated requirement, with the output power accuracy requirement defined at the RIB during the transmitter ON period for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, for BS type 1-O and BS type 2-O.

The BS rated carrier TRP output power shall be within limits as specified in table 6.3.1-1.

Table 6.3.1-1: BS rated carrier TRP output power limits for BS type 1-O

| BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (note) |
| Medium Range BS | ≤ + 47 dBm |
| Local Area BS | ≤ + 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

Despite the general requirements for the BS output power described in subclauses 9.3.2 – 9.3.3, additional regional requirements might be applicable.

NOTE:	In certain regions, power limits corresponding to BS classes may apply for BS type 2-O.

### 6.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in 3GPP TS 38.104 [2], subclause 9.3.2.

The minimum requirement for BS type 2-O is in 3GPP TS 38.104 [2], subclause 9.3.3.

### 6.3.3	Test purpose

The test purpose is to verify the accuracy of the maximum carrier TRP (Pmax,c,TRP) across the frequency range for all RIBs.

### 6.3.4	Method of test

#### 6.3.4.1	Initial conditions

For BS type 1-O:

Test environment:	Normal, see annex B.2.

RF channels to be tested:	FFS; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: FFS in single band operation; see subclause 4.9.1.

FFS in multi-band operation, see subclause 4.9.1.

Beams to be tested: 	The [narrowest declared beam] (see table 4.6-1, [D9.3, D9.11]).

Directions to be tested: FFS

For BS type 2-O:

Test environment:	Normal, see annex B.2.

RF channels to be tested:	FFS; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: FFS in single band operation; see subclause 4.9.1.

FFS in multi-band operation, see subclause 4.9.1.

Beams to be tested: 	The [narrowest declared beam] (see table 4.6-1, [D9.3, D9.11]).

Directions to be tested: FFS

#### 6.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.6-1, D9.2) of the BS with the test system.

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Configure the beam peak direction of the BS according to the required conditions for the TRP test.

5)	Set the BS to transmit according to the applicable test configuration in clause 5 using the corresponding test model(s) in subclause 4.12.2.

In addition, for a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.11.

6)	Set the BS in the direction of the appropriated TRP measurement grid (see annex xx).

Editor’s note: annex xx will describe how the measurement grid conforms to the specified TRP estimation uncertainty – it is important it is finished before the final approval.

7)	Measure EIRP by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex xx).

9)	Calculate TRP using the EIRP measurements.

For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 6.3.5	Test requirement

#### 6.3.5.1	BS type 1-O

The TRP measurement result in step 9 of subclause 6.3.4.2 shall remain:

-	within +2+X1 dB and -2-X1 dB of the manufacturer's declared rated carrier TRP output power Prated,c,TRP  carrier frequency f  ≤ 3.0 GHz;

-	within +2+X2 dB and –2-X2 dB of the manufacturer's declared rated carrier TRP output power Prated,c,TRP for carrier frequency 3.0 GHz < f ≤ 4.2 GHz.

-	within +2+X3 dB and –2-X3 dB of the manufacturer's declared rated carrier TRP output power Prated,c,TRP for carrier frequency 4.2 GHz < f ≤ 6.0 GHz.

#### 6.3.5.2	BS type 2-O

The TRP measurement result in step 9 of subclause 6.3.4.2 shall remain:

-	within +3+FFS dB and -3-FFS dB of the manufacturer's declared rated carrier TRP output power Prated,c,TRP  carrier frequency 28 GHz < f ≤  FFS GHz.

-	within +3+FFS dB and –3-FFS dB of the manufacturer's declared rated carrier TRP output power Prated,c,TRP for carrier frequency FFS GHz < f ≤  FFS GHz.

Editor’s note: more frequency divisions for the measuring accuracy may be introduced.

## 6.4	OTA output power dynamics

### 6.4.1	General

The requirements in subclause 6.4 apply during the transmitter ON period. Transmit signal quality (as specified in subclause 6.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are single direction requirements and apply to the beam peak directions over the OTA peak directions set.

### 6.4.2	OTA RE power control dynamic range

#### 6.4.2.1	Definition and applicability

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

#### 6.4.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in 3GPP TS 38.104 [2], subclause 9.4.2.2.

#### 6.4.2.4	Method of test

No specific test or test requirements are defined for RE Power control dynamic range. The Error Vector Magnitude test, as described in subclause 6.6 provides sufficient test coverage for this requirement.

### 6.4.3	OTA total power dynamic range

#### 6.4.3.1	Definition and applicability

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP). The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS, PBCH or synchronization signals.

#### 6.4.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in 3GPP TS 38.104 [2], subclause 9.4.3.2.

The minimum requirement for BS type 2-O is in 3GPP TS 38.104 [2], subclause 9.4.3.3.

#### 6.4.3.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 6.4.3.4	Method of test

##### 6.4.3.4.1	Initial conditions

For BS type 1-O:

Test environment:	Normal, see annex B.2.

RF channels to be tested:	FFS; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: FFS in single band operation; see subclause 4.9.1.

Beams to be tested: 	The [narrowest declared beam] (see table 4.6-1, [D9.3, D9.11]).

Directions to be tested: The reference beam direction pair (see table 4.6-1, [D9.7]).

For BS type 2-O:

Test environment:	Normal, see annex B.2.

RF channels to be tested:	FFS; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: FFS in single band operation; see subclause 4.9.1.

Beams to be tested: 	The [narrowest declared beam] (see table 4.6-1, [D9.3, D9.11]).

Directions to be tested: The reference beam direction pair (see table 4.6-1, [D9.7]).

##### 6.4.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.6-1, [D9.2]) of the BS with the test system.

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit using [E-TM 3.1] for BS type 1-O or [E-TM x.x] for BS type 2-O, in 3GPP TS 38.141-1 [3] subclause 6.1.1.1 at the manufacturers declared rated carrier output power (PRated,c,EIRP).

6)	Measure the average OFDM symbol power as defined in annex F in 3GPP TS 38.141-1 [3].

7)	Set the BS to transmit using [E-TM 2] for BS type 1-O or [E-TM x.x] for BS type 2-O, in 3GPP TS 38.141-1 [3] subclause 6.1.1.1 at the manufacturers declared rated carrier output power (PRated,c,EIRP).

8)	Measure the average OFDM symbol power as defined in annex F 3GPP TS 38.141 [xx] by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS, PBCH or synchronisation signals.

9)	If BS supports 256QAM, set the channel set-up of the transmitted signal according to [E-TM 3.1a] for BS type 1-O or [E-TM x.x] for BS type 2-O and repeat step 1. Set to transmit a signal according to [E-TM 2a] for BS type 1-O or [E-TM x.x] for BS type 2-O and repeat step 3.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.4.3.5	Test requirement

##### 6.4.3.5.1	BS type 1-O

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.3.5.1-1.

Table 6.4.3.5.1-1: Total power dynamic range

| BS channel bandwidth [MHz] | Total power dynamic range [dB] |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.9-FFS | 10.4-FFS | N/A |
| 10 | 17.1-FFS | 13.8-FFS | 10.4-FFS |
| 15 | 18.9-FFS | 15.7-FFS | 12.5-FFS |
| 20 | 20.2-FFS | 17-FFS | 13.8-FFS |
| 25 | 21.2-FFS | 18.1-FFS | 14.9-FFS |
| 30 | 22-FFS | 18.9-FFS | 15.7-FFS |
| 40 | 23.3-FFS | 20.2-FFS | 17-FFS |
| 50 | 24.3-FFS | 21.2-FFS | 18.1-FFS |
| 60 | N/A | 22-FFS | 18.9-FFS |
| 70 | N/A | 22.7-FFS | 19.6-FFS |
| 80 | N/A | 23.3-FFS | 20.2-FFS |
| 90 | N/A | 23.8-FFS | 20.8-FFS |
| 100 | N/A | 24.3-FFS | 21.3-FFS |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

NOTE 2:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in subclause 6.6.

##### 6.4.3.5.2	BS type 2-O

OTA total power dynamic range minimum requirement for BS type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 6.4.3.5.2-1.

Table 6.4.3.5.2-1: Minimum requirement for BS type 2-O total power dynamic range

| SCS [kHz] | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | OTA total power dynamic range [dB] |  |  |  |
| 60 | 18.1-FFS | 21.2-FFS | 24.2-FFS | N.A |
| 120 | 15.0-FFS | 18.1-FFS | 21.2-FFS | 24.2-FFS |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

NOTE 2:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in subclause 6.6.

## 6.5	OTA transmit ON/OFF power

Detailed structure of the subclause is TBD.

## 6.6	OTA transmitted signal quality

Detailed structure of the subclause is TBD.

## 6.7	OTA unwanted emissions

### 6.7.1	General

This subclause describes relations between OTA unwanted emissions requirements.

### 6.7.2	OTA occupied bandwidth

### 6.7.2.1	Definition and applicability

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [13].

The value of /2 shall be taken as 0.5%.

The OTA occupied bandwidth requirement applies during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer’s declared OTA coverage range at the RIB.

### 6.7.2.2	Minimum requirement

The minimum requirement for BS type 1-O and BS type 2-O is in 3GPP TS 38.104 [2], subclause 9.7.2.2.

### 6.7.2.3	Test purpose

The test purpose is to verify that the emission at the RIB does not occupy an excessive bandwidth for the service to be provided and is, therefore, not likely to create interference to other users of the spectrum beyond undue limits.

### 6.7.2.4	Method of test

#### 6.7.2.4.1	Initial conditions

For BS type 1-O:

Test environment: Normal, see annex B.2.

RF channels to be tested:

-	B, M and T; see subclause 4.9.1.

Directions to be tested:

- The reference beam direction pair (see table 4.6-x, Dx.x).

Aggregated Channel Bandwidth positions to be tested for contiguous carrier aggregation:

-	BBW Channel CA, MBW Channel CA and TBW Channel CA; see subclause 4.x.

For a BS declared to be capable of single carrier operation, start transmission according to [E-TM1.1], subclause 4.9.3 at manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

For a BS declared to be capable of contiguous carrier aggregation operation, set the base station to transmit according to [E- TM1.1] on all carriers configured using the applicable test configuration and corresponding power setting specified in annex X.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in annex X.

For BS type 2-O:

Test environment: Normal, see annex B.2.

RF channels to be tested:

-	B, M and T; see subclause 4.9.1.

Directions to be tested:

- The reference beam direction pair (see table 4.6.x, Dx.x).

Aggregated Channel Bandwidth positions to be tested for contiguous carrier aggregation:

-	BBW Channel CA, MBW Channel CA and TBW Channel CA; see subclause 4.x.

For a BS declared to be capable of single carrier operation, start transmission according to [E-TM1.1], subclause 4.9.3 at manufacturers declared rated carrier output EIRP (PRated,c,EIRP).

For a BS declared to be capble of contiguas carrier aggregation operation, set the base station to transmit according to [E- TM1.1] on all carriers configured using the applicable test configuration and corresponding power setting specified in annex X.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause X.

#### 6.7.2.4.2	Procedure

The OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.6.x, Dx.x) of the BS with the test system.

3) Set the BS so that the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested is toward the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit signal.

6)	Measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.7.2.4.2-1 and table  6.7.2.4.2-2. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

Table 6.7.2.4.2-1: Span and number of measurement points for OBW measurements for NR

| Channel bandwidth BWChannel (MHz) | 5 | 10 | 15 | 20 | > 20 |
| --- | --- | --- | --- | --- | --- |
| Span (MHz) | 10 | 20 | 30 | 40 |  |
| Minimum number of measurement points | 400 | 400 | 400 | 400 |  |

7)	Compute the total of the EIRP, P0, (in power units, not decibel units) of all the measurement cells in the measurement span. Compute P1, the EIRP outside the occupied bandwidth on each side. P1 is half of the total EIRP outside the bandwidth. P1 is half of (100 % - (occupied percentage)) of P0. For the occupied percentage of 99 %, P1 is 0.005 times P0. The EIRP calculation depends on whether the test facility supports dual polarization:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Measure and sum the EIRP on both polarizations to obtain P0 or P1.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power as the sum over both polarizations to obtain P0 or P1

8)	Determine the lowest frequency, f1, for which the sum of all EIRP in the measurement cells from the beginning of the span to f1 exceeds P1.

9)	Determine the highest frequency, f2, for which the sum of all EIRP in the measurement cells from the end of the span to f2 exceeds P1.

10)	Compute the OTA occupied bandwidth as f2 - f1.

In addition, for multi-band RIB(s), the following steps shall apply:

11) For multi-band RIBs and single band tests, repeat the steps 6) - 10) above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 6.7.2.5 	Test requirement

#### 6.7.2.5.1	BS type 1-O

The OTA occupied bandwidth for each NR carrier shall be less than the channel bandwidth as defined in table 5.x-x. For contiguous CA, the occupied bandwidth shall be less than or equal to the Aggregated Channel Bandwidth as defined in subclause 5.x.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause x,x,x and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex X.

#### 6.7.2.5.2	BS type 2-O

The OTA occupied bandwidth for each NR carrier shall be less than the channel bandwidth as defined in table 5.x-x. For contiguous CA, the occupied bandwidth shall be less than or equal to the Aggregated Channel Bandwidth as defined in subclause 5.x.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause x,x,x and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex X.

### 6.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

Detailed structure of the subclause is TBD.

### 6.7.4	OTA operating band unwanted emissions

Detailed structure of the subclause is TBD.

### 6.7.5	OTA transmitter spurious emissions

Detailed structure of the subclause is TBD.

## 6.8	OTA transmitter intermodulation

Detailed structure of the subclause is TBD.

# 7	Radiated receiver characteristics

## 7.1	General

General test conditions for receiver tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in clause 4.5.

Unless otherwise stated the requirements in clause 7 apply during the BS receive period.

[The throughput requirements defined for the receiver characteristics in this clause do not assume HARQ transmissions.]

When the BS is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

Each requirement shall be met over the RoAoA specified.

For requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for the reference direction.

And

ΔOTAREFSENS = 41.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for all other directions.

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS (dB)

## 7.2	OTA sensitivity

### 7.2.1	Definition and applicability

The OTA sensitivity requirement is based upon the declaration of one or more OTA sensitivity direction declarations (OSDD), related to a BS type 1-H and BS type 1-O receiver.

The BS type 1-H and BS type 1-O receiver may optionally be capable of redirecting/changing the receiver target by means of adjusting BS settings resulting in multiple sensitivity RoAoA. The sensitivity RoAoA resulting from the current AAS BS settings is the active sensitivity RoAoA.

If the AAS BS is capable of redirecting the receiver target related to the OSDD then the OSDD shall include:

-	BS channel bandwidth and declared minimum EIS level applicable to any active sensitivity RoAoA inside the receiver target redirection range in the OSDD.

-	A declared receiver target redirection range, describing all the angles of arrival that can be addressed for the OSDD through alternative settings in the BS.

-	Five declared sensitivity RoAoA comprising the conformance testing directions as detailed in [x].

-	The receiver target reference direction.

NOTE 1:	Some of the declared sensitivity RoAoA may coincide depending on the redirection capability.

NOTE 2:	In addition to the declared sensitivity RoAoA, several sensitivity RoAoA may be implicitly defined by the receiver target redirection range without being explicitly declared in the OSDD.

If the BS is not capable of redirecting the receiver target related to the OSDD, then the OSDD includes only:

-	BS channel bandwidth and declared minimum EIS level applicable to the sensitivity RoAoA in the OSDD.

-	One declared active sensitivity RoAoA.

-	The receiver target reference direction.

NOTE 3:	For BS without target redirection capability, the declared (fixed) sensitivity RoAoA is always the active sensitivity RoAoA.

The OTA sensitivity EIS level declaration shall apply to each supported polarization, under the assumption of polarization match.

### 7.2.2		Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in 3GPP TS 38.104 [2] subclause 7.2.2 shall be met when the level of the arriving signal is equal to the minimum EIS level in the respective declared set of EIS level and BS channel bandwidth.

### 7.2.3	Test Purpose

The test purpose is to verify that the BS can meet the BER or throughput requirement for a specified measurement channel at the EIS level and the range of angles of arrival declared in the OSDD.

### 7.2.4	Method of test

#### 7.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: B, M and T; see subclause 4.9.1.

Directions to be tested:

- receiver target reference direction (see table 4.6-x, Dx.x),

- conformance test directions (see table 4.6-x, Dx.x).

#### 7.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex x.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction of the BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band and RAT as the OSDD being tested according to the appropriate test configuration in clause 5.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in subclause 7.2.5.4.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in subclause 7.2.5.

9)	Measure:

-	Throughput according to annex A for each supported polarization.

10)	Repeat steps 3 to 9 for all OSDD(s) declared for the BS (see table 4.6-x, Dx.x).

For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.2.5	Test requirements

#### 7.2.5.1	General

The minimum EIS level is a declared figure for each OSDD (see table 4.6-x, D10.6). The test requirement is calculated from the declared value offset by the EIS Test Tolerance specified in subclause 4.1.

#### 7.2.5.2	Test requirements for BS type 1-H and BS type 1-O

For each measured carrier, the throughput measured in step 9 of subclause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [2] annex A.1 with parameters specified in table 7.2.5.2-1.

Table 7.2.5.2-1: EIS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EIS level [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 in clause A.1 in [1] | Declared minimum EIS + [1.3] | Declared minimum EIS + [1.4] | Declared minimum EIS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 in clause A.1 in [1] | Declared minimum EIS + [1.3] | Declared minimum EIS + [1.4] | Declared minimum EIS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 in clause A.1 in [1] | Declared minimum EIS + [1.3] | Declared minimum EIS + [1.4] | Declared minimum EIS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 in clause A.1 in [1] | Declared minimum EIS + [1.3] | Declared minimum EIS + [1.4] | Declared minimum EIS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 in clause A.1 in [1] | Declared minimum EIS + [1.3] | Declared minimum EIS + [1.4] | Declared minimum EIS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 in clause A.1 in [1] | Declared minimum EIS + [1.3] | Declared minimum EIS + [1.4] | Declared minimum EIS + TBD |

#### 7.2.5.3	Test requirements for BS type 2-O

There is no OTA sensitivity requirement for FR2, the OTA sensitivity is the same as the OTA reference sensitivity in subclause 7.3.

## 7.3	OTA reference sensitivity level

### 7.3.1	Definition and applicability

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel. The OTA REFSENS requirement shall apply to each supported polarization, under the assumption of polarization match.

### 7.3.2	Minimum requirement

For BS type 1-O the minimum requirement is in 3GPP TS 38.104 [1], subclause 10.3.2.

For BS type 2-O the minimum requirement is in 3GPP TS 38.104 [1], subclause 10.3.3.

### 7.3.3	Test Purpose

The test purpose is to verify that the BS can meet the throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

### 7.3.4	Method of test

#### 7.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested:

-	B, M and T; see subclause 4.9.1.

Directions to be tested:

- OTA REFSENS receiver target reference direction (see table 4.6-x, Dx.x),

- OTA REFSENS conformance test directions (see table 4.6-x, Dx.x).

#### 7.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction of the BS according to the OTA REFSENS RoAoA for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in clause 5.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in subclause 7.3.5.4.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in subclause 7.3.5.

9)	Measure:

-	Throughput according to annex x for each supported polarization.

10)	Repeat steps 3 to 9 for all OTA REFSENS conformance test directions of the BS (see table 4.6-x, Dx.x).

For multi-band capable FR1 BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.3.5	Test requirements

#### 7.3.5.1	General

The FR1 EISREFSENS level is the conducted REFSENS requirement value offset by ΔOTAREFSENS. The test requirement is calculated from the EISREFSENS level offset by the EISREFSENS Test Tolerance specified in subclause 4.1.

#### 7.3.5.2	Test requirements for BS type 1-O

For each measured carrier, the throughput measured in step 9 of subclause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex TS 38.104 [2] A.1 with parameters specified in tables 7.3.5.2-1 to 7.3.5.2-3.

Table 7.3.5.2-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -101.7 – ΔOTAREFSENS + [1.3 dB] | -101.7 – ΔOTAREFSENS + [1.4 dB] | -101.7 – ΔOTAREFSENS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -101.8 – ΔOTAREFSENS + [1.3 dB] | -101.8 – ΔOTAREFSENS + [1.4 dB] | -101.8 – ΔOTAREFSENS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -98.9 – ΔOTAREFSENS + [1.3 dB] | -98.9 – ΔOTAREFSENS + [1.4 dB] | -98.9 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -95.3 – ΔOTAREFSENS + [1.3 dB] | -95.3 – ΔOTAREFSENS + [1.4 dB] | -95.3 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -95.6 – ΔOTAREFSENS + [1.3 dB] | -95.6 – ΔOTAREFSENS + [1.4 dB] | -95.6 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -95.7 – ΔOTAREFSENS + [1.3 dB] | -95.7 – ΔOTAREFSENS + [1.4 dB] | -95.7 – ΔOTAREFSENS + TBD |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -96.7 – ΔOTAREFSENS + [1.3 dB] | -96.7 – ΔOTAREFSENS + [1.4 dB] | -96.7 – ΔOTAREFSENS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -96.8 – ΔOTAREFSENS + [1.3 dB] | -96.8 – ΔOTAREFSENS + [1.4 dB] | -96.8 – ΔOTAREFSENS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -93.9 – ΔOTAREFSENS + [1.3 dB] | -93.9 – ΔOTAREFSENS + [1.4 dB] | -93.9 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -90.3 – ΔOTAREFSENS + [1.3 dB] | -90.3 – ΔOTAREFSENS + [1.4 dB] | -90.3 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -90.6 – ΔOTAREFSENS + [1.3 dB] | -90.6 – ΔOTAREFSENS + [1.4 dB] | -90.6 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -90.7 – ΔOTAREFSENS + [1.3 dB] | -90.7 – ΔOTAREFSENS + [1.4 dB] | -90.7 – ΔOTAREFSENS + TBD |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-3: Local Area BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -93.7 – ΔOTAREFSENS + [1.3 dB] | -93.7 – ΔOTAREFSENS + [1.4 dB] | -93.7 – ΔOTAREFSENS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -93.8 – ΔOTAREFSENS + [1.3 dB] | -93.8 – ΔOTAREFSENS + [1.4 dB] | -93.8 – ΔOTAREFSENS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -90.9 – ΔOTAREFSENS + [1.3 dB] | -90.9 – ΔOTAREFSENS + [1.4 dB] | -90.9 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -87.3 – ΔOTAREFSENS + [1.3 dB] | -87.3 – ΔOTAREFSENS + [1.4 dB] | -87.3 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -87.6 – ΔOTAREFSENS + [1.3 dB] | -87.6 – ΔOTAREFSENS + [1.4 dB] | -87.6 – ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -87.7 – ΔOTAREFSENS + [1.3 dB] | -87.7 – ΔOTAREFSENS + [1.4 dB] | -87.7 – ΔOTAREFSENS + TBD |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

#### 7.3.5.2	Test requirements for BS type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and TS 38.104 [2] annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the FR2 OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50MHZ BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50MHz BS channel bandwidth it does not imply that BS has to support 50MHz BS channel bandwidth.

For wide area BS, EISREFSENS_50MC is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For medium range BS, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For local area BS, EISREFSENS_50M is an integer value in the range -86- to -109 dBm. The specific value is declared by the vendor.

Table 10.3.3-1 FR2 OTA reference sensitivity requirement

| BS channel Bandwidth [MHz] | Sub-carrier spacing [kHz] | FRC | EISREFSENS level [dBm] |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + TBD |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + TBD |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + TBD |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in table 10.3.3-2. |  |  |  |

## 7.4	OTA dynamic range

### 7.4.1	Definition and applicability

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth.

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the FR1 OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

### 7.4.2	Minimum requirement

For BS type 1-O, the minimum requirement is in 3GPP TS 38.104 [2], subclause 10.4.2.

### 7.4.3	Test purpose

The test purpose is to verify that at the BS receiver dynamic range, the relative throughput shall fulfil the specified limit.

### 7.4.4	Method of test

#### 7.4.4.1	Initial conditions

Test environment:

-	Normal: see annex B.2.

RF channels to be tested:

-	[B, M and T]; see subclause 4.9.1.

Directions to be tested:

- OTA REFSENS receiver target reference direction (see table 4.6-x, Dx.x),

#### 7.4.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.x.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.4.5.2-1 to 7.4.5.2-3.

b)	Set the signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.4.5.2-1 to 7.4.5.2-3.

6)	Measure the throughput according to annex x for each supported polarization.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.4.5	Test requirement

#### 7.4.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA dynamic range Test Tolerance specified in subclause 4.1.

#### 7.4.5.2	Test requirements for BS type 1-O

For each measured carrier, the throughput measured in step 6 of subclause 7.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [2] annex A.2 with parameters specified in tables 7.4.5.2-1 to 7.4.5.2-3.

Table 7.4.5.2-1: Wide Area BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -70.7 – ΔOTAREFSENS + TBD | -70.7 – ΔOTAREFSENS + TBD | -70.7 – ΔOTAREFSENS + TBD | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -71.4 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS + TBD |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.7 – ΔOTAREFSENS + TBD | -70.7 – ΔOTAREFSENS + TBD | -70.7 – ΔOTAREFSENS + TBD | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -71.4 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-3 | -68.4 – ΔOTAREFSENS + TBD | -68.4 – ΔOTAREFSENS + TBD | -68.4 – ΔOTAREFSENS + TBD |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 – ΔOTAREFSENS + TBD | -70.7 – ΔOTAREFSENS + TBD | -70.7 – ΔOTAREFSENS + TBD | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -71.4 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-3 | -68.4 – ΔOTAREFSENS + TBD | -68.4 – ΔOTAREFSENS + TBD | -68.4 – ΔOTAREFSENS + TBD |  |  |
| 20 | 15 | G- FR1-A2-4 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 25 | 15 | G- FR1-A2-4 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 30 | 15 | G- FR1-A2-4 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 40 | 15 | G- FR1-A2-4 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 50 | 15 | G- FR1-A2-4 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -72.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 60 | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -71.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 70 | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -70.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 80 | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 90 | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -69.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| 100 | 30 | G- FR1-A2-5 | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -64.5 – ΔOTAREFSENS + TBD | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD | -64.8 – ΔOTAREFSENS + TBD |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-2: Medium Range BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -65.7 – ΔOTAREFSENS + TBD | -65.7 – ΔOTAREFSENS + TBD | -65.7 – ΔOTAREFSENS + TBD | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -66.4 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS + TBD |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.7 – ΔOTAREFSENS + TBD | -65.7 – ΔOTAREFSENS + TBD | -65.7 – ΔOTAREFSENS + TBD | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -66.4 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-3 | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.7 – ΔOTAREFSENS + TBD | -65.7 – ΔOTAREFSENS + TBD | -65.7 – ΔOTAREFSENS + TBD | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -66.4 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-3 | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD |  |  |
| 20 | 15 | G- FR1-A2-4 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 25 | 15 | G- FR1-A2-4 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 30 | 15 | G- FR1-A2-4 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 40 | 15 | G- FR1-A2-4 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 50 | 15 | G- FR1-A2-4 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | --59.5 – ΔOTAREFSENS + TBD | --59.5 – ΔOTAREFSENS + TBD | --59.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 60 | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 70 | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -65.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 80 | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 90 | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -64.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| 100 | 30 | G- FR1-A2-5 | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -59.5 – ΔOTAREFSENS + TBD | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD | -59.8 – ΔOTAREFSENS + TBD |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-3: Local Area BS dynamic range

| BS channel bandwidth [MHz] | Subcarrier spacing [kHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -62.7 – ΔOTAREFSENS + TBD | -62.7 – ΔOTAREFSENS + TBD | -62.7 – ΔOTAREFSENS + TBD | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.7 – ΔOTAREFSENS + TBD | -62.7 – ΔOTAREFSENS + TBD | -62.7 – ΔOTAREFSENS + TBD | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-3 | -60.4 – ΔOTAREFSENS + TBD | -60.4 – ΔOTAREFSENS + TBD | -60.4 – ΔOTAREFSENS + TBD |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.7 – ΔOTAREFSENS + TBD | -62.7 – ΔOTAREFSENS + TBD | -62.7 – ΔOTAREFSENS + TBD | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-2 | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-3 | -60.4 – ΔOTAREFSENS + TBD | -60.4 – ΔOTAREFSENS + TBD | -60.4 – ΔOTAREFSENS + TBD |  |  |
| 20 | 15 | G- FR1-A2-4 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 25 | 15 | G- FR1-A2-4 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 30 | 15 | G- FR1-A2-4 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 40 | 15 | G- FR1-A2-4 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 50 | 15 | G- FR1-A2-4 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -64.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD |  |  |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 60 | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -63.4 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 70 | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -62.8 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 80 | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 90 | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -61.6 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| 100 | 30 | G- FR1-A2-5 | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -56.5 – ΔOTAREFSENS + TBD | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G- FR1-A2-6 | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD | -56.8 – ΔOTAREFSENS + TBD |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

## 7.5	OTA in-band selectivity and blocking

### 7.5.1	OTA adjacent channel selectivity

#### 7.5.1.1	Definition and applicability

OTA Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 7.5.1.2	Minimum requirement

For BS type 1-O, the minimum requirement is in 3GPP TS 38.104 [2], subclause 10.5.1.2.

For BS type 2-O, the minimum requirement is in 3GPP TS 38.104 [2], subclause 10.5.1.3.

#### 7.5.1.3	Test purpose

The test purpose is to verify the ability of the BS receiver filter to suppress interfering signals in the channels adjacent to the wanted channel.

#### 7.5.1.4	Method of test

##### 7.5.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested:

-	[B, M and T]; see subclause 4.9.1.

-	Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA: MRFBW in single-band operation, see subclause 4.9.1; BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

- OTA REFSENS receiver target reference direction (see table 4.6-x, Dx.x),

##### 7.5.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.x.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation match is accomplished such that all the power from the test antenna is captured by the BS under test.

5)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.1.5.1-1 for BS type 1-O and table 7.5.1.5.2-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in table 7.5.1.5.1-1 for BS type 1-O and table 7.5.1.5.2-1 for BS type 2-O.

6)	Measure throughput according to annex x for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclause 4.8.2.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 7.5.1.5	Test requirement

##### 7.5.1.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA ACS Test Tolerance specified in subclause 4.1.

##### 7.5.1.5.2	Test requirements for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 7.5.1.5.2-1 and table 7.5.1.5.2-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.2 and is further specified in TS 38.104 [2] annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.2-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.5.1.5.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.5.1.5.2-1: OTA ACS requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (Note 2) |  |  | Interfering signal mean power [dBm] |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | EISminSENS + 6dB + TBD | EISminSENS + 6dB + TBD | EISminSENS + 6dB + TBD | Wide Area: -52 – ΔminSENS Medium Range: -47– ΔminSENS Local Area: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth NOTE 2: EISminSENS depends on the BS channel bandwidth. |  |  |  |  |

Table 7.5.1.5.2-2: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±2.5025] | 5MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 10 | [±2.5075] | 5MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 15 | [±2.5125] | 5MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 20 | [±2.5025] | 5MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 25 | [±9.535] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 30 | [±9.585] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 40 | [±9.535] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 50 | [±9.485] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 60 | [±9.585] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 70 | [±9.535] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 80 | [±9.485] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 90 | [±9.585] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |
| 100 | [±9.535] | 20MHz DFT-s-OFDM NR signal SCS: 15kHz |

##### 7.5.1.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the FR2 OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 7.5.1.5.3-1 and table 7.5.1.5.3-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.3 and is further specified in TS 38.104 [2] annex A. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.1.5.3-1: OTA ACS requirement for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] |  | Interfering signal mean power [dBm] |
| --- | --- | --- | --- |
|  | 24.24 GHz < f ≤ 33.4 GHz | 37 GHz < f ≤ 52.6 GHz |  |
| 50, 100, 200, 400 | EISREFSENS + 6dB + TBD (Note 3) | EISREFSENS + 6dB + TBD (Note 3) | EISREFSENS + 27.7 (Note 1) EISREFSENS + 26.7 (Note 2) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 3: EISREFSENS is given in subclause 7.3.5.3. |  |  |  |

Table 7.5.1.5.3-2: OTA ACS interferer frequency offset for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 | ±24.29 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |
| 100 | ±24.31 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |
| 200 | ±24.29 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |
| 400 | ±24.31 | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |

### 7.5.2	OTA in-band blocking

#### 7.5.2.1	Definition and applicability

The OTA in-band blocking characteristics is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.5.2.2	Minimum requirement

For BS type 1-O, the minimum requirement is in 3GPP TS 38.104 [2], subclause 10.5.2.2.

For BS type 2-O, the minimum requirement is in 3GPP TS 38.104 [2], subclause 10.5.2.3.

#### 7.5.2.3	Test purpose

The test purpose is to verify the ability of the BS receiver to withstand high-levels of in-band interference from unwanted signals at specified frequency offsets without undue degradation of its sensitivity.

#### 7.5.2.4	Method of test

##### 7.5.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested:

-	[B, M and T]; see subclause 4.9.1.

-	Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA: MRFBW in single-band operation, see subclause 4.9.1; BRFBW_T’RFBW and B’RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested:

- OTA REFSENS receiver target reference direction (see table 4.6-x, Dx.x),

- OTA REFSENS conformance test directions (see table 4.6-x, Dx.x).

##### 7.5.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.x.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.1-1 for BS type 1-O and table 7.5.2.5.2-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in table 7.5.2.5.1-1 for BS type 1-O and table 7.5.2.5.2-1 for BS type 2-O.

6)	Measure throughput according to annex x for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclause 4.8.2.

7)	Repeat steps 3 to 6 for all the specified measurement directions.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

#### 7.5.2.5	Test requirement

##### 7.5.2.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA in-band blocking Test Tolerance specified in subclause 4.1.

##### 7.5.2.5.2	Test requirements for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the FR1 OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 7.5.2.5.2-1, table 7.5.2.5.2-2 and table 7.5.2.5.2-3 for general OTA and narrowband OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.2 and is further specified in TS 38.104 [2] annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS type 1-O the OTA in-band blocking requirement apply from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. The ΔfOOB for BS type 1-O is defined in table 7.5.2.5.2-0.

Table 7.5.2.5.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB [MHz] |
| --- | --- | --- |
| BS type 1-O | FUL_high – FUL_low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 7.5.2.5.2-1 and 7.5.2.5.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.5.2.5.2-1 and 7.5.2.5.2-3.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA narrowband blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the interfering signal minimum offset in table 7.5.2.5.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band RIBs, the OTA blocking requirements apply in the narrowband blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the interfering signal minimum offset in table 7.5.2.5.2-3.

Table 7.5.2.5.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |  |
| 5, 10, 15, 20 | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | ±7.5 | 5 MHz DFT-s-OFDM NR signal SCS: 15 kHz |
|  | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | Wide Area: -43 - ΔminSENS Medium Range: -38 - ΔminSENS Local Area: -35 - ΔminSENS | ±7.5 | 5 MHz DFT-s-OFDM NR signal SCS: 15 kHz |
| 25 ,30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | ±30 | 20 MHz DFT-s-OFDM NR signal SCS: 15 kHz |
|  | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | Wide Area: -43 - ΔminSENS Medium Range: -38 - ΔminSENS Local Area: -35 - ΔminSENS | ±30 | 20 MHz DFT-s-OFDM NR signal SCS: 15 kHz |

Table 7.5.2.5.2-2: OTA narrowband blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] |  |  | OTA Interfering signal mean power [dBm] |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 10, 15, 20 | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | Wide Area: -49 - ΔminSENS Medium Range: -44 - ΔminSENS Local Area: -41 - ΔminSENS |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | EISREFSENS + 6 dB + TBD | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | EISminSENS + 6 dB + TBD | Wide Area: -49 - ΔminSENS Medium Range: -44 - ΔminSENS Local Area: -41 - ΔminSENS |
| NOTE: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. |  |  |  |  |

Table 7.5.2.5.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | ±([342.5] + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 1 RB SCS: 15 kHz |
| 10 | ±([347.5] + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±([352.5] + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±([342.5] + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±([557.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 | 20 MHz DFT-s-OFDM NR signal, 1 RB SCS: 15 kHz |
| 30 | ±([562.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 40 | ±([557.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 50 | ±([552.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 60 | ±([562.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 70 | ±([557.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 80 | ±([552.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 90 | ±([562.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 100 | ±([557.5] + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| NOTE: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |

##### 7.5.2.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the FR2 OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2 BS, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 10.5.2.3-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in subclause 7.3.5.3 and is further specified in TS 38.104 [2] annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges. The OTA blocking requirements apply in the in-band blocking frequency range, which is from [XX] MHz below the lowest frequency of the uplink operating band up to [XX] MHz above the highest frequency of the uplink operating band for BS type 2-O in an operating band less than [XX] MHz wide, but excludes the downlink frequency range of the FDD operating band.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.5.2.5.3-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.2.5.3-1: General OTA blocking requirement for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received [MHz] | OTA wanted signal mean power [dBm] |  | OTA interfering signal mean power [dBm] | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
|  | 24.24 GHz < f ≤ 33.4 GHz | 37 GHz < f ≤ 52.6 GHz |  |  |  |
| 50, 100, 200, 400 | EISREFSENS + 6dB + TBD | EISREFSENS + 6dB + TBD | EISREFSENS_50M + 33dB | ±75 | 50 MHz DFT-s-OFDM NR signal 60 kHz SCS |
| NOTE: EISREFSENS and EISREFSENS_50M are given in subclause 7.3.5.3. |  |  |  |  |  |

## 7.6	OTA out-of-band blocking

Detailed structure of the subclause is TBD.

## 7.7	OTA receiver spurious emissions

Detailed structure of the subclause is TBD.

## 7.8	OTA receiver intermodulation

### 7.8.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. The requirement is defined as a directional requirement at the RIB.

### 7.8.2	Minimum requirement

The minimum requirement for BS type 1-O is in 3GPP TS 38.104 [2], subclause 10.8.2.

The minimum requirement for BS type 2-O is in 3GPP TS 38.104 [2], subclause 10.8.3.

### 7.8.3	Test purpose

To verify that the BS receiver dynamic range, the relative throughput shall fulfil the specified limit.

### 7.8.4	Method of test

#### 7.8.4.1	Initial conditions

Test environment: Normal, annex B.2.

RF channels to be tested for single carrier:		FFS; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested: 	For single-band RIB(s): FFS. For multi-band RIB(s): FFS

Directions to be tested: OTA REFSENS receiver target reference direction (see table 4.6-1, Dx.x).

#### 7.8.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction of the BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band and RAT as the OSDD being tested according to the appropriate test configuration in clause 5.

7)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.8.5.1-1 and 7.8.5.1-3 for BS type 1-O and table 7.8.5.2-1 for BS type 2-O.

b)	Set the Signal generator for the interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.8.5.1-1 and 7.8.5.1-3 for BS type 1-O and table 7.8.5.2-1 for BS type 2-O.

8)	Measure the throughput according to annex E, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclause 5.3.4.

9)	Repeat for all the specified measurement directions.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.8.5	Test requirement

#### 7.8.5.1	BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the FR1 OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB with the conditions specified in tables 7.8.5.1-1 and 7.8.5.1-2 for intermodulation performance and in tables 7.8.5.1-3 and 7.8.5.1-4 for narrowband intermodulation performance.

The reference measurement channel for the wanted signal is identified in table 7.3.5.2-1, table 7.3.5.2-2 and table 7.3.5.2-3 for each BS channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal should be 30kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the BS channel bandwidth of the NR interfering signal in tables 7.8.5.1-2 and 7.8.5.1-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

[For multi-band RIBs, the intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

[For multi-band RIBs, the narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in tables 7.8.5.1-2 and 7.8.5.1-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Table 7.8.5.1-1: General intermodulation requirement

| BS class | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 + ΔOTAREFSENS | EISREFSENS + 6 dB +FFS | See table 7.8.5.1-2 |
|  | -52 + ΔminSENS | EISminSENS + 6 dB +FFS |  |
| Medium Range BS | -47 + ΔOTAREFSENS | EISREFSENS + 6 dB +FFS |  |
|  | -47 + ΔminSENS | EISminSENS + 6 dB +FFS |  |
| Local Area BS | -44 + ΔOTAREFSENS | EISREFSENS + 6 dB +FFS |  |
|  | -44 + ΔminSENS | EISminSENS + 6 dB +FFS |  |
| NOTE: EISREFSENS and EISminSENS depend on the BS class and on the BS channel bandwidth, see subclause 7.3 and 7.2. |  |  |  |

Table 7.8.5.1-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±7.5] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 10 | [±7.45] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 15 | [±7.43] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 20 | [±7.38] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 30 | [±7.43] | CW |
|  | [±25] | 5MHz DFT-S-OFDM NR signal |
| 25 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 40 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 50 | [±7.35] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 60 | [±7.49] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 70 | [±7.42] | CW |
|  | [±25] | 5MHz DFT-S-OFDM NR signal |
| 80 | [±7.44] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 90 | [±25] | CW |
|  | [±7.43] | 5MHz DFT-S-OFDM NR signal |
| 100 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |

Table 7.8.5.1-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB +FFS (Note 1) | -52 - ΔOTAREFSENS | See table 7.8.5.1-4 |
|  | EISminSENS + 6 dB +FFS (Note 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB +FFS (Note 1) | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB +FFS (Note 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB +FFS (Note 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB +FFS (Note 1) | -44 - ΔminSENS |  |
| NOTE: EISREFSENS / EISminSENS depends on the BS channel bandwidth, see subclause 7.3 and 7.2. |  |  |  |

Table 7.8.5.1-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±360] | CW |
|  | [±1420] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 10 | [±325] | CW |
|  | [±1780] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | [±380] | CW |
|  | [±1600] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | [±345] | CW |
|  | [±1780] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | [±325] | CW |
|  | [±1990] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | [±320] | CW |
|  | [±1990] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | [±310] | CW |
|  | [±2710] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | [±330] | CW |
|  | [±3250] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | [±350] | CW |
|  | [±3790] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | [±400] | CW |
|  | [±4870] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | [±390] | CW |
|  | [±4870] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

#### 7.8.5.2	BS type 2-O

Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted signal at the assigned channel frequency and two OTA interfering signals provided at the RIB using the parameters in tables 7.8.5.2-1 and 7.8.5.2-2. All of the OTA test signals arrive from the same direction, and the requirement is valid if the signals arrive from any direction within the FR2 OTA REFSENS RoAoA. The reference measurement channel for the wanted signal is identified in [table 7.3.2-1] for each BS channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 7.8.5.2-1: General intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Mean power of interfering signals [dBm] | Wanted signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS_50M + 25dB +FFS | EISREFSENS + 6dB | See table 7.8.5.2-2 |
| NOTE: EISREFSENS and EISREFSENS_50M are given in subclause 10.3.3. |  |  |  |

Table 7.8.5.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 MHz | [±7.5] | CW |
|  | [±40] | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |
| 100 MHz | [±6.88] | CW |
|  | [±40] | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |
| 200 MHz | [±5.64] | CW |
|  | [±40] | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |
| 400 MHz | [±6.02] | CW |
|  | [±45] | 50MHz DFT-s-OFDM NR signal 60 kHz SCS |

## 7.9	OTA in-channel selectivity

### 7.9.1	Definition and applicability

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal as specified in annex A and shall be time aligned with the wanted signal.

### 7.9.2	Minimum requirement

The minimum requirement for BS type 1-O is in 3GPP TS 38.104 [2], subclause 10.9.2.

The minimum requirement for BS type 2-O is in 3GPP TS 38.104 [2], subclause 10.9.3.

### 7.9.3	Test purpose

# The purpose of this test is to verify the BS receiver ability to suppress the IQ leakage.

### 7.9.4	Method of test

#### 7.9.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:		FFS; see subclause 4.9.1.

Directions to be tested: OTA REFSENS receiver target reference direction (see table 4.6-1, Dx.x).

#### 7.9.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

For each supported NR channel BW:

7)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified as follows:

a)	Adjust the signal generator for the wanted signal as specified in:

For BS type 1-O, table 7.9.5.1-1 for BS of Wide Area BS class, in table 7.9.5.1-2 for BS of Local Area BS class and in table 7.9.5.1-3 for BS of  Medium Range BS class on one side of the FC.

For BS type 2-O, table 7.9.5.2-1 on one side of the FC.

b)	Adjust the signal generator for the interfering signal as specified in:

For BS type 1-O, table 7.9.5.1-1 for BS of Wide Area BS class, in table 7.9.5.1-2 for BS of Local Area BS class and in table 7.9.5.1-3 for BS of  Medium Range BS class at opposite side of the FC and adjacent to the wanted signal.

For BS type 2-O, table 7.9.5.2-1 at opposite side of the FC and adjacent to the wanted signal.

4)	Measure throughput according to annex E.

5)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

9)	Repeat for all the specified measurement directions.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

### 7.9.5	Test requirement

#### 7.9.5.1	BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to all supported polarizations, under the assumption of polarization matching.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For BS type 1-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A with parameters specified in table 7.9.5.1-1 for Wide Area BS, in table 7.9.5.1-2 for Medium Range BS and in table 7.9.5.1-3 for Local Area BS.

Table 7.9.5.1-1: Wide Area BS in-channel selectivity

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -100.6-ΔminSENS + FFS | -100.6-ΔminSENS + FFS | -100.6-ΔminSENS + FFS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -98.7-ΔminSENS + FFS | -98.7-ΔminSENS + FFS | -98.7-ΔminSENS + FFS | -77.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40, 50 | 15 | G-FR1-A1-4 | -92.3-ΔminSENS + FFS | -92.3-ΔminSENS + FFS | -92.3-ΔminSENS + FFS | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 15 kHz, 100 PRB |
| 5 | 30 | G-FR1-A1-8 | -101.3-ΔminSENS + FFS | -101.3-ΔminSENS + FFS | -101.3-ΔminSENS + FFS | -81.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -98.8-ΔminSENS + FFS | -98.8-ΔminSENS + FFS | -98.8-ΔminSENS + FFS | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 30 kHz, 10 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -92.6-ΔminSENS + FFS | -92.6-ΔminSENS + FFS | -92.6-ΔminSENS + FFS | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 30 kHz, 50 PRB |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -98.2-ΔminSENS + FFS | -98.2-ΔminSENS + FFS | -98.2-ΔminSENS + FFS | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -92.7-ΔminSENS + FFS | -92.7-ΔminSENS + FFS | -92.7-ΔminSENS + FFS | -71.6 - ΔminSEN | DFT-s-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-2:  Medium Range BS in-channel selectivity

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -95.6-ΔminSENS + FFS | -95.6-ΔminSENS + FFS | -95.6-ΔminSENS + FFS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -93.7-ΔminSENS + FFS | -93.7-ΔminSENS + FFS | -93.7-ΔminSENS + FFS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40, 50 | 15 | G-FR1-A1-4 | -87.3-ΔminSENS + FFS | -87.3-ΔminSENS + FFS | -87.3-ΔminSENS + FFS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 100 PRB |
| 5 | 30 | G-FR1-A1-8 | -96.3-ΔminSENS + FFS | -96.3-ΔminSENS + FFS | -96.3-ΔminSENS + FFS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -93.8-ΔminSENS + FFS | -93.8-ΔminSENS + FFS | -93.8-ΔminSENS + FFS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 10 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -87.6-ΔminSENS + FFS | -87.6-ΔminSENS + FFS | -87.6-ΔminSENS + FFS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 50 PRB |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -93.2-ΔminSENS + FFS | -93.2-ΔminSENS + FFS | -93.2-ΔminSENS + FFS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -87.7-ΔminSENS + FFS | -87.7-ΔminSENS + FFS | -87.7-ΔminSENS + FFS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-3: Local area BS in-channel selectivity

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] |  |  | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -92.6-ΔminSENS + FFS | -92.6-ΔminSENS + FFS | -92.6-ΔminSENS + FFS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -90.7-ΔminSENS + FFS | -90.7-ΔminSENS + FFS | -90.7-ΔminSENS + FFS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40, 50 | 15 | G-FR1-A1-4 | -84.3-ΔminSENS + FFS | -84.3-ΔminSENS + FFS | -84.3-ΔminSENS + FFS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 15 kHz, 100 PRB |
| 5 | 30 | G-FR1-A1-8 | -93.3-ΔminSENS + FFS | -93.3-ΔminSENS + FFS | -93.3-ΔminSENS + FFS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -90.8-ΔminSENS + FFS | -90.8-ΔminSENS + FFS | -90.8-ΔminSENS + FFS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 10 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -84.6-ΔminSENS + FFS | -84.6-ΔminSENS + FFS | -84.6-ΔminSENS + FFS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 30 kHz, 50 PRB |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -90.2-ΔminSENS + FFS | -90.2-ΔminSENS + FFS | -90.2-ΔminSENS + FFS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -84.7-ΔminSENS + FFS | -84.7-ΔminSENS + FFS | -84.7-ΔminSENS + FFS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. |  |  |  |  |  |  |  |

#### 7.9.5.2	BS type 2-O

For BS type 2-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A with parameters specified in table 7.9.5.2-1.

Table 7.9.5.2-1: OTA in-channel selectivity requirement for BS type 2-O

| BS channel bandwidth [MHz] | Subcarrier spacing [KHz] | Reference measurement channel | Wanted signal mean power [dBm] (Note 2) | Interfering signal mean power [dBm] (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M +FFS | EISREFSENS_50M + 10 | DFT-s-OFDM NR signal, SCS 60 kHz, 32 PRB |
| 100,200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 3 +FFS | EISREFSENS_50M + 13 | DFT-s-OFDM NR signal, SCS 60 kHz, 64 PRB |
| 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M +FFS | EISREFSENS_50M + 10 | DFT-s-OFDM NR signal, SCS 120 kHz, 16 PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M +3 +FFS | EISREFSENS_50M + 13 | DFT-s-OFDM NR signal, SCS 120 kHz, 32 PRB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 and located in the interfering signal. NOTE 2: EISREFSENS_50M is defined in subclause 7.3.3. |  |  |  |  |  |

# 8	Radiated performance requirements

Detailed structure of the clause is TBD.

Annex A (normative):
Reference measurement channels

Annex B (normative):
Environmental requirements for the BS equipment

# B.1	General

For each test in the present document, the environmental conditions under which the BS is to be tested are defined.

# B.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table D.1.

Table B.1: Limits of conditions for normal test environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15 C | 30 C |
| Relative humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

NOTE:	This may, for instance, be the case for measurements of radiated emissions performed on an open field test site.

# B.3	Extreme test environment

The manufacturer shall declare one of the following:

1)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-3 [7];

2)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-4 [8];

3)	The equipment that does not comply with the mentioned classes, the relevant classes from IEC 60 721 [9] documentation for temperature, humidity and vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions is not tested in the present document. These may be stated and tested separately.

## B.3.1	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-1 [10].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-2 [11].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

# B.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer’s declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60 068-2-6 [12]. Other environmental conditions shall be within the ranges specified in annex B.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

# B.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [10] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [10] Test Ab/Ad and IEC 60 068-2-2 [11] Test Bb/Bd: Dry heat.

# B.6	Measurement of test environments

The measurement accuracy of the BS test environments defined in annex B shall be:

Pressure:	5 kPa

Temperature:	2 degrees

Relative humidity:	5 %

DC voltage:	1.0 %

AC voltage:	1.5 %

Vibration:	10 %

Vibration frequency:	0.1 Hz

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

Annex C (informative):
Test tolerances and derivation of test requirements

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification 3GPP TS 38.104 [2] using the test tolerances (TT) defined here. When the TT value is zero, the test requirement will be the same as the minimum requirement. When the TT value is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

The TTOTA values are derived from OTA Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the TTOTA values may sometimes be set to zero.

The TTOTA values should not be modified for any reason e.g. to take account of commonly known OTA Test System errors (such as mismatch, cable loss, etc.).

Note that a formula for applying TTOTA values is provided for all OTA tests, even those with a test tolerance of zero. This is necessary in the case where the OTA Test System uncertainty is greater than that allowed in subclause 4.1.2. In this event, the excess error shall be subtracted from the defined TTOTA value in order to generate the correct tightened test requirements as defined in this annex.

# C.1	Measurement of transmitter

Detailed structure of the subclause is TBD.

Table C.1-1: Derivation of test requirements (OTA transmitter tests)

| Test | Minimum requirement in 3GPP TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See 3GPP TS 38.104 [2], subclause 9.2 | FR1: 1.0 dB, f ≤ 3.0 GHz 1.2 dB, 3.0 GHz < f ≤ 4.2 GHz TBD, 4.2 GHz < f ≤ 6.0 GHz | Formula: Upper limit + TT, Lower limit – TT |
|  |  | FR2: TBD |  |

# C.2	Measurement of receiver

Detailed structure of the subclause is TBD.

Table C.2-1: Derivation of test requirements (OTA receiver tests)

| Test | Minimum requirement in 3GPP TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See 3GPP TS 38.104 [2], subclause 10.2 | FR1: 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz TBD, 4.2 GHz < f ≤ 6.0 GHz | Formula: Declared Minimum EIS + TT |
|  |  | FR2: TBD |  |

Annex D (normative):
Calibration

Annex E (informative):
OTA test system set-up

# E.1	Transmitter

## E1.1	Radiated transmit power

Figure E.1.1-1: Measurement set up for radiated transmit power

# E.2	Receiver

## E.2.1	OTA sensitivity

Figure E.2.1-1: Measurement set up for OTA sensitivity

Annex F (normative):
Estimation of Measurement Uncertainty

Annex G (informative):
Transmitter Spatial emissions Declaration

## G.1	General

The transmitter spatial emission declaration is an optional declaration which provides additional information on the power level of emission in the intended (in cell) spatial directions and the unintended (out of cell) spatial directions. The declarations are only valid when the beam is configured in one of the EIRP conformance directions.

Figure G.1-1: Example of out of cell directions set and declared single beam at a single extreme steering direction

The declaration of unwanted spatial emission may in many circumstances not directly relate to system performance on its own. This is because it is often not possible to differentiate wanted and unwanted radiation, and furthermore because the benefits of optimizing beamforming performance may outweigh the impacts of “unwanted” radiation, leading to systems with apparently higher unwanted radiation also providing superior throughput performance. System performance should additionally be characterized taking all factors into account.

## G.2	Declarations

Table G.2-1: Optional manufacturer declarations

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| Dxx.1 | Out of cell directions set | The set of directions which are outside the intended directions of radiation or outside the wanted cell. Declared per operating band. |
| Dxx.2 | Out of cell power level | Declared in band average power inside each of the out of cell directions set(s) (DE.1) declared for each of the 5 conformance directions (D9.x) |
| Dxx.3 | In cell power level | Declared in band average power outside the out of cell directions set(s) (DE.1) declared for each of the 5 conformance directions (D9.x) |
| Dxx.4 | Average out of cell power level | Declared in band average power inside each of the out of cell directions set(s) (DE.1) averaged over the 5 conformance directions (D9.x). |
| DE.5 | Average in cell power level | Declared in band average power inside each of the out of cell directions set(s) (DE.1) averaged over the 5 conformance directions (D9.x) |

NOTE 1:	The declaration of unwanted spatial emission may in many circumstances not directly relate to system performance on its own. This is because it is often not possible to differentiate wanted and unwanted radiation, and furthermore because the benefits of optimizing beamforming performance may outweigh the impacts of “unwanted” radiation, leading to systems with apparently higher unwanted radiation also providing superior throughput performance. System performance should additionally be characterized taking all factors into account.

NOTE 2:	The average out of cell power level reflects the impact of out of cell radiation on other cells more accurately than the out of cell power level for individual test beams.

Annex H (informative):
Format and interpretation of tests

Each test has a standard format:

X	Title

All tests are applicable to all equipment within the scope of the present document, unless otherwise stated.

X.1	Definition and applicability

This subclause gives the general definition of the parameter under consideration and specifies whether the test is applicable to all equipment or only to a certain subset. Required manufacturer declarations may be included here.

X.2	Minimum requirement

This subclause contains the reference to the subclause to the 3GPP reference (or core) specification which defines the minimum requirement.

X.3	Test purpose

This subclause defines the purpose of the test.

X.4	Method of test

X.4.1	General

In some cases there are alternative test procedures or initial conditions. In such cases, guidance for which initial conditions and test procedures can be applied are stated here. In the case only one test procedure is applicable, that is stated here.

X.4.2y 	First test method

X.4.2y.1	Initial conditions

This subclause defines the initial conditions for each test, including the test environment, the RF channels to be tested and the basic measurement set-up. The OTA Test System is assumed to be correctly calibrated as part of the initial conditions. Calibration is not explicitly mentioned.

X.4.2y.2	Procedure

This subclause describes the steps necessary to perform the test and provides further details of the test definition like domain (e.g. frequency-span), range, weighting (e.g. bandwidth), and algorithms (e.g. averaging). The procedure may comprise data processing of the measurement result before comparison with the test requirement (e.g. average result from several measurement positions).

X.4.3y		Alternative test method (if any)

If there are alternative test methods, each is described with its initial conditions and procedures.

X.5	Test requirement

This subclause defines the pass/fail criteria for the equipment under test, see subclause 4.1.3 (Interpretation of measurement results). Test requirements for every minimum requirement referred in subclause X.2 are listed here. Cases where minimum requirements do not apply need not be mentioned.

Annex I (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017/11 | R4-84bis | R4-1711983 | - | - | - | TS skeleton | 0.0.1 |
| 2018/04 | R4-86bis | R4-1805875, R4-1805876, R4-1804931, R4-1805877, R4-1805386, R4-1805916, R4-1805915, R4-1805878 | - | - | - | Implementation of TPs agreed during RAN4#86bis, on top of the agreed R4-1803913: - R4-1805875 TP for TS 38.141-2: Addition of applicability table in sub-clause 4.7.2 - R4-1805876 TP for TS 38.141-2: Addition of co-location reference antenna description to sub-clause 4.13 - R4-1804931 TP to TS 38.141-2: clauses 1-3 - R4-1805877 TP to TS 38.141-2: clauses 4, 5 - R4-1805386 TP to TS 38.141-2: clause 6 - R4-1805916 TP to TS 38.141-2: clause 7 - R4-1805915 TP to TS 38.141-2: NR BS OTA sensitivity conformance test (7.2) - R4-1805878 TP to TS 38.141-2: NR BS OTA REFSENS conformance test (7.3) | 0.1.0 |
| 2018/06 | R4-87 | R4-1714157, R4-1806599, R4-1807591, R4-1807747, R4-1808325, R4-1808329, R4-1808331, R4-1808332, R4-1808333, R4-1808334, R4-1808336, R4-1808337, R4-1808483, R4-1808487 | - | - | - | Implementation of TPs agreed during RAN4#87, on top of R4-1807255: - R4-1714157 TP to TS 38.141-2 - annex with spatial declarations definitions - R4-1806599 TP to TS 38.141-2: NR BS OTA dynamic range conformance test (7.4) - R4-1807591 TP to TS 38.141-2 - update to Rx general section (7.1) - R4-1807747 TP to TS 38.141-2: Annexes - R4-1808325 TP to TS 38.141-2: Improvement of RIB interface in Figures 4.2-1, 4.2-2 and 4.2-3, in sub-clause 4.2 - R4-1808329 TP to TS 38.141-2: NR BS OTA in-band selectivity and blocking conformance test (7.5) - R4-1808331 TP to TS 38.141-2: Definitions, symbols and abbreviations (Sections 3) - R4-1808332 TP to TS 38.141-2: NR BS OTA occupied bandwidth (6.7.2) - R4-1808333 TP to TR 38.141-2: Clarifications on OTA sensitivity requirement (7.2.1, 7.3.1) - R4-1808334 TP to TS 38.141-2 - OTA base station output power (6.3) - R4-1808336 TP to TS 38.141-2 - OTA in-band receiver intermodulation (7.8) - R4-1808337 TP to TS 38.141-2 - OTA in-channel selectivity (7.9) - R4-1808483 TP to TS 38.141-2: multi-band operation - R4-1808487 TP to TS 38.141-2 - OTA output power dynamics (6.4) | 0.2.0 |