3GPP TR 38.814 V1.0.0 (2018-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

New frequency range for NR (4.4-5.0 GHz)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	6

5	NR Frequency band definition	6

6	Channel numbering and channel bandwidth	6

7	NR band 4.4-4.99 GHz issues	6

7.1	UE specific	6

7.1.1	Transmitter characteristics	6

7.1.1.1	UE maximum output power	6

7.1.1.2	UE maximum output power for modulation / channel bandwidth	7

7.1.1.3	UE maximum output power with additional requirements	7

7.1.1.3	Spectrum emission mask	7

7.1.1.4	Additional spectrum emission mask	7

7.1.1.5	Adjacent Channel Leakage Ratio (ACLR)	8

7.1.1.6	Spurious emission band UE co-existence	8

7.1.1.7	Additional spurious emissions	8

7.1.1.8	A-MPR	8

7.1.2	Receiver characteristics	8

7.1.2.1	Reference sensitivity	8

7.1.2.2	Adjacent Channel Selectivity (ACS)	8

7.1.2.3	Blocking	8

7.1.2.3.1	Out-of-band blocking	8

7.2	BS specific	9

7.2.1	Operating band unwanted emissions	9

7.2.2	Additional spurious emissions requirements	9

7.2.3	Co-location with other base stations	9

7.2.4	General blocking requirement	9

7.2.5	Blocking requirement for co-location with other base stations	9

8	   Required changes to NR, E-UTRA, UTRA and MSR specifications	9

Annex <A> (normative): <Normative annex title>	10

Annex B: Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

<Text to be added>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# 4	Background

<Text to be added>

# 5	NR Frequency band definition

<Text to be added>

# 6	Channel numbering and channel bandwidth

<Text to be added>

# 7	NR band 4.4-4.99 GHz issues

## 7.1	UE specific

<Text to be added>

### 7.1.1	Transmitter characteristics

#### 7.1.1.1	UE maximum output power

The MOP requirements for Band 42 and 43 single-carrier were specified as 23 dBm +2/-3 dB for power class 3 based on simulation results shown in Table 7.1.1.1-1 in August 2011.

Table 7.1.1.1-1.  Simulation results for combined Band 42 and Band 43 filter [R4-114656]

|  | Bandwidth | Max IL (corner) | 2f0 rejection | 2.7 GHz rejection |
| --- | --- | --- | --- | --- |
| Design 7 | 400 MHz | 1.9 | 20 dB | 30 dB |
| Design 8 | 400 MHz | 2.3 | 15 dB | 10 dB |
| Design 9 | 400 MHz | 2.0 | 20 dB | 15 dB |

BPFs for Band n79 are provided in Table 7.1.1.1-2 and 7.1.1.1-3.

Table 7.1.1.1-1.  Simulation results for combined Band 42 and Band 43 filter [R4-114656]

| F [GHz] | 4.4 | 4.8 | 4.99 | 5.0 |
| --- | --- | --- | --- | --- |
| IL +25C [dB] | 0.88 | 0.60 | 0.59 | 0.59 |
| IL ETC -40~+90C [dB] | 1.1 | 0.8 | 0.80 | 0.80 |

Table 7.1.1.1-2.  Simulation results for combined Band 42 and Band 43 filter [R4-114656]

|  | typ IL [dB] |  |  | worst IL [dB] |  |  | Attenuation [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frequencies [GHz] | 4.4 | 4.8 | 5 | 4.4 | 4.8 | 5 | 2.3-2.69 | 0.617-2.2 | 3.3-4.2 | 3.3-3.8 | 5.15-5.925 | 5.250-5.925 |
| n79 filter TR | 2.3 | 1 | 2.3 | 2.5 | 1.2 | 2.5 | >35 | >25 | >15 | >15 | >40 | >40 |
| comment | 4.4-4.5GHz range must demonstrate coexistence with altimeter band for China |  |  |  |  |  |  |  |  |  |  |  |
|  | improved rejection in WiFi band |  |  |  |  |  |  |  |  |  |  |  |

From these tables, no significant filter IL degradation for Band n79 can be seen compared to that of Band 42 and 43 at ETC condition. Therefore, MOP for Band n79 should be aligned with those of Band 42 and 43 at least.

Agreement: MOP for Band n79 is to be 23 dBm +2/-3 dB

#### 7.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

#### 7.1.1.3	UE maximum output power with additional requirements

<Text to be added>

#### 7.1.1.3	Spectrum emission mask

<Text to be added>

#### 7.1.1.4	Additional spectrum emission mask

<Text to be added>

#### 7.1.1.5	Adjacent Channel Leakage Ratio (ACLR)

<Text to be added>

#### 7.1.1.6	Spurious emission band UE co-existence

<Text to be added>

#### 7.1.1.7	Additional spurious emissions

<Text to be added>

#### 7.1.1.8	A-MPR

<Text to be added>

### 7.1.2	Receiver characteristics

#### 7.1.2.1	Reference sensitivity

With the same investigation as UE maximum output power described in 7.1.1.1, no significant filter IL degradation for Band n79 can be seen compared to that of Band 42 and 43 at ETC condition. In addition, from LNA perspective, the fractional bandwidth of Band n79 pass-band is 12.8 % which is smaller than that of Band n78 (14.1 %). Hence, NF of Band n79 shouldn’t be higher than that of Band n78. Therefore, reference sensitivity for Band n79 should also be aligned with those of Band n78 at least.

Agreement: REFSENS for Band n79 should be 1 dB larger than that of bands which have NR smallest sensitivity (less RF challenges) such as Band n1.

Table 7.1.2.1-1: Reference sensitivity for Band n79

| Operating Band | SCS kHz | 40 | 50 | 60 | 80 | 100 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz |  |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| n79 | 15 | -89.6 | -88.6 |  |  |  | TDD |
|  | 30 | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  | 60 | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |

#### 7.1.2.2	Adjacent Channel Selectivity (ACS)

<Text to be added>

#### 7.1.2.3	Blocking

##### 7.1.2.3.1	Out-of-band blocking

Based on filter performance shown in Table 7.1.1.1-1 and 7.1.1.1-2, out-of-band blocking requirement needs to be optimized as with LTE band 42 and 43 as below.

Table 7.1.2.3.1-1: Out-of-band blocking parameters for Band n79

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified TBD with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is TBD NOTE 3: The REFSENS power level is TBD |  |  |  |  |  |  |

Table 7.1.2.3.1-2: Out of band blocking for Band n79

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| n79 (NOTE 2) | FInterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MIN(150,3CBW) or MIN(150,3CBW) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MIN(150,3CBW) or FDL_high + MIN(150,3CBW) ≤ f ≤ 12750 |
| NOTE 1: CBW denotes the channel bandwidth of the wanted signal NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW equal or larger than 50 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*CBW from the band edge. |  |  |  |  |  |

## 7.2	BS specific

<Text to be added>

### 7.2.1	Operating band unwanted emissions

<Text to be added>

### 7.2.2	Additional spurious emissions requirements

<Text to be added>

### 7.2.3	Co-location with other base stations

<Text to be added>

### 7.2.4	General blocking requirement

<Text to be added>

### 7.2.5	Blocking requirement for co-location with other base stations

<Text to be added>

# 8				Required changes to NR, E-UTRA, UTRA and MSR specifications

The required changes to the 3GPP specifications for the NR band 4.4-4.99GHz are summarised in a Table 8-1.

Table 8-1: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 08/2017 | RAN4#84 | R4-1708172 |  |  | TR skeleton | N/A | 0.0.1 |
| 01/2018 | RAN4-#85 | R4-1800831 |  |  | RAN4 #84bis R4-1711836 TP for TR 38.814 UE RF requirements of Band n79 NTT DOCOMO, INC. RAN4 #85 R4-1712816 TP for TR 38.814 Finalization of Band n79 NTT DOCOMO, INC. | 0.0.1 | 0.1.0 |