3GPP TSG RAN WG2 Meeting #104               	R2-1817617

Spokane, USA, 12 - 16 November 2018

Agenda item:		11.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Resource allocation and coordination for IAB

WID/SID:	FS_NR_IAB

Document for:		Discussion and Decision

# 1	Introduction

The study of Integrated Access and Backhaul (IAB) is an essential component of NR deployments providing a mechanism to achieve coverage and reliability targets in the absence of wired backhaul connection. This contribution deals with resource allocation aspects between access and BH links. More specifically, we consider the following objectives of the approved IAB study item [1]:

- Dynamic resource allocation between the backhaul and access links [RAN1, RAN2], e.g.,

  - Mechanisms to efficiently multiplex access and backhaul links (for both DL and UL directions) in time, frequency, or space under a per-link half-duplex constraint across one or multiple backhaul link hops for both TDD and FDD operation.

RAN1 has already agreed the baseline approach for resource configuration principles and resource types that would be needed to control the radio resource usage on the parent/child BH/access links. Relevant agreements made in RAN1#93 [2], RAN1#94 [3] and RAN1#94bis [4] are listed in the Appendix.

# 2	Schedulable resources for IAB BH link(s)

## 2.1. IAB node schedulable resources

Figure 1 [3] shows basic connections between the IAB nodes and access UEs. From the middle IAB node perspective there will be parent BH links as well as child BH and access links, all for both UL and DL. According to RAN1 agreements [2]:

- Downlink IAB transmissions (transmissions from an IAB node to child IAB nodes and UEs directly under the IAB node) should be scheduled by the IAB node itself.

- Uplink IAB transmission (transmissions from an IAB node to its parent node) should be scheduled by the parent node.

RAN1 has also agreed that “from MT point of view, time domain resources available for parent link are the same as in NR Rel-15

- Downlink time resources facilitate DL Parent BH link

- Uplink time resources facilitate UL Parent BH link

- Flexible time resources facilitate dynamic capacity allocation between DL/UL and between Parent/Child links.”

The question is now how to determine schedulable resources for the IAB Node (DU).

Figure 1 Definition of the IAB nodes and links between IAB nodes and access UEs

Main principles for the resource allocation and coordination can be following:

- IAB node capabilities, primarily the half duplex constraint (TDM/FDM/SDM), needs to be taken into account when configuring and determining time domain resources for IAB Child link.

- Semi-static configuration of time domain resources is coordinated by the Donor CU by RRC signalling.

  - NR Rel-15 supports only three resource types: Downlink, Uplink, Flexible. It is foreseen that additional resource types need be supported for Rel-16 IAB scenario.

  - Additional resource types cover two flavours, hard and soft. Based on RAN1 agreement:

    - Hard: The corresponding time resource is always available for the DU child link

    - Soft: The availability of the corresponding time resource for the DU child link is explicitly and/or implicitly controlled by the parent node.

- Soft resources facilitate dynamic capacity adaptation of time resources between Parent and Child links. This can be done by means of DCI and/or higher layer configuration by parent node.

  - Soft resources are seen as Flexible resources from MT point of view

  - IAB-MT can determine the actual resource usage for flexible/soft time domain resources (defined by parent node) based on rules defined in Rel.15 (TS 38.213) and according to received DCI and/or higher layer configuration.

Slot configuration:

Existing slot configuration principle defined in NR Rel-15 can be a starting point also for IAB scenarios by reusing the signalling principles such layered structure (with TDD-UL-DL-ConfigurationCommon, TDD-UL-DL-ConfigurationDedicated, DCI format 2_0), adaptation periods, granularities etc. Configured flexible/soft resources will provide means to adapt to traffic requirements by enabling dynamic capacity allocation between DL/UL and between Parent/Child links, as needed.

Observation 1: Semi-static resource allocation can be done with existing RRC signalling for slot configuration

Observation 2: Flexible resources can be used to facilitate dynamic capacity allocation between DL/UL and between Parent/Child links.

Proposal 1: It is proposed to adopt the principles described above as the basis for IAB resource allocation and coordination.

## 2.2. Resource pool coordination

According to RAN1#93 agreements, distributed and centralized coordination mechanisms between IAB-donor and IAB-nodes should be studied. Resource pool coordination covers both frequency domain (with granularity of one PRB) and time domain (with granularity of one OFDM symbol). The semi-static coordination scheme depends on the architecture and protocol choices:

- IAB of Group 1 architecture operates according to centralized coordination according to example shown in Figure 2.

- IAB of Group 2 architecture may operate according to distributed coordination.

Parent node is responsible for DL/UL scheduling for the Parent BH link(s) within the configured resource pool (and according to the agreements made in RAN1#93). Parent node is responsible also for the dynamic resource pool adaptation between Parent and Child links.

According to example shown in Figure 2, CU makes the semi-static resource configuration separately for each IAB node

- A single resource configuration covers both parent and child links of the IAB node.

- Resource configuration defined in Rel-15 is enhanced by additional resource types (to be defined by RAN1)

- Parent node is responsible for allocating Flexible (soft) resources in the Parent BH link(s) in order to facilitate dynamic capacity allocation between DL/UL and between Parent and Child links.

When operating according to Group 1 architecture, resource configuration made for different IAB nodes must be consistent (e.g. w.r.t duplexing constraints).

- 3GPP needs to consider sufficient conflict resolution rules to manage possible configuration/scheduling errors made by the CU and/or Parent node.

- In order to avoid/mitigate conflicting situations due to dynamic adaptation, it makes sense to align flexible/soft resources across all links (parent, child, grand-child). Furthermore, it makes sense to have a hierarchical adaptation where the nodes down in the tree topology have to wait until all (grand-) parent nodes release the resources.

Figure 2. Resource configuration for group 1 architecture.

Observation 3: The semi-static resource pool coordination scheme (distributed, centralized) depends on the architecture and protocol choices.

Proposal 2: In the case of group 1 architecture, CU makes the semi-static resource configuration separately for each IAB node. A single resource configuration covers both parent and child links of the IAB node.

Proposal 3: For multi-hop scenarios, consider conflict resolution rules to manage possible configuration/scheduling errors made by the CU and/or Parent node.

# 3	Conclusions

In this contribution, we have discussed resource allocation aspects between Parent BH and Child links. The main question was how to define schedulable resources for IAB DU. We made following observations:

Observation 1: Semi-static resource allocation can be done with existing RRC signalling for slot configuration

Observation 2: Flexible resources can be used to facilitate dynamic capacity allocation between DL/UL and between Parent/Child links.

Observation 3: The semi-static resource pool coordination scheme (distributed, centralized) depends on the architecture and protocol choices.

Based on the discussion and observations above, we would propose:

Proposal 1: It is proposed to adopt the principles described above as the basis for IAB resource allocation and coordination.

Proposal 2: In the case of L2 architecture, CU makes the semi-static resource configuration separately for each IAB node. A single resource configuration covers both parent and child links of the IAB node.

Proposal 3: For multi-hop scenarios, consider conflict resolution rules to manage possible configuration/scheduling errors made by the CU and/or Parent node.

# References

- RP-172290, “Study on Integrated Access and Backhaul for NR”, AT&T, Qualcomm, Samsung

- 3GPP, “RAN1 Chairman’s Notes”, RAN1#93

- 3GPP, “RAN1 Chairman’s Notes”, RAN1#94

- 3GPP, “RAN1 Chairman’s Notes”, RAN1#94bis

# Appendix

The following agreements were related to resource allocation were made in RAN1#93 [2]:

Agreements:

- IAB supports TDM, FDM, and SDM between Access and BH links at an IAB node, subject to a half-duplex constraint. Further study the following solutions for the different multiplexing options:

  - Mechanisms for orthogonal partitioning of time slots or frequency resources between access and backhaul links across one or multiple hops

  - Utilization of different DL/UL slot configurations for access and backhaul links

  - DL and UL power control enhancements and timing requirements to allow for intra-panel FDM and SDM of backhaul and access links.

  - Interference management including cross-link interference

  - Note: the level of required enhancement or optimization for the different options is FFS

Agreements:

- Downlink IAB transmissions (transmissions from an IAB node to child IAB nodes and UEs directly under the IAB node) should be scheduled by the IAB node itself.

- Uplink IAB transmission (transmissions from an IAB node to its parent node) should be scheduled by the parent node.

- Semi-static (on the timescale of RRC signalling) should be supported for resource (frequency, time in terms of slot/slot format, etc.) coordination between IAB nodes.

- The following aspects should be further studied:

  - Distributed or centralized coordination mechanisms

  - Resource granularity of the required signalling (e.g. TDD configuration pattern)

  - Exchange of L1 and/or L3 measurements between IAB nodes

  - Exchange of topology related information (e.g. hop order) impacting RAN1 study

  - Resource (frequency, time in terms of slot/slot format, etc.) coordination which is faster than semi-static coordination.

The following agreements were related to resource allocation were made in RAN1#94 [3]:

Agreements:

Capture the following definitions in the TR:

- Access link: a link between an access UE and an IAB node or IAB donor (LA,DL or LA,UL)

- Backhaul link: a link between an IAB node and an IAB child node (LC,DL or LC,UL)or IAB parent node (LP,DL or LP,UL)

- Note: the IAB node may have its functions for UL access and child BH respectively in the same location or different locations, which is not the scope of RAN1 discussion

- Note: For a given BH link for an IAB node, it may be a parent BH or a child BH, depending on the topology/architecture, which is not the scope of RAN1 disucssion

Agreements:

An IAB node can support the following cases using existing MU-MIMO or sectorization mechanisms:

- Case A: multiplexing of DL transmissions to access UEs and child IAB nodes

- Case B: multiplexing of UL transmissions from access UEs and child IAB nodes

Agreements:

Clarify the SDM/FDM scenario definition:

- SDM/FDM Tx: An IAB node simultaneous transmits in the DL (to an access UE and/or child IAB node) and transmits in the UL (to a parent IAB node)

- SDM/FDM Rx: An IAB node simultaneous receives in the DL (a transmission from a parent node) and receives in the UL (from an access UE and/or child IAB node)

Agreements:

- At least Case #1 is supported for both access and backhaul link transmission timing.

- Further study includes additionally the following two cases (in addition to other cases #2/3/4/5)

  - Case #6 (Case#1 DL transmission timing + Case #2 UL transmission timing):

    - the DL transmission timing for all IAB nodes is aligned with the parent IAB node or donor DL timing (e.g. TA/2 adjustment as in Case #1)

    - the UL transmission timing of an IAB node can be aligned with the IAB node’s DL transmission timing

  - Case #7 (Case#1 DL transmission timing + Case #3 UL reception timing):

    - the DL transmission timing for all IAB nodes is aligned with the parent IAB node or donor DL timing (e.g. TA/2 adjustment as in Case #1)

    - the UL reception timing of an IAB node can be aligned with the IAB node’s DL reception timing

  - FFS: TA required for IAB nodes to support these cases

  - For Case #6 and Case #7 further consider the potential impact of imperfect timing adjustment, overhead of required DL/UL switching gaps, and scheduling impact on access UEs and child IAB nodes

  - Study to include aspects (including feasibility) when the IAB node is connected to one or multiple parent nodes

Agreements:

- For the support of TDM, at least the following cases are supported:

|  | TDM Between: |  |  |
| --- | --- | --- | --- |
| Case | Link 1 | Link 2 | Supported by a pattern? |
| 1 | LP,DL | LC,DL | Yes |
| 2 | LP,UL | LC,UL | Yes |
| 3 | LP,DL | LC,UL | Yes |
| 4 | LP,UL | LC,DL | Yes |
| 5 | LP,DL | LA,DL | Yes |
| 6 | LP,UL | LA,UL | Yes |
| 7 | LP,DL | LA,UL | Yes |
| 8 | LP,UL | LA,DL | Yes |
| 9 | LP,DL | LA,DL and LC,DL | Yes |
| 10 | LP,UL | LA,UL and LC,UL | Yes |
| 11 | LP,DL | LA,UL and LC,UL | Yes |
| 12 | LP,UL | LA,DL and LA,DL | Yes |
| 13 | LC,DL | LA,DL | * At least Rel. 15 mechanisms can be used, FFS enhancements |
| 14 | LC,UL | LA,UL | * At least Rel. 15 mechanisms can be used, FFS enhancements |
| 15 | LC,DL | LA,UL | * At least Rel. 15 mechanisms can be used, FFS enhancements |
| 16 | LC,UL | LA,DL | * At least Rel. 15 mechanisms can be used, FFS enhancements |

Note: A given pattern may include support for multiple cases, details FFS.

At least for Cases 1-12, an IAB node is configured with IAB-node specific resources in time available for the links:

- Further study details of the adaptation period and granularity (e.g. slot or symbol-level) of the pattern provided to the IAB node, including

  - Explicit or implicit indication of the resources

  - Enhancements to existing signaling mechanisms to indicate the pattern

- Further study the indication of resources within the configuration which can be dynamically and flexibly used for different links, including

  - The need to consider the scheduling delay, IAB node processing delays, or information required to be available for the use of flexible resources

  - Mechanisms to schedule flexible resources (e.g. GC-PDCCH)

The following agreements were related to resource allocation were made in RAN1#94bis [4]:

Agreements:

- From an MT point-of-view, the following time-domain resources can be indicated for the parent link:

- Downlink time resource,

- uplink time resource,

- flexible time resource

- as in release 15.

- From a DU point-of-view, the child link has the following types of time resources

- Downlink time resource,

- uplink time resource,

- flexible time resource

- not available time resources (not to be used for communication on the DU child links)

- Exact configuration for DU is FFS

Agreements:

- For each of the downlink, uplink and flexible time-resource types of the DU child link there are two flavors, hard and soft:

  - Hard: The corresponding time resource is always available for the DU child link

  - Soft: The availability of the corresponding time resource for the DU child link is explicitly and/or implicitly controlled by the parent node.