3GPP TSG CT Meeting #67	CP-150186

# Shanghai, P.R. China; 9th – 10th March 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.008 | CR | 2699 | Rev | - | Current version: | 12.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unsuccessful PDP context activation with #50 or #51 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, HTC, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, HTC, Samsung, TeliaSonera, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11 |  |  |  |  |  |  | Date: |  | 2015-02-25 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Revision 2 of the CR is submitted direct to plenary to fix some issues on the cover sheet of the CR in C1-150653 (in CP-150064), i.e., the Category and WI fields are corrected in order to show “A” and “TEI11”. Hence, the cover sheet of the CR correctly indicates that is a mirror to the agreed CR2725 in C1-150810 (to Rel-11). Present specification for PDP context activation with #50 or #51 prevents the MS implementation from initiating the PDP context activation procedure for the same APN in any equivalent PLMN (ePLMN) if the list of ePLMNs exists in the MS. This means that the causes #50 and #51 cut across ePLMNs.This can result in problem: - The user will not be able to get packet service even if a PLMN in the list of ePLMNs than the registered PLMN provides that service. - An operator is required that the PLMNs on a ePLMN list support exactly the same IP version configuration. That’s to say, an operator cannot make use of the ePLMN list feature or deploy a network with ePLMNs which have different support for IP stack, e.g., the PLMN A cannot just support IPv4 and the PLMN B cannot support IPv6. - Any MS implemention supporting Rel-8 to Rel-10 version of the specification is allowed to initiate the PDP context activation procedure but new Rel-11 MS won’t be able to do so. This results in two different user experiences depending on the scenario, and also operators may need to manage two different groups of MSes as long as legacy MSes are in use. CT1 has discussed this issue for long and there are two different views and some operators object to be hurt in their existing deployment for an issue which is not theirs (reject cause #50 or #51 sent for a PDP context activation request seems to be used in few network deployments). |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is an implementation option if the MS considers a new PLMN neither as the registered PLMN nor in the list of equivalent PLMNs. Similarly the reject cause #66 is proposed to be fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MS implementations won’t be able to get packet service even if a PLMN in the ePLMN list provides service. This results in negatively affecting the user experience of packet services. An operator is required that all PLMNs on a ePLMN list support exactly the same IP version configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

##### 6.1.3.1.3	Unsuccessful PDP context activation initiated by the MS

Upon receipt of an ACTIVATE PDP CONTEXT REQUEST message the network may reject the MS initiated PDP context activation by sending an ACTIVATE PDP CONTEXT REJECT message to the MS. The message shall contain a cause code that typically indicates one of the following causes:

# 8:	Operator Determined Barring;

# 26:	insufficient resources;

# 27:	missing or unknown APN;

# 28:	unknown PDP address or PDP type;

# 29:	user authentication failed;

# 30:	activation rejected by GGSN, Serving GW or PDN GW;

# 31:	activation rejected, unspecified;

# 32:	service option not supported;

# 33:	requested service option not subscribed;

# 34:	service option temporarily out of order;

# 35: NSAPI already used. The network shall not send this cause code (see note 1);

# 50:	PDP type IPv4 only allowed;

# 51:	PDP type IPv6 only allowed;

# 65:	maximum number of PDP contexts reached;

# 66:	requested APN not supported in current RAT and PLMN combination;

# 95 - 111:	protocol errors; or

#112:	APN restriction value incompatible with active PDP context.

NOTE 1:	Pre-R99 network may send this cause code.

If the SM cause value is #26 "insufficient resources" or #27 "missing or unknown APN", the network may include a value for timer T3396 in the ACTIVATE PDP CONTEXT REJECT message. If the SM cause value is #26 "insufficient resources" and if the request type in the ACTIVATE PDP CONTEXT REQUEST was set to "emergency", the network shall not include a value for timer T3396.

Upon receipt of an ACTIVATE PDP CONTEXT REJECT message, the MS shall stop timer T3380 and enter/remain in state PDP-INACTIVE.

If the ACTIVATE PDP CONTEXT REQUEST message was sent with request type set to "emergency" and the MS receives an ACTIVATE PDP CONTEXT REJECT message, then the MS may inform the upper layers of the failure to establish the emergency bearer.

NOTE:	This can result in the upper layers requesting establishment of a CS emergency call (if not already attempted in the CS domain) or initiating other implementation specific mechanisms, e.g. procedures specified in 3GPP TS 24.229 [95] can result in the emergency call being attempted to another IP-CAN.

If the SM cause value is #26 "insufficient resources" and T3396 value IE is included:

-	the MS shall take different actions depending on the timer value received for timer T3396 (if the MS is configured for dual priority, exceptions are specified in subclause 6.1.3.12):

i)	if the timer value of the timer T3396 indicates neither zero nor deactivated, the MS shall stop timer T3396 associated with the corresponding APN, if it is running. The MS shall then start timer T3396 with the value provided in the T3396 value IE and:

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message for the same APN that was sent by the MS, until timer T3396 expires, the timer T3396 is stopped, the MS is switched off or the SIM/USIM is removed; and

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message without an APN if the APN was not included in the ACTIVATE PDP CONTEXT REQUEST message, until timer T3396 expires, the MS is switched off or the SIM/USIM is removed;

ii)	if the timer value indicates that this timer is deactivated, the MS:

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message for the same APN that was sent by the MS, until the MS is switched off or the SIM/USIM is removed or the MS receives a REQUEST PDP CONTEXT ACTIVATION or REQUEST SECONDARY PDP CONTEXT ACTIVATION or MODIFY PDP CONTEXT REQUEST message with the same APN from the network; and

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message without an APN if the APN was not included in the ACTIVATE PDP CONTEXT REQUEST message, until the MS is switched off or the SIM/USIM is removed; and

iii)	if the timer value indicates that this timer is zero, the MS may send an ACTIVATE PDP CONTEXT REQUEST message for the same APN;

-	if the MS is switched off when the timer T3396 is running, the MS shall behave as follows when the MS is switched on:

-	let t1 be the time remaining for T3396 timeout at switch off and let t be the time elapsed between switch off and switch on. If t1 is greater than t, then the timer shall be restarted with the value t1 – t. If t1 is equal to or less than t, then the timer need not be restarted. If the MS is not capable of determining t, then the MS shall restart the timer with the value t1; and

-	if prior to switch off, timer T3396 was running because an ACTIVATE PDP CONTEXT REQUEST, ACTIVATE SECONDARY PDP CONTEXT REQUEST, MODIFY PDP CONTEXT REQUEST or ACTIVATE MBMS CONTEXT REQUEST message containing the low priority indicator set to "MS is configured for NAS signalling low priority" was rejected with timer T3396, and if timer T3396 is restarted at switch on, then the MS configured for dual priority shall handle session management requests as indicated in subclause 6.1.3.12.

If the SM cause value is #27 "missing or unknown APN" and T3396 value IE is included:

-	the MS shall take different actions depending on the timer value received for timer T3396:

i)	if the timer value of the timer T3396 indicates neither zero nor deactivated, the MS shall start timer T3396 and:

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message for the same APN that was sent by the MS, until timer T3396 expires, the MS is switched off or the SIM/USIM is removed; and

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message without an APN if the APN was not included in the ACTIVATE PDP CONTEXT REQUEST message, until timer T3396 expires, the MS is switched off or the SIM/USIM is removed;

ii)	if the timer value indicates that this timer is deactivated, the MS:

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message for the same APN that was sent by the MS, until the MS is switched off or the SIM/USIM is removed; and

-	shall not send another ACTIVATE PDP CONTEXT REQUEST message without an APN if the APN was not included in the ACTIVATE PDP CONTEXT REQUEST message, until the MS is switched off or the SIM/USIM is removed; and

iii)	if the timer value indicates that this timer is zero, the MS may send an ACTIVATE PDP CONTEXT REQUEST message for the same APN;

If the T3396 value IE is not included, the MS may send an ACTIVATE PDP CONTEXT REQUEST message for the same APN. If the APN was not included in the previous ACTIVATE PDP CONTEXT REQUEST message, the MS may send an ACTIVATE PDP CONTEXT REQUEST message without an APN.

The MS can access the network with access class 11 – 15 or initiate a PDP context activation procedure for emergency bearer services even if the timer T3396 is running.

If the SM cause value is #50 "PDP type IPv4 only allowed" or #51 "PDP type IPv6 only allowed", the MS shall not automatically send another ACTIVATE PDP CONTEXT REQUEST message for the same APN that was sent by the MS using the same PDP type, until:

-	the MS is registered to a new PLMN;

-	the PDP type which is used to access to the APN is changed;

-	the MS is switched off; or

-	the SIM/USIM is removed.

Furthermore as an implementation option, for the SM cause values #50 "PDP type IPv4 only allowed" and #51 "PDP type IPv6 only allowed", the MS may decide not to automatically send another ACTIVATE PDP CONTEXT REQUEST message for the same APN that was sent by the MS using the same PDP type, if the MS registered to a new PLMN which is in the list of equivalent PLMNs.

NOTE 1:	Request to send another ACTIVATE PDP CONTEXT REQUEST message with a specific PDP type has to come from upper layers.

If the SM cause value is #65 "maximum number of PDP contexts reached", the MS shall determine the PLMN's maximum number of PDP contexts in Iu or A/Gb mode (see subclause 6.1.3.0) as the number of active PDP contexts it has.

NOTE 2: 	In some situations, when attempting to establish multiple PDP contexts, the number of active PDP contexts that the UE has when SM cause #65 is received is not equal to the maximum number of PDP contexts reached in the network.

NOTE 3:	When the network supports emergency bearer services, it is not expected that SM cause #65 is returned by the network when the UE requests a PDP context for emergency bearer services.

The PLMN's maximum number of PDP contexts in A/Gb or Iu mode applies to the PLMN in which the SM cause #65 "maximum number of PDP contexts reached" is received. When the MS is switched off or when the USIM is removed, the MS shall clear all previous determinations representing any PLMN's maximum number of PDP contexts in Iu or A/Gb mode (see subclause 6.1.3.0). When the MS selects a new PLMN, the MS may clear previous determinations representing any PLMN's maximum number of PDP contexts in Iu or A/Gb mode.

If the SM cause value is #66 "requested APN not supported in current RAT and PLMN combination", the MS shall not send an ACTIVATE PDP CONTEXT REQUEST message for the same APN in the current PLMN in A/Gb or Iu mode until the MS is switched off or the SIM/USIM is removed. . Furthermore as an implementation option, the MS may decide not to automatically send another ACTIVATE PDP CONTEXT REQUEST message for the same APN in a PLMN which is in the list of equivalent PLMNs.