3GPP TSG CT Meeting #81	CP-182230

# Gold Coast, Australia; 10th – 11th September 2018								(was CP-182194)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0798 | rev | 4 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the control plane based SoR related EF and use only the EF-UST. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2018-09-10 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Steering of UE in VPLMN EF is defined to contain only one indication. That can be achieved in a less expensive way, by using just the service bit. The unnecessary EF read leads to slower USIM initialization as compared to earlier generations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the Steering of UE in VPLMN EF from 3GPP TS 31.102. - The text in other sections was enhanced to specify correctly the usage of the UST bit, that is, it is used to indicate that the UE is to receive the control plane based SoR information in the Registration Accept message during initial registration in a VPLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | USIM initialization time increases by the order of tens of milliseconds, thereby slowing the initial registration. Revision 3 (CT#81): In revision 2, by mistake the description of the case when service no 127 is “available” and the description of the case when service no 127 is not “available” in subclause 5.3.49 were the same. This is fixed in revision 3 by changing “the Registration Accept shall contain” to “the Registration Accept may contain” for the case when service no 127 is not “available”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8, 4.4.xx.1 (introduced in C6-180419), 4.4.xx.9 (introduced in C6-180419), 4.7, 5.3.49, Annex A, Annex E, Annex H.y (introduced in C6-180419). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | - This CR is a revision of the C6-180373 that was postponed from CT6#89-Bis. - Changes in this CR are made on top of the already agreed text from C6-180419 (in CT6#89-Bis). So, C6-180419 shall be implemented before implementing this CR. |  |  |  |  |  |  |  |  |

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n8 |  |  | M |  | 1 byte |
| 2 | Services n9 to n16 |  |  | O |  | 1 byte |
| 3 | Services n17 to n24 |  |  | O |  | 1 byte |
| 4 | Services n25 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n(8X-7) to n(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | … |  |
|  | Service n°126 | UAC Access Identities support |
|  | Service n°127 | Steering of UE in VPLMN |
|  | Service n°128 | Call control on PDU Session by USIM |

***** Next change *****

### 4.4.xx	Contents of files at the DF5GS level

#### 4.4.xx.1	Introduction

This clause describes the files that are specific for 5GS.

DF5GS shall be present at the ADFUSIM level if any of the following services are "available" in EFUST (USIM Service Table):

| Service n°122 | 5GS Mobility Management Information |
| --- | --- |
| Service n°123 | 5G Security Parameters |
| Service n°124 | Subscription identifier privacy support |
| Service n°125 | SUCI calculation by the USIM |
| Service n°126 | UAC Access Identities support |
| Service n°127 | Steering of UE in VPLMN |

…

-

***** Next change *****

## 4.7	Files of USIM

This clause contains two figures depicting the file structure of the UICC and the ADFUSIM. ADFUSIM shall be selected using the AID and information in EFDIR.

…

Figure 4.1: File identifiers and directory structures of UICC

|  | ADFUSIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | EFLI |  |  | EFARR |  |  |  |  | EFIMSI |  |  |  |  |  | EFKeys |  |  |  |  |  | EFKeysPS |  |  |  |
|  |  |  |  |  |  |  | '6F05' |  |  | '6F06' |  |  |  |  | '6F07' |  |  |  |  |  | '6F08' |  |  |  |  |  | '6F09' |  |  |  |
|  |  |  |  |  |  |  | EFDCK |  |  | EFHPPLMN |  |  |  |  |  | EFCNL |  |  |  |  |  | EFACMmax |  |  |  |  |  | EFUST |  |  |  |
|  |  |  |  |  |  |  | '6F2C' |  |  | '6F31' |  |  |  |  |  | '6F32' |  |  |  |  |  | '6F37' |  |  |  |  |  | '6F38' |  |  |  |
|  |  |  |  |  |  |  | EFACM |  |  | EFFDN |  |  |  |  |  | EFSMS |  |  |  |  |  | EFGID1 |  |  |  |  |  | EFGID2 |  |  |  |
|  |  |  |  |  |  |  | '6F39' |  |  | '6F3B' |  |  |  |  |  | '6F3C' |  |  |  |  |  | '6F3E' |  |  |  |  |  | '6F3F' |  |  |  |
|  |  |  |  |  |  |  | EFMSISDN |  |  | EFPUCT |  |  |  |  |  | EFSMSP |  |  |  |  |  | EFSMSS |  |  |  |  |  | EFCBMI |  |  |  |
|  |  |  |  |  |  |  | '6F40' |  |  | '6F41' |  |  |  |  |  | '6F42' |  |  |  |  |  | '6F43' |  |  |  |  |  | '6F45' |  |  |  |
|  |  |  |  |  |  |  | EFSPN |  |  | EFSMSR |  |  |  |  |  | EFCBMID |  |  |  |  |  | EFSDN |  |  |  |  |  | EFEXT2 |  |  |  |
|  |  |  |  |  |  |  | '6F46' |  |  | '6F47' |  |  |  |  |  | '6F48' |  |  |  |  |  | '6F49' |  |  |  |  |  | '6F4B' |  |  |  |
|  |  |  |  |  |  |  | EFEXT3 |  |  | EFBDN |  |  |  |  |  | EFEXT5 |  |  |  |  |  | EFCCP2 |  |  |  |  |  | EFCBMIR |  |  |  |
|  |  |  |  |  |  |  | '6F4C' |  |  | '6F4D' |  |  |  |  |  | '6F4E' |  |  |  |  |  | '6F4F' |  |  |  |  |  | '6F50' |  |  |  |
|  |  |  |  |  |  |  | EFEXT4 |  |  | EFEST |  |  |  |  |  | EFACL |  |  |  |  |  | EFCMI |  |  |  |  |  | EFSTART-HFN |  |  |  |
|  |  |  |  |  |  |  | '6F55' |  |  | '6F56' |  |  |  |  |  | '6F57' |  |  |  |  |  | '6F58' |  |  |  |  |  | '6F5B' |  |  |  |
|  |  |  |  |  |  |  | EFTHRESHOLD |  |  | EFPLMNwAcT |  |  |  |  |  | EFOPLMNwAcT |  |  |  |  |  | EFHPLMNwAcT |  |  |  |  |  | EFPSLOCI |  |  |  |
|  |  |  |  |  |  |  | '6F5C' |  |  | '6F60' |  |  |  |  |  | '6F61' |  |  |  |  |  | '6F62' |  |  |  |  |  | '6F73' |  |  |  |
|  |  |  |  |  |  |  | EFACC |  |  | EFFPLMN |  |  |  |  |  | EFLOCI |  |  |  |  |  | EFICI |  |  |  |  |  | EFOCI |  |  |  |
|  |  |  |  |  |  |  | '6F78' |  |  | '6F7B' |  |  |  |  |  | '6F7E' |  |  |  |  |  | '6F80' |  |  |  |  |  | '6F81' |  |  |  |
|  |  |  |  |  |  |  | EFICT |  |  | EFOCT |  |  |  |  |  | EFAD |  |  |  |  |  | EFVGCS |  |  |  |  |  | EFVGCSS |  |  |  |
|  |  |  |  |  |  |  | '6F82' |  |  | '6F83' |  |  |  |  |  | '6FAD' |  |  |  |  |  | '6FB1' |  |  |  |  |  | '6FB2' |  |  |  |
|  |  |  |  |  |  |  | EFVBS |  |  | EFVBSS |  |  |  |  |  | EFeMLPP |  |  |  |  |  | EFAaeM |  |  |  |  |  | EFECC |  |  |  |
|  |  |  |  |  |  |  | '6FB3' |  |  | '6FB4' |  |  |  |  |  | '6FB5' |  |  |  |  |  | '6FB6' |  |  |  |  |  | '6FB7' |  |  |  |
|  |  |  |  |  |  |  | EFHiddenkey |  |  | EFNETPAR |  |  |  |  |  | EFPNN |  |  |  |  |  | EFOPL |  |  |  |  |  | EFMBDN |  |  |  |
|  |  |  |  |  |  |  | '6FC3' |  |  | '6FC4' |  |  |  |  |  | '6FC5' |  |  |  |  |  | '6FC6' |  |  |  |  |  | '6FC7' |  |  |  |
|  |  |  |  |  |  |  | EFEXT6 |  |  | EFMBI |  |  |  |  |  | EFMWIS |  |  |  |  |  | EFCFIS |  |  |  |  |  | EFEXT7 |  |  |  |
|  |  |  |  |  |  |  | '6FC8' |  |  | '6FC9' |  |  |  |  |  | '6FCA' |  |  |  |  |  | '6FCB' |  |  |  |  |  | '6FCC' |  |  |  |
|  |  |  |  |  |  |  | EFSPDI |  |  | EFMMSN |  |  |  |  |  | EFEXT8 |  |  |  |  |  | EFMMSICP |  |  |  |  |  | EFMMSUP |  |  |  |
|  |  |  |  |  |  |  | '6FCD' |  |  | '6FCE' |  |  |  |  |  | '6FCF' |  |  |  |  |  | '6FD0' |  |  |  |  |  | '6FD1' |  |  |  |
|  |  |  |  |  |  |  | EFMMSUCP |  |  | EFNIA |  |  |  |  |  | EFVGCSCA |  |  |  |  |  | EFVBSCA |  |  |  |  |  | EFGBAP |  |  |  |
|  |  |  |  |  |  |  | '6FD2' |  |  | '6FD3' |  |  |  |  |  | '6FD4' |  |  |  |  |  | '6FD5' |  |  |  |  |  | '6FD6' |  |  |  |
|  |  |  |  |  |  |  | EFMSK |  |  | EFMUK |  |  |  |  |  | EFEHPLMN |  |  |  |  |  | EFGBANL |  |  |  |  |  | EFEHPLMNPI |  |  |  |
|  |  |  |  |  |  |  | '6FD7' |  |  | '6FD8' |  |  |  |  |  | '6FD9' |  |  |  |  |  | '6FDA' |  |  |  |  |  | '6FDB' |  |  |  |
|  |  |  |  |  |  |  | EFLRPLMNSI |  |  | EFNAFKCA |  |  |  |  |  | EFSPNI |  |  |  |  |  | EFPNNI |  |  |  |  |  | EFNCP-IP |  |  |  |
|  |  |  |  |  |  |  | '6FDC' |  |  | '6FDD' |  |  |  |  |  | '6FDE' |  |  |  |  |  | '6FDF' |  |  |  |  |  | '6FE2' |  |  |  |
|  |  |  |  |  |  |  | EFEPSLOCI |  |  | EFEPSNSC |  |  |  |  |  | EFUFC |  |  |  |  |  | EFUICCIARI |  |  |  |  |  | EFNASCONFIG |  |  |  |
|  |  |  |  |  |  |  | '6FE3' |  |  | '6FE4' |  |  |  |  |  | '6FE6' |  |  |  |  |  | '6FE7' |  |  |  |  |  | '6FE8' |  |  |  |
|  |  |  |  |  |  |  | EFPWS |  |  | EFFDNURI |  |  |  |  |  | EFBDNURI |  |  |  |  |  | EFSDNURI |  |  |  |  |  | EFIWL |  |  |  |
|  |  |  |  |  |  |  | '6FEC' |  |  | '6FED' |  |  |  |  |  | '6FEE' |  |  |  |  |  | '6FEF' |  |  |  |  |  | '6FF0' |  |  |  |
|  |  |  |  |  |  |  | EFIPS |  |  | EFIPD |  |  |  |  |  | EFePDGId |  |  |  |  |  | EFePDGSelection |  |  |  |  |  | EFePDGIdEm |  |  |  |
|  |  |  |  |  |  |  | '6FF1' |  |  | '6FF2' |  |  |  |  |  | '6FF3' |  |  |  |  |  | '6FF4' |  |  |  |  |  | '6FF5' |  |  |  |
|  |  |  |  |  |  |  | EFePDGSelectionEm |  |  | EFFromPreferred |  |  |  |  |  | EFIMSConfigData |  |  |  |  |  | EF3GPPPSDATAOFF |  |  |  |  |  | EF3GPPPSDATAOFFservicelist |  |  |  |
|  |  |  |  |  |  |  | '6FF6' |  |  | '6FF7' |  |  |  |  |  | '6FF8' |  |  |  |  |  | '6FF9' |  |  |  |  |  | '6FFA' |  |  |  |
|  |  |  |  |  |  |  | EFTVCONFIG |  |  | EFXCAPConfigData |  |  |  |  |  | EFEARFCNList |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '6FFB' |  |  | '6FFC' |  |  |  |  |  | '6FFD' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFPHONEBOOK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F3A' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFPSC |  |  | EFCC |  |  |  |  |  | EFPUID |  |  |  |  |  | EFPBR |  |  |  |  |  | EFUID |  |  |  |
|  |  |  |  |  |  |  | '4F22' |  |  | '4F23' |  |  |  |  |  | '4F24' |  |  |  |  |  | '4F30' |  |  |  |  |  | '4FXX' |  |  |  |
|  |  |  |  |  |  |  | EFCCP1 |  |  | EFIAP |  |  |  |  |  | EFADN |  |  |  |  |  | EFEXT1 |  |  |  |  |  | EFPBC |  |  |  |
|  |  |  |  |  |  |  | '4FXX' |  |  | '4FXX' |  |  |  |  |  | '4FXX' |  |  |  |  |  | '4FXX' |  |  |  |  |  | '4FXX' |  |  |  |
|  |  |  |  |  |  |  | EFGRP |  |  | EFAAS |  |  |  |  |  | EFGAS |  |  |  |  |  | EFANR |  |  |  |  |  | EFSNE |  |  |  |
|  |  |  |  |  |  |  | '4FXX' |  |  | '4FXX' |  |  |  |  |  | '4FXX' |  |  |  |  |  | '4FXX' |  |  |  |  |  | '4FXX' |  |  |  |
|  |  |  |  |  |  |  | EFEMAIL |  |  | EFPURI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4FXX' |  |  | '4FXX' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFGSM-ACCESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F3B' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFKc |  |  | EFKcGPRS |  |  |  |  |  | EFCPBCCH |  |  |  |  |  | EFinvSCAN |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F20' |  |  | '4F52' |  |  |  |  |  | '4F63' |  |  |  |  |  | '4F64' |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFMexE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F3C' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFMexE-ST |  |  | EFORPK |  |  |  |  |  | EFARPK |  |  |  |  |  | EFTPRK |  |  |  |  |  | EFTKCDF |  |  |  |
|  |  |  |  |  |  |  | '4F40' |  |  | '4F41' |  |  |  |  |  | '4F42' |  |  |  |  |  | '4F43' |  |  |  |  |  | '4FXX' |  |  |  |
|  |  |  |  | DFSoLSA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F70' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFSAI |  |  | EFSLL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F30' |  |  | '4F31' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFWLAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F40' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFPseudo |  |  | EFUPLMNWLAN |  |  |  |  |  | EF0PLMNWLAN |  |  |  |  |  | EFUWSIDL |  |  |  |  |  | EFOWSIDL |  |  |  |
|  |  |  |  |  |  |  | '4F41' |  |  | '4F42' |  |  |  |  |  | '4F43' |  |  |  |  |  | '4F44' |  |  |  |  |  | '4F45' |  |  |  |
|  |  |  |  |  |  |  | EFWRI |  |  | EFHWSIDL |  |  |  |  |  | EFWEHPLMNPI |  |  |  |  |  | EFWHPI |  |  |  |  |  | EFWLRPLMN |  |  |  |
|  |  |  |  |  |  |  | '4F46' |  |  | '4F47' |  |  |  |  |  | '4F48' |  |  |  |  |  | '4F49' |  |  |  |  |  | '4F4A' |  |  |  |
|  |  |  |  |  |  |  | EFHPLMNDAI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F4B' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFHNB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F50' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFACSGL |  |  | EFCSGT |  |  |  |  |  | EFHNBN |  |  |  |  |  | EFOCSGL |  |  |  |  |  | EFOCSGT |  |  |  |
|  |  |  |  |  |  |  | '4F81' |  |  | '4F82' |  |  |  |  |  | '4F83' |  |  |  |  |  | '4F84' |  |  |  |  |  | '4F85' |  |  |  |
|  |  |  |  |  |  |  | EFOHNBN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F86' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFProSe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5F90' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFPROSE_MON |  |  | EFPROSE_ANN |  |  |  |  |  | EFPROSEFUNC |  |  |  |  |  | EFPROSE_RADIO_COM |  |  |  |  |  | EFPROSE_RADIO_MON |  |  |  |
|  |  |  |  |  |  |  | '4F01' |  |  | '4F02' |  |  |  |  |  | '4F03' |  |  |  |  |  | '4F04' |  |  |  |  |  | '4F05' |  |  |  |
|  |  |  |  |  |  |  | EFPROSE_RADIO_ANN |  |  | EFPROSE_POLICY |  |  |  |  |  | EFPROSE_PLMN |  |  |  |  |  | EFPROSE_GC |  |  |  |  |  | EFPST |  |  |  |
|  |  |  |  |  |  |  | '4F06' |  |  | '4F07' |  |  |  |  |  | '4F08' |  |  |  |  |  | '4F09' |  |  |  |  |  | '4F10' |  |  |  |
|  |  |  |  |  |  |  | EFPROSE_UIRC |  |  | EFPROSE_GM_DISCOVERY |  |  |  |  |  | EFPROSE_RELAY |  |  |  |  |  | EFPROSE_RELAY_DISCOVERY |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F11' |  |  | '4F12' |  |  |  |  |  | '4F13' |  |  |  |  |  | '4F14' |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFACDC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5FA0' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFACDC_LIST |  |  | EFACDC_OS_CONFIG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F01' |  |  | '4FXX' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DFTV '5FB0' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EFTVUSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4FXX' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DF5GS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | '5FC0' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EF5GS3GPPLOCI |  |  | EF5GSN3GPPLOCI |  |  |  |  |  | EF5GS3GPPNSC |  |  |  |  |  | EF5GSN3GPPNSC |  |  |  |  |  | EF5GAUTHKEYS |  |  |  |
|  |  |  |  |  |  |  | '4F01' |  |  | '4F02' |  |  |  |  |  | '4F03' |  |  |  |  |  | '4F04' |  |  |  |  |  | '4F05' |  |  |  |
|  |  |  |  |  |  |  | EFUAC_AIC |  |  | EFSUCI_Calc_Info |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '4F06' |  |  | '4F07' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Figure 4.2: File identifiers and directory structures of USIM

NOTE 5:	The value '6F65' under ADFUSIM was used in earlier versions of this specification, and should not be re-assigned in future versions.

***** Next change *****

### 5.3.49	Control plane-based Steering of Roaming related procedures

If service n°127 is "available" in the USIM Service Table, the Registration Accept shall contain control plane based Steering of Roaming information during the initial registration in a VPLMN.

If service n°127 is not "available" in the USIM Service Table, the Registration Accept may contain control plane based Steering of Roaming information during the initial registration in a VPLMN.

The control plane-based Steering of Roaming procedure and the related information from the HPLMN are specified in 3GPP TS 23.122 [31].

***** Next change *****

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification |  | Description |  | Change advised |  |
| --- | --- | --- | --- | --- | --- |
| '2F00' |  | Application directory |  | Caution |  |
| '2F05' |  | Preferred languages |  | Yes |  |
| … |  |  |  |  |  |
| '4F07' |  | ProSe Policy Parameters |  | Yes |  |
| '4F07' |  | Subscriber Concealed Identifier Calculation Information |  | Yes |  |
| '4F08' |  | ProSe PLMN Parameters |  | Yes |  |
| '4F09' |  | ProSe Group Counter |  | No |  |
| … |  |  |  |  |  |
| '6FFC' |  | XCAP Configuration Data |  | Yes |  |
| '6FFD' |  | EARFCN List for MTC/NB-IOT UEs |  | Yes |  |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, EFEPSLOCI, EF5GS3GPPLOCI and EF5GS3GPPLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. |  |  |  |  |  |

***** Next change *****

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification |  | Description |  | Value |  |
| --- | --- | --- | --- | --- | --- |
| '2F00' |  | Application directory |  | Card issuer/operator dependent |  |
| '2F05' |  | Preferred languages |  | 'FF…FF' |  |
| '2F06' |  | Access rule reference |  | Card issuer/operator dependent |  |
| … |  |  |  |  |  |
| '4F06' |  | UAC Access Identities Configuration |  | Operator dependent |  |
| '4F07' |  | ProSe Policy Parameters |  | Operator dependent |  |
| '4F07' |  | Subscriber Concealed Identifier Calculation Information |  | Operator dependent |  |
| '4F08' |  | ProSe PLMN Parameters |  | Operator dependent |  |
| '4F09' |  | ProSe Group Counter |  | 'FF…FF' |  |
| … |  |  |  |  |  |
| '6FFC’ |  | XCAP Configuration Data |  | Operator dependent |  |
| '6FFD' |  | EARFCN list for MTC/NB-IOT UEs |  | Operator dependent |  |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxxxxx stands for any valid MCC and MNC, coded according to TS 24.008 [9].

***** Next change *****

# H.y	List of SFI Values at the DF 5GS Level

| File Identification | SFI | Description |
| --- | --- | --- |
| '4F01' | '01' | 5GS 3GPP location information (Note) |
| '4F02' | '02' | 5GS non-3GPP location information (Note) |
| '4F03' | '03' | 5GS 3GPP Access NAS Security Context |
| '4F04' | '04' | 5GS non-3GPP Access NAS Security Context |
| '4F05' | '05' | 5G authentication keys |
| '4F06' | '06' | UAC Access Identities Configuration |
| '4F07' | '07' | Subscriber Concealed Identifier Calculation Information |
| NOTE: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EF5GS3GPPLOCI and EF5GSN3GPPLOCI accordingly. |  |  |

All other SFI values are reserved for future use.