3GPP TSG-CT Meeting #82	CP-183171

# Sorrento, Italy, 10 - 11 Decemebr 2018										Revision of C4-188278

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0107 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UDM group Id |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2018-12-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | udmGroupId, udrGroupId and ausfGroupId attributes are used in various APIs and should use a common type NfGroupId. Revision 1: The CR was not marked as Agreed by mistake during CT4#87. NfGroup is corrected to NfGroupId in subclause 6.2.3.2.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the data type used for the UDM, UDR and AUSF Group ID in the NRF APIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent data types used for the UDM, UDR and AUSF Group ID in 5GC APIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.6, 6.1.6.2.7, 6.1.6.2.8, 6.2.3.2.3.1, 6.2.6.1, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 29.571 CR 0062 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.6.1	General

This subclause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnrf service based interface protocol.

Table 6.1.6.1-1: Nnrf_NFManagement specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| NFProfile | 6.1.6.2.2 |  |
| NFService | 6.1.6.2.3 |  |
| DefaultNotificationSubscription | 6.1.6.2.4 | Data structure for specifying the notifications the NF service subscribes by default along with callback URI. |
| IpEndPoint | 6.1.6.2.5 |  |
| UdrInfo | 6.1.6.2.6 |  |
| UdmInfo | 6.1.6.2.7 |  |
| AusfInfo | 6.1.6.2.8 |  |
| SupiRange | 6.1.6.2.9 |  |
| IdentityRange | 6.1.6.2.10 |  |
| AmfInfo | 6.1.6.2.11 |  |
| SmfInfo | 6.1.6.2.12 |  |
| UpfInfo | 6.1.6.2.13 | Information related to UPF |
| SnssaiUpfInfoItem | 6.1.6.2.14 |  |
| DnnUpfInfoItem | 6.1.6.2.15 |  |
| SubscriptionData | 6.1.6.2.16 |  |
| NotificationData | 6.1.6.2.17 |  |
| NFRegistrationData | 6.1.6.2.18 |  |
| NFServiceVersion | 6.1.6.2.19 | Contains the version details of an NF service. |
| PcfInfo | 6.1.6.2.20 |  |
| BsfInfo | 6.1.6.2.21 |  |
| Ipv4AddressRange | 6.1.6.2.22 |  |
| Ipv6PrefixRange | 6.1.6.2.23 |  |
| InterfaceUpfInfoItem | 6.1.6.2.24 |  |
| UriList | 6.1.6.2.25 |  |
| N2InterfaceAmfInfo | 6.1.6.2.26 | AMF N2 interface information |
| TaiRange | 6.1.6.2.27 |  |
| TacRange | 6.1.6.2.28 |  |
| Fqdn | 6.1.6.3.2 |  |
| NFType | 6.1.6.3.3 |  |
| NotificationType | 6.1.6.3.4 |  |
| TransportProtocol | 6.1.6.3.5 |  |
| NotificationEventType | 6.1.6.3.6 |  |
| NFStatus | 6.1.6.3.7 |  |
| DataSetId | 6.1.6.3.8 |  |
| UPInterfaceType | 6.1.6.3.9 |  |
| ServiceName | 6.1.6.3.11 |  |
| NFServiceStatus | 6.1.6.3.12 |  |

Table 6.1.6.1-2 specifies data types re-used by the Nnrf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf service based interface.

Table 6.1.6.1-2: Nnrf_NFManagement re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| N1MessageClass | 3GPP TS 29.518 [6] | The N1 message type |
| N2InformationClass | 3GPP TS 29.518 [6] | The N2 information type |
| IPv4Addr | 3GPP TS 29.571 [7] |  |
| IPv6Addr | 3GPP TS 29.571 [7] |  |
| IPv6Prefix | 3GPP TS 29.571 [7] |  |
| Uri | 3GPP TS 29.571 [7] |  |
| Dnn | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| Snssai | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] |  |
| Guami | 3GPP TS 29.571 [7] |  |
| Tai | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| LinksValueSchema | 3GPP TS 29.571 [7] | 3GPP Hypermedia link |
| UriScheme | 3GPP TS 29.571 [7] |  |
| AmfName | 3GPP TS 29.571 [7] |  |
| DateTime | 3GPP TS 29.571 [7] |  |
| NfGroupId | 3GPP TS 29.571 [7] | Network Function Group Id |

* * * Next Change * * * *

##### 6.1.6.2.6	Type: UdrInfo

Table 6.1.6.2.6-1: Definition of type UdrInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| groupId | NfGroupId | O | 0..1 | Identity of the UDR group that is served by the UDR instance |
| supiRanges | array(SupiRange) | O | 0..N | List of ranges of SUPI's whose profile data is available in the UDR instance (NOTE 1) |
| gpsiRanges | array(IdentityRange) | O | 0..N | List of ranges of GPSIs whose profile data is available in the UDR instance (NOTE 1) |
| externalGroupIdentifiersRanges | array(IdentityRange) | O | 0..N | List of ranges of external groups whose profile data is available in the UDR instance (NOTE 1) |
| supportedDataSets | array(DataSetId) | O | 0..N | List of supported data sets in the UDR instance |
| NOTE 1: If none of these parameters is provided, the UDR can serve any external group and any SUPI or GPSI. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.7	Type: UdmInfo

Table 6.1.6.2.7-1: Definition of type UdmInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| groupId | NfGroupId | O | 0..1 | Identity of the UDM group that is served by the UDM instance |
| supiRanges | array(SupiRange) | O | 0..N | List of ranges of SUPIs whose profile data is available in the UDM instance (NOTE 1) |
| gpsiRanges | array(IdentityRange) | O | 0..N | List of ranges of GPSIs whose profile data is available in the UDM instance (NOTE 1) |
| externalGroupIdentifiersRanges | array(IdentityRange) | O | 0..N | List of ranges of external groups whose profile data is available in the UDM instance (NOTE 1) |
| routingIndicators | array(string) | O | 0..N | List of Routing Indicator information that allows to route network signalling with SUCI (see 3GPP TS 23.003 [12]) to the UDM instance. |
| NOTE 1: If none of these parameters is provided, the UDM can serve any external group and any SUPI or GPSI. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.8	Type: AusfInfo

Table 6.1.6.2.8-1: Definition of type AusfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| groupId | NfGroupId | O | 0..1 | Identity of the AUSF group |
| supiRanges | array(SupiRange) | O | 0..N | List of ranges of SUPIs that can be served by the AUSF instance. If not provided, the AUSF can serve any SUPI. |
| routingIndicators | array(string) | O | 0..N | List of Routing Indicator information that allows to route network signalling with SUCI (see 3GPP TS 23.003 [12]) to the AUSF instance. |

* * * Next Change * * * *

###### 6.2.3.2.3.1	GET

This operation retrieves a list of NF Instances, and their offered services, currently registered in the NRF, satisfying a number of filter criteria, such as those NF Instances offering a certain service name, or those NF Instances of a given NF type (e.g., AMF).

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| target-nf-type | NFType | M | 1 | This IE shall contain the NF type of the NF Service Producer being discovered. |
| requester-nf-type | NFType | M | 1 | This IE shall contain the NF type of the NF Service Consumer that is invoking the Nnrf_NFDiscovery service. |
| service-names | array(string) | O | 1..N | If included, this IE shall contain an array of service names for which the NRF is queried to provide the list of NF profiles. The NRF shall return the NF profiles that have at least one NF service matching the NF service names in this list. The NF service names returned by the NRF shall be an intersection of the NF service names requested and the NF service names registered in the NF profile. If not included, the NRF shall return all the NF service names registered in the NF profile. |
| requester-nf-instance-fqdn | Fqdn | O | 0..1 | If included, this IE shall contain the FQDN of the NF Service Consumer that is invoking the Nnrf_NFDiscovery service. The NRF shall use this to return only those NF profiles that include at least one NF service containing an entry in the "allowedNfDomains" list (see subclause 6.1.6.2.3) that matches the domain of the requester NF. |
| target-plmn | PlmnId | C | 0..1 | This IE shall be included when NF services in a different PLMN need to be discovered. When included, this IE shall contain the PLMN ID of the different PLMN. |
| requester-plmn | PlmnId | C | 0..1 | This IE shall be included when NF services in a different PLMN need to be discovered. When included, this IE shall contain the PLMN ID of the requester NF. |
| target-nf-instance-id | NfInstanceId | O | 0..1 | Identity of the NF instance being discovered. |
| target-nf-fqdn | Fqdn | O | 0..1 | FQDN of the target NF instance being discovered. |
| hnrf-uri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFDiscovery Service (see subclause 6.2.1) of the home NRF. It shall be included if the NF Service Consumer has previously received such API URI to be used for service discovery (e.g., from the NSSF in the home PLMN). |
| snssais | array(Snssai) | O | 0..N | If included, this IE shall contain the list of S-NSSAI that are served by the services being discovered. The NRF shall use this to identify the NF services that have registered their support for these S-NSSAIs. The NRF shall return the NF profiles that have at least one S-NSSAI matching the S-NSSAIs in this list. The S-NSSAIs included in the NF services returned by the NRF shall be an intersection of the S-NSSAIs requested and the S-NSSAIs registered in the NF profile. |
| nsi-list | array(string) | O | 0..N | If included, this IE shall contain the list of NSI IDs that are served by the services being discovered. |
| dnn | Dnn | O | 0..1 | If included, this IE shall contain the DNN for which NF services serving that DNN is discovered. DNN may be included if the target NF type is "SMF" or "UPF". |
| smf-serving-area | string | O | 0..1 | If included, this IE shall contain the serving area of the SMF. It may be included if the target NF type is "UPF". |
| tai | Tai | O | 0..1 | Tracking Area Identity. |
| amf-region-id | string | O | 0..1 | AMF Region Identity. |
| amf-set-id | string | O | 0..1 | AMF Set Identity. |
| guami | Guami | O | 0..1 | Guami used to search for an appropriate AMF. (NOTE 1) |
| supi | Supi | O | 0..1 | If included, this IE shall contain the SUPI of the requester UE to search for an appropriate NF. SUPI may be included if the target NF type is e.g. "PCF", "AUSF", "UDM" or "UDR". |
| ue-ipv4-address | Ipv4Addr | O | 0..1 | The IPv4 address of the UE for which a BSF needs to be discovered. |
| ue-ipv6-prefix | Ipv6Prefix | O | 0..1 | The IPv6 prefix of the UE for which a BSF needs to be discovered. |
| pgw-ind | boolean | O | 0..1 | When present, this IE indicates whether a combined SMF/PGW-C or a standalone SMF needs to be discovered. true: A combined SMF/PGW-C is requested to be discovered; false: A standalone SMF is requested to be discovered. (See NOTE 2) |
| pgw | Fqdn | O | 0..1 | If included, this IE shall contain the PGW FQDN which is received by the AMF from the MME to find the combined SMF/PGW. |
| gpsi | Gpsi | O | 0..1 | If included, this IE shall contain the GPSI of the requester UE to search for an appropriate NF. GPSI may be included if the target NF type is "UDM" or "UDR". |
| external-group-identity | GroupId | O | 0..1 | If included, this IE shall contain the external group identifier of the requester UE to search for an appropriate NF. This may be included if the target NF type is "UDM" or "UDR". |
| data-set | DataSetId | O | 0..1 | Indicates the data set to be supported by the NF to be discovered. May be included if the target NF type is "UDR". |
| routing-indicator | string | O | 0..1 | Routing Indicator information that allows to route network signalling with SUCI (see 3GPP TS 23.003 [12]) to an AUSF and UDM instance capable to serve the subscriber. May be included if the target NF type is "AUSF" or "UDM". |
| group-id-list | array(NfGroupId) | O | 1..N | Identity of the group(s) of the NFs of the target NF type to be discovered. May be included if the target NF type is "UDR", "UDM" or "AUSF". |
| supported-features | SupportedFeatures | O | 0..1 | List of features required to be supported by the target Network Function. |
| NOTE 1: If this parameter is present and no AMF supporting the requested GUAMI is available due to AMF Failure or planned AMF removal, the NRF shall return in the response AMF instances acting as a backup for AMF failure or planned AMF removal respectively for this GUAMI. The NRF can detect if an AMF has failed, using the Heartbeat procedure. The NRF will receive a de-registration request from an AMF performing a planned removal. NOTE 2: If the combined SMF/PGW-C is requested to be discovered, the NRF shall return in the response the SMF instances registered with the SmfInfo containing pgwFqdn. |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P |  | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- | --- |
| SearchResult | M |  | 1 | 200 OK | The response body contains the result of the search over the list of registered NF Instances. |
| n/a |  |  |  | 307 Temporary Redirect | The response shall be used when the intermediate NRF redirects the service discovery request. |
| ProblemDetails | M |  | 1 | 400 Bad Request | The response body contains the error reason of the request message. |
| ProblemDetails |  | M | 1 | 403 Forbidden | This response shall be returned if the NF Service Consumer is not allowed to discover the NF Service(s) being queried. |
| ProblemDetails | M |  | 1 | 500 Internal Server Error | The response body contains the error reason of the request message. |

* * * Next Change * * * *

#### 6.2.6.1	General

This subclause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nnrf service based interface protocol.

Table 6.2.6.1-1: Nnrf_NFDiscovery specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| SearchResult | 6.2.6.2.2 |  |
| NFProfile | 6.2.6.2.3 |  |
| NFService | 6.2.6.2.4 |  |

Table 6.2.6.1-2 specifies data types re-used by the Nnrf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf service based interface.

Table 6.2.6.1-2: Nnrf_NFDiscovery re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] |  |
| Dnn | 3GPP TS 29.571 [7] |  |
| Tai | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| Uri | 3GPP TS 29.571 [7] |  |
| Gpsi | 3GPP TS 29.571 [7] |  |
| GroupId | 3GPP TS 29.571 [7] |  |
| Guami | 3GPP TS 29.571 [7] |  |
| IPv4Addr | 3GPP TS 29.571 [7] |  |
| IPv6Addr | 3GPP TS 29.571 [7] |  |
| UriScheme | 3GPP TS 29.571 [7] |  |
| DefaultNotificationSubscription | 3GPP TS 29.510 | See clause 6.1.6.2.4 |
| IPEndPoint | 3GPP TS 29.510 | See clause 6.1.6.2.5 |
| NFType | 3GPP TS 29.510 | See clause 6.1.6.3.3 |
| UdrInfo | 3GPP TS 29.510 | See clause 6.1.6.2.6 |
| UdmInfo | 3GPP TS 29.510 | See clause 6.1.6.2.7 |
| AusfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.8 |
| SupiRange | 3GPP TS 29.510 | See clause 6.1.6.2.9 |
| AmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.11 |
| SmfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.12 |
| UpfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.13 |
| PcfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.20 |
| BsfInfo | 3GPP TS 29.510 | See clause 6.1.6.2.21 |
| NFServiceVersion | 3GPP TS 29.510 | See clause 6.1.6.2.19 |
| NFStatus | 3GPP TS 29.510 | See clause 6.1.6.3.7 |
| DataSetId | 3GPP TS 29.510 | See clause 6.1.6.3.8 |
| ServiceName | 3GPP TS 29.510 | See clause 6.1.6.3.11 |
| NFServiceStatus | 3GPP TS 29.510 | See clause 6.1.6.3.12 |
| NfGroupId | 3GPP TS 29.571 [7] | Identifier of a NF Group |

* * * Next Change * * * *

## A.2	Nnrf_NFManagement API

openapi: 3.0.0

info:

[…]

UdrInfo:

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

externalGroupIdentityfiersRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

supportedDataSets:

type: array

items:

$ref: '#/components/schemas/DataSetId'

[…]

UdmInfo:

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

externalGroupIdentityfiersRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

routingIndicators:

type: array

items:

type: string

AusfInfo:

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

routingIndicators:

type: array

items:

type: string

* * * Next Change * * * *

## A.3	Nnrf_NFDiscovery API

openapi: 3.0.0

[…]

paths:

/nf-instances:

get:

summary: Search a collection of NF Instances

operationId: SearchNFInstances

tags:

- NF Instances (Store)

parameters:

- name: target-nf-type

in: query

description: Type of the target NF

required: true

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- name: requester-nf-type

in: query

description: Type of the requester NF

required: true

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- name: service-names

in: query

description: Name of the service offered by the NF

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: requester-nf-instance-fqdn

in: query

description: FQDN of the requester NF

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

- name: target-plmn

in: query

description: Id of the PLMN where the target NF is located

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: requester-plmn

in: query

description: Id of the PLMN where the NF issuing the Discovery request is located

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: target-nf-instance-id

in: query

description: Identity of the NF instance being discovered

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: target-nf-fqdn

in: query

description: FQDN of the NF instance being discovered

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

- name: hnrf-uri

in: query

description: Uri of the home NRF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

- name: snssais

in: query

description: Slice info of the target NF

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: dnn

in: query

description: Dnn supported by the SMF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: nsi-list

in: query

description: NSI IDs that are served by the services being discovered

schema:

type: array

items:

type: string

style: form

explode: false

- name: smf-serving-area

in: query

schema:

type: string

- name: tai

in: query

description: Tracking Area Identity

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

- name: amf-region-id

in: query

description: AMF Region Identity

schema:

type: string

- name: amf-set-id

in: query

description: AMF Set Identity

schema:

type: string

- name: guami

in: query

description: Guami used to search for an appropriate AMF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

- name: supi

in: query

description: SUPI of the user

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: ue-ipv4-address

in: query

description: IPv4 address of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ue-ipv6-prefix

in: query

description: IPv6 prefix of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

- name: pgw-ind

in: query

description: Combined PGW-C and SMF or a standalone SMF

schema:

type: boolean

- name: pgw

in: query

description: PGW FQDN of a combined PGW-C and SMF

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

- name: gpsi

in: query

description: GPSI of the user

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

- name: external-group-identity

in: query

description: external group identifier of the user

schema:

type: string

- name: data-set

in: query

description: data set supported by the NF

schema:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/DataSetId'

- name: routing-indicator

in: query

description: routing indicator in SUCI

schema:

type: string

- name: group-id-list

in: query

description: Group IDs of the NFs being discovered

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in IETF RFC 7232, 3.2

schema:

type: string

* * *End Of Change * * * *