3GPP TSG CT Meeting #82	CP-183247

# Sorento, Italy; 10th - 11th Dec 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0828 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce 5G NSI EF and procedure. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Gemalto, Idemia, ST Microelectronics, G+D MS |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce 5G NSI EF and procedure as specified in SA3 and CT4 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An EF is introduced to store a SUPI of type network specific identifier in NAI format. Related procedure is also described. - EF_NSI can only contain a SUPI consisting of a network specific identifier in NAI format - The impact of update of EF_NSI is marked as FFS because the current reference to TS 31.111 is not sufficient |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SUPI type network specific identifier cannot be configured in the USIM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8, 4.4.11.1.1, 4.4.11.XX, 4.7, 5.2.X, Annex A, Annex E, H.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n8 |  |  | M |  | 1 byte |
| 2 | Services n9 to n16 |  |  | O |  | 1 byte |
| 3 | Services n17 to n24 |  |  | O |  | 1 byte |
| 4 | Services n25 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n(8X-7) to n(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | … |  |
|  | Service n°119 | V2X |
|  | Service n°120 | XCAP Configuration Data |
|  | Service n°yyy | Support for SUPI of type network specific identifier |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

Service n°46 can only be declared "available" if service n°45 is declared "available".

Service n°95, n°99 and n°115 shall not be declared "available" if an ISIM application is present on the UICC.

[…]

***** Next change *****

### 4.4.11	Contents of files at the DF5GS level

#### 4.4.11.1	Introduction

This clause describes the files that are specific for 5GS.

DF5GS shall be present at the ADFUSIM level if any of the following services are "available" in EFUST (USIM Service Table):

| Service n°122 | 5GS Mobility Management Information |
| --- | --- |
| Service n°123 | 5G Security Parameters |
| Service n°124 | Subscription identifier privacy support |
| Service n°125 | SUCI calculation by the USIM |
| Service n°126 | UAC Access Identities support |
| Service n°127 | Steering of UE in VPLMN |
| Service n°yyy | Support for SUPI of type network specific identifier |

***** Next change *****

### 4.4.11.XX	EFNSI (Network Specific Identifier)

If service n°yyy is "available", this file shall be present.

This EF contains the SUPI consisting of a network specific identifier in NAI format, as specified in TS 23.003 [25]. The SUPI contained in this EF shall not be an IMSI.

| Identifier: '4Fxx' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: 'yy' |  |  |  |  |  |  |
| File size: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | Network specific identifier in NAI format |  |  | M |  | X bytes |

Network Specific Identifier in NAI format:

Contents:

- Network specific identifier in NAI format as Subscription Permanent Identifier, as specified in 3GPP TS 23.003 [4].

Coding:

- NAI constructed as specified in IETF RFC 7542 and encoded as UTF-8 string.

***** Next change *****

## 4.7	Files of USIM

This clause contains two figures depicting the file structure of the UICC and the ADFUSIM. ADFUSIM shall be selected using the AID and information in EFDIR.

…

Figure 4.1: File identifiers and directory structures of UICC

|  | ADFUSIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | EFLI |  |  |  | EFARR |  |  |  |  |  |  |  | EFIMSI |  |  |  |  | EFKeys |  |  | EFKeysPS |  |
|  |  |  |  |  |  |  | '6F05' |  |  |  | '6F06' |  |  |  |  |  |  |  | '6F07' |  |  |  |  | '6F08' |  |  | '6F09' |  |
|  |  |  |  |  |  |  | EFDCK |  |  |  | EFHPPLMN |  |  |  |  |  |  |  | EFCNL |  |  |  |  | EFACMmax |  |  | EFUST |  |
|  |  |  |  |  |  |  | '6F2C' |  |  |  | '6F31' |  |  |  |  |  |  |  | '6F32' |  |  |  |  | '6F37' |  |  | '6F38' |  |
|  |  |  |  |  |  |  | EFACM |  |  |  | EFFDN |  |  |  |  |  |  |  | EFSMS |  |  |  |  | EFGID1 |  |  | EFGID2 |  |
|  |  |  |  |  |  |  | '6F39' |  |  |  | '6F3B' |  |  |  |  |  |  |  | '6F3C' |  |  |  |  | '6F3E' |  |  | '6F3F' |  |
|  |  |  |  |  |  |  | EFMSISDN |  |  |  | EFPUCT |  |  |  |  |  |  |  | EFSMSP |  |  |  |  | EFSMSS |  |  | EFCBMI |  |
|  |  |  |  |  |  |  | '6F40' |  |  |  | '6F41' |  |  |  |  |  |  |  | '6F42' |  |  |  |  | '6F43' |  |  | '6F45' |  |
|  |  |  |  |  |  |  | EFSPN |  |  |  | EFSMSR |  |  |  |  |  |  |  | EFCBMID |  |  |  |  | EFSDN |  |  | EFEXT2 |  |
|  |  |  |  |  |  |  | '6F46' |  |  |  | '6F47' |  |  |  |  |  |  |  | '6F48' |  |  |  |  | '6F49' |  |  | '6F4B' |  |
|  |  |  |  |  |  |  | EFEXT3 |  |  |  | EFBDN |  |  |  |  |  |  |  | EFEXT5 |  |  |  |  | EFCCP2 |  |  | EFCBMIR |  |
|  |  |  |  |  |  |  | '6F4C' |  |  |  | '6F4D' |  |  |  |  |  |  |  | '6F4E' |  |  |  |  | '6F4F' |  |  | '6F50' |  |
|  |  |  |  |  |  |  | EFEXT4 |  |  |  | EFEST |  |  |  |  |  |  |  | EFACL |  |  |  |  | EFCMI |  |  | EFSTART-HFN |  |
|  |  |  |  |  |  |  | '6F55' |  |  |  | '6F56' |  |  |  |  |  |  |  | '6F57' |  |  |  |  | '6F58' |  |  | '6F5B' |  |
|  |  |  |  |  |  |  | EFTHRESHOLD |  |  |  | EFPLMNwAcT |  |  |  |  |  |  |  | EFOPLMNwAcT |  |  |  |  | EFHPLMNwAcT |  |  | EFPSLOCI |  |
|  |  |  |  |  |  |  | '6F5C' |  |  |  | '6F60' |  |  |  |  |  |  |  | '6F61' |  |  |  |  | '6F62' |  |  | '6F73' |  |
|  |  |  |  |  |  |  | EFACC |  |  |  | EFFPLMN |  |  |  |  |  |  |  | EFLOCI |  |  |  |  | EFICI |  |  | EFOCI |  |
|  |  |  |  |  |  |  | '6F78' |  |  |  | '6F7B' |  |  |  |  |  |  |  | '6F7E' |  |  |  |  | '6F80' |  |  | '6F81' |  |
|  |  |  |  |  |  |  | EFICT |  |  |  | EFOCT |  |  |  |  |  |  |  | EFAD |  |  |  |  | EFVGCS |  |  | EFVGCSS |  |
|  |  |  |  |  |  |  | '6F82' |  |  |  | '6F83' |  |  |  |  |  |  |  | '6FAD' |  |  |  |  | '6FB1' |  |  | '6FB2' |  |
|  |  |  |  |  |  |  | EFVBS |  |  |  | EFVBSS |  |  |  |  |  |  |  | EFeMLPP |  |  |  |  | EFAaeM |  |  | EFECC |  |
|  |  |  |  |  |  |  | '6FB3' |  |  |  | '6FB4' |  |  |  |  |  |  |  | '6FB5' |  |  |  |  | '6FB6' |  |  | '6FB7' |  |
|  |  |  |  |  |  |  | EFHiddenkey |  |  |  | EFNETPAR |  |  |  |  |  |  |  | EFPNN |  |  |  |  | EFOPL |  |  | EFMBDN |  |
|  |  |  |  |  |  |  | '6FC3' |  |  |  | '6FC4' |  |  |  |  |  |  |  | '6FC5' |  |  |  |  | '6FC6' |  |  | '6FC7' |  |
|  |  |  |  |  |  |  | EFEXT6 |  |  |  | EFMBI |  |  |  |  |  |  |  | EFMWIS |  |  |  |  | EFCFIS |  |  | EFEXT7 |  |
|  |  |  |  |  |  |  | '6FC8' |  |  |  | '6FC9' |  |  |  |  |  |  |  | '6FCA' |  |  |  |  | '6FCB' |  |  | '6FCC' |  |
|  |  |  |  |  |  |  | EFSPDI |  |  |  | EFMMSN |  |  |  |  |  |  |  | EFEXT8 |  |  |  |  | EFMMSICP |  |  | EFMMSUP |  |
|  |  |  |  |  |  |  | '6FCD' |  |  |  | '6FCE' |  |  |  |  |  |  |  | '6FCF' |  |  |  |  | '6FD0' |  |  | '6FD1' |  |
|  |  |  |  |  |  |  | EFMMSUCP |  |  |  | EFNIA |  |  |  |  |  |  |  | EFVGCSCA |  |  |  |  | EFVBSCA |  |  | EFGBAP |  |
|  |  |  |  |  |  |  | '6FD2' |  |  |  | '6FD3' |  |  |  |  |  |  |  | '6FD4' |  |  |  |  | '6FD5' |  |  | '6FD6' |  |
|  |  |  |  |  |  |  | EFMSK |  |  |  | EFMUK |  |  |  |  |  |  |  | EFEHPLMN |  |  |  |  | EFGBANL |  |  | EFEHPLMNPI |  |
|  |  |  |  |  |  |  | '6FD7' |  |  |  | '6FD8' |  |  |  |  |  |  |  | '6FD9' |  |  |  |  | '6FDA' |  |  | '6FDB' |  |
|  |  |  |  |  |  |  | EFLRPLMNSI |  |  |  | EFNAFKCA |  |  |  |  |  |  |  | EFSPNI |  |  |  |  | EFPNNI |  |  | EFNCP-IP |  |
|  |  |  |  |  |  |  | '6FDC' |  |  |  | '6FDD' |  |  |  |  |  |  |  | '6FDE' |  |  |  |  | '6FDF' |  |  | '6FE2' |  |
|  |  |  |  |  |  |  | EFEPSLOCI |  |  |  | EFEPSNSC |  |  |  |  |  |  |  | EFUFC |  |  |  |  | EFUICCIARI |  |  | EFNASCONFIG |  |
|  |  |  |  |  |  |  | '6FE3' |  |  |  | '6FE4' |  |  |  |  |  |  |  | '6FE6' |  |  |  |  | '6FE7' |  |  | '6FE8' |  |
|  |  |  |  |  |  |  | EFPWS |  |  |  | EFFDNURI |  |  |  |  |  |  |  | EFBDNURI |  |  |  |  | EFSDNURI |  |  | EFIWL |  |
|  |  |  |  |  |  |  | '6FEC' |  |  |  | '6FED' |  |  |  |  |  |  |  | '6FEE' |  |  |  |  | '6FEF' |  |  | '6FF0' |  |
|  |  |  |  |  |  |  | EFIPS |  |  |  | EFIPD |  |  |  |  |  |  |  | EFePDGId |  |  |  |  | EFePDGSelection |  |  | EFePDGIdEm |  |
|  |  |  |  |  |  |  | '6FF1' |  |  |  | '6FF2' |  |  |  |  |  |  |  | '6FF3' |  |  |  |  | '6FF4' |  |  | '6FF5' |  |
|  |  |  |  |  |  |  | EFePDGSelectionEm |  |  |  | EFFromPreferred |  |  |  |  |  |  |  | EFIMSConfigData |  |  |  |  | EF3GPPPSDATAOFF |  |  | EF3GPPPSDATAOFFservicelist |  |
|  |  |  |  |  |  |  | '6FF6' |  |  |  | '6FF7' |  |  |  |  |  |  |  | '6FF8' |  |  |  |  | '6FF9' |  |  | '6FFA' |  |
|  |  |  |  |  |  |  | EFTVCONFIG |  |  |  | EFXCAPConfigData |  |  |  |  |  |  |  | EFEARFCNList |  |  |  |  | EF5GSLOCI |  |  | EF5GS3GPPNSC |  |
|  |  |  |  |  |  |  | '6FFB' |  |  |  | '6FFC' |  |  |  |  |  |  |  | '6FFE |  |  |  |  | '6FFF' |  |  | '6F00' |  |
|  |  |  |  |  |  |  | EF5GSN3GPPNSC |  |  |  | EFSteering_of_UE_in_VPLMN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | '6F01 |  |  |  | '6F05' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DF5GS '5FC0' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | EF5GS3GPPLOCI |  |  |  | EF5GSN3GPPLOCI |  |  |  |  |  |  |  | EF5GS3GPPNSC |  |  |  |  | EF5GSN3GPPNSC |  |  | EF5GAUTHKEYS |  |
|  |  |  |  |  |  |  | '4F01' |  |  |  | '4F02' |  |  |  |  |  |  |  | '4F03' |  |  |  |  | '4F04' |  |  | '4F05' |  |
|  |  |  |  |  |  |  |  | EFUAC_AIC |  |  |  | EFSUCI_Calc_Info |  |  |  |  |  |  |  | EFNSI |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '4F06' |  |  |  | '4F07' |  |  |  |  |  |  |  | '4Fxx' |  |  |  |  |  |  |  |  |

***** Next change *****

5.2	USIM security related procedures

…

### 5.2.X	NSI request

Requirement:		Service n°yyy is "available".

Request:	The ME performs the reading procedure with EFNSI.

The ME shall ignore the content of this EF if the EF contains an IMSI.

***** Next change *****

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification |  | Description |  | Change advised |  |
| --- | --- | --- | --- | --- | --- |
| '2F00' |  | Application directory |  | Caution |  |
| '2F05' |  | Preferred languages |  | Yes |  |
| … |  |  |  |  |  |
| '4Fxx' |  | Network Specific Identifier |  | Caution |  |
| '4F09' |  | ProSe Group Counter |  | No |  |
| '4F10' |  | ProSe Service Table |  | Caution |  |
| … |  |  |  |  |  |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, and EFEPSLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. |  |  |  |  |  |

Editor’s Note:	The REFRESH for NSI update needs to be further specified.

***** Next change *****

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description |  | Value |  |
| --- | --- | --- | --- | --- |
| '2F00' | Application directory |  | Card issuer/operator dependent |  |
| … | … |  | … |  |
| '4F08' | ProSe PLMN Parameters |  | Operator dependent |  |
| '4Fxx' | Network Specific Identifier |  | Operator dependent |  |
| '4F09' | ProSe Group Counter |  | 'FF…FF' |  |
| '4F10' | ProSe Service Table |  | Operator dependent |  |
| '4F11' | ProSe UsageInformationReportingConfiguration | Operator dependent |  |
| … | … |  | … |  |
| '6FC5' | PLMN Network Name |  | Operator dependent |  |
| '6FC6' | Operator Network List |  | Operator dependent |  |
| '6FC7' | Mailbox Dialling Numbers |  | Operator dependent |  |
| … | … |  | … |  |

***** Next change *****

# H.9	List of SFI Values at the DF 5GS Level

| File Identification | SFI | Description |
| --- | --- | --- |
| '4F01' | '01' | 5GS 3GPP location information (Note) |
| '4F02' | '02' | 5GS non-3GPP location information (Note) |
| '4F03' | '03' | 5GS 3GPP Access NAS Security Context |
| '4F04' | '04' | 5GS non-3GPP Access NAS Security Context |
| '4F05' | '05' | 5G authentication keys |
| '4F06' | '06' | UAC Access Identities Configuration |
| '4F07' | '07' | Subscriber Concealed Identifier Calculation Information |
| '4Fxx' | 'yy' | Network Specific Identifier |
| NOTE: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EF5GS3GPPLOCI and EF5GSN3GPPLOCI accordingly. |  |  |

All other SFI values are reserved for future use.

***** End of changes *****