3GPP TSG CT WG4 Meeting #89	C4-190479

Montreal, CANADA; 25th Feb – 01st Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.511 | CR | 0012 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GPSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2019-02-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To convey the GPSI as an optional parameter to the 5G-EIR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add GPSI to Check Equipment Identity request as an optional parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GPSI is not available at the EIR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 6.1.3.2.3.1, 6.1.4.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI specification file TS29511_N5g-eir_EquipmentIdentityCheck with impacts to API N5g-eir_EquipmentIdentityCheck in 3GPP TS 29.511. . |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.2.2	Procedure using CheckEquipmentIdentity Operation

The NF Service Consumer (e.g. AMF) shall check the PEI by using the HTTP GET method as shown in Figure 5.2.2.2.2-1.

Figure 5.2.2.2.2-1: PEI status check by the NF Service Consumer

1.	The NF Service Consumer (e.g. AMF) sends a GET request to the resource representing the PEI equipment Status. It shall include the PEI as a query parameter and, optionally, the SUPI and/or GPSI may also be included.

2a.	On success, "200 OK" with the message body containing the equipment status of the PEI.

2b.	If the PEI is not known, "404 Not Found" with the message body containing a ProblemDetails object, with the "details" attribute set to "ERROR_EQUIPMENT_UNKNOWN". When receiving the response from the 5G-EIR, the NF Service Consumer (e.g. AMF) shall check the equipment Status and the detailed problem. Dependent upon the result, the NF Service Consume will decide its subsequent actions (e.g. sending a Registration Reject if the 5G-EIR indicates that the PEI is unknown or blacklisted).

The definition of the equipment-status resource is specified in subclause 6.1.3.

* * * Next Change * * * *

###### 6.1.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pei | Pei | M | 1 | The PEI of the UE shall be included for equipment identify checking |
| supi |  | Supi |  | O |  | 0..1 |  | The SUPI of the UE |  |
| gpsi | Gpsi | O | 0..1 | The GPSI of the UE |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EirResponseData | M | 1 | 200 OK | Upon success, a response body containing the Equipment Status shall be returned |
| ProblemDetails | M | 1 | 404 Not Found | The equipment identify checking has failed. The "cause" attribute shall be set to the following application error: - ERROR_EQUIPMENT_UNKNOWN See table 6.1.5.3-1 for the description of this error. |

* * * Next Change * * * *

#### 6.1.4.1	General

This subclause specifies the application data model supported by the API.

Table 6.1.4.1-1 specifies the data types defined for the n5g-eir-eic service based interface protocol.

Table 6.1.4.1-1: n5g-eir-eic specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| EirResponseData | 6.1.4.2.2 |  |
| EquipmentStatus | 6.1.4.3.3 | Equipment status of the PEI, this data type is string. |

Table 6.1.6.1-2 specifies data types re-used by the N<NF> service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 6.1.4.1-2: 5g-eir-eic  re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Pei | 3GPP TS 29.571[6] | Data type representing the PEI of the UE. |
| Supi | 3GPP TS 29.571 [6] | Data type representing the SUPI of the subscriber. pattern: "(imsi-[0-9]{5,15}¦nai-.+)" |
| ProblemDetails | 3GPP TS 29.571 [6] | Common data type for error responses |
| Gpsi | 3GPP TS 29.571 [6] | Data type representing the GPSI of the subscriber. |

* * * Next Change * * * *

## A.2	N5g-eir_EquipmentIdentityCheck Service API

openapi: 3.0.0

info:

version: '1.0.0'

title: '5G-EIR Equipment Identity Check'

description: '5G-EIR Equipment Identity Check Service'

servers:

- url: '{apiRoot}/n5g-eir-eic/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- n5g-eir-eic

paths:

/equipment-status:

get:

summary: Retrieves the status of the UE

operationId: GetEquipmentStatus

tags:

- Equipment Status (Document)

parameters:

- name: pei

in: query

description: PEI of the UE

required: true

schema:

$ref: '#/components/schemas/Pei'

- name: supi

in: query

description: SUPI of the UE

required: false

schema:

$ref: '#/components/schemas/Supi'

- name: gpsi

in: query

description: GPSI of the UE

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/EirResponseData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'        '404':

description: PEI Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

n5g-eir-eic: Access to the N5g-eir_EquipmentIdentityCheck API

schemas:

EirResponseData:

type: object

required:

- status

properties:

status:

$ref: '#/components/schemas/EquipmentStatus'

Pei:

type: string

pattern: "([0-9]{14})"

Supi:

type: string

pattern: "(imsi-[0-9]{5,15}|nai-.+)"

EquipmentStatus:

type: string

enum:

- WHITELISTED

- BLACKLISTED

- GREYLISTED

ProblemDetails:

description: 'https://www.rfc-editor.org/rfc/rfc7807.txt'

type: object

required:

- type

properties:

type:

type: string

title:

type: string

status:

type: integer

detail:

type: string

instance:

type: string

externalDocs:

description: 3GPP TS 29.511 V15.2.0; 5G System; Equipment Identity Register Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.511/'

* * * End Of Change * * * *