3GPP TSG-CT Meeting #83	CP-190176

# Shenzhen, China, 18-19 March 2019

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0843 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IDEMIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | IDEMIA |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  |  | Date: |  | 2019-03-12 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During implementation of CR#0717 and CR#0716 a reference to a Note 5 was re-used in CR#0717, while the Note 5 itself was deleted in CR#0716 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference to non-existing Note 5 deleted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference to a non-existing Note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification |  | Description |  | Change advised |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| '2F00' |  | Application directory |  | Caution |  |  |
| '2F05' |  | Preferred languages |  | Yes |  |  |
| '2F06' |  | Access rule reference |  | Caution |  |  |
| '2F08' |  | UICC Maximum Power Consumption |  | No |  |  |
| '2FE2' |  | ICC identification |  | No |  |  |
| '4F01' |  | ProSe Monitoring Parameters |  | Yes |  |  |
| '4F01' |  | ACDC List |  | Yes |  |  |
| '4F01' |  | MCS Service Table |  | Yes |  |  |
| '4F01' |  | V2X Service Table |  | Yes |  |  |
| ‘4F01’ |  | 5GS 3GPP location information |  | Caution (Note 1) |  |  |
| '4F02' |  | V2X configuration data |  | Yes |  |  |
| '4F02' |  | ProSe Announcing Parameters |  | Yes |  |  |
| '4F02' |  | MCS configuration data |  | Yes |  |  |
| ‘4F02’ |  | 5GS non-3GPP location information |  | Caution (Note 1) |  |  |
| '4F03' |  | HPLMN ProSe Function |  | Yes |  |  |
| ‘4F03’ |  | 5GS 3GPP Access NAS Security Context |  | Caution |  |  |
| '4F04' |  | ProSe Direct Communication Radio Parameters |  | Yes |  |  |
| ‘4F04’ |  | 5GS non-3GPP Access NAS Security Context |  | Caution |  |  |
| '4F05' |  | ProSe Direct Discovery Monitoring Radio Parameters |  | Yes |  |  |
| ‘4F05’ |  | 5G authentication keys |  | No |  |  |
| '4F06' |  | ProSe Direct Discovery Announcing Radio Parameters |  | Yes |  |  |
| '4F06' |  | UAC Access Identities Configuration |  | Caution |  |  |
| '4F07' |  | ProSe Policy Parameters |  | Yes |  |  |
| '4F07' |  | Subscriber Concealed Identifier Calculation Information |  | Yes |  |  |
| '4F09' |  | Network Specific Identifier |  | Caution |  |  |
| '4F09' |  | ProSe Group Counter |  | No |  |  |
| '4F10' |  | ProSe Service Table |  | Caution |  |  |
| '4F11' |  | ProSe UsageInformationReportingConfiguration |  | Caution (Note 4) |  |  |
| '4F12' |  | ProSe Group Member Discovery Parameters |  | Yes |  |  |
| '4F13' |  | ProSe Relay Parameters |  | Yes |  |  |
| '4F14' |  | ProSe Relay Discovery Parameters |  | Yes |  |  |
| '4F20' |  | Image data |  | Yes |  |  |
| ‘4F20’ |  | GSM Ciphering key Kc |  | No |  |  |
| '4FXX' |  | Image Instance data Files |  | Yes |  |  |
| '4FXX' |  | ACDC OS Configuration |  | Yes |  |  |
| '4F21' |  | ICE graphics |  | Yes |  |  |
| '4FXX' |  | Unique identifier |  | Yes |  |  |
| '4F22' |  | Phone book synchronisation counter |  | Yes |  |  |
| '4F23' |  | Change counter |  | Yes |  |  |
| '4F24' |  | Previous unique identifier |  | Yes |  |  |
| '4F30' |  | Phone book reference file |  | Yes |  |  |
| '4FXX' |  | Capability configuration parameters 1 |  | Yes |  |  |
| '4F30' |  | SoLSA Access Indicator |  | Caution |  |  |
| '4F31' |  | SoLSA LSA List |  | Caution |  |  |
| '4FXX' |  | LSA Descriptor files |  | Caution |  |  |
| ‘4F52’ |  | GPRS Ciphring key KcGPRS |  | No |  |  |
| '4F63' |  | CPBCCH Information |  | No |  |  |
| '4F64’ |  | Investigation Scan |  | Caution |  |  |
| '4FXX' |  | Additional number alpha string |  | Yes |  |  |
| '4FXX' |  | Additional number |  | Yes |  |  |
| '4FXX' |  | Second name entry |  | Yes |  |  |
| '4FXX' |  | Grouping information alpha string |  | Yes |  |  |
| '4FXX' |  | Phone book control |  | Yes |  |  |
| '4FXX' |  | E-mail addresses |  | Yes |  |  |
| '4FXX' |  | Index administration phone book |  | Yes |  |  |
| '4FXX' |  | Extension 1 |  | Yes |  |  |
| '4FXX' |  | Abbreviated dialling numbers |  | Yes |  |  |
| '4FXX' |  | Grouping file |  | Yes |  |  |
| '4FXX' |  | TV User Service Description |  | Yes |  |  |
| '4F41' |  | Pseudonym |  | Caution |  |  |
| '4F42' |  | User controlled PLMN selector for I-WLAN |  | No |  |  |
| '4F43' |  | Operator controlled PLMN selector for I-WLAN |  | Caution |  |  |
| '4F44' |  | User controlled WSID List |  | No |  |  |
| '4F45' |  | Operator controlled WSID List |  | Caution |  |  |
| '4F46' |  | WLAN Reauthentication Identity |  | No |  |  |
| 4F47' |  | Home I-WLAN Specific Identifier List |  | Yes |  |  |
| '4F47' |  | Multimedia Messages List |  | Yes |  |  |
| '4F48' |  | I-WLAN Equivalent HPLMN Presentation Indication |  | Yes |  |  |
| '4F48' |  | Multimedia Messages Data File |  | Yes |  |  |
| '4F49' |  | I-WLAN HPLMN Priority Indication |  | Yes |  |  |
| '4F4A' |  | I-WLAN Last Registered PLMN |  | Caution |  |  |
| '4F4B' |  | HPLMN Direct Access Indicator |  | Yes |  |  |
| '4F81' |  | Allowed CSG lists |  | Yes |  |  |
| '4F82' |  | CSG Type |  | Yes |  |  |
| '4F83' |  | HNB name |  | Yes |  |  |
| '4F84' |  | Operator CSG lists |  | Yes |  |  |
| '4F85' |  | Operator CSG Type |  | Yes |  |  |
| '4F86' |  | Operator HNB name |  | Yes |  |  |
| '6F05' |  | Language indication |  | Yes |  |  |
| ‘6F06’ |  | Access rule reference (under ADFUSIM and DFTELECOM) |  | Caution |  |  |
| '6F07' |  | IMSI |  | Caution (Note 1) |  |  |
| '6F08' |  | Ciphering and integrity keys |  | No |  |  |
| '6F09' |  | Ciphering and integrity keys for packet switched domain |  | No |  |  |
| '6F2C' |  | De-personalization Control Keys |  | Caution |  |  |
| '6F31' |  | Higher Priority PLMN search period |  | Caution |  |  |
| '6F32' |  | Co-operative network list |  | Caution |  |  |
| '6F37' |  | ACM maximum value |  | Yes |  |  |
| '6F38' |  | USIM service table |  | Caution |  |  |
| '6F39' |  | Accumulated call meter |  | Yes |  |  |
| '6F3B' |  | Fixed dialling numbers |  | Yes (Note 2) |  |  |
| '6F3C' |  | Short messages |  | Yes |  |  |
| '6F3E' |  | Group identifier level 1 |  | Yes |  |  |
| '6F3F' |  | Group identifier level 2 |  | Yes |  |  |
| '6F40' |  | MSISDN storage |  | Yes |  |  |
| '6F41' |  | PUCT |  | Yes |  |  |
| '6F42' |  | SMS parameters |  | Yes |  |  |
| '6F43' |  | SMS status |  | Yes |  |  |
| '6F45' |  | CBMI |  | Caution |  |  |
| '6F46' |  | Service provider name |  | Yes |  |  |
| '6F47' |  | Short message status reports |  | Yes |  |  |
| '6F48' |  | CBMID |  | Yes |  |  |
| '6F49' |  | Service Dialling Numbers |  | Yes (Note 2) |  |  |
| '6F4B' |  | Extension 2 |  | Yes |  |  |
| '6F4C' |  | Extension 3 |  | Yes |  |  |
| '6F4D' |  | Barred dialling numbers |  | Yes |  |  |
| '6F4E' |  | Extension 5 |  | Yes |  |  |
| '6F4F' |  | Capability configuration parameters 2 |  | Yes |  |  |
| '6F50' |  | CBMIR |  | Yes |  |  |
| '6F54' |  | SetUp Menu Elements |  | Yes |  |  |
| ‘6F55’ |  | Extension 4 |  | Yes |  |  |
| '6F56' |  | Enabled services table |  | Caution |  |  |
| '6F57' |  | Access point name control list |  | Yes |  |  |
| '6F58' |  | Comparison method information |  | Yes |  |  |
| '6F5B' |  | Initialisation value for Hyperframe number |  | Caution |  |  |
| '6F5C' |  | Maximum value of START |  | Yes |  |  |
| '6F60' |  | User controlled PLMN selector with Access Technology |  | No |  |  |
| '6F61' |  | Operator controlled PLMN selector with Access Technology |  | Caution |  |  |
| '6F62' |  | HPLMN selector with Access Technology |  | Caution |  |  |
| '6F73' |  | Packet switched location information |  | Caution (Note 1) |  |  |
| '6F78' |  | Access control class |  | Caution |  |  |
| '6F7B' |  | Forbidden PLMNs |  | Caution |  |  |
| '6F7E' |  | Location information |  | No (Note 1) |  |  |
| '6F80' |  | Incoming call information |  | Yes |  |  |
| '6F81' |  | Outgoing call information |  | Yes |  |  |
| '6F82' |  | Incoming call timer |  | Yes |  |  |
| '6F83' |  | Outgoing call timer |  | Yes |  |  |
| '6FAD' |  | Administrative data |  | Caution |  |  |
| '6FB1' |  | Voice Group Call Service |  | Yes |  |  |
| '6FB2' |  | Voice Group Call Service Status |  | Yes |  |  |
| '6FB3' |  | Voice Broadcast Service |  | Yes |  |  |
| '6FB4' |  | Voice Broadcast Service Status |  | Yes |  |  |
| '6FB5' |  | Enhanced Multi Level Pre-emption and Priority |  | Yes |  |  |
| '6FB6' |  | Automatic Answer for eMLPP Service |  | Yes |  |  |
| '6FB7' |  | Emergency Call Codes |  | Caution |  |  |
| '6FC3' |  | Key for hidden phone book entries |  | No |  |  |
| '6FC4' |  | Network Parameters |  | No |  |  |
| '6FC5' |  | PLMN Network Name |  | Yes |  |  |
| '6FC6' |  | Operator Network List |  | Yes |  |  |
| '6FC7' |  | Mailbox Dialling Numbers |  | Yes |  |  |
| '6FC8' |  | Extension 6 |  | Yes |  |  |
| '6FC9' |  | Mailbox Identifier |  | Caution |  |  |
| '6FCA' |  | Message Waiting Indication Status |  | Caution |  |  |
| '6FCB' |  | Call Forwarding Indication Status |  | Caution |  |  |
| '6FCC' |  | Extension 7 |  | Yes |  |  |
| '6FCD' |  | Service Provider Display Information |  | Yes |  |  |
| '6FCE' |  | MMS Notification |  | Yes |  |  |
| '6FCF' |  | Extension 8 |  | Yes |  |  |
| '6FD0' |  | MMS Issuer Connectivity Parameters |  | Yes |  |  |
| '6FD1' |  | MMS User Preferences |  | Yes |  |  |
| '6FD2' |  | MMS User Connectivity Parameters |  | Yes |  |  |
| '6FD3' |  | Network's indication of alerting (NIA) |  | Caution |  |  |
| '6FD4' |  | Voice Group Call Service Ciphering Algorithm |  | Yes |  |  |
| '6FD5' |  | Voice Broadcast Service Ciphering Algorithm |  | Yes |  |  |
| '6FD6' |  | GBA Bootstrapping parameters |  | Caution |  |  |
| '6FD7' |  | MBMS Service Keys List |  | Caution |  |  |
| '6FD8' |  | MBMS User Key |  | Caution |  |  |
| '6FD9' |  | EHPLMN |  | Caution |  |  |
| '6FDA' |  | GBA NAF List |  | Caution |  |  |
| '6FDB' |  | EHPLMN Presentation Indication |  | Caution |  |  |
| '6FDC' |  | Last RPLMN Selection Indication |  | Caution |  |  |
| '6FDD' |  | NAF Key Centre Address |  | Caution |  |  |
| '6FDE' |  | Service Provider Name Icon |  | Yes |  |  |
| '6FDF' |  | PLMN Network Name Icon |  | Yes |  |  |
| '6FE0' |  | In Case of Emergency – Dialling Number |  | Yes |  |  |
| '6FE1' |  | In Case of Emergency – Free Format |  | Yes |  |  |
| '6FE2' |  | Network Connectivity Parameters for UICC IP connections |  | Yes |  |  |
| '6FE3' |  | EPS location information |  | Caution (Note 1) |  |  |
| '6FE4' |  | EPS NAS Security Context |  | Caution |  |  |
| '6FE5' |  | Public Service Identity of the SM-SC |  | Yes |  |
| '6FE6' |  | USAT Facility Control |  | Caution |  |
| '6FE7' |  | UICC IARI |  | Caution (Note 3) |  |
| '6FE8' |  | Non Access Stratum Configuration |  | Yes |  |
| '6FE9' |  | UICC certificate |  | Yes |  |
| '6FEA' |  | Relay Node ID |  | Yes |  |
| '6FEB' |  | Max value of Secure Channel counter |  | Caution |  |
| '6FEC' |  | Public Warning System |  | Yes |  |
| '6FED' |  | FDN URI |  | Yes (Note 2) |  |
| '6FEE' |  | BDN URI |  | Yes |  |
| '6FEF' |  | SDN URI |  | Yes (Note 2) |  |
| '6FF0' |  | IMEI(SV) White List |  | Yes |  |
| '6FF1' |  | IMEI(SV) Pairing Status |  | Caution |  |
| '6FF2' |  | IMEI(SV) Pairing Devices |  | Caution |  |
| '6FF3' |  | Home ePDG Identifier |  | Yes |  |
| '6FF4' |  | ePDG Selection Information |  | Yes |  |
| '6FF5' |  | Emergency ePDG Identifier |  | Yes |  |
| '6FF6' |  | ePDG Selection Information for Emergency Services |  | Yes |  |
| '6FF7' |  | From Preferred |  | Yes |  |
| '6FF8' |  | IMSConfigData |  | Caution) |  |
| '6FF9' |  | 3GPPPSDATAOFF |  | Caution |  |
| '6FFA' |  | 3GPPPSDATAOFFservicelist |  | Caution |  |
| '6FFB' |  | TV Configuration |  | Yes |  |
| '6FFC' |  | XCAP Configuration Data |  | Yes |  |
| '6FFD' |  | EARFCN List for MTC/NB-IOT UEs |  | Yes |  |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, EFEPSLOCI and EF5GSLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. |  |  |  |  |  |  |

Editor’s Note:	The REFRESH for NSI update needs to be further specified.