3GPP TSG-CT Meeting #83	CP-190201

# Shenzhen, P. R. China; 18th – 19th March 2019

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	CT aspects of Access Traffic Steering, Switch and Splitting support in 5G system

## Acronym:	ATSSS

## Unique identifier:	830016

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |
| 820044 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820014 | Wireless and Wireline Convergence for the 5G system architecture (5WWC) | The ATSSS-CT targets any kind of Non 3GPP access (untrusted, trusted, wireline) (as ATSSS-CT parent WID). This work item (5WWC) works out how to support trusted non-3GPP access to 5GC. |

Dependency on non-3GPP (draft) specification:

## 3	Justification

The Rel-16 normative work on Access Traffic Steering, Switching and Splitting (ATSSS) is being progressed in SA2. This new work item aims to support ATSSS from CT aspects.

## 4	Objective

The objective of this work item is to provide the stage 3 solutions for the functionalities defined in stage 2 requirements under the ATSSS WID in the TSG SAs working groups.

Stage 3 work shall be started only after the applicable normative stage 2 work is available.

The following functionalities, as supported in stage 2, will be specified in CT1, CT3 and CT4 respectively.

CT1:

-	Support of Multi Access-PDU (MA-PDU) session over one 3GPP access and one non-3GPP access

-	Transport of ATSSS rules in NAS messages

-	Support of access performance measurements and measurement reporting over the user plane

-	Support of steering functionalities (i.e. MPTCP functionality and ATSSS Low-Layer functionality) on the UE and the network side which enable steering, switching and splitting a data flow traffic between/among 3GPP access and non-3GPP access

NOTE:	The ATSSS feature can be supported over any type of access network, including untrusted and trusted non-3GPP access networks, wireline 5G access networks, wireline 5G cable access networks, etc., as long as a MA-PDU Session can be established over this type of access network.

CT3:

-	Support of MA-PDU session related management in PCF

-	Handling of PCC rules including ATSSS control information

CT4:

-	Impacts to NRF services for selection of ATSSS capable SMF/UPF

-	Impacts to N4 reference point for support of Multi-Access Rules and MPTCP proxy control

-	Possible impacts to UDM and UDR services for new subscription information

-	Possible impacts to SMF PDU Session service for support of ATSSS

-	Impacts to AMF Namf_Communication service for ATSSS

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TS | 24.193 | Access Traffic Steering, Switching and Splitting; Stage 3 | TSG#85 (September 2019) | TSG#86 (December 2019) | CT1 responsibility Rapporteur: ZHOU, Xingyue (Joy), ZTE, zhou.xingyue@zte.com.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.501 | Update of NAS signalling to support ATSSS | TSG#86 (December 2019) | CT1 responsibility |
| 24.526 | Extend URSP to support multi-access PDU session selection | TSG#86 (December 2019) | CT1 responsibility |
| 24.502 | Potential impacts to support ATSSS | TSG#86 (December 2019) | CT1 responsibility |
| 29.512 | Update Npcf_SMPolicyControl service and enhance the PCC rules to support ATSSS | TSG#86 (December 2019) | CT3 responsibility |
| 29.513 | Update the call flows to support ATSSS | TSG#86 (December 2019) | CT3 responsibility |
| 29.244 | N4 extensions to support ATSSS. | TSG#86 (December 2019) | CT4 responsibility |
| 29.502 | Possible impacts to SMF PDUSession API for support of ATSSS. | TSG#86 (December 2019) | CT4 responsibility |
| 29.503 | Possible impacts to UDM services for new subscription information | TSG#86 (December 2019) | CT4 responsibility |
| 29.505 | Possible impacts to UDR services for new subscription information | TSG#86 (December 2019) | CT4 responsibility |
| 29.518 | Impacts to Namf_Communication service to support ATSSS | TSG#86 (December 2019) | CT4 responsibility |
| 29.510 | Impacts to NRF services for selection of ATSSS capable SMF and UPF. | TSG#86 (December 2019) | CT4 responsibility |
| 23.008 | Possible impacts to subscriber data | TSG#86 (December 2019) | CT4 responsibility |

## 6	Work item Rapporteur(s)

ZHOU, Xingyue (Joy), ZTE, zhou.xingyue@zte.com.cn

## 7	Work item leadership

CT1

## 8	Aspects that involve other WGs

Possible charging aspects will be covered by SA5.

## 9	Supporting Individual Members

Supporting IM name

China Telecom

ZTE

Nokia

Nokia Shanghai Bell

Motorola Mobility

Lenovo

Verizon

Charter Communications

LG Electronics

Huawei

HiSilicon

Ericsson

Samsung

vivo

InterDigital

BlackBerry UK Ltd.

Deutsche Telekom

CATT