3GPP TSG CT WG1 Meeting #88bis						C1-143768

Sophia Antipolis (France), 20-24 October 2014

Source:	Acer Inc.

Title:	Timer for ProSe Discovery Request

Agenda item:	12.5

Document for:	Discussion & Approval

# Introduction

The DISCOVERY_REQUEST time out timer and Announce request procedure retransmitting timer is not yet defined in 3GPP TS 24.334, and consequently it is not explained with which value it is initialized in the UE and which is the UE behavior when it expires.

# 2. Proposal

# UE completes announce request procedure by receiving DISCOVERY_RESPONSE message from ProSe Function. But DISCOVERY_RESPONSE message can get lost for many reasons. This contribution proposes that UE can handle this by setting a timeout when it sends DISCOVERY_REQUEST message, and if UE isn’t received DISCOVERY_RESPONSE message when the timeout expires, it reinitiate announce request procedure by retransmitting the DISCOVERY_REQUEST message.

# 2.1. Timer T3901

The purpose of this timer is to indicate to the UE the period of time it should wait for DISCOVERY_RESPONSE message from ProSe Function.

- When UE has submitted the DISCOVERY_REQUEST message to ProSe Function, it shall start timer T3901.

- When the UE receives DISCOVERY_RESPONSE (response-announce) message, it shall stop the timer T3901.

- When the UE receives DISCOVERY_RESPONSE (response-reject) message, it shall stop the timer T3901.

,

# 2.1.1.	UE behaviour when Timer T3901 expires

If timer T3901 has expired in the UE, the UE shall:

- close the existing secure connection to the ProSe Function.

- double the current value for timer T3902 but not exceeding the maximum value defined for this timer.

- start timer T3902 and wait for it to expire.

# 2.2. Timer T3902:

The purpose of this timer is to indicate to the UE the minimum period of time it should wait before attempting the Announce request procedure again.

# 2.2.1 	UE behaviour when Timer T3902 expires

If timer T3902 has expired in the UE, the UE shall restart the Announce request procedure as defined in sub-clause 6.2.2.2. of 3GPP TS 24.334.

# 2.3. Timer Value

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- |
| T3901 | 30sec | Sending of DISCOVERY REQUEST from UE. | Reception of DISCOVERY ACCEPT or DISCOVERY REJECT | See sub-clause 2.1.1 |
| T3902 | Initial value is 1 minute. Maximum value is 30 minutes | T3901 expiry as defined in 2.1.1 |  | See sub-clause 2.2.1 |