3GPP TSG-CT WG1 Meeting #88bis 	C1-143907

# Sophia-Antipolis (France), 20-24 October 2014

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.334 | CR | 0030 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce Timer T3901 for ProSe Discovery Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Acer Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ProSe-CT |  |  |  |  |  |  | Date: |  | 2014-10-08 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The Announce request procedure DISCOVERY_REQUEST timeout timer is not yet defined in 3GPP TS 24.334, and consequently it is not explained with which value it is initialized in the UE and which is the UE behavior when it expires. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE completes announce request procedure by receiving DISCOVERY_RESPONSE message from ProSe Function. But DISCOVERY_RESPONSE message can get lost for many reasons. If UE isn’t received DISCOVERY_RESPONSE message when the timeout expires, it reinitiate announce request procedure by retransmitting the DISCOVERY_REQUEST message. This contribution proposes the timer T3901 to indicate to the UE the period of time it should wait for DISCOVERY_RESPONSE message from ProSe Function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not know how long UE needs to wait for DISCOVERY_RESPONSE from ProSe function. It will lead UE to wait for very long until the secure connection close. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2, 6.2.2.4, 6.2.2.5, 6.2.2.6.1, Figure 6.2.2.2.1, 13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Change *****

#### 6.2.2.2	Announce request procedure initiation

Before initiating the announce request procedure, the UE is configured with the data structure of the ProSe Application IDs appropriate for its HPLMN. This step is performed using mechanisms out of scope of 3GPP.

If the UE is authorised to perform ProSe direct discovery announcing in the registered PLMN, it shall initiate an announce request procedure:

a)	when the UE is triggered by an upper layer application to announce a ProSe Application ID and the UE has no valid corresponding ProSe Application Code for that upper layer application;

b)	when the validity timer T4000 assigned by the ProSe Function to a ProSe Application Code has expired and the request from upper layers to announce the ProSe Application ID corresponding to that ProSe Application Code is still in place; or

c)	when the UE selects a new PLMN while announcing a ProSe Application Code.

NOTE 1:	To ensure service continuity if the UE needs to keep announcing a ProSe Application Code corresponding to the same ProSe Application ID, the UE can initiate the announce request procedure before the TTL timer T4000 assigned by the ProSe Function for a Prose Application Code expires.

The UE initiates the announce request procedure by sending a DISCOVERY_REQUEST message with a new transaction ID, the ProSe Application ID set to the ProSe Application ID received from upper layers, the command set to "announce", the UE identity set to the UE's IMSI, and the Application Identity set to the Application Identity of the upper layer application that requested the announcing.

When UE has submitted the DISCOVERY_REQUEST message to ProSe Function, it shall start timer T3901.

NOTE 2:	A UE can include one or multiple transactions in one DISCOVERY_REQUEST message for different ProSe Application IDs, and receive corresponding <response-announce> element or <response-reject> element in a DISCOVERY_RESPONSE message for each respective transaction. In the following description of the announce request procedure, only one transaction is included.

Figure 6.2.2.2.1 illustrates the interaction of the UE and the ProSe Function in the announce request procedure.

***** Next change *****

Figure 6.2.2.2.1: Announce request procedure

***** Next change ****

#### 6.2.2.4	Announce request procedure completion by the UE

Upon receipt of the DISCOVERY_RESPONSE message, the UE shall stop the timer T3901, if the transaction ID contained in the <response-announce> element matches the value sent by the UE in a DISCOVERY_REQUEST message with the command set to "announce", the UE shall start performing direct discovery announcing as described below. Otherwise the UE shall discard the DISCOVERY_RESPONSE message.

The UE requests the ProSe direct discovery parameters from the lower layers (see 3GPP TS 36.331 [12]). If the lower layers indicate that ProSe direct discovery is not supported by the network, the UE shall not perform direct discovery announcing. Else if the lower layers indicate that ProSe direct discovery is supported by the network:

-	if the lower layers indicate that the UE is not required to send a discovery indication to the eNodeB, the UE shall request the time parameter for the next discovery transmission opportunity from lower layers;

-	if the lower layers indicate that the UE is required to send a discovery indication to the eNodeB and the UE is in EMM-IDLE mode, the UE shall perform a Service Request procedure to send a discovery indication for ProSe direct discovery announcing as specified in 3GPP TS 24.301 [11] to transfer to EMM-CONNECTED mode. Upon successful completion of the Service Request procedure, the UE shall trigger the lower layers to send the discovery indication to the eNodeB as specified in 3GPP TS 36.331 [12]. Upon indication from the lower layers that the discovery indication has been sent successfully to the eNodeB, the UE shall request the time parameter for the next discovery transmission opportunity from lower layers; or

-	if the lower layers indicate that the UE is required to send a discovery indication to the eNodeB and the UE is in EMM-CONNECTED mode, the UE shall trigger the lower layers to send the discovery indication to the eNodeB as specified in 3GPP TS 36.331 [12]. Upon indication from lower layers that the discovery indication has been sent successfully to the eNodeB, the UE shall request the time parameter for the next discovery transmission opportunity from lower layers;

If the time parameter obtained from lower layers is valid, the UE uses this parameter to compute the MIC field for the PC5_DISCOVERY message as described in 3GPP TS 33.303 [6].

The UE shall use the ProSe Application Code received in the DISCOVERY_RESPONSE message, along with the Discovery Key, in order to construct a PC5_DISCOVERY message, according to the format defined in subclause 11.2.5.

The UE then passes the PC5_DISCOVERY message to the lower layers for transmission if:

-	the UE is currently authorised to perform direct discovery announcing in the registered PLMN;

-	the validity timer T4000 for the allocated ProSe Application Code has not expired; and

-	a request from upper layers to announce the ProSe Application ID associated with both the ProSe Application Code and the authorised Application Identity is still in place.

***** Next change ****

#### 6.2.2.5	Announce request procedure not accepted by the ProSe Function

If the DISCOVERY_REQUEST message cannot be accepted by the ProSe Function, the ProSe Function sends a DISCOVERY_RESPONSE message containing a <response-reject> element to the UE including an appropriate PC3 Control Protocol cause value.

Upon receipt of the DISCOVERY_RESPONSE message containing a <response-reject>, the UE shall stop the timer T3901,

If the application corresponding to the Application Identity contained in the DISCOVERY_REQUEST message is not authorised for ProSe direct discovery announcing in the registered PLMN, the ProSe Function shall send the DISCOVERY_RESPONSE message containing a <response-reject> element with PC3 Control Protocol cause value #1 "Invalid application".

If the UE is not authorised for ProSe direct discovery announcing, the ProSe Function shall send the DISCOVERY_RESPONSE message containing a <response-reject> element with PC3 Control Protocol cause value #3 "UE authorisation failure".

If the UE requests a country-specific ProSe Application ID for a country that does not correspond to the country of its HPLMN, and the ProSe Function has not authorized the UE to announce in that country, the ProSe Function shall send a DISCOVERY_RESPONSE message containing a <response-reject> element with PC3 Control Protocol cause value #8 "Scope Violation in Prose Application ID".

If the UE requests a country-specific ProSe Application ID for a country that does not correspond to the country of its HPLMN, and the ProSe Function has no agreement to access the country-wide ProSe Application ID database of that country, the ProSe Function shall send a DISCOVERY_RESPONSE message containing a <response-reject> element with PC3 Control Protocol cause value #8 "Scope Violation in Prose Application ID".

***** Next change *****

#### 6.2.2.6	Abnormal cases

##### 6.2.2.6.1	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Indication from the transport layer of transmission failure of DISCOVERY_REQUEST message (e.g. after TCP retransmission timeout)

The UE shall close the existing secure connection to the ProSe Function, establish a new secure connection and then restart the announce request procedure.

b)	T3901 expiry

If timer T3901 has expired in the UE:

- the UE shall close the existing secure connection to the ProSe Function,

- the UE shall retransmit the DISCOVERY_REQUEST message.

Editor's note: the timer to trigger retransmission and the maximum number of allowed retransmission are FFS.

c)	Indication from upper layers that the request to announce the ProSe Application ID is no longer in place after sending the DISCOVERY_REQUEST message, but before the announce request procedure is completed

The UE shall acknowledge the DISCOVERY_RESPONSE message received from the ProSe Function but discard its contents and then abort the procedure.

d)	Change of PLMN

If a PLMN change occurs before the announce request procedure is completed, the procedure shall be aborted. If the UE is authorized to announce in the new PLMN, the procedure shall be restarted once the UE is registered on the new PLMN.

## 13.2	Timers of ProSe direct services procedures

Table 13.2.1: ProSe direct services timers – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- |
| T3901 | 30sec | Sending of DISCOVERY REQUEST from UE. | Reception of DISCOVERY ACCEPT or DISCOVERY REJECT | Ref 6.2.2.6.1 |
| T4000 | NOTE 1 | Upon receiving a ProSe Application Code with an associated T4000 timer in a DISCOVERY_RESPONSE message whose transaction ID contained in the <response-announce> element matches the value sent by the UE in a DISCOVERY_REQUEST message with the command set to "announce", as described in subclause 6.2.2.4. | Upon receiving a new T4000 timer value for the same ProSe Application Code or receiving a new Timer associated with a new ProSe Application Code for the same ProSe Application ID in a DISCOVERY_RESPONSE message. When the UE selects a new PLMN. | Stop announcing the associated ProSe Application Code over the PC5 interface and re-initiate the announce request procedure if the request from upper layers to announce the ProSe Application ID corresponding to the associated ProSe Application Code is still in place. |
| T4002 | NOTE 2 | Upon receiving a Discovery Filter with an associated T4002 timer in a DISCOVERY_RESPONSE message whose transaction ID contained in the <response-monitor> element matches the value sent by the UE in a DISCOVERY_REQUEST message with the command set to "monitor", as described in subclause 6.2.3.4. | Upon receiving a new T4002 timer value for the same Discovery Filter in a DISCOVERY_RESPONSE message. | Stop using the associated Discovery Filter for ProSe direct discovery monitoring over the PC5 interface and re-initiate the monitor request procedure, if the request from upper layers to monitor the ProSe Application ID corresponding to the associated Discovery Filter is still in place. |
| T4004 | NOTE 3 | Upon receiving a T4004 timer in a MATCH_REPORT_ACK message whose transaction ID contained in the <match-ack> element matches the value sent by the UE in a MATCH_REPORTmessage, as described in subclause 6.2.4.4. | Upon receiving a new T4004 timer value for the same ProSe Application Code in a MATCH_REPORT_ACK message. | The UE may inform the upper layers that the corresponding ProSe Application ID is not longer matched. |
| T4005 | NOTE 4 | Upon receiving a monitoring, announcing or communication policy for a given PLMN with an associated T4005 value in the ProSe service authorisation as described in subclause 5.1.3 and subclause 5.1.4. | When the service authorisation for the corresponding PLMN is revoked by the ProSe Function. Upon receiving a new T4005 timer value for the same operation (monitoring, announcing or communication) in the same PLMN. | Stop the monitoring, announcing or communication operation in the corresponding PLMN and re-initiate the service authorisation procedure if the UE wants to continue performing announcing, monitoring or communication in that PLMN. |
| NOTE 1: The value of this timer is provided by the ProSe Function during the announce request procedure. NOTE 2: The value of this timer is provided by the ProSe Function during the monitor request procedure. NOTE 3: The value of this timer is provided by the ProSe Function during the match report procedure. NOTE 4: The value of this timer is provided by the ProSe Function during service authorisation procedure. |  |  |  |  |

Table 13.2.2: ProSe direct services timers – ProSe Function side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- |
| T4001 | NOTE 1 | Upon assigning a ProSe Application Code with an associated T4000 value to the UE, as described in subclause 6.2.2.3. | Upon receiving a new DISCOVERY_REQUEST message from the UE with the command set to "announce" for the same ProSe Application ID. | Delete the association between the UE, the requested ProSe Application ID and the corresponding ProSe Application Code allocated by the ProSe Function. |
| T4003 | NOTE 2 | Upon assigning a Discovery Filter with an associated T4002 value to the UE, as described in subclause 6.2.3.3. | Upon receiving a new DISCOVERY_REQUEST message from the UE with the command set to "monitor" for the same ProSe Application ID | Delete the association between the UE, the requested ProSe Application ID and the corresponding Discovery Filter allocated by the ProSe Function. |
| NOTE 1: The value of this timer is assigned by the ProSe Function during the announce request procedure. NOTE 2: The value of this timer is assigned by the ProSe Function during the monitor request procedure. |  |  |  |  |

NOTE:	Multiple timers T4001 and T4003 can run simultaneously in the ProSe Function.

***** End of change *****