3GPP TSG-CT WG4 Meeting #93	C4-193493

# Wroclaw, Poland; 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.389 | CR | 0004 | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft-ietf-dime-load published as RFC 8583 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DLoCMe |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | the IETF draft on Diameter Load Information Conveyance has been published as RFC8583. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the reference to the I-D "draft-ietf-dime-load" by a reference to the RFC 8583 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential Correction. Missing normative reference, which might lead to misimplementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3.1, 6.3.10, D.1, D.2, D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.285: "Architecture enhancements for V2X services; Stage 2".

[3]	IETF RFC 6733: "Diameter Base Protocol".

[4]	3GPP TS 33.210: "3G Security; Network Domain Security; IP Network Layer Security".

[5]	IETF RFC 4960: "Stream Control Transmission Protocol".

[6]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol".

[7]	IETF RFC 5234: "Augmented BNF for Syntax Specifications: ABNF".

[8]	3GPP TS 23.003: "Numbering, addressing and identification".

[9]	3GPP TS 29.228: "IP multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and Message Elements".

[10]	3GPP TS 29.272: "Evolved Packet System; MME and SGSN Related Interfaces Based on Diameter Protocol".

[11]	3GPP TS 29.329: "Sh Interface based on the Diameter protocol".

[12]	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[13]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[14]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[15]	IETF RFC 7944: "Diameter Routing Message Priority".

[16]	3GPP TS 29.388: "V2X Control Function to V2X Control Function to Home Subscriber Server (HSS) aspects (V4)".

[17]	3GPP TS 29.345: "Inter-Proximity-services (ProSe) Function signalling aspects".

[18]	IETF RFC 8583: "Diameter Load Information Conveyance".

* * * Next Change * * * *

### 6.3.1	General

The following table (table 6.3.1-1) specifies the Diameter AVPs defined for the V6 interface, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32 e.g., PRR-Flags, bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x0001 shall be used.

Table 6.3.1-1: V6 specific Diameter AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| V2X-Authorization-Data | 4700 | 6.3.2 | Grouped |  |  |  |  |  |
| V2X-Permission-in-VPLMN | 4701 | 6.3.3 | Unsigned32 |  |  |  |  |  |
| V2X-Application-Server | 4702 | 6.3.4 | Grouped |  |  |  |  |  |
| NOTE 1: The AVP header bit denoted as "M", indicates whether support of the AVP is required. The AVP header bit denoted as "V" indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [3]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table (table 6.3.1-2) specifies the Diameter AVPs re-used by the V6 interface from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within V6 interface.

Any other AVPs from existing Diameter Applications, except for the AVPs from the Diameter base protocol, do not need to be supported. The AVPs from the Diameter base protocol are not included in table 6.3.1-2.

Table 6.3.1-2: V6 re-used Diameter AVPs

| Attribute Name | Reference | Comments | M-bit |
| --- | --- | --- | --- |
| Application-Server | 3GPP TS 32.299 [14] | See subclause 6.3.5 |  |
| Geographical-Information | 3GPP TS 29.272 [10] | See subclause 6.3.6 |  |
| Visited-PLMN-Id | 3GPP TS 29.272 [10] |  |  |
| User-Identifier | 3GPP TS 29.336 [13] |  |  |
| User-Name | IETF RFC 6733 [3] |  |  |
| MSISDN | 3GPP TS 29.329 [11] |  |  |
| Supported-Features | 3GPP TS 29.229 [6] |  |  |
| Feature-List-ID | 3GPP TS 29.229 [6] |  |  |
| Feature-List | 3GPP TS 29.229 [6] |  |  |
| OC-Supported-Features | IETF RFC 7683 [12] | See subclause 6.3.7 | Must set |
| OC-OLR | IETF RFC 7683 [12] | See subclause 6.3.8 | Must set |
| DRMP | IETF RFC 7944 [15] | See subclause 6.3.9 | Must not set |
| Load | IETF RFC 8583 [18] | See subclause 6.3.10 | Must not set |
| NOTE 1: The M-bit settings for re-used AVPs override those of the defining specifications that are referenced. Values include: "Must set", "Must not set". If the M-bit setting is blank, then the defining specification applies. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |

* * * Next Change * * * *

### 6.3.10	Load

The Load AVP is of type Grouped and it is defined in IETF RFC 8583 [18]. This AVP is used to support Diameter load control mechanism, see Annex D for more information.

* * * Next Change * * * *

## D.1	General

Diameter load control mechanism is an optional feature.

IETF RFC 8583 [18] specifies a Diameter load control mechanism which includes the definition and the transfer of related AVPs between Diameter nodes.

It is recommended to make use of IETF draft-ietf-dime-load-08 [18] on the V6 interface where, when applied, the requesting V2X Control Function shall behave as a reacting node and the responding V2X Control Function as a reporting node.

* * * Next Change * * * *

## D.2	Responding V2X Control Function behaviour

The responding V2X Control Function may report its current load by including a Load AVP of type HOST in answer commands as described in IETF RFC 8583 [18].

The responding V2X Control Function calculates its current load by implementation specific means. For example, the responding V2X Control Function may take into account the traffic over the V6 interface or other interfaces, the level of usage of internal resources (e.g. CPU, memory), the access to external resources, etc.

The responding V2X Control Function determines when to send Load AVPs of type HOST by implementation specific means.

* * * Next Change * * * *

## D.3	Requesting V2X Control Function behaviour

When performing next hop Diameter Agent selection for requests that are routed based on realm, the requesting V2X Control Function may take into account load values from Load AVPs of type PEER received from candidate next hop Diameter nodes, as per IETF RFC 8583 [18].

* * * End of Changes * * * *