3GPP TSG CT Meeting #71	CP-160158

# Gothenburg, Sweden; 7th – 8th March 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 2321 | rev | 7 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhacements to the EPS attach procedure for CIoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Verizon, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Verizon, Intel |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT-CT |  |  |  |  |  |  | Date: |  | 2016-03-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | At SA2#111 meeting, SA2 has reached agreements on CIoT architecture for NB-IOT and sent an LS in C1-154066/S2-153695 to CT1 to inform about the agreements on the normative work which is to progress th solution 2 from TR 23.720 as a mandatory feature for the UE and the network and solution 18 as an optional feature. At SA2#113 meeting, SA2 have agreed severals CRs on CIoT with one particularly on attach procedure (i.e., S2-160836) which shows the necessary updates to the EPS attach procedure for CIoT. Other agreed CRs need also to be taken into account when considering stage 3 impacts to the EPS attach procedure (in S2-160907, S2-160909), and also the SA2 LS reply in S2-160906. Based on the above information. It is proposed to enhance the existing EPS attach procedure for CIoT. Rev7, changes after the agreed CT1 version in JeJu (#96) MCC has identified that this agreed CT1 CR (in C1-161548) and the agreed CT1 CR2314 in C1-160569 propose changes to the figure 9.9.3.34.1. Particularly, both CRs change the bit 2 of octet 8 (that is currently spare). The rapporteur of TS 24.301 proposes that this is solved at the plenary by revising this CR but just proposing the following bit of octet 8 (i.e., bit 3). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EPS attach procedure is enhanced so that CIoT could become possible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The particular optimizations because of CIoT are not supported by stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 5.5.1.1, 5.5.1.2.2, 5.5.1.2.4, 5.5.1.2.5, 6.2.2, 6.5.1.2, 8.2.1.1, 8.2.1.11, 8.2.4.12, 8.2.4.13, 9.9.3.0A, 9.9.3.0B 9.9.3.34, 9.9.4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 23.401 CR2951 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Based on CR2951r8 |  |  |  |  |  |  |  |  |

***** Next change *****

## 4.2	Linkage between the protocols for EPS mobility management and EPS session management

During the EPS attach procedure, the network can activate a default EPS bearer context (i.e. if the UE requests PDN connectivity in the attach request). Additionally, the network can activate one or several dedicated EPS bearer contexts in parallel for PDN connections of IP PDN type. To this purpose the EPS session management messages for the default EPS bearer context activation are transmitted in an information element in the EPS mobility management messages. In this case, the UE and the network execute the attach procedure, the default EPS bearer context activation procedure, and the dedicated EPS bearer context activation procedure in parallel. The UE and network shall complete the combined default EPS bearer context activation procedure and the attach procedure before the dedicated EPS bearer context activation procedure is completed. The success of the attach procedure is dependent on the success of the default EPS bearer context activation procedure. If the attach procedure fails, then the ESM procedures also fail.

Except for the attach procedure and the service request procedure, during EMM procedures the MME shall suspend the transmission of ESM messages. During the service request procedure the MME may suspend the transmission of ESM messages.

Except for the attach procedure, during EMM procedures the UE shall suspend the transmission of ESM messages.

***** Next change *****

#### 5.5.1.1	General

The attach procedure is used to attach to an EPC for packet services in EPS.

The attach procedure is used for three purposes:

-	by a UE in PS mode of operation to attach for EPS services only;

-	by a UE in CS/PS mode 1 or CS/PS mode 2 of operation to attach for both EPS and non-EPS services;

-	by a UE supporting NB-S1 mode only in PS mode of operation to attach for EPS services and "SMS only"; or

-	to attach for emergency bearer services.

The lower layers indicate to NAS that the network does not support emergency bearer services for the UE in limited service state (3GPP TS 36.331 [22]). This information is taken into account when deciding whether to initiate attach for emergency bearer services in WB-S1 mode.

If the MME does not support an attach for emergency bearer services, the MME shall reject any request to attach with an attach type set to "EPS emergency attach".

With a successful attach procedure, a context is established for the UE in the MME. Furthermore, if the UE requested PDN connectivity, a default bearer is established between the UE and the PDN GW, thus enabling always-on IP connectivity to the UE. The network may also initiate the activation of dedicated bearers as part of the attach procedure.

With a successful attach procedure in NB-S1 mode, a context is established for the UE in the MME. If the attach request included information to request PDN connectivity, a default bearer is also established between the UE and the PDN.

During the attach procedure with default bearer establishment, the UE may also obtain the home agent IPv4 or IPv6 address or both.

In a shared network, the UE shall choose one of the PLMN identities as specified in 3GPP TS 23.122 [6]. The UE shall construct the TAI of the cell from this chosen PLMN identity and the TAC received as part of the broadcast system information. The chosen PLMN identity shall be indicated to the E-UTRAN (see 3GPP TS 36.331 [22]). Whenever an ATTACH REJECT message with the EMM cause #11 "PLMN not allowed" is received by the UE, the chosen PLMN identity shall be stored in the "forbidden PLMN list" and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a. Whenever an ATTACH REJECT message with the EMM cause #14 "EPS services not allowed in this PLMN" is received by the UE, the chosen PLMN identity shall be stored in the "forbidden PLMNs for GPRS service" and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a. Whenever an ATTACH REJECT message is received by the UE with the EMM cause #12 "tracking area not allowed", #13 "roaming not allowed in this tracking area", or #15 "no suitable cells in tracking area", the constructed TAI shall be stored in the suitable list.

An attach attempt counter is used to limit the number of subsequently rejected attach attempts. The attach attempt counter shall be incremented as specified in subclause 5.5.1.2.6. Depending on the value of the attach attempt counter, specific actions shall be performed. The attach attempt counter shall be reset when:

-	the UE is powered on;

-	a USIM is inserted;

-	an attach or combined attach procedure is successfully completed;

-	a GPRS attach or combined GPRS attach procedure is successfully completed in A/Gb or Iu mode;

-	a combined attach procedure is completed for EPS services only with cause #2, #16, #17, #18 or #22;

-	an attach or combined attach procedure is rejected with cause #11, #12, #13, #14, #15, #25 or #35:

-	a network initiated detach procedure is completed with cause #11, #12, #13, #14, #15 or #25; or

-	a new PLMN is selected.

Additionally the attach attempt counter shall be reset when the UE is in substate EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH and:

-	a new tracking area is entered;

-	timer T3402 expires; or

-	timer T3346 is started.

***** Next change *****

##### 5.5.1.2.1	General

This procedure can be used by a UE to attach for:

-	EPS services only; or

-	EPS services and "SMS only" if the UE supports NB-S1 mode only.

When the UE initiates the attach procedure for normal service, the UE shall indicate "EPS attach" in the EPS attach type IE.

When the UE initiates the attach procedure for emergency bearer services, the UE shall indicate "EPS emergency attach" in the EPS attach type IE. The attach procedure for emergency bearer services is not applicable for NB-S1 mode (see 3GPP TS 23.401 [10]).

***** Next change *****

##### 5.5.1.2.2	Attach procedure initiation

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

The UE shall include the IMSI in the EPS mobile identity IE in the ATTACH REQUEST message if the selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs and:

-	the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]; or

-	the UE is in NB-S1 mode.

For all other cases, the UE shall handle the EPS mobile identity IE in the ATTACH REQUEST message as follows:

If the UE supports neither A/Gb mode nor Iu mode:

-	the UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. In addition, the UE shall include Old GUTI type IE with GUTI type set to "native GUTI". If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

If the UE supports A/Gb mode or Iu mode or both:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	The mapping of the P-TMSI and the RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI".

-	If the TIN is deleted and

-	the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI";

-	the UE does not hold a valid GUTI but holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE; or

-	the UE does not hold a valid GUTI, P-TMSI or RAI, the UE shall include the IMSI in the EPS mobile identity IE.

-	Otherwise the UE shall include the IMSI in the EPS mobile identity IE.

If the UE is attaching for emergency bearer services and does not hold a valid GUTI, P-TMSI or IMSI as described above, the IMEI shall be included in the EPS mobile identity IE.

If the UE supports A/Gb mode or Iu mode or if the UE needs to indicate its UE specific DRX parameter to the network, the UE shall include the UE specific DRX parameter in the DRX parameter IE in the ATTACH REQUEST message.

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the ATTACH REQUEST message.

If the UE supports SRVCC to GERAN/UTRAN, the UE shall set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported".

If the UE supports vSRVCC from S1 mode to Iu mode, then the UE shall set the H.245 after handover capability bit to "H.245 after SRVCC handover capability supported" and additionally set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported" in the ATTACH REQUEST message.

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

If the UE supports ProSe direct discovery, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct discovery bit to "ProSe direct discovery supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports ProSe direct communication, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct communication bit to "ProSe direct communication supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE is in NB-S1 mode, then the UE shall set the control plane CIoT EPS optimization bit to "control plane CIoT EPS optimization supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE is in NB-S1 mode, supports CIoT EPS optimizations and requests to attach for EPS services and "SMS only", the UE shall indicate SMS only requested bit to "SMS only" in the additional update type IE and shall set the EPS attach type IE to "EPS attach" in the ATTACH REQUEST message.

The UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container IE to request PDN connectivity If a valid EPS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the message included in the ESM message container IE. When the UE does not have a valid EPS security context, the ATTACH REQUEST message combined with the message included in the ESM message container IEis not integrity protected.

Figure 5.5.1.2.2.1: Attach procedure and combined attach procedure

***** Next change *****

##### 5.5.1.2.4	Attach accepted by the network

During an attach for emergency bearer services, if not restricted by local regulations, the MME shall not check for mobility and access restrictions, regional restrictions, subscription restrictions, or perform CSG access control when processing the ATTACH REQUEST message. The network shall not apply subscribed APN based congestion control during an attach procedure for emergency bearer services.

If the attach request is accepted by the network, the MME shall send an ATTACH ACCEPT message to the UE and start timer T3450.

If the attach request included the PDN CONNECTIVITY REQUEST message in the ESM message container information element to request PDN connectivity, the MME when accepting the attach request shall send the ATTACH ACCEPT message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message contained in the ESM message container information element to activate the default bearer (see subclause 6.4.1). The network may also initiate the activation of dedicated bearers towards the UE by invoking the dedicated EPS bearer context activation procedure (see subclause 6.4.2).

If the attach request is accepted by the network, the MME shall delete the stored UE radio capability information, if any.

If the UE has included the UE network capability IE or the MS network capability IE or both in the ATTACH REQUEST message, the MME shall store all octets received from the UE, up to the maximum length defined for the respective information element.

NOTE 1:	This information is forwarded to the new MME during inter-MME handover or to the new SGSN during inter-system handover to A/Gb mode or Iu mode.

NOTE 2:	For further details concerning the handling of the MS network capability and UE network capability in the MME see also 3GPP TS 23.401 [10].

If the UE specific DRX parameter was included in the DRX Parameter IE in the ATTACH REQUEST message, the MME shall replace any stored UE specific DRX parameter with the received parameter and use it for the downlink transfer of signalling and user data.

The MME shall include the extended DRX parameters IE in the ATTACH ACCEPT message only if the extended DRX parameters IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of eDRX.

Editor’s note [WI eDRX-CT; CR#2212]:	The MME handling of the UE specific DRX cycle versus the extended idle mode DRX cycle when they are both used is FFS.

The MME shall assign and include the TAI list the UE is registered to in the ATTACH ACCEPT message. The UE, upon receiving an ATTACH ACCEPT message, shall delete its old TAI list and store the received TAI list.

The MME may include T3412 extended value IE in the ATTACH ACCEPT message only if the UE indicates support of the extended periodic timer T3412 in the MS network feature support IE in the ATTACH REQUEST message.

The MME shall include the T3324 value IE in the ATTACH ACCEPT message only if the T3324 value IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of PSM.

If the MME supports and accepts the use of PSM, and the UE included the T3412 extended value IE in the ATTACH REQUEST message, then the MME shall take into account the T3412 value requested when providing the T3412 value IE and the T3412 extended value IE in the ATTACH ACCEPT message.

NOTE 3:	Besides the value requested by the UE, the MME can take local configuration  or subscription data provided by the HSS into account when selecting a value for T3412 (3GPP TS 23.401 [10] subclause 4.3.17.3).

Upon receiving the ATTACH ACCEPT message, the UE shall stop timer T3410.

The GUTI reallocation may be part of the attach procedure. When the ATTACH REQUEST message includes the IMSI or IMEI, or the MME considers the GUTI provided by the UE is invalid, or the GUTI provided by the UE was assigned by another MME, the MME shall allocate a new GUTI to the UE. The MME shall include in the ATTACH ACCEPT message the new assigned GUTI together with the assigned TAI list. In this case the MME shall enter state EMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.4.1.

For a shared network, the TAIs included in the TAI list can contain different PLMN identities. The MME indicates the selected core network operator PLMN identity to the UE in the GUTI (see 3GPP TS 23.251 [8B]).

If the ATTACH ACCEPT message contains a GUTI, the UE shall use this GUTI as the new temporary identity. The UE shall delete its old GUTI and store the new assigned GUTI. If no GUTI has been included by the MME in the ATTACH ACCEPT message, the old GUTI, if any available, shall be kept.

If A/Gb mode or Iu mode is supported in the UE, the UE shall set its TIN to "GUTI" when receiving the ATTACH ACCEPT message.

If the ATTACH ACCEPT message contains the T3412 extended value IE, then the UE shall use the value in T3412 extended value IE as periodic tracking area update timer (T3412). If the ATTACH ACCEPT message does not contain T3412 extended value IE, then the UE shall use the value in T3412 value IE as periodic tracking area update timer (T3412).

If the ATTACH ACCEPT message contains the T3324 value IE, then the UE shall use the included timer value for T3324 as specified in 3GPP TS 24.008 [13], subclause 4.7.2.8.

The MME may also include a list of equivalent PLMNs in the ATTACH ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, and if the attach procedure is not for emergency bearer services, the UE shall remove from the list any PLMN code that is already in the list of "forbidden PLMNs" or in the list of "forbidden PLMNs for GPRS service". In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the ATTACH ACCEPT message. If the ATTACH ACCEPT message does not contain a list, then the UE shall delete the stored list.

If the attach procedure is not for emergency bearer services, and if the PLMN identity of the registered PLMN is a member of the list of "forbidden PLMNs" or the list of "forbidden PLMNs for GPRS service", any such PLMN identity shall be deleted from the corresponding list(s).

The network informs the UE about the support of specific features, such as IMS voice over PS session, location services (EPC-LCS, CS-LCS) or emergency bearer services, in the EPS network feature support information element. In a UE with IMS voice over PS capability, the IMS voice over PS session indicator and the emergency bearer services indicator shall be provided to the upper layers. The upper layers take the IMS voice over PS session indicator into account as specified in 3GPP TS 23.221 [8A], subclause 7.2a and subclause 7.2b, when selecting the access domain for voice sessions or calls. When initiating an emergency call, the upper layers also take both the IMS voice over PS session indicator and the emergency bearer services indicator into account for the access domain selection. In a UE with LCS capability, location services indicators (EPC-LCS, CS-LCS) shall be provided to the upper layers. When MO-LR procedure is triggered by the UE's application, those indicators are taken into account as specified in 3GPP TS 24.171 [13C].

If the UE has initiated the attach procedure due to manual CSG selection and receives an ATTACH ACCEPT message; and the UE sent the ATTACH REQUEST message in a CSG cell, the UE shall check if the CSG ID and associated PLMN identity of the cell are contained in the Allowed CSG list. If not, the UE shall add that CSG ID and associated PLMN identity to the Allowed CSG list and the UE may add the HNB Name (if provided by lower layers) to the Allowed CSG list if the HNB Name is present in neither the Operator CSG list nor the Allowed CSG list.

When the UE receives the ATTACH ACCEPT message combined with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, it shall forward the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to the ESM sublayer. Upon receipt of an indication from the ESM sublayer that the default EPS bearer context has been activated, the UE shall send an ATTACH COMPLETE message together with an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message contained in the ESM message container information element to the network.

Additionally, the UE shall reset the attach attempt counter, service request attempt counter and tracking area updating attempt counter, enter state EMM-REGISTERED and set the EPS update status to EU1 UPDATED.

When the UE receives any ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST messages during the attach procedure, the UE shall forward the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message(s) to the ESM sublayer. The UE shall send a response to the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message(s) after successful completion of the attach procedure.

If the attach procedure was initiated in S101 mode, the lower layers are informed about the successful completion of the procedure.

Upon receiving an ATTACH COMPLETE message, the MME shall stop timer T3450, enter state EMM-REGISTERED and consider the GUTI sent in the ATTACH ACCEPT message as valid.

***** Next change *****

##### 5.5.1.2.5	Attach not accepted by the network

If the attach request cannot be accepted by the network, the MME shall send an ATTACH REJECT message to the UE including an appropriate EMM cause value.

If the attach procedure fails due to:

-	EPS attached without PDN connectivity is not supported;

-	a default EPS bearer setup failure;

-	an ESM procedure failure; or

-	operator determined barring is applied on default EPS bearer context activation during attach procedure,

the MME shall:

-	combine the ATTACH REJECT message with a PDN CONNECTIVITY REJECT message contained in the ESM message container information element. In this case the EMM cause value in the ATTACH REJECT message shall be set to #19 "ESM failure"; or

-	send the ATTACH REJECT message with the EMM cause set to #15 "No suitable cells in tracking area", if the PDN connectivity reject is due to ESM cause #29 subject to operator policies (see 3GPP TS 29.274 [16D] for further details). In this case, the network may additionally include the Extended EMM cause IE with value "E-UTRAN not allowed".

If the attach request is rejected due to NAS level mobility management congestion control, the network shall set the EMM cause value to #22 "congestion" and assign a back-off timer T3346.

Upon receiving the ATTACH REJECT message, if the message is integrity protected or contains a reject cause other than EMM cause value #25, the UE shall stop timer T3410.

If the ATTACH REJECT message with EMM cause #25 was received without integrity protection, then the UE shall discard the message.

The UE shall take the following actions depending on the EMM cause value received in the ATTACH REJECT message.

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(EPS services and non-EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. The UE shall consider the USIM as invalid for EPS services and non-EPS services until switching off or the UICC containing the USIM is removed or the timer T3245 expires as described in subclause 5.3.7a. Additionally, the UE shall delete the list of equivalent PLMNs and enter state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI and ciphering key sequence number, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

NOTE:	The possibility to configure a UE so that the radio transceiver for a specific RAT is not active, although it is implemented in the UE, is out of scope of the present specification.

#7		(EPS services not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. The UE shall consider the USIM as invalid for EPS services until switching off or the UICC containing the USIM is removed or the timer T3245 expires as described in subclause 5.3.7a. Additionally, the UE shall delete the list of equivalent PLMNs and enter state EMM-DEREGISTERED.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#11	(PLMN not allowed); or

#35	(Requested service option not authorized in this PLMN);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. Additionally, the UE shall delete the list of equivalent PLMNs and reset the attach attempt counter.

In S1 mode, the UE shall store the PLMN identity in the "forbidden PLMN list" and enter state EMM-DEREGISTERED.PLMN-SEARCH and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a. The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

In S101 mode, the UE shall store the PLMN identity provided with the indication from the lower layers to prepare for an S101 mode to S1 mode handover in the list of "forbidden PLMNs for attach in S101 mode" and enter the state EMM-DEREGISTERED.NO-CELL-AVAILABLE and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the MM parameters update status, TMSI, LAI, ciphering key sequence number and location update attempt counter, and the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value and no RR connection exists.

#12	(Tracking area not allowed);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. Additionally, the UE shall reset the attach attempt counter.

In S1 mode, the UE shall store the current TAI in the list of "forbidden tracking areas for regional provision of service" and enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

In S101 mode, the UE shall store the PLMN identity provided with the indication from the lower layers to prepare for an S101 mode to S1 mode handover in the list of "forbidden PLMNs for attach in S101 mode" and enter the state EMM-DEREGISTERED.NO-CELL-AVAILABLE and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#13	(Roaming not allowed in this tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. The UE shall delete the list of equivalent PLMNs and reset the attach attempt counter.

In S1 mode, the UE shall store the current TAI in the list of "forbidden tracking areas for roaming". Additionally, the UE shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE or optionally EMM-DEREGISTERED.PLMN-SEARCH. The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

In S101 mode, the UE shall store the PLMN identity provided with the indication from the lower layers to prepare for an S101 mode to S1 mode handover in the list of "forbidden PLMNs for attach in S101 mode" and enter the state EMM-DEREGISTERED.NO-CELL-AVAILABLE and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#14	(EPS services not allowed in this PLMN);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. Additionally, the UE shall delete the list of equivalent PLMNs and reset the attach attempt counter.

In S1 mode, the UE shall store the PLMN identity in the "forbidden PLMNs for GPRS service" list. Additionally, the UE shall enter state EMM-DEREGISTERED.PLMN-SEARCH and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a. The UE shall perform a PLMN selection according to 3GPP TS 23.122 [6].

In S101 mode, the UE shall store the PLMN identity provided with the indication from the lower layers to prepare for an S101 mode to S1 mode handover in the list of "forbidden PLMNs for attach in S101 mode" and enter the state EMM-DEREGISTERED.NO-CELL-AVAILABLE and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#15	(No suitable cells in tracking area);

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.3) and shall delete any GUTI, last visited registered TAI, TAI list and eKSI. Additionally, the UE shall reset the attach attempt counter.

In S1 mode, the UE shall:

-	store the current TAI in the list of "forbidden tracking areas for roaming" and enter the state EMM-DEREGISTERED.LIMITED-SERVICE. AND

-	if the Extended EMM cause IE with value "E-UTRAN not allowed" is included in the ATTACH REJECT message, the UE supports "E-UTRA Disabling for EMM cause #15", and the "E-UTRA Disabling Allowed for EMM cause #15" parameter as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17] is present and set to enabled; then the UE shall disable the E-UTRA capability as specified in subclause 4.5 and search for a suitable cell in GERAN or UTRAN radio access technology; otherwise. the UE shall search for a suitable cell in another tracking area or in another location area according to 3GPP TS 36.304 [21].

In S101 mode, the UE shall store the PLMN identity provided with the indication from the lower layers to prepare for an S101 mode to S1 mode handover in the list of "forbidden PLMNs for attach in S101 mode" and enter the state EMM-DEREGISTERED.NO-CELL-AVAILABLE and if the UE is configured to use timer T3245 (see 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]) then the UE shall start timer T3245 and proceed as described in subclause 5.3.7a.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status, P-TMSI, P-TMSI signature, RAI, GPRS ciphering key sequence number and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#22	(Congestion);

If the T3346 value IE is present in the ATTACH REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below; otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.5.1.2.6.

The UE shall abort the attach procedure, reset the attach attempt counter, set the EPS update status to EU2 NOT UPDATED and enter state EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH.

The UE shall stop timer T3346 if it is running.

If the ATTACH REJECT message is integrity protected, the UE shall start timer T3346 with the value provided in the T3346 value IE.

If the ATTACH REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [13].

The UE stays in the current serving cell and applies the normal cell reselection process. The attach procedure is started if still needed when timer T3346 expires or is stopped.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#25	(Not authorized for this CSG);

EMM cause #25 is only applicable when received from a CSG cell. EMM cause #25 received from a non-CSG cell is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.6.

The UE shall set the EPS update status to EU3 ROAMING NOT ALLOWED (and store it according to subclause 5.1.3.3). Additionally, the UE shall reset the attach attempt counter and shall enter the state EMM-DEREGISTERED.LIMITED-SERVICE.

If the CSG ID and associated PLMN identity of the cell where the UE has sent the ATTACH REQUEST message are contained in the Allowed CSG list, the UE shall remove the entry corresponding to this CSG ID and associated PLMN identity from the Allowed CSG list.

If the CSG ID and associated PLMN identity of the cell where the UE has sent the ATTACH REQUEST message are contained in the Operator CSG list, the UE shall apply the procedures defined in 3GPP TS 23.122 [6] subclause 3.1A.

The UE shall search for a suitable cell according to 3GPP TS 36.304 [21].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

#42	(Severe network failure);

The UE shall set the EPS update status to EU2 NOT UPDATED, and shall delete any GUTI, last visited registered TAI, TAI list, eKSI, and list of equivalent PLMNs, and set the attach attempt counter to 5. The UE shall start an implementation specific timer setting its value to 2 times the value of T as defined in 3GPP TS 23.122 [6]. While this timer is running, the UE shall not consider the PLMN + RAT combination that provided this reject cause a candidate for PLMN selection. The UE then enters state EMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [6].

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition set the GMM state to GMM-DEREGISTERED, GPRS update status to GU2 NOT UPDATED, and shall delete the P-TMSI, P-TMSI signature, RAI and GPRS ciphering key sequence number.

Other values are considered as abnormal cases. The behaviour of the UE in those cases is specified in subclause 5.5.1.2.6.

***** Next change *****

### 6.2.2	IP address allocation via NAS signalling

The UE shall set the PDN type IE in the PDN CONNECTIVITY REQUEST message, based on its IP stack configuration if it requests IP connectivity (e.g. the per APN settings specified in 3GPP TS 23.401 [10]) as follows:

a)-	A UE, which is IPv6 and IPv4 capable and

-	has not been allocated an IP address for this APN, shall set the PDN type IE to IPv4v6.

-	has been allocated an IPv4 address for this APN and received the ESM cause #52 "single address bearers only allowed", and is requesting an IPv6 address, shall set the PDN type IE to IPv6.

-	has been allocated an IPv6 address for this APN and received the ESM cause #52 "single address bearers only allowed", and is requesting an IPv4 address, shall set the PDN type IE to IPv4.

b)	A UE, which is only IPv4 capable, shall set the PDN type IE to IPv4.

c)	A UE, which is only IPv6 capable, shall set the PDN type IE to IPv6.

d)	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall set the PDN type IE to IPv4v6.

If the UE wants to use DHCPv4 for IPv4 address assignment, it shall indicate that to the network within the Protocol Configuration Options IE in the PDN CONNECTIVITY REQUEST.

If the UE wants to get PDN connectivity without IP, the UE shall set the PDN type IE in the PDN CONNECTIVITY REQUEST message to "non IP".

On receipt of the PDN CONNECTIVITY REQUEST message sent by the UE, the network when allocating an IP address shall take into account the PDN type IE, the operator policies of the home and visited network, and the user's subscription data and:

-	if the UE requests for PDN type IPv4v6, but the subscription is limited to IPv4 only or IPv6 only for the requested APN, the network shall override the PDN type requested by the UE to be limited to a single address PDN type (IPv4 or IPv6). In the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent to the UE, the network shall set the PDN type value to either "IPv4" or "IPv6" and the ESM cause value to #50 "PDN type IPv4 only allowed", or #51 "PDN type IPv6 only allowed", respectively. The UE shall not subsequently initiate another UE requested PDN connectivity procedure to the same APN to obtain a PDN type different from the one allowed by the network until:

a)	all EPS bearer contexts to the given APN are deactivated at the UE as a result of:

i)	EPS bearer context synchronization during tracking area updating or service request procedure;

ii)	an EPS bearer context deactivation procedure initiated by the network;

iii)	a local EPS bearer context deactivation without NAS signalling as specified in subclause 6.4.4.6;

iv)	a detach procedure; or

v)	a tracking area updating procedure or service request procedure that is rejected with cause #40 "No EPS bearer context activated"; or

b)	the PDN type which is used to access to the APN is changed;

NOTE 1:	Request to send another PDN CONNECTIVITY REQUEST message with a specific PDN type has to come from upper layers.

-	if the UE requests PDN type IPv4v6, but the PDN GW configuration dictates the use of IPv4 addressing only or IPv6 addressing only for this APN, the network shall override the PDN type requested by the UE to limit it to a single address PDN type (IPv4 or IPv6). In the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent to the UE, the network shall set the PDN type value to either "IPv4" or "IPv6" and the ESM cause value to #50 "PDN type IPv4 only allowed", or #51 "PDN type IPv6 only allowed", respectively. The UE shall not subsequently initiate another UE requested PDN connectivity procedure to the same APN to obtain a PDN type different from the one allowed by the network until:

a)	all EPS bearer contexs to the given APN are deactivated at the UE and the network as a result of:

i)	EPS bearer context synchronization during tracking area updating or service request procedure;

ii)	an EPS bearer context deactivation procedure initiated by the network;

iii)	a local EPS bearer context deactivation without NAS signalling as specified in subclause 6.4.4.6;

iv)	a detach procedure; or

v)	a tracking area updating procedure or service request procedure that is rejected with cause #40 "No EPS bearer context activated"; or

b)	the PDN type which is used to access to the APN is changed;

NOTE 2:	Request to send another PDN CONNECTIVITY REQUEST message with a specific PDN type has to come from upper layers.

-	if the UE requests PDN type IPv4v6, but the operator uses single addressing per bearer, e.g. due to interworking with nodes of earlier releases, the network shall override the PDN type requested by the UE to a single IP version only. In the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent to the UE, the network shall set the PDN type value to either "IPv4" or "IPv6" and the ESM cause value to #52 "single address bearers only allowed". The UE should subsequently request another PDN connection for the other IP version using the UE requested PDN connectivity procedure to the same APN with a single address PDN type (IPv4 or IPv6) other than the one already activated;

NOTE 3:	If the MT and TE are separated, the UE might not be able to use ESM cause #52 "single address bearers only allowed" as a trigger for activating a second single-IP-stack EPS bearer context.

-	if the network sets the PDN type to IPv4 or IPv4v6, the network shall include an IPv4 address in the PDN address information. In this case, if the IPv4 address is to be configured using DHCPv4, the network shall set the IPv4 address to 0.0.0.0; and

-	if the network sets the PDN type to IPv6 or IPv4v6, the network shall include the interface identifier that the UE shall use for the link local address in the PDN address information.

The network shall include the PDN type and the PDN address information within the PDN address IE in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent to the UE.

***** Next change *****

#### 6.5.1.2	UE requested PDN connectivity procedure initiation

In order to request connectivity to an additional PDN, the UE shall send a PDN CONNECTIVITY REQUEST message to the MME, start timer T3482 and enter the state PROCEDURE TRANSACTION PENDING (see example in figure 6.5.1.2.1).

When the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message, the UE shall not start timer T3482 and shall not include the APN.

NOTE 1:	If the UE needs to provide protocol configuration options which require ciphering or provide an APN, or both, during the attach procedure, the ESM information transfer flag is included in the PDN CONNECTIVITY REQUEST. The MME then at a later stage in the PDN connectivity procedure initiates the ESM information request procedure in which the UE can provide the MME with protocol configuration options or APN or both.

In order to request a PDN connection for emergency bearer services, the UE shall not include an APN in the PDN CONNECTIVITY REQUEST message or, when applicable, in the ESM INFORMATION RESPONSE message.

In order to request connectivity to a PDN using the default APN, the UE includes the Access point name IE in the PDN CONNECTIVITY REQUEST message or, when applicable, in the ESM INFORMATION RESPONSE message, according to the following conditions:

-	if use of a PDN using the default APN requires PAP/CHAP, then the UE should include the Access point name IE; and

-	in all other conditions, the UE need not include the Access point name IE.

In order to request connectivity to an additional PDN using a specific APN, the UE shall include the requested APN in the PDN CONNECTIVITY REQUEST message.

In the PDN type information element the UE shall either indicate the IP version capability of the IP stack associated with the UE or non IP as specified in subclause 6.2.2.

The UE shall set the request type to "initial request" when the UE is establishing a new PDN connectivity to a PDN in an attach procedure or in a stand-alone PDN connectivity procedure. The UE shall set the request type to "emergency" when the UE is requesting a new PDN connectivity for emergency bearer services. The UE shall set the request type to "handover" when the connectivity to a PDN is established upon handover from a non-3GPP access network and the UE was connected to that PDN before the handover to the 3GPP access network, or when the UE initiates the procedure to add 3GPP access to the PDN connection which is already established over WLAN.

NOTE 2:	For emergency bearer services, the handover from non-3GPP access to E-UTRA is not supported.

If the UE supports DSMIPv6, the UE may include a request for obtaining the IPv6 address and optionally the IPv4 address of the home agent in the Protocol configuration options IE in the PDN CONNECTIVITY REQUEST message. The UE may also include a request for obtaining the IPv6 Home Network Prefix. The UE shall request the IPv6 Home Network Prefix only if the UE has requested the home agent IPv6 address. The requested home agent address(es) and the Home Network Prefix are related to the APN the UE requested connectivity for.

The UE may set the ESM information transfer flag in the PDN CONNECTIVITY REQUEST message to indicate that it has ESM information, i.e. protocol configuration options, APN, or both, that needs to be sent after the NAS signalling security has been activated between the UE and the MME.

If the UE supports A/Gb mode or Iu mode or both, the UE shall indicate the support of the network requested bearer control procedures (see 3GPP TS 24.008 [13]) in A/Gb mode or Iu mode in the Protocol configuration options IE.

Protocol configuration options provided in the ESM INFORMATION RESPONSE message replace any protocol configuration options provided in the PDN CONNECTIVITY REQUEST message.

When the UE initiates the procedure to add 3GPP access to the PDN connection that is already established over WLAN, the UE shall provide the same APN as that of the PDN connection established over WLAN in the PDN connectivity procedure as specified in the subclause 6.2.2 of 3GPP TS 23.161 [34].

Figure 6.5.1.2.1: UE requested PDN connectivity procedure

***** Next change *****

#### 8.2.1.11	Additional update result

The network may include this IE to provide the UE with additional information about the result of:

-	a combined attach procedure if the procedure was successful for EPS services and non-EPS services, or for EPS services and "SMS only"; or

-	an attach procedure requested for CIoT EPS optimizations if the procedure was successful for EPS services and "SMS only".

***** Next change *****

#### 8.2.4.12	Additional update type

The UE shall include this IE if the UE requests "SMS only" or CIoT EPS optimizations.

***** Next change *****

#### 8.2.4.13	Voice domain preference and UE's usage setting

This IE shall be included if the UE in WB-S1 mode supports CS fallback and SMS over SGs, or if the UE is configured to support IMS voice, but does not support 1xCS fallback.

***** Next change *****

#### 9.9.3.0A	Additional update result

The purpose of the Additional update result information element is to provide additional information about the result of a combined attach procedure or a combined tracking area updating procedure.

The Additional update result information element is coded as shown in figure 9.9.3.0A.1 and table 9.9.3.0A.1.

The Additional update result is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional update result IEI |  |  |  | ANB-CIoT |  | Additional update result value |  | Octet 1 |

Figure 9.9.3.0A.1: Additional update result information element

Table 9.9.3.0A.1: Additional update result information element

| Additional update result value (octet 1) |  |  |
| --- | --- | --- |
| Bits |  |  |
| 2 | 1 |  |
| 0 | 0 | no additional information |
| 0 | 1 | CS Fallback not preferred |
| 1 | 0 | SMS only |
| 1 | 1 | reserved |
| Negotiated CIoT network behaviour (NNB-CIoT) (octet 1) |  |  |
| 4 | 3 |  |
| 0 | 0 | no additional information |
| 0 | 1 | control-plane CIoT EPS optimization |
| 1 | 0 | user-plane EPS optimization |
| 1 | 1 | reserved |

***** Next change *****

#### 9.9.3.0B	Additional update type

The purpose of the Additional update type information element is to provide additional information about the type of request for a combined attach or a combined tracking area updating procedure.

The Additional update type information element is coded as shown in figure 9.9.3.0B.1 and table 9.9.3.0B.1.

The Additional update type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Additional update type IEI |  |  |  | PNB-CIoT |  |  | 0 Spare |  | AUTV | octet 1 |

Figure 9.9.3.0B.1: Additional update type information element

Table 9.9.3.0B.1: Additional update type information element

| Additional update type value (AUTV) (octet 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 1 |  |  |
| 0 | no additional information. If received it shall be interpreted as request for combined attach or combined tracking area updating. |  |
| 1 | SMS only |  |
| Preferred CIoT network behaviour (PNB-CIoT) (octet 1) |  |  |
| Bit |  |  |
| 4 | 3 |  |
| 0 | 0 | no additional information |
| 0 | 1 | control-plane CIoT EPS optimization |
| 1 | 0 | user-plane EPS optimization |
| 1 | 1 | reserved |
| Bit 2 of octet 1 is spare and shall be coded as zero. |  |  |

***** Next change *****

#### 9.9.3.12A	EPS network feature support

The purpose of the EPS network feature support information element is to indicate whether certain features are supported by the network.

The EPS network feature support information element is coded as shown in figure 9.9.3.12A.1 and table 9.9.3.12A.1.

The EPS network feature support is a type 4 information element with a length of 3 octets.

| 8 |  | 7 |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPS network feature support IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of EPS network feature support contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  | EAw/oPDN |  | ESR PS |  | CS-LCS |  |  | EPC-LCS |  | EMC BS |  | IMS VoPS |  | octet 3 |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.9.3.12A.1: EPS network feature support information element

Table 9.9.3.12A.1: EPS network feature support information element

| IMS voice over PS session indicator (IMS VoPS) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | IMS voice over PS session in S1 mode not supported |
| 1 |  |  |  | IMS voice over PS session in S1 mode supported |
| Emergency bearer services indicator (EMC BS) (octet 3, bit 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | emergency bearer services in S1 mode not supported |
| 1 |  |  |  | emergency bearer services in S1 mode supported |
| Location services indicator in EPC (EPC-LCS) (octet 3, bit 3) |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | location services via EPC not supported |
| 1 |  |  |  | location services via EPC supported |
| Location services indicator in CS (CS-LCS) (octet 3, bit 4 to 5) |  |  |  |  |
| Bit |  |  |  |  |
| 5 | 4 |  |  |  |
| 0 | 0 |  |  | no information about support of location services via CS domain is available |
| 0 | 1 |  |  | location services via CS domain supported |
| 1 | 0 |  |  | location services via CS domain not supported |
| 1 | 1 |  |  | reserved |
| Support of EXTENDED SERVICE REQUEST for packet services (ESRPS) (octe t3, bit 6) |  |  |  |  |
| Bit |  |  |  |  |
| 6 |  |  |  |  |
| 0 |  |  |  | network does not support use of EXTENDED SERVICE REQUEST to request for packet services |
| 1 |  |  |  | network supports use of EXTENDED SERVICE REQUEST to request for packet services |
| EPS attached w/o PDN connectivity (EAw/oPDN) (octet 3, bit 7) |  |  |  |  |
| This bit indicates the capability for EPS attached without PDN connectivity |  |  |  |  |
| Bit |  |  |  |  |
| 6 |  |  |  |  |
| 0 |  |  |  | EPS attached without PDN connectivity not supported |
| 1 |  |  |  | EPS attached without PDN connectivity supported |
| Bit 8 of octet 3 is spare and shall be coded as zero. |  |  |  |  |

***** Next change *****

#### 9.9.3.34	UE network capability

The purpose of the UE network capability information element is to provide the network with information concerning aspects of the UE related to EPS or interworking with GPRS. The contents might affect the manner in which the network handles the operation of the UE. The UE network capability information indicates general UE characteristics and it shall therefore, except for fields explicitly indicated, be independent of the frequency band of the channel it is sent on.

The UE network capability information element is coded as shown in figure 9.9.3.34.1 and table 9.9.3.34.1.

The UE network capability is a type 4 information element with a minimum length of 4 octets and a maximum length of 15 octets.

NOTE:	The requirements for the support of UMTS security algorithms in the UE are specified in 3GPP TS 33.102 [18], and the requirements for the support of EPS security algorithms in 3GPP TS 33.401 [19].

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE network capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of UE network capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| EEA0 |  | 128- EEA1 |  | 128- EEA2 |  | 128- EEA3 |  | EEA4 |  | EEA5 |  | EEA6 |  | EEA7 |  | octet 3 |  |
| EIA0 |  | 128- EIA1 |  | 128- EIA2 |  | 128- EIA3 |  | EIA4 |  | EIA5 |  | EIA6 |  | EIA7 |  | octet 4 |  |
| UEA0 |  | UEA1 |  | UEA2 |  | UEA3 |  | UEA4 |  | UEA5 |  | UEA6 |  | UEA7 |  | octet 5* |  |
| UCS2 |  | UIA1 |  | UIA2 |  | UIA3 |  | UIA4 |  | UIA5 |  | UIA6 |  | UIA7 |  | octet 6* |  |
| ProSe-dd |  | ProSe |  | H.245-ASH |  | ACC-CSFB |  | LPP |  | LCS |  | 1xSR VCC |  | NF |  | octet 7* |  |
| 0 Spare |  | 0 Spare |  | EAw/oPDN |  | S1-U data |  | UP CIoT |  | CP CIoT |  | 0 Spare |  | ProSe-dc |  | octet 8* |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 9* -15* |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.9.3.34.1: UE network capability information element

Table 9.9.3.34.1: UE network capability information element

| EPS encryption algorithms supported (octet 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EPS encryption algorithm EEA0 supported (octet 3, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA0 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA0 supported |
| EPS encryption algorithm 128-EEA1 supported (octet 3, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA1 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA1 supported |
| EPS encryption algorithm 128-EEA2 supported (octet 3, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA2 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA2 supported |
| EPS encryption algorithm 128-EEA3 supported (octet 3, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA3 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA3 supported |
| EPS encryption algorithm EEA4 supported (octet 3, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA4 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA4 supported |
| EPS encryption algorithm EEA5 supported (octet 3, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA5 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA5 supported |
| EPS encryption algorithm EEA6 supported (octet 3, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA6 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA6 supported |
| EPS encryption algorithm EEA7 supported (octet 3, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA7 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA7 supported |
| EPS integrity algorithms supported (octet 4) |  |  |  |  |  |
| EPS integrity algorithm EIA0 supported (octet 4, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA0 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA0 supported |
| EPS integrity algorithm 128-EIA1 supported (octet 4, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA1 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA1 supported |
| EPS integrity algorithm 128-EIA2 supported (octet 4, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA2 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA2 supported |
| EPS integrity algorithm 128-EIA3 supported (octet 4, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA3 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA3 supported |
| EPS integrity algorithm EIA4 supported (octet 4, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA4 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA4 supported |
| EPS integrity algorithm EIA5 supported (octet 4, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA5 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA5 supported |
| EPS integrity algorithm EIA6 supported (octet 4, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA6 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA6 supported |
| EPS integrity algorithm EIA7 supported (octet 4, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA7 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA7 supported |
| UMTS encryption algorithms supported (octet 5) |  |  |  |  |  |
| UMTS encryption algorithm UEA0 supported (octet 5, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA0 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA0 supported |
| UMTS encryption algorithm UEA1 supported (octet 5, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA1 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA1 supported |
| UMTS encryption algorithm UEA2 supported (octet 5, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA2 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA2 supported |
| UMTS encryption algorithm UEA3 supported (octet 5, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA3 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA3 supported |
| UMTS encryption algorithm UEA4 supported (octet 5, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA4 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA4 supported |
| UMTS encryption algorithm UEA5 supported (octet 5, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA5 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA5 supported |
| UMTS encryption algorithm UEA6 supported (octet 5, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA6 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA6 supported |
| UMTS encryption algorithm UEA7 supported (octet 5, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA7 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA7 supported |
| UCS2 support (UCS2) (octet 6, bit 8) |  |  |  |  |  |
| This information field indicates the likely treatment of UCS2 encoded character strings by the UE. |  |  |  |  |  |
| 0 |  |  |  |  | The UE has a preference for the default alphabet (defined in |
|  |  |  |  |  | 3GPP TS 23.038 [3]) over UCS2 (see ISO/IEC 10646 [29]). |
| 1 |  |  |  |  | The UE has no preference between the use of the default alphabet and |
|  |  |  |  |  | the use of UCS2. |
| UMTS integrity algorithms supported (octet 6) |  |  |  |  |  |
| UMTS integrity algorithm UIA1 supported (octet 6, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA1 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA1 supported |
| UMTS integrity algorithm UIA2 supported (octet 6, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA2 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA2 supported |
| UMTS integrity algorithm UIA3 supported (octet 6, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA3 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA3 supported |
| UMTS integrity algorithm UIA4 supported (octet 6, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA4 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA4 supported |
| UMTS integrity algorithm UIA5 supported (octet 6, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA5 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA5 supported |
| UMTS integrity algorithm UIA6 supported (octet 6, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA6 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA6 supported |
| UMTS integrity algorithm UIA7 supported (octet 6, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA7 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA7 supported |
| NF capability (octet 7, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | notification procedure not supported |
| 1 |  |  |  |  | notification procedure supported |
| 1xSRVCC capability (octet 7, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | SRVCC from E-UTRAN to cdma2000® 1x CS not supported |
| 1 |  |  |  |  | SRVCC from E-UTRAN to cdma2000® 1x CS supported |
|  |  |  |  |  | (see 3GPP TS 23.216 [8]) |
| Location services (LCS) notification mechanisms capability (octet 7, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | LCS notification mechanisms not supported |
| 1 |  |  |  |  | LCS notification mechanisms supported (see 3GPP TS 24.171 [13C]) |
| LTE Positioning Protocol (LPP) capability (octet 7, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | LPP not supported |
| 1 |  |  |  |  | LPP supported (see 3GPP TS 36.355 [22A]) |
| Access class control for CSFB (ACC-CSFB) capability (octet 7, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | eNodeB-based access class control for CSFB not supported |
| 1 |  |  |  |  | eNodeB-based access class control for CSFB supported (see 3GPP TS 22.011 [1A]) |
| H.245 After SRVCC Handover capability (H.245-ASH) (octet 7, bit 6) This bit indicates the capability for H.245 with support and use of pre-defined codecs, and if needed, H.245 codec negotiation after SRVCC handover. |  |  |  |  |  |
| 0 |  |  |  |  | H.245 after SRVCC handover capability not supported |
| 1 |  |  |  |  | H.245 after SRVCC handover capability supported (see 3GPP TS 23.216 [8]) |
| ProSe (octet 7, bit 7) This bit indicates the capability for ProSe. |  |  |  |  |  |
| 0 |  |  |  |  | ProSe not supported |
| 1 |  |  |  |  | ProSe supported |
| ProSe direct discovery (ProSe-dd) (octet 7, bit 8) This bit indicates the capability for ProSe direct discovery. |  |  |  |  |  |
| 0 |  |  |  |  | ProSe direct discovery not supported |
| 1 |  |  |  |  | ProSe direct discovery supported |
| ProSe direct communication (ProSe-dc) (octet 8, bit 1) This bit indicates the capability for ProSe direct communication. |  |  |  |  |  |
| 0 |  |  |  |  | ProSe direct communication not supported |
| 1 |  |  |  |  | ProSe direct communication supported |
| Control-plane CIoT EPS optimization (CP CIoT) (octet 8, bit 3) This bit indicates the capability for control-plane CIoT EPS optimization. |  |  |  |  |  |
| 0 |  |  |  |  | Control-plane CIoT EPS optimization not supported |
| 1 |  |  |  |  | Control-plane CIoT EPS optimization supported |
| User-plane CIoT EPS optimization (UP CIoT) (octet 8, bit 4) This bit indicates the capability for user-plane CIoT EPS optimization. |  |  |  |  |  |
| 0 |  |  |  |  | User-plane CIoT EPS optimization not supported |
| 1 |  |  |  |  | User-plane CIoT EPS optimization supported |
| S1-u data transfer (S1-U data) (octet 8, bit 5) This bit indicates the capability for S1-u data transfer. |  |  |  |  |  |
| 0 |  |  |  |  | S1-U data transfer not supported |
| 1 |  |  |  |  | S1-U data transfer supported |
| EPS attached w/o PDN connectivity (EAw/oPDN) (octet 8, bit 6) This bit indicates the capability for EPS attached without PDN connectivity. |  |  |  |  |  |
| 0 |  |  |  |  | EPS attached without PDN connectivity not supported |
| 1 |  |  |  |  | EPS attached without PDN connectivity supported |
| Bits 8 to 7 of octet 8 are spare and shall be coded as zero. |  |  |  |  |  |
| All other bits in octet 9 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |

***** Next change *****

#### 9.9.4.10	PDN type

The purpose of the PDN type information element is to indicate either:

-	the IP version capability of the IP stack associated with the UE; or

-	non IP.

The PDN type information element is coded as shown in figure 9.9.4.10.1 and table 9.9.4.10.1.

The PDN type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDN type IEI |  |  |  | 0 Spare | PDN type value |  |  | octet 1 |

Figure 9.9.4.10.1: PDN type information element

Table 9.9.4.10.1: PDN type information element

| PDN type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | IPv4 |
| 0 | 1 | 0 |  | IPv6 |
| 0 | 1 | 1 |  | IPv4v6 |
| 1 | 0 | 0 |  | unused; shall be interpreted as "IPv6" if received by the network |
| 1 | 0 | 1 |  | non IP |
| All other values are reserved. |  |  |  |  |
| Bit 4 of octet 1 is spare and shall be coded as zero. |  |  |  |  |