NB M2M - System Information Broadcasting for Cellular IoT

Introduction

At GERAN#62 a new SI was agreed to study Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things.

An NB M2M radio access technology was introduced and a number of functions have been added to be supported in Rel-13, such as Access Control and network sharing. To support NB M2M access and these new functions, the way and the content of system information broastcast should be updated accordingly.

In this contribution, the specific design for system information is introduced, which is applicable for both Gb and S1-based architecture.

Contents of System Information

System Information Category

According to the System Information category provided in [1] and the size limit of a SIB, there requires at least four different system information (SI) types. To identify different message types, an SI type field is added for each SI.

The remaining part of this section will discuss details of these four SIs.

System Information Type 1

This message is sent on the PBSCH by the network to all mobile stations within the cell giving providing information to allow UEs early access to the network. The information elements are shown in Table 1, the total length of this category is 118 bits including 3 coverage classes.

Table 1. SI1 message content

| Information element | Description | Presence | length (bit) | Range |
| --- | --- | --- | --- | --- |
| SI type |  | M | 3 |  |
| Super Frame Number | Super frame Number, updated for every super frame. | M | 16 |  |
| SystemInfo Value Tag | SI change indication. The change of this IE indicates the change of SI except the SI containing EAB. | M | 6 |  |
| SystemInfo Value Tag for EAB | SI containing PLMN specific EAB, i.e. SI4, change indication. |  | 6 |  |
| Cell Access Related Info |  | M |  |  |
| > PLMN identity |  |  | 24 |  |
| > Cell Identity |  |  | 28 |  |
| > Tracking Area Code / Location Area Code |  |  | 16 |  |
| > Cell Barred |  |  | 1 |  |
| Access Control for Common PLMN |  |  |  |  |
| > eab-Category |  | M | 2 |  |
| > eab-BarringBitmap |  | M | 10 |  |
| Cell Selection Parameters |  |  |  |  |
| > q-RxLevMin | Parameter “Qrxlevmin” in [4], Minimum required RX level in the cell. | M | 6 |  |

System Information Type 2

This message is sent on the PBCH by the network to all mobile stations within the cell to provide information about L2/L3 specific parameters for data (re)transmission. The following table shows the contents as below with the total size of 143 bits, including five types of timers and three types of coverage class:

Table 2. SI2 message content

| Information element | Description | Presence | length (bit) | Range |
| --- | --- | --- | --- | --- |
| SI type |  | M | 3 |  |
| SI type BCH | To indicate existence of SIs on BCH_1 and BCH_2. |  | 6 |  |
| PCCH Config |  | M |  |  |
| > Default Paging Cycle | Default paging cycle |  | 4 |  |
| > Ns list |  |  |  | 1 to <maxNrOfCoverage category-1> |
| >> Ns |  |  | 3 |  |
| Uplink Power Control Parameter |  | O | ~16 |  |
| Timer list | Timer/Counter value list for different application | O |  | 1 to <maxNrOftimer> |
| > timer |  |  | 2 |  |
| DCI configuration list |  | M |  | 1 to <maxNrOfCoverage category> |
| > DCI-interval |  | M | 4 |  |
| > Channel ID |  | M | 4 |  |
| > MCS |  | M | 3 |  |
| Common RACH config list |  | O |  | 1 to <maxNrOfCoverage category> |
| > Resource Allocation |  |  | ~12 |  |
| > MCS |  |  | 3 |  |
| RACH control parameter |  |  |  |  |
| > Power control | Power control for RACH | O | ~16 |  |
| > Max trans | Maximum number of RACH transmission | M | 2 |  |
| > Ra-ResponseWindowSize | The window size based on DCI interval where corresponding Random Access Response will appear after random access initiation. | M | 2 |  |

System Information Type 3

This message is sent on the PBCH by the network to all mobile stations within the cell giving information to support UE autonomous cell selection/reselection. The UE needs to get thresholds parameters and neighbor cell information as shown in Table 2, assuming there are 5 neighbor cells the total length of this category is 73 bits. If more than 5 neighbor cells need to be supported, SI3 could be splited into several segments and therefore a Seg Number IE is included.

Table 3. Cell selection and cell re-selection related information message content

| Information element | Description | Presence | length (bit) | Range |
| --- | --- | --- | --- | --- |
| SI type |  | M | 3 |  |
| Seg Number |  |  | 3 |  |
| Cell Reselection Parameters | Power level and offset for cell selection and reselection | M |  |  |
| > q-Hyst | Parameter Qhyst in [2], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on. |  | 4 |  |
| > s-Search | Parameter “SSearch” in [2]. |  | 5 |  |
| > t-Reselection | Parameter “Treselection” in [2]. |  | 3 |  |
| Neighbour Cell Description |  |  |  | 1 to <maxNrOfNeighbourCells> |
| > Physical Cell ID | As an intra-frequency system, Physical Cell Identity (PCI) will be used to identify a unique NB M2M cell. PCI is signalled in PSS and SSS on PBSCH | M | 6 |  |
| > Cell Reselect Offset | Offset if available should be taken into account when evaluating candidates for cell re-selection. Parameter “Qoffset” in [2]. | O | 5 |  |

System Information Type 4

This message is sent on the PBCH by the network giving information of Network sharing. UEs with FULL_MOCN capability shall be able to select a preferred PLMN based on the information shown in Table 4, assuming there is one Common PLMN with 4 additional PLMN supported, the total size of this information is 147 bits.

Table 4. SI4 message content

| Information element | Description | Presence | length (bit) | Range |
| --- | --- | --- | --- | --- |
| SI type |  | M | 3 |  |
| Multi-PLMN list |  |  |  | 1 to <maxNrOfAdditionalPLMN> |
| > PLMN-Identity |  | M | 24 |  |
| > eab-Category |  | M | 2 |  |
| > eab-BarringBitmap |  | M | 10 |  |

System Information Distribution

PBSCH and PBCH will be used for SI distribution. The Channel mapping is as shown in Figure 1.

Figure 1. Channel mapping for SI distribution

PBSCH

PBSCH is designated for synchronization and broadcast information, where the synchronization and broadcast burst is used. One BCH block consists of 8 synchronization and broadcast and appears periodically in PBSCH with payload of 132 bits. SI1 will be allocated within one BCH block

UEs in a good coverage class may only need to receive one BCH block, whereas UEs in worse coverage classes may need to receive it more times, before they successfully decode the SI1 information.  For example if a UE needs to receive it four times, the total receive time for SI1 is 2.56s (4 * 640ms).

As a result

Figure 2. BCH block structure

PBCH

PBCH is a dedicated physical channel for system information only. System Information, except SI1, will be transmitted on PBCH.

Figure 3. BCH_0, BCH_1, BCH_2 block structure

There are three type of BCH block on PBCH. They are all 640ms long. The corresponding payload and system information are showed below in Table 5.

UEs in a good coverage class may only need to receive one BCH block, whereas UEs in worse coverage classes may need to receive it more times, before they successfully decode the system information.  For example if a UE needs to receive it four times, the total receive time for SI2/SI3/SI4 is 2.56s (4 * 640ms).

Table 5. BCH block and corresponding SI

| BCH block type | Payload | System Information |
| --- | --- | --- |
| BCH_0 | 152 bits | SI2 (143 bits) |
| BCH_1 | 152 bits | SI4 (147 bits) |
| BCH_2 | 92 bits | SI3 (73 bits) |

Extended SI

As described above, if more than 5 neighbour cells need to be supported, SI3 could be split into several segments with a Seg Number IE. In this case, the SIs could be transmitted as below:

Figure 3. SI segmentation

Table 6. BCH block and corresponding SI

| BCH block type | System Information |
| --- | --- |
| BCH_0 | SI2 |
| BCH_1 | SI4 |
| BCH_2 | SI3 (Seg1) |
| BCH_0 | SI2 |
| BCH_1 | SI4 |
| BCH_2 | SI3 (Seg2) |

If other new SI is defined in the future, for example, a new SI5 which includes MBMS related parameters, BCH_1 and BCH_2 could be multiplexed with SIB5, as shown in Table 7.

Figure 4. New SI introduced

Table 7. BCH block and corresponding SI

| BCH block type | System Information |
| --- | --- |
| BCH_0 | SI2 |
| BCH_1 | SI4 |
| BCH_2 | SI3 |
| BCH_0 | SI2 |
| BCH_1 | SI5 |
| BCH_2 | SI3 |

System Information Reading

Idle mode

When cell reselection occurs, the UE shall read the necessary SIs in the selected cell.

When the UE wakes up from the Power Saving Mode, the UE shall read SI1. If the SystemInfo Value Tag field in the SI1 has been changed compared with last received SI1, the UE should read the corresponding SIs on PBCH as indicated by the System value tag field. The 6 bit SystemInfo Value Tag guarantees a UEs knows whether the SI has been changed in the last 4.55 hours (2^6*4*640ms).

When the UE wakes up from DRX for Paging, the UE should read the SI1. If the System value tag field in the SI1 has been changed compared with the last received SI1, the UE should read the corresponding SIs on PBCH as indicated by the System value tag field.

Connected Mode

Considering SIs are changed infrequently, therefore in most cases may will not change during a M2M data transmission period, in order to save battery life, UE in connected mode does not need to read SIs and can read updated SIs in idle mode after data transmission.

Preliminary Analysis

Based on the above illustration, the maximum total size of all the system information is around 478 bits (~60bytes) and the whole system information can be obtained in 5.12s. As referred in [3], the acquisition of system information is calculated by mixing all these system information together and thus the time seems not quite long. However in sourcing company’s understanding, this is not realistic for future-proof design as different content categories might have different extension requirements, and some spare space might be reserved for additional functions introduced in the future. Therefore the frequency and split for each system information message shall consider the content category with their change probality/frequency, instead of simply mixing all the information together. This is under further investigation from the sourcing company.

Conclusions

In this contribution, the split of different system information messages and System Information distribution and receiving has been introduced. It is proposed to adopt the system information category in Cellular IoT and adopt the SI distribution above for NB M2M in the TR.

References

“NB M2M - System Information Broadcasting for Cellular IoT”, Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd. , FS_IoT_LC_Telco#6.

GP-140848, “NB M2M - Cell Selection and Reselection”, Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd. , GERAN#64.

GP-140603, “GSM Evolution for cellular IoT – BCCH Overview”, Ericsson.