# Pseudo CR 45.820 – EC-GSM, AGCH assignment messages

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62, see [1].

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

New (EC-)AGCH assignment messages applicable to the EC-GSM solution are described in [2] and need to be included in the TR (i.e. new sections need to be added in the TR).

## 1.3	Summary of change

The TR is updated to include the assignment messages to be sent on the EC-AGCH on TS1, as well as the new assignment message for a fixed uplink allocation on the AGCH on TS0.

Four EC-AGCH messages are included:

- Fixed Uplink Allocation 1, sent in response to an EC-RACH request using an AB

- Fixed Uplink Allocation 2, sent in response to an EC-RACH request using a NB

- Downlink Allocation, sent to a device whose Ready timer is running for the assignment of DL TBF resources

- Assignment Reject, sent to reject access requests received on the EC-RACH

The new Fixed Uplink Allocation Assignment message, to be sent on the AGCH on TS0 in response to a device performing access request on the RACH on TS0, is also included.

## 1.4	References

[1]		GP-140421, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, source VODAFONE Group Plc. GERAN#62

[2]		GP-150167, “EC-GSM, AGCH assignment messages”, source Ericsson LM. GERAN#65

# pCR to 3GPP TR 45.820-v0.4.0

First modification (added sub-clause)

##### 6.2.5.x	EC-AGCH Fixed Uplink Allocation 1 message

The EC-AGCH Fixed Uplink Allocation 1 message is sent on the EC-AGCH of TS1 to assign uplink resources to a device. The uplink resources consist of a fixed uplink allocation.

The message is sent to a device in response to an access request on the EC-RACH on TS1 using Access Burst (AB). The device is addressed with the CIoT_REQUEST_REFERENCE.

Table 6.2-xx EC-AGCH Fixed Uplink Allocation 1 information elements

< Message Type : bit (6) > < Fixed Uplink Allocation 1 message content > ::= < PAGE_MODE : bit (2) > < QUARTER_HYPERFRAME_INDICATOR : bit (2) > < USED_DL_COVERAGE_CLASS : bit (3) > -- # repetitions used when transmitting this message < CIoT_REQUEST_REFERENCE : bit (11) > < TIMING_ADVANCE_VALUE : bit (6) > < TSC : bit (3) > < SI_FP_index : bit (3) > -- reference to frequency parameters in System Information < UL Fixed allocation : < UL Fixed Allocation struct > > < DL_COVERAGE_CLASS_ASSIGNMENT : bit (3) >; -- DL coverage class for PUAN <UL Fixed Allocation struct > ::= < TIMESLOT_NUMBERS_UL_ASSIGNMENT : bit (8) > -- indication of what TNs that are part of the allocation < UPLINK_TFI_ASSIGNMENT : bit (5) > < CDMA_CODE : bit (2) > -- to be used during the uplink transfer < UL_COVERAGE_CLASS_ASSIGNMENT : bit (3) > < NUMBER_OF_UL_ALLOCATED_RLC_DATA_BLOCKS : bit (4) > < START_FN_FIRST_UL_ RLC_DATA _BLOCK : bit (5) > { 0 < START_FN_NEXT_ UL_ RLC_DATA _BLOCK : bit (3) > -- relative to end of previous UL RLC Data block ¦ 1 } -- next UL radio block starts directly after end of previous UL radio block This part is repeated (NUMBER_OF_UL_ALLOCATED_RLC_DATA_BLOCKS – 1) times { 0 -- MCS-1 to be used during the transfer ¦ 1 < MCS_VALUE : bit (3) > } -- indicates what MCS to use, if different from MCS-1 (for devices in normal coverage)

QUARTER_HYPERFRAME_INDICATOR (2 bits) The Quarter Hyperframe Indicator IE indicates what quarter hyperframe the last part of the assignment message is received in. USED_DL_COVERAGE_CLASS (3 bits) The Used DL Coverage Class IE indicates the number of repetitions that are used when transmitting this specific message. The IE is intended for other devices, with a downlink coverage class that requires fewer repetitions, which are listening to the same EC-AGCH blocks. Those devices can use this IE to determine that they can stop listening to the channel for some time. SI_FP_index (3 bits) The SI_FP_index IE specifies what frequency parameters set, as defined in the System Information on the EC-BCCH, that applies for the assignment.

Next modification (added sub-clause)

##### 6.2.5.y	EC-AGCH Fixed Uplink Allocation 2 message

The EC-AGCH Fixed Uplink Allocation 2 message is sent on the EC-AGCH of TS1 to assign uplink resources to a device. The uplink resources consist of a fixed uplink allocation. The message is sent to a device in response to an access request on the EC-RACH on TS1 using Normal Burst (NB). The device is addressed with the last 17 bits of the MS_ID included in the access request.

Table 6.2-xx EC-AGCH Fixed Uplink Allocation 2 information elements

< Message Type : bit (6) > < Fixed Uplink Allocation 2 message content > ::= < PAGE_MODE : bit (2) > < QUARTER_HYPERFRAME_INDICATOR : bit (2) > < USED_DL_COVERAGE_CLASS : bit (3) > -- # repetitions used when transmitting this message < MS_ID : bit (17) > -- could e.g. be the last 17 bits of the TLLI < TSC : bit (3) > < SI_FP_index : bit (3) > -- reference to frequency parameters in System Information < UL Fixed allocation : < UL Fixed Allocation struct > > < DL_COVERAGE_CLASS_ASSIGNMENT : bit (3) >; -- DL coverage class for PUAN <UL Fixed Allocation struct > ::= < TIMESLOT_NUMBERS_UL_ASSIGNMENT : bit (8) > -- indication of what TNs that are part of the allocation < UPLINK_TFI_ASSIGNMENT : bit (5) > < CDMA_CODE : bit (2) > -- to be used during the uplink transfer < UL_COVERAGE_CLASS_ASSIGNMENT : bit (3) > < NUMBER_OF_UL_ALLOCATED_RLC_DATA_BLOCKS : bit (4) > < START_FN_FIRST_UL_ RLC_DATA _BLOCK : bit (5) > { 0 < START_FN_NEXT_ UL_ RLC_DATA _BLOCK : bit (3) > -- relative to end of previous UL RLC Data block ¦ 1 } -- next UL radio block starts directly after end of previous UL radio block This part is repeated (NUMBER_OF_UL_ALLOCATED_RLC_DATA_BLOCKS – 1) times { 0 -- MCS-1 to be used during the transfer ¦ 1 < MCS_VALUE : bit (3) > } -- indicates what MCS to use, if different from MCS-1 (for devices in normal coverage)

QUARTER_HYPERFRAME_INDICATOR (2 bits) The Quarter Hyperframe Indicator IE indicates what quarter hyperframe the last part of the assignment message is received in. USED_DL_COVERAGE_CLASS (3 bits) The Used DL Coverage Class IE indicates the number of repetitions that are used when transmitting this specific message. The IE is intended for other devices, with a downlink coverage class that requires fewer repetitions, which are listening to the same EC-AGCH blocks. Those devices can use this IE to determine that they can stop listening to the channel for some time. SI_FP_index (3 bits) The SI_FP_index IE specifies what frequency parameters set, as defined in the System Information on the EC-BCCH, that applies for the assignment.

Next modification (added sub-clause)

##### 6.2.5.z	EC-AGCH Downlink Allocation message

The EC-AGCH Downlink Allocation message is sent on the EC-AGCH of TS1 to assign downlink resources to a device. The device is addressed with the TLLI.

Table 6.2-xx EC-AGCH Downlink Allocation information elements

< Message Type : bit (6) > < Downlink Allocation message content > ::= < PAGE_MODE : bit (2) > < QUARTER_HYPERFRAME_INDICATOR : bit (2) > < USED_DL_COVERAGE_CLASS : bit (3) > -- # repetitions used when transmitting this message < TLLI : bit (32) > < TSC : bit (3) > < SI_FP_index : bit (3) > -- reference to frequency parameters in System Information < DL_COVERAGE_CLASS_ASSIGNMENT : bit (3) >; -- DL Coverage Class used during TBF < TIMESLOT_NUMBERS_DL_ASSIGNMENT : bit (8) > -- indication of what TNs that are part of the allocation < DOWNLINK_TFI_ASSIGNMENT : bit (5) > < MINIMUM_WAIT_TIME : bit (5) > -- indicates if the device can wait before starting to read the assigned PDTCH < UL_COVERAGE_CLASS_ASSIGNMENT : bit (3) > -- to be used for subsequent UL signaling

QUARTER_HYPERFRAME_INDICATOR (2 bits) The Quarter Hyperframe Indicator IE indicates what quarter hyperframe the last part of the assignment message is received in. USED_DL_COVERAGE_CLASS (3 bits) The Used DL Coverage Class IE indicates the number of repetitions that are used when transmitting this specific message. The IE is intended for other devices, with a downlink coverage class that requires fewer repetitions, which are listening to the same EC-AGCH blocks. Those devices can use this IE to determine that they can stop listening to the channel for some time. SI_FP_index (3 bits) The SI_FP_index IE specifies what frequency parameters set, as defined in the System Information on the EC-BCCH, that applies for the assignment.

Next modification (added sub-clause)

##### 6.2.5.w	EC-AGCH Assignment Reject message

The EC-AGCH Assignment Reject message is sent on the EC-AGCH of TS1 to up to four devices to reject their access requests. The device(s) is/are addressed with the CIoT_Request_Reference, in case the access request was performed using Access Burst (AB), or with the last 17 bits of the MS_ID included in the access request, in case the access request was performed using Normal Burst (NB).

Table 6.2-xx EC-AGCH Assignment Reject information elements

< Message Type : bit (6) > < Assignment Reject message content > ::= < PAGE_MODE : bit (2) > < USED_DL_COVERAGE_CLASS : bit (3) > -- # repetitions used when transmitting this message -- First rejected device { 0 < CIoT_Request_Reference_1 : bit (11) > -- Identity for access request performed using AB ¦ 1 < MS_ID_1 : bit (17) > } -- Identity for access request performed using NB < WAIT_INDICATION_1 : bit (5) > -- waiting time for first device -- Second rejected device { 0 < CIoT_Request_Reference_2 : bit (11) > -- Identity for access request performed using AB ¦ 1 < MS_ID_2 : bit (17) > } -- Identity for access request performed using NB < WAIT_INDICATION_2 : bit (5) > -- waiting time for second device -- Third rejected device { 0 < CIoT_Request_Reference_3 : bit (11) > -- Identity for access request performed using AB ¦ 1 < MS_ID_3 : bit (17) > } -- Identity for access request performed using NB < WAIT_INDICATION_3 : bit (5) > -- waiting time for third device -- Fourth rejected device < CIoT_Request_Reference_4 : bit (11) > -- Identity for access request performed using AB < WAIT_INDICATION_4 : bit (5) > -- waiting time for fourth device

USED_DL_COVERAGE_CLASS (3 bits) The Used DL Coverage Class IE indicates the number of repetitions that are used when transmitting this specific message. The IE is intended for other devices, with a downlink coverage class that requires fewer repetitions, which are listening to the same EC-AGCH blocks. Those devices can use this IE to determine that they can stop listening to the channel for some time. WAIT_INDICATION_n (5 bits) The Wait Indication IE provides the time the device shall wait before attempting another request.

Next modification (added sub-clause)

##### 6.2.5.u	AGCH Fixed Uplink Allocation Assignment message

The AGCH Fixed Uplink Allocation Assignment message is sent on the AGCH of TS0 to assign uplink resources to a device. The uplink resources consist of a fixed uplink allocation.

The message is sent to a device in response to an access request on the RACH on TS0 using either Access Burst (AB), with a 11 bit format starting with ‘100’, or Normal Burst (NB). The device is addressed either with the Request_reference and Request_FN_info, in case the access request was performed using AB, or with the 30 last bits of the MS_ID included in the access request, in case the access request was performed using NB.

Table 6.2-xx EC-AGCH Fixed Uplink Allocation 1 information elements

| IEI | Information element | Type / Reference | Presence | Format | length |
| --- | --- | --- | --- | --- | --- |
|  | L2 Pseudo Length | L2 Pseudo Length 10.5.2.19 | M | V | 1 |
|  | RR management Protocol Discriminator | Protocol Discriminator 10.2 | M | V | 1/2 |
|  | Skip Indicator | Skip Indicator 10.3.1 | M | V | 1/2 |
|  | Fixed Uplink Allocation Assignment Message Type | Message Type 10.4 | M | V | 1 |
|  | Page Mode | Page Mode 10.5.2.26 | M | V | 1/2 |
|  | Feature Indicator | Feature Indicator 10.5.2.76 | M | V | 1/2 |
|  | FUAA Rest Octets | FUAA Rest Octets 10.5.2.xx | M | V | 19 |

# Table 10.5.2.xx.1: FUAA Rest Octets information element

<FUAA Rest Octets> ::= { 0 < Request_reference : bit (11) > -- in case of AB request < Request_FN_info : bit (16) > -- Indicating FN of received access request message < TIMING_ADVANCE_VALUE : bit (6) > ¦ 1 < MS_ID : bit (30) > } -- in case of NB request < TSC : bit (3) > < SI_FP_index : bit (3) > -- reference to frequency parameters in EC-BCCH System Information < UL Fixed allocation : < UL Fixed Allocation struct > > <UL Fixed Allocation struct > ::= < TIMESLOT_NUMBERS_UL_ASSIGNMENT : bit (8) > -- indication of what TNs that are part of the allocation < UPLINK_TFI_ASSIGNMENT : bit (5) > < CDMA_CODE : bit (2) > -- to be used during the uplink transfer < NUMBER_OF_UL_ALLOCATED_RLC_DATA_BLOCKS : bit (4) > < START_FN_FIRST_UL_ RLC_DATA _BLOCK : bit (5) > { 0 < START_FN_NEXT_ UL_ RLC_DATA _BLOCK : bit (3) > -- relative to end of previous UL RLC Data block ¦ 1 } -- next UL radio block starts directly after end of previous UL radio block This part is repeated (NUMBER_OF_UL_ALLOCATED_RLC_DATA_BLOCKS – 1) times < EGPRS_CHANNEL_CODING_COMMAND : bit (4) >

SI_FP_index (3 bits) The SI_FP_index IE specifies what frequency parameters set, as defined in the System Information on the EC-BCCH, that applies for the assignment.

Last modification

### 6.2.6	Bibliography

[6.2-1]	GP-020645, “Discussion paper – Fixed Allocation”, source Nokia. GERAN#9.

[6.2-2]	GPC150064, “EC GSM – EC-SCH design, performance and mapping”, source Ericsson LM, GERAN1 Adhoc#1 on FS_IoT_LC

[6.2-3]	GP-140603, “GSM Evolution for cellular IoT – BCCH overview”, source Ericsson LM. GERAN#63.

[6.2-n]	GP-150167, “EC-GSM, AGCH assignment messages”, source Ericsson LM. GERAN#65

End of modifications