# Pseudo CR 45.820 – Narrowband LTE Coverage Performance Evaluation (update of GP-150799)

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62.

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

In GERAN#67, a new system design, i.e., NB LTE, based on the current LTE system but using only 200 kHz bandwidth is proposed in GP-150779. This document contains the coverage performance evaluation of NB LTE.

## 1.3	Summary of change

The coverage evaluation of NB LTE.

# pCR to 3GPP TR 45.820-v1.4.0

### 7.4.6 Concept evaluation

#### 7.4.6.1 Coverage evaluation

Omitted

##### 7.4.6.1.3 Downlink shared data and common control channels

###### 7.4.6.1.3.1 Evaluation methodology and assumptions

The evaluation methodology as detailed in section 5.1 is fully adopted. The relevant parameters are set according to Annex C and included in Table 7.4.6.1.3-1 for easy reference.

Table 7.4.6.1.3-1: Assumptions for link level simulations.

| Parameter | Value |
| --- | --- |
| Frequency band | 900 MHz |
| Propagation channel model | TU |
| Doppler spread | 1 Hz |
| Interference/noise | Sensitivity |
| Antenna configuration | BS: 1T2R MS: 1T1R |
| Frequency error | F_offset(t) = F_est_error + (F_drift_inactive *T_inactive) + (F_drift_active * t). |
| NB LTE specific frequency error (F_est_error) | Randomly chosen from [-50, 50] Hz |
| Frequency drift rate (F_drift_active) | 22.5 Hz/second |
| BS transmit power per 200KHz (dBm) | 43 |
| MS transmit power (dBm) | 23 dBm |
| Thermal noise density (dBm/Hz) | -174 |
| BS Receiver noise figure (dB) | 3 |
| MS Receiver noise figure (dB) | 5 |
| Interference margin (dB) | 0 |
| Receiver processing gain (dB) | 0 |

The modulation and coding scheme (MCS) considered is to identify the maximum coupling loss (MCL) that can meet the 10% BLER target. The used MCSs in the evaluation are summarized in Table 7.4.6.1.3-2. For M-PDSCH, the data rate above SNDCP is 3.1 kbps.

Table 7.4.6.1.3-2: Summary of the MCSs used for different downlink channels.

|  | M-PDSCH | M-PBCH | M-PDCCH | M-EPDCCH |
| --- | --- | --- | --- | --- |
| Coding scheme | Conv. Code | Conv. Code | Conv. Code | Conv. Code |
| Code rate (per repetition) | 0.34 | 0.08 | 0.11 | 0.067 |
| modulation | QPSK | QPSK | QPSK | QPSK |
| # repetition | 24 | 4 | 8 | 12 |
| BW (Hz) | 180000 | 180000 | 180000 | 180000 |

###### 7.4.6.1.3.2 Results

The MCLs for M-PDSCH, M-PBCCH, M-PDCCH and M-EPDCCH are summarized in table 7 .4.6.1.3-3

Table 7.4.6.1.3-3: MCL of NB LTE

| Logical channel name | M-PDSCH | M-PBCH | M-PDCCH | M-EPDCCH |
| --- | --- | --- | --- | --- |
| (1) Tx power (dBm) | 43 | 43 | 43 | 43 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (3) Receiver noise figure (dB) | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 180,000 | 180,000 | 180,000 | 180,000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -116.4 | -116.4 | -116.4 | -116.4 |
| (7) Required SINR (dB) | -4.7 | -11.7 | -6 | -6 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -121.1 | -128.1 | -122.4 | -122.4 |
| (9) Rx processing gain | 0 | 0 | 0 | 0 |
| (10) MCL = (1) (8) + (9) (dB) | 164.1 | 171.1 | 165.4 | 165.4 |

##### 7.4.6.1.4 Uplink shared data channel

###### 7.4.6.1.4.1 Evaluation methodology and assumptions

The evaluation methodology as detailed in section 5.1 is fully adopted. The relevant parameters are set according to Annex C and included in Table 7.4.6.1.3-1 for easy reference.

In NB LTE, there is only uplink share data channel, i.e., M-PUSCH, which carries both uplink control and data. In the extreme coverage case, Turbo code with rate 0.21 is used. One subcarrier, i.e., 2.5 KHz is used, and the modulation is BPSK. The data rate above SNDCP is 354 bps.

###### 7.4.6.1.4.2 Results

The MCLs for M-PUSCH is given in Table 7.4.6.1.4-1.

Table 7.4.6.1.3-3: MCL of NB LTE

| Logical channel name | M-PUSCH |
| --- | --- |
| (1) Tx power (dBm) | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 3 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 2,500 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -137.0 |
| (7) Required SINR (dB) | -5.6 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -142.6 |
| (9) Rx processing gain | 0 |
| (10) MCL = (1) (8) + (9) (dB) | 165.6 |