# Pseudo CR 45.820 – Simulation Results for EC-GSM Coexistence with UTRA and E-UTRA (Update of GP-150765)

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62.

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2 	Reason for change

This contribution evaluates the impact on EC-GSM from UTRA or E-UTRA aggressors in two typical GSM frequency deployments.

## 1.3 	Summary of change

Simulation results for EC-GSM with UTRA and E-UTRA. The updated text is highlighted in blue.

# pCR to 3GPP TR 45.820-v1.4.0

First modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Release specifications".

[3]	3GPP TR 36.888: "Study on provision of low cost Machine-Type Communications (MTC) user Equipment (UEs) based on LTE (v12.0.0)".

[4]	3GPP TR 45.914: "Circuit switched voice capacity evolution for GSM/EDGE Radio Access Network (GERAN)".

[5] 	3GPP TS 45.005: "Radio transmission and reception (v12.2.0) ".

[6]	3GPP TS 24.008: "radio interface Layer 3 specification; Core network protocols; Stage 3".

[7]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[8]	3GPP TS 36.331: "Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[10]	3GPP TS 45.001, "Physical layer on the radio path".

[11]	3GPP TS 45.002: "GSM/EDGE Radio Access Network; Multiplexing and multiple access on the radio path".

[12]	3GPP TS 45.003: "GSM/EDGE Radio Access Network; Channel coding".

[13]	3GPP TS 45.004: "GSM/EDGE Radio Access Network; Modulation".

[14]	3GPP TS 44.060: "GSM/EDGE Radio Access Network; Radio Link Control / Medium Access Control (RLC/MAC) protocol".

[15]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulations".

[16]	3GPP TR 33.863: “Study on battery efficient security for very low throughput Machine Type Communication Devices“

[17]	3GPP TR 33.860: “Study on Access Security Enhancements with relation to cellular IoT”[18]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[19]	3GPP TS 24.007 "Mobile radio interface signalling layer 3; General Aspects”

[20]	3GPP TS 24.301 "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3"

[21]	3GPP TS 25.942: “Radio Frequency (RF) system scenarios”.

[22]	3GPP TS 36.942: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[23]	3GPP TS 36.413: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”

[24] 	3GPP TS 27.007: “AT command set for User Equipment (UE)”

[25]	3GPP TS 23.236: “Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes”

[26]	3GPP TS 45.010, “Radio subsystem synchronization”.

[27]	3GPP TS 37.014, “E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”

Second modification

#### 6.2.6.X	Co-existence simulations with GSM aggressor EC-GSM victim

##### 6.2.6.x.1 	Simulation assumptions

Table 6.2.6.x-1 shows the four simulated cases. The 4/12 frequency reuse is intended to capture a BCCH layer, while the 3/9 reuse is intended to capture a TCH traffic layer.

Table 6.2.6.x-1 Simulation cases.

| Cases | Aggressor | Victim | Link direction | EC-GSM frequency reuse |
| --- | --- | --- | --- | --- |
| 1 | UTRA | EC-GSM | Downlink | 4/12 |
| 2 | UTRA | EC-GSM | Uplink | 4/12 |
| 3 | UTRA | EC-GSM | Downlink | 3/9 |
| 4 | UTRA | EC-GSM | Uplink | 3/9 |
| 5 | E-UTRA | EC-GSM | Downlink | 4/12 |
| 6 | E-UTRA | EC-GSM | Uplink | 4/12 |
| 7 | E-UTRA | EC-GSM | Downlink | 3/9 |
| 8 | E-UTRA | EC-GSM | Uplink | 3/9 |

In general, the agreed assumptions in Annex G.2 have been followed.  Table 6.2.6.x-2 highlights some of the most important agreed parameters, as well as presents a few new parameters specific for EC-GSM system. In addition to the ISD of 750 meters, the results with an ISD of 1732 meters are also provided.

Table 6.2.6.x-2 Simulation assumptions for EC-GSM

| Parameter | Setting |
| --- | --- |
| BS maximum transmit power | 43 dBm |
| BS Minimum transmit power | Not applicable, since no power control was active for the EC-GSM victim. |
| MS maximum transmit power | 23 dBm |
| MS minimum transmit power | Not applicable, since no power control was active for the EC-GSM victim. |
| MS antenna gain | -4 dBi |
| Frequency reuse | 4/12: BCCH layer or 3/9: TCH layer |
| Frequency deployment | 1 carrier per cell, both in case of 4/12 and 3/9 reuse. |
| Building Penetration Loss | Scenario 1 with inter-site correlation coefficient 0.5 |
| System load | Alt 1: Fully loaded system, i.e. all TS occupied. Alt 2: Lightly loaded system, only on TS occupied. |
| ACS | 58 dB Based on the bandwidth of the guard band, ACS for EC-GSM BS and MS are derived from 3GPP TS 45.005 (sub-clause 6.3.1) reference interference levels at the third adjacent channel [3]. |
| ACLR1 | UTRA: 45 dB for BS and 33 dB for UE E-UTRA: 45 dB for BS and 30 dB for UE |
| Gap between center frequency of EC-GSM and UTRA or E-UTRA | EC-GSM and UTRA: 2.8 MHz2 EC-GSM and E-UTRA: 5.2 MHz2 |
| Note1: Bandwidth conversion is done according to [6.2-Y1]. Note 2: The gap is based on Foffset,RAT and the allowed channel raster for each RAT, see clauses 4.5.2 & 4.6.1 [27] |  |

Two types of load in the victim EC-GSM system have been assumed. In the first step, a fully loaded system is assumed to provoke maximum absolute outage. In the second step, the scenarios displaying maximum outage, have been re-simulated with a low load, i.e. only one TS occupied by EC-GSM. In such a setup, we can establish a baseline of the observed total outage before introducing the UTRA or E-UTRA aggressors to the system, which can be used to reflect the maximum outage increase after activating the UTRA or E-UTRA aggressors.

##### 6.2.6.x.2 	Results

Table 6.2.6.x.-3 to Table 6.2.6.x.-6 summarizes the results from the UTRA or ETURA as the aggressor, respectively. The relative performance degradation, i.e., increase in outage, of EC-GSM performance when exposed to an uncoordinated UTRA or E-UTRA aggressor is at most 0.6 percentage points for the UL when EC-GSM is fully loaded. Negligible degradation for the DL is observed. When the EC-GSM load is reduced to a single TS the relative outage degradation increases, but the levels never go above 1 percent-unit.

The absolute outage levels are low for the DL, but are as expected somewhat impacted in the UL due to the reduction in MS output power to 23 dBm. In absolute terms at most 3.2% outage is observed in the performed set of simulations.

Table 6.2.6.x-3 Summary of EC-GSM performance loss due to interference from a UTRA aggressor, ISD 1732 meters.

| Cases | Aggressor | Victim | Link direction | EC-GSM frequency reuse | EC-GSM system load | Outage [%] | Relative outage degradation [%-points] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UTRA | EC-GSM | Downlink | 4/12 | 8TS | 0.2% | 0.0 |
| 2 | UTRA | EC-GSM | Uplink | 4/12 | 8TS | 3.0% | 0.6 |
| 2’ | UTRA | EC-GSM | Uplink | 4/12 | 1TS | 2.5% | 1.0 |
| 3 | UTRA | EC-GSM | Downlink | 3/9 | 8TS | 0.2% | 0.1 |
| 4 | UTRA | EC-GSM | Uplink | 3/9 | 8TS | 3.2% | 0.6 |
| 4’ | UTRA | EC-GSM | Uplink | 3/9 | 1TS | 2.6% | 0.6 |

Table 6.2.6.x-4 Summary of EC-GSM performance loss due to interference from a UTRA aggressor, ISD 750 meters.

| Cases | Aggressor | Victim | Link direction | EC-GSM frequency reuse | EC-GSM system load | Outage [%] | Relative outage degradation [%-points] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UTRA | EC-GSM | Downlink | 4/12 | 8TS | 0.1% | 0.1 |
| 2 | UTRA | EC-GSM | Uplink | 4/12 | 8TS | 0.4% | 0.0 |
| 3 | UTRA | EC-GSM | Downlink | 3/9 | 8TS | 0.1% | 0.1 |
| 4 | UTRA | EC-GSM | Uplink | 3/9 | 8TS | 0.6% | 0.1 |

Table 6.2.6.x-5 Summary of EC-GSM performance loss due to interference from a E-UTRA aggressor, ISD 1732 meters

| Cases | Aggressor | Victim | Link direction | EC-GSM frequency reuse | EC-GSM system load | Outage [%] | Relative outage degradation [%-points] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | E-UTRA | EC-GSM | Downlink | 4/12 | 8TS | 0.2% | 0.0 |
| 2 | E-UTRA | EC-GSM | Uplink | 4/12 | 8TS | 2.7% | 0.3 |
| 2’ | E-UTRA | EC-GSM | Uplink | 4/12 | 1TS | 2.3% | 0.8 |
| 3 | E-UTRA | EC-GSM | Downlink | 3/9 | 8TS | 0.1% | 0.0 |
| 4 | E-UTRA | EC-GSM | Uplink | 3/9 | 8TS | 2.9% | 0.3 |
| 4’ | E-UTRA | EC-GSM | Uplink | 3/9 | 1TS | 2.5% | 0.5 |

Table 6.2.6.x-6 Summary of EC-GSM performance loss due to interference from a E-UTRA aggressor, ISD 750 meters

| Cases | Aggressor | Victim | Link direction | EC-GSM frequency reuse | EC-GSM system load | Outage [%] | Relative outage degradation [%-points] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | E-UTRA | EC-GSM | Downlink | 4/12 | 8TS | 0.0% | 0 |
| 2 | E-UTRA | EC-GSM | Uplink | 4/12 | 8TS | 0.4% | 0 |
| 3 | E-UTRA | EC-GSM | Downlink | 3/9 | 8TS | 0.0% | 0 |
| 4 | E-UTRA | EC-GSM | Uplink | 3/9 | 8TS | 0.5% | 0 |

##### 6.2.6.x.4 	Conclusions

This contribution presents the impact on EC-GSM system capacity from an adjacent UTRA or E-UTRA deployment. The results indicate that the impact from the aggressor on EC-GSM ranges between 0 and 1 percent units in outage. Based on this observation it can be concluded that the impact from UTRA or E-UTRA onto EC-GSM can be considered minimal.

For more details on the investigation and the results, see [6.2-Y].

Second modification

# Annex H: Bibliography

[6.2-1]	GP-020645, "Discussion paper – Fixed Allocation", source Nokia. GERAN#9.

[6.2-2]	GPC150064, "EC GSM – EC-SCH design, performance and mapping", source Ericsson LM, GERAN1 Adhoc#1 on FS_IoT_LC

[6.2-3]	GP-140603, "GSM Evolution for cellular IoT – BCCH overview", source Ericsson LM. GERAN#63.

[6.2-4]	GP-150208, "EC-GSM, EC-SCH design, performance and mapping", source Ericsson LM, GERAN#65

[6.2-5]	GPC150089, "EC-GSM, FCCH overview", source Ericsson LM, GERAN1 Adhoc#1 on FS_IoT_LC

[6.2-6]	GPC150437, "EC-GSM, Network synchronization with alternative EC-SCH design (update of GPC150198)", source Ericsson LM. GERAN#65.

[6.2-7]	GP-150155, "EC-GSM, Performance evaluation - Coverage improvement target", source Ericsson LM, GERAN#65.

[6.2-8]	GPC150064, "EC-GSM, EC-SCH design, performance and mapping", source Ericsson LM. GERAN Ad Hoc #1 on FS_IoT_LC.

[6.2-9]	GP-150140, "EC-GSM, Sensitivity to carrier frequency offset", source Ericsson LM. GERAN#65.

[6.2-10]	GP-150429, "EC-GSM, On the impact to legacy GSM/EDGE Base stations", source Ericsson LM, GERAN#66.

[6.2-11]	GP-150355, "EC-GSM, Overlaid CDMA for extended coverage", source Ericsson LM. GERAN#66.

[6.2-12]	GP-150139, "EC-GSM, Incremental redundancy and chase combining" source Ericsson LM. GERAN#65.

[6.2-13]	GPC150485, “EC-GSM, Simulation Results for Coexistence with GSM (update of GPC150445)”, source Ericsson LM. GERAN Ad Hoc #3 on FS_IoT_LC.

[6.2-Y]	GP-150764 EC-GSM, “Simulation Results for Coexistence with UTRA and E-UTRA”, source Ericsson LM, GERAN#67.

[6.2-Y1]	GPC150304, “CIoT-Coexistence with LTE and UMTS (update of GPC150182)”, source VODAFONE Group Plc., GERAN Ad hoc#2 on FS_IoT_LC.

[8.3.1-1]	GP-150496, " NB M2M - Evaluation of Transmission Efficiency (update of GPC150153)" source Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd. GERAN#66.

[6.3-1]	Militzer, Burkhard, et.al.,  “Evolutionary search for low autocorrelated binary sequences”,  Evolutionary Computation, IEEE Transactions, Vol. 2, 1998.