# Pseudo CR 45.820 – Link Layer – Scheduling

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62, see [1].

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

In GERAN#67, a new system design, i.e., Narrowband LTE (NB-LTE), based on the current LTE system but use only 200 KHz bandwidth is proposed in GP-150779. This document contains the description the scheduling principles for the PDSCH and PUSCH and the related Downlink Control Information (DCI).

## 1.3	Summary of change

Addition of the scheduling principles for the PDSCH and PUSCH and the related Downlink Control Information (DCI) on (E)PDCCH.

# pCR to 3GPP TR 45.820-v1.4.0

## 7.4.4	Link Layer

<Unmodified sections omitted>

### 7.4.4.2 	Scheduling

As in LTE, the uplink and downlink shared channels of NB-LTE are fully scheduled by the base station. To do so, the base station sends downlink control information (downlink assignments and uplink grants) to the MS via a control channel. There are two variants of such a control channel in LTE, namely the PDCCH (Physical Downlink Control Channel) and the EPDCCH (Enhanced Physical Downlink Control Channel).

Due to benefits in terms of performance (see section 7.4.6.1.3) and other benefits outlined below, the physical downlink control channel for NB-LTE should follow an EPDCCH-like design and is herein denoted as M-EPDCCH.

Furthermore, the following sections outline a bundling scheme applicable to M-PDSCH, M-PUSCH and M-EPDCCH. To allow robust transmission of sufficiently large transport blocks and DCI in challenging coverage situations it follows the TTI Bundling mechanism defined in [3GPP TS 36.321].

#### 7.4.4.2.1	Scheduling of M-PDSCH

In an M-subframe, the minimum scheduling unit is 1 PRB (1 ms x 180 kHz). Hence, in principle, up to 6 MSs may be scheduled in an M-subframe. Following the principle of LTE, a transport block is mapped to the scheduling units (PRBs) assigned to a MS in one M-subframe. Unlike LTE, these scheduling units now appear in time dimension (see section 7.4.2.2) which results in the structure shown in Figure 7.4.4.2.1-1. Besides that, the structure and DCI formats of the existing LTE PDCCH are re-used.

After the M-EPDCCH occurrence in M-subframe n, the MS performs the blind decoding. If it detects a valid DCI on the M-EPDCCH, it attempts to receive the corresponding M-PDSCH in M-subframe n+k. That means a downlink assignment received in M-subframe n is valid for M-subframe n+k. In order to further relax the processing demand on the MS and to account for timing advance or DL/UL retuning of half-duplex MSs, it may be preferable to expand the pre-scheduling time. That means, a grant or assignment received on M-EPDCCH in M-subframe n becomes valid only in M-subframe n+k where k >= 2. The following figures illustrate the case where k=1.

Figure 7.4.4.2.1-1:  M-PDSCH resource allocation for NB-LTE using existing M-EPDCCH formats for 1.4 MHz in combination with forward scheduling.

For MSs in basic coverage, the EPDDCH occupies only a subset of the subframes within the M-subframe as shown in Figure 7.4.4.2.1-1, allowing MS to sleep longer. Secondly, the MS has time to decode the DCI and, in case of uplink, to prepare the corresponding transport block. The reduced time diversity compared to the PDCCH based approach may have slightly negative impact on the reception performance. However, as shown in section 7.4.6.1.3 the M-EPDCCH generally outperforms the M-PDCCH.

With decreasing channel quality the number of bits that can be carried in the M-PDSCH of an M-subframe decreases, the transport block size that can be received with a desirable block error rate decreases, increasing the  relative L1/2 protocol overhead. For the highest targeted Maximum Coupling Loss (MCL), the expected M-PDSCH data rate is in the order of a few kbps (see section 7.4.6.1.3), corresponding to a few octets in each M-subframe. This is in the same order or even less than the L2 protocol overhead and the L1 CRC. In order to reduce the relative protocol overhead, the size of a transport block needs to be increased by expanding it in the time domain beyond one M-subframe. The same basic principle is supported in LTE and is known as “TTI bundling” (see section 5.4.2.1 of [3GPP TS 36.321]).

When applied in the context of NB-LTE as described above, a downlink assignment received in M-subframe n is not only valid for an initial transmission in M-subframe n+1 but also for a number of retransmissions in the subsequent M-subframes. On the receiver side (here the MS), the transmission attempts are soft-combined. In order to efficiently address the coverage extension of 20 dB, the bundling level in NB-LTE should be configurable on a per-MS basis. It could be configured semi-statically by RRC or be signalled as part of the downlink control information. In section 7.4.6.1.3 bundling levels of 1, 6 and 36 have been used. That means, an assignment is valid for the following 1, 6 or 36 M-subframes.

But not only the M-PDSCH but also the M-EPDCCH requires repetitions in order to ensure successful reception by MSs in “robust” or “extreme” coverage situations. The M-EPDCCH evaluation in section 7.4.6.1.3 shows that the transmission of a DCI block (UL grant, DL assignment) of 64 bit (48 DCI + 16 CRC) requires 0.5, 4 and 48 PRBs (subframes) in basic, robust and extreme coverage, respectively.

The example in Figure 7.4.4.2.1-2 shows how an M-EPDCCH using 4 PRBs provides a DL assignment to a MS in robust coverage.

Figure 7.4.4.2.1-2: M-EPDCCH for a MS in robust coverage extension mode
(M-EPDCCH on 4 PRBs followed by M-PDSCH on 6 entire M-subframes)

In “extreme” coverage conditions (MCL ~ 164 dB) the M-EPDCCH resources configurable in one M-subframe will not be sufficient as shown in section 7.4.6.1.3. Hence, the M-EPDCCH transmission needs to be expanded across several M-subframes. This expansion does not follow naturally from the time domain expansion principle by a factor of 6 outlined in section 7.4.2.2. However, it is very similar to the bundling scheme introduced for the M-PDSCH. An example for extreme coverage is shown in Figure 7.4.4.2.1-3 (using the repetition levels determined in section 7.4.6.1.3).

Figure 7.4.4.2.1-3: M-EPDCCH for a MS in extreme coverage extension mode
NOTE: Each square in this figure depicts an M-subframe (6 ms) consisting of 6 subframes!

A MS should not be required to read and blindly decode M-EPDCCH continuously in all M-subframes. It could be configurable whether the M-EPDCCH recurs only once every M-PDSCH bundling window or e.g. twice or four times within this cycle. If, as in the figure above, the MS decodes a valid assignment in M-subframe n, it knows that the next possible M-EPDCCH occurrence will be after “bundling factor” subframes. Furthermore, the MS would only attempt to decode M-EPDCCH in M-subframes where it is not in DRX.

It is also worth noting that the forward scheduling across many M-subframes enables temporary use of M-EPDCCH resource elements for M-PDSCH. In the example depicted in Figure 7.4.4.2.1-3, the M-EPDCCH on the left assigns all 6 PRBs in each of 36 subsequent M-subframes to a single MS. Consequently, this UE does not attempt to receive M-EPDCCH in M-subframes n+8 to n+43 but rather expects these additional resource elements to be allocated to the M-PDSCH (except for static allocations such as MIB, SIB, PSS/SSS). Other MSs might try to receive M-EPDCCH in those M-subframes (whenever they are not in DRX) but will not find a valid DCI. Only from M-subframe n+44 onwards the eNB will start sending M-EPDCCH on the M-EPDCCH resource elements.

#### 7.4.4.2.2	Scheduling of M-PUSCH

With the time expansion principle there are still 6 PRBs per M-subframe that can be allocated to the M-PUSCH of up to 6 MSs. Figure 7.4.4.2.2-1 shows that the M-EPDCCH structure outlined above for the downlink is also well-suited to allocate M-PUSCH resources.

Figure 7.4.4.2.2-1: Scheduling M-PUSCH resources by M-EPDCCH

Without any changes, the DCI formats for scheduling the LTE PUSCH could be re-used to assign M-PUSCH resources on a NB-LTE carrier. As mentioned above, it is however desirable to allocate even narrower frequency resources for MSs in bad channel conditions. Due to the time domain expansion a PRB covers now 30 kHz bandwidth, i.e., 12 subcarriers of 2.5 kHz each. In “extreme” coverage (~ 164 dB MCL) it is favourable to allocate as little as one subcarrier to a MS. Consequently, the smallest scheduling unit of the M-PUSCH is 6 ms (1 M-subframe) in time- and 1 subcarrier (2.5 kHz) in frequency domain. To achieve this, the M-EPDCCH needs to support the additional granularity in resource allocation.

As for the downlink, a single M-subframe is not sufficient for the M-PUSCH to carry a transport block of reasonable size. Therefore, multiple subsequent M-subframes are bundled and the receiver (eNB) accumulates the M-PUSCH across all these M-subframes. This in combination with the robust M-EPDCCH transmission is depicted in Figure 7.4.4.2.2-2. Performance evaluation results for the M-PUSCH are provided in section 7.4.6.1.4.

Figure 7.4.4.2.2-2: Scheduling M-PUSCH resources by M-EPDCCH in extreme coverage