# Pseudo CR 45.820 – Narrowband LTE Random Access Performance Evaluation (update of GP-150947)

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62.

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

In GERAN#67, a new system design, i.e., NB LTE, based on the current LTE system but using only 200 kHz bandwidth is proposed in GP-150779. This document contains the random access performance evaluation of NB LTE.

## 1.3	Summary of change

The random access performance evaluation of NB LTE. The updated text is highlighted in red.

# pCR to 3GPP TR 45.820-v1.4.0

First modification

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Release specifications".

[3]	3GPP TR 36.888: "Study on provision of low cost Machine-Type Communications (MTC) user Equipment (UEs) based on LTE (v12.0.0)".

[4]	3GPP TR 45.914: "Circuit switched voice capacity evolution for GSM/EDGE Radio Access Network (GERAN)".

[5] 	3GPP TS 45.005: "Radio transmission and reception (v12.2.0) ".

[6]	3GPP TS 24.008: "radio interface Layer 3 specification; Core network protocols; Stage 3".

[7]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[8]	3GPP TS 36.331: "Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[10]	3GPP TS 45.001, "Physical layer on the radio path".

[11]	3GPP TS 45.002: "GSM/EDGE Radio Access Network; Multiplexing and multiple access on the radio path".

[12]	3GPP TS 45.003: "GSM/EDGE Radio Access Network; Channel coding".

[13]	3GPP TS 45.004: "GSM/EDGE Radio Access Network; Modulation".

[14]	3GPP TS 44.060: "GSM/EDGE Radio Access Network; Radio Link Control / Medium Access Control (RLC/MAC) protocol".

[15]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulations".

[16]	3GPP TR 33.863: “Study on battery efficient security for very low throughput Machine Type Communication Devices“

[17]	3GPP TR 33.860: “Study on Access Security Enhancements with relation to cellular IoT”[18]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[19]	3GPP TS 24.007 "Mobile radio interface signalling layer 3; General Aspects”

[20]	3GPP TS 24.301 "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3"

[21]	3GPP TS 25.942: “Radio Frequency (RF) system scenarios”.

[22]	3GPP TS 36.942: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[23]	3GPP TS 36.413: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”

[24] 	3GPP TS 27.007: “AT command set for User Equipment (UE)”

[25]	3GPP TS 23.236: “Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes”

[26]	3GPP TS 45.010, “Radio subsystem synchronization”.

[29]     3GPP Technical Specification 36.104, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception,” www.3gpp.org.

Second modification

### 7.4.6 Concept evaluation

#### 7.4.6.1 Coverage evaluation

Omitted

##### 7.4.6.1.2 Random access request and uplink ToA estimation

###### 7.4.6.1.2.1 Simulation settings

According the agreed methodology in subchapter 5.7 and the simulation assumptions in Annex C, various aspects of the random access channels are evaluated. The simulation assumptions used are summarized in Table 7.4.6.1.2-1. The PRACH formatted used in the evaluations are described in subchapter 7.4.3.3.1.

Table 7.4.6.1.2-1: Simulation parameters

| Parameter | Value |
| --- | --- |
| Channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | 1 Tx; 2 Rx |
| Cell size | 35 km |
| Timing uncertainty | Randomly drawn from the interval [0, 233.3 us]. See Note 1. |
| Frequency error | Randomly drawn from the set {-50 Hz, 50 Hz}. See Note 2. |
| MS frequency drift | Randomly drawn from the set {-22.5 Hz/s, 22.5 Hz/s}. |
| Preambles | Zadoff-Chu sequences with roots 1,…, 64. |
| Number of channel realizations | 5,000 |
| Sampling rate at the receiver | 160 KHz |
| Note 1: 233.3 us is the maximum round-trip delay in the cell of size 35 km. A timing error of +/- 4.7 μs is added implicitly in the simulation. Note 2: 99% frequency errors are within 50 Hz in NB-LTE cell search. |  |

###### 7.4.6.1.2.2 PRACH coverage performance

For PRACH preamble detection at the receiver, a threshold is set such that false alarm detection rate is sufficiently low when thermal noise is fed to the receiver. When repetitions are used (PRACH formats 2 and 3), detection statistics of each three PRACH segments that are in the same PRACH slot are coherently combined, and detection statistics are non-coherently combined across PRACH slots and receive antennas.

As reference values, LTE specifies that PRACH false alarm rate is 0.1% and PRACH detection rate is 99% (TS 36.104 [29]).

Simulations have been run with 100,000 realizations to test false alarm rates. The results are given in Table 7.4.6.1.2-2.  It can be seen that the predetermined thresholds achieve extremely small false alarm rates.

Table 7.4.6.1.2-2: PRACH false alarm rate

| Number of segments | 1 | 6 | 18 |
| --- | --- | --- | --- |
| SNR (dB) | -2.4 | -12.4 | -22.4 |
| False alarm rate | 1/100,000 | 0/100,000 | 0/100,000 |

For PRACH preamble detection at the receiver, a misdetection is counted if either (1) the peak of accumulated detection statistic does not exceed the pre-determined threshold or (2) the detected preamble index is incorrect when the peak of accumulated detection statistic exceeds the threshold.

PRACH detection rate performance under different number of PRACH segments are presented in Table7.4.6.1.2-3. The results show that the three PRACH formats can achieve greater than 99% detection rates under -2.6 dB, -12.4 dB, -22.4 dB SNR, respectively.

Table 7.4.6.1.2-3: PRACH detection rate

| Number of segments | 1 | 6 | 18 |
| --- | --- | --- | --- |
| SNR (dB) | -2.4 | -12.4 | -22.4 |
| Detection rate (%) | 99.86 | 99.14 | 99.42 |

Based on the misdetection rate performance, Table 7.4.6.1.2-4 summarizes NB-LTE PRACH coverage performance. It can be seen the 164 dB MCL target can be met by the design.

Table 7.4.6.1.2-4: PRACH coverage performance

| 1) Tx power (dBm) | 23 | 23 | 23 |
| --- | --- | --- | --- |
| 2) PSD (dBm/Hz) | -174 | -174 | -174 |
| 3) Rx noise figure (dB) | 3 | 3 | 3 |
| 4) Interference margin (dB) | 0 | 0 | 0 |
| 5) BW (Hz) | 160000 | 160000 | 160000 |
| 6) Effective noise (dBm) 2)+3)+4)+10*log10(5)) | -118.9588 | -118.9588 | -118.9588 |
| 7) Required SINR (dB) | -2.4 | -12.4 | -22.4 |
| 8) Rx sensitivity (dBm) 6)+7) | -124.5588 | -133.3588 | -141.3588 |
| 9) Rx processing gain (dB) | 0 | 0 | 0 |
| 10) MCL (dB) 1)-8)+9) | 144.3583 | 154.3583 | 164.3583 |

###### 7.4.6.1.2.3 Time of arrival estimation performance

Among others, a main objective of random access is to achieve uplink synchronization, which is important for maintaining the uplink orthogonality in LTE. To this end, the receiver (i.e., base station) estimates the time-of-arrival from the received preamble. Figure 7.4.6.1.2-1 shows the distributions of time-of-arrival estimation errors for the three PRACH formats under -2.4 dB, -12.4 dB, -22.4 dB SNR, respectively. The results show that the estimation errors are within 3 PRACH samples, or equivalently, 18.75 us. In NB-LTE, the shortest CP is 28.2 us. Therefore, the time-of-arrival estimation errors (up to 18.75 us) can be handled by the CP and thus the time-of-arrival estimation accuracy is satisfactory.

Figure 7.4.6.1.2-1: Distribution of time-of-arrival estimation errors. Blue curve: Format 0 at -2.4 dB SNR; red curve: Format 1 at -12.2 dB SNR; green curve: Format 2 at -22.4 dB SNR.

###### 7.4.6.1.2.4 Random access capacity

In this subsection, random access preamble capacity is evaluated under Building Penetration Loss (BPL) Scenario 2 with inter-site correlation coefficient 0.75. The corresponding coupling loss statistics collected are given in Table 7.4.6.1.2-5. Coupling loss statistics were also collected for three other scenarios (i.e., BPL Scenario 1 with inter-site correlation coefficient 0.5, BPL Scenario 1 with inter-site correlation coefficient 0.75, and BPL Scenario 2 with inter-site correlation coefficient 0.5). It turned out that Scenario 2 with inter-site correlation coefficient 0.75 is the most challenging configuration and yields the highest amount of devices with high coupling loss, and thus it is the focus of random access capacity evaluation.

For MAR traffic, periodic inter-arrival times are given as 1 day (40%), 2 hours (40%), 1 hour (15%), and 30 minutes (5%). There are 52547 devices per sector.  Table 7.4.6.1.2-5 gives the average arrival rates for users in basic, robust and extreme coverage modes.

Table 7.4.6.1.2-5: Coupling loss statistics and traffic arrival rates: Scenario 2 with 0.75 inter-site correlation coefficient

| Coupling loss | <144 dB | [144 dB, 154 dB] | >154 dB |
| --- | --- | --- | --- |
| Percentage | 88.42% | 9.01% | 2.57% |
| MAR mean arrival rate (/s) | 6.0229 | 0.6137 | 0.1751 |

As one example configuration, suppose a cell reserves 10 preambles out of the total 64 preambles for contention-free random access. The preamble dimensioning among the three coverage classes goes as follows. Recall users in basic, robust, and extreme coverage modes require 1, 6, 18 PRACH segments, respectively. To ensure fair random access opportunities, one way to dimension preambles may go as follows.

88.42*1*NO≈9.01*6*N1≈2.57*18*N2

N0+N1+N2=54

Here recall 88.42%, 9.01%, 2.57 % are the respectively percentages of devices in the three coverage classes, and 1, 6, 18 are the respectively number of PRACH segments used for preamble transmission.  Setting (N0, N1, N2) = (24, 16, 14) roughly satisfies the above equations.

With the above preamble dimensioning among the three coverage classes, Table 7.4.6.1.2-6 shows the average numbers of users per second that can perform random access while meeting 1% preamble collision rate. Here a preamble collision is counted if two or more users select the same preambles when they are using the same PRACH format (i.e. in the same coverage class). It can be seen that the random access preamble capacity of NB-LTE can well handle the MAR traffic demand indicated in Table 7.4.6.1.2-5.

Table 7.4.6.1.2-6: Random access preamble capacity performance

| Format | Number of preambles | Random access capacity (/s) @1% collision rate |  |
| --- | --- | --- | --- |
|  |  | 60 ms PRACH period | 120 ms PRACH period |
| 0 | 24 | 178 | 89 |
| 1 | 16 | 20 | 10 |
| 2 | 14 | 6 | 3 |