3GPP TSG-GERAN #69 	GP-160049

# Malta, 14-18 Feb, 2016							    							         (Revision of GP-151222)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0186 | rev | 5 | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS, Mapping of logical channels onto physical channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-02-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of the mapping of logical EC-channels onto physical channels is introduced. Compared to GP-151222, track changes with user name “R5, EAB” has been used. Earlier made changes in GP-151222 are marked with “EAB”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1, 6.3.2.2.1, 6.3.2.2.5 (new), 6.3.2.3.1, 6.3.2.3.3b (new), 6.3.2.3.4b (new), 6.3.2.4b (new), 6.4.1, 6.5.1, 6.5.2b, 6.5.3a, 6.5.6a, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1611,1612 TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184,0185 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

### 6.3.2	Mapping in time of packet logical channels onto physical channels

#### 6.3.2.1	General

A physical channel allocated to carry packet logical channels is called a packet data channel (PDCH). A PDCH shall carry packet logical channels only.

In RTTI configuration, physical channels are paired, forming PDCH-pairs. The two physical channels shall have the same parameters (see subclause 5.6.3) except for the timeslot number (TN). The two PDCHs constituting a PDCH-pair shall be located on the same carrier. The two PDCHs constituting a PDCH-pair need not be contiguous. In each direction, physical channels shall be assigned so that PDCH-pairs do not partially overlap.

On a given PDCH, PDTCHs in BTTI configuration and RTTI configuration (assigned to different MSs) may be carried. Alternatively, both PDCHs forming a PDCH-pair may be assigned to only carry PDTCHs in RTTI configuration.

Packet switched logical channels are mapped dynamically onto a 52-multiframe.

-	For a PDCH/F in BTTI configuration the 52-multiframe consists of 12 blocks of 4 consecutive frames, 2 idle frames and 2 frames used for the PTCCH (see 3GPP TS 45.010), as shown in Figure 9. Table 6 in clause 7, indicates the frame numbers for each of the blocks (B0...B11) transmitted in the multiframe. The ordered list of block is defined as B0, B6, B3, B9, B1, B7, B4, B10, B2, B8, B5, B11.

-	For PDCH/H, the 52-multiframe consists of 6 blocks of 4 frames each, and two idle frames.  Table 6 in clause 7 indicates the frame numbers for each of the blocks (B0…B5) transmitted in the multiframe.

-	For a PDCH-pair in RTTI configuration the 52-multiframe consists of 24 RTTI blocks of 2 consecutive frames, plus 2 idle frames and 2 frames used for the PTCCH (see 3GPP TS 45.010) on each PDCH of the PDCH-pair, as shown in Figure 9a. Table 6 in clause 7 indicates the frame numbers for each of the 24 RTTI blocks (B0a, B0b, ...B11a, B11b) transmitted in the 52-multiframe.

-	For a PDCH, when in EC-EGPRS operation, the 52-multiframe consists of 12 BTTI blocks of 4 consecutive frames, and 4 idle frames, as shown in Figure 9b. In case blind physical layer transmissions are used, multiple BTTI blocks constitute the block that is mapped onto the physical channel, see Figure 9b. Table 6a in clause 7 indicates the frame numbers of each of the blocks depending on the downlink Coverage Class or uplink Coverage Class respetively.

-	Depending on the intended coverage range, EC-PDTCH/F is defined for different Coverage Classes (CCs), see 3GPP TS 43.064. The CCs are mapped onto different number of PDCHs according to:

-	CC1: 				1 PDCH

-	CC2, CC3, CC4: 4 PDCHs

The PDCHs that an EC-PDTCH/F is mapped to are contiguous, see Table 6a in clause 7.

In case overlaid CDMA is used, see 3GPP TS 43.064 [6], up to four MS can be multiplexed on the same physical resource, simultaneously transmitting, using different orthogonal codes assigned by the network, see 3GPP TS 44.060 [11]. The codes are applied per burst over the assigned PDCHs for EC-PDTCH and EC-PACCH in each TDMA frame from the lowest to the highest numbered assigned TN, according to Table 6.3-2, and are only applied in case blind physical layer transmissions are used (i.e. for CC2, CC3 and CC4). The code value is either 0 or 1. In case 0 is used, the burst is transmitted as if no code was applied. In case 1 is used, the whole burst is shifted in phase by  radians, see 3GPP TS 45.004 [14]. Overlaid CDMA is only applied for transmissions on the uplink, i.e. for EC-PACCH/U and EC-PDTCH/U.

Table 6.3-2. Overlaid CDMA codes

| Overlaid CDMA code1 | Code sequence2 |
| --- | --- |
| 0 | 0000 |
| 1 | 0101 |
| 2 | 0011 |
| 3 | 0110 |
| NOTE1: see 3GPP TS 44.060 [11] NOTE2: The first number in the sequence is applied to the lowest numbered assigned PDCH. The second number to the second lowest numbered assigned PDCH etc. |  |

A block allocated to a given logical channel comprises one radio block, or in the case of uplink only, 4 random access bursts.  The type of channel may vary on a block-by-block basis.

In the downlink direction, the logical channel type shall be indicated by the message type contained in the block header part.

In the uplink part for channels other than (EC-)PACCH transmitted as access bursts or PRACH, CPRACH, or EC-RACH, the logical channel type shall be indicated by the message type contained in the block header part for GPRS, or indicated by Stealing Flags for EGPRS and EGPRS2, or detected by the MS in case of EC-EGPRS operation. For (EC-)PACCH transmitted as access bursts, the logical channel type is indicated by the corresponding polling message on the downlink (see 3GPP TS 44.060). For the PRACH or CPRACH case the logical channel type is indicated by the USF (see 3GPP TS 44.060), set on the downlink on a block by block basis.

For COMPACT, timeslot mapping and rotation of the control channels is used such that control channels belonging to a serving time group are rotated over odd timeslot numbers as follows: 7, 5, 3, 1, 7, 5  . The rotation occurs between frame numbers (FN) mod 52 = 3 and 4. The mapping of the control channels on timeslot numbers is defined by the following formula:

-	for 0  FN mod 52  3, TN = ((6 x ((FN div 52) mod 4)) + 1 + (2 x TG)) mod 8;

-	for 4  FN mod 52  51, TN = ((6 x ((FN div 52) mod 4)) + 7 + (2 x TG)) mod 8.

Packet switched logical channels PDTCH, PACCH, and PTCCH are never rotated.

#### 6.3.2.2	Mapping of the uplink channels

##### 6.3.2.2.1	Mapping of uplink packet traffic channel ((EC-)PDTCH/U) and (EC-)PACCH/U

###### 6.3.2.2.1.1	BTTI configuration

The PDCH's where the MS may expect occurrence of its (EC-)PDTCH/U(s) or (EC-)PACCH/U for a mobile originated transfer is indicated in resource assignment messages (see 3GPP TS 44.060). (EC-)PACCH/U shall be allocated respecting the resources assigned to the MS. For PACCH/U the MS multislot class shall also be respected. For each PDCH assigned to the MS, an Uplink State Flag (R0... R7) is given to the MS. An exception is when using Fixed Uplink Allocation where the Uplink State Flag is not used to allocate resources, and hence no Uplink State Flag is given to the MS.

In case of USF based allocation, the occurrence of the PDTCH/U and/or the PACCH/U at given block(s) Bx (where Bx = B0...Bn; n=5 for the PDTCH/HU and n=11 for the PDTCH/FU) in the 52-multiframe structure for a given MS on a given PDCH shall be indicated by the value of the Uplink State Flag (USF) contained in the header of the preceding block transmitted in the downlink of the same PDCH (or in the case of shifted USF on the downlink of a PDCH with a relationship to the uplink PDCH as defined in 3GPP TS 44.060), that is to say B(x-1) in the same multiframe if x1 or B(n) in the previous multiframe if x=0. If the USF in block B(x-1) indicates that block B(x) shall be used by an MS for which the USF_GRANULARITY is set to 1 (corresponding to 4 blocks) in the last assignment message, that MS shall also use the three following blocks. The USF corresponding to the last three blocks shall be set to an unused value. The MS may transmit a PDTCH block or a PACCH block on any of the uplink blocks used by the MS. The occurrence of the PACCH/U associated to a PDTCH/D shall be indicated by the network by polling the MS (see 3GPP TS 44.060).

In case of Fixed Uplink Allocation, the occurrence of the EC-PDTCH/U at given block(s) in the 52-multiframe structure for a given MS on given PDCH(s) shall be indicated by an EC-Immediate Assignment Type 1, EC-Immediate Assignment Type 2, EC-EGPRS Packet Uplink Ack/Nack or EC-EGPRS Packet Uplink Assignment message. The number of blocks available per 52-multiframe depends on the uplink Coverage Class as shown in table 6.3-3. The mapping of each block onto the 52-multiframe is shown in table 6a, and illustrated in figure 9b.

Table 6.3-3: Blocks per 52-multiframe depending on uplink Coverage Class for EC-EGPRS

| Uplink Coverage Class | Number of blocks per 52-multiframe |
| --- | --- |
| 1 | 12 |
| 2 | 12 |
| 3 | 6 |
| 4 | 3 |

The occurrence of the EC-PACCH/U associated with EC-PDTCH/D shall be indicated by the network by polling the MS (see 3GPP TS 44.060).

NOTE 1:	This subclause specifies how the network shall signal that the MS is allowed to use the uplink. The operation of the MS is specified in 3GPP TS 44.060. In particular cases of extended dynamic allocation or exclusive allocation, the MS may not need to monitor the USF on all the downlink timeslots corresponding to the assigned uplink PDCHs. In case of Fixed Uplink Allocation the MS shall not monitor the USF.

NOTE 2:	The PDCH/HU is only assigned in exclusive allocation (see 3GPP TS 44.060).

NOTE 3: 	A MS using packet uplink traffic channels mapped to the same physical channel than an uplink PCCCH in extended dynamic allocation MAC mode is not required to check if allocated uplink PDTCH/U or PACCH/U blocks also belong to the PRACH.

In a dual carrier or a DLMC configuration, the uplink block(s) shall be allocated on the corresponding physical channel on which the USF or poll (see 3GPP TS 44.060) is received. Uplink blocks shall not be allocated on physical channels having different frequency domain descriptions (see subclause 5.4) simultaneously in the same block period.

For COMPACT, USF_GRANULARITY should be set to 0 (corresponding to 1 block) for dynamic allocation for the following cases:

i)	for odd timeslot numbers (TN) 1, 3, 5, and 7 in nominal and large cells;

ii)	for even timeslot numbers (TN) 0, 2, 4, and 6 in large cells.

###### 6.3.2.2.1.2	RTTI configuration

The PDCH-pairs where the MS may expect occurrence of its PDTCH/U(s) or PACCH/U for a mobile originated transfer are indicated in resource assignment messages (see 3GPP TS 44.060). PACCH/U shall be allocated respecting the resources assigned to the MS and the MS multislot class.

For each PDCH-pair assigned to the MS, one or two USFs (R0... R7) are given to the MS. For each assigned uplink PDCH-pair the network may signal in resource assignment messages a “corresponding downlink PDCH-pair” where the USF is monitored. The timeslot numbers of the PDCHs constituting the corresponding downlink PDCH-pair may be different from those of the PDCHs constituting the uplink PDCH-pair. If no indication is provided, the corresponding downlink PDCH-pair shall be the one with the same timeslot numbers as the uplink PDCH-pair.

For an assigned uplink PDCH-pair, for the transmission of the USF the network can use one of two modes:

-	BTTI USF mode: USFs are sent in a basic radio block period, i.e. a USF is mapped on four bursts transmitted on one of the PDCHs of a downlink PDCH-pair during four consecutive TDMA frames;

-	RTTI USF mode: a USF is sent in a reduced radio block period, i.e. a USF is mapped on four bursts transmitted on both PDCHs of a downlink PDCH-pair during two consecutive TDMA frames.

The network shall signal the USF mode in the resource assignment messages (see 3GPP TS 44.060). The USF mode shall be the same for all the uplink PDCH-pairs assigned to one mobile station. Also, on a given downlink PDCH-pair, all USFs shall be sent with the same USF mode.

If the BTTI USF mode is used, for each PDCH-pair assigned to the MS the network shall give two USFs (R0... R7) to the MS, one for each PDCH of the corresponding downlink PDCH-pair. If the RTTI USF mode is used, for each PDCH-pair assigned the network shall give to the MS one Uplink State Flag (R0... R7) to the MS.

When a given downlink PDCH is configured to provide USFs for PDTCHs operating in RTTI configuration (as part of a corresponding downlink PDCH-pair) and PDTCHs operating in BTTI configuration, the BTTI USF mode shall be used on both PDCHs of the corresponding downlink PDCH-pair to schedule uplink blocks for mobile stations using RTTI configuration. Additionally, if any of the downlink PDTCHs mapped onto either PDCH of a corresponding PDCH-pair operates in BTTI configuration, the BTTI USF mode shall be used on both PDCHs. In this case, if x1, the first uplink RTTI radio block Bxa shall be allocated by the USF contained in block B(x-1) of the downlink PDCH having the lowest TN and the second uplink RTTI radio block Bxb shall be allocated by the USF contained in block B(x-1) of the highest numbered PDCH TN of the “corresponding downlink PDCH-pair” to the uplink PDCH-pair (see Figure 9a and 3GPP TS 44.060). If x=0 the corresponding USFs will be carried within block B11 of the previous 52-multiframe. In case of dual carrier or a DLMC configuration in the downlink, this same relationship between USFs and uplink RTTI radio blocks shall apply. If the USF_GRANULARITY is set to 1 (corresponding to 4 blocks) in the last assignment message, the MS shall also use the next three consecutive RTTI radio blocks of sub-index a or sub-index b (according to the PDCH in which the USF was received), see 3GPP TS 44.060. The USF corresponding to the last three blocks shall be set to an unused value.

When on a downlink PDCH belonging to a PDCH-pair a USF is sent scheduling an uplink block belonging to an MS in BTTI configuration, the USF on the other PDCH of the “corresponding PDCH-pair” shall also schedule an uplink block belonging to an MS in BTTI configuration.

When both PDCHs of a corresponding downlink PDCH-pair are configured to provide USFs for uplink PDTCHs all operating in RTTI configuration, and when all the downlink PDTCHs mapped onto both PDCHs operate in RTTI configuration, the RTTI USF mode shall be used on both PDCHs. In this case, if x1, the uplink RTTI radio block Bxa shall be allocated by the USF contained in RTTI block B(x-1)b and uplink RTTI radio block Bxb shall be allocated by the USF contained in RTTI block Bxa of the “corresponding downlink PDCH-pair” to the uplink PDCH-pair (see Figure 9a and 3GPP TS 44.060). If x=0 the USFs will be carried within RTTI block B11b of the previous 52-multiframe and B0a respectively. If the USF_GRANULARITY is set to 1 (corresponding to 4 blocks) in the last assignment message, the MS shall also use the next three consecutive RTTI radio blocks, see 3GPP TS 44.060. The USF corresponding to the last three blocks shall be set to an unused value.

The MS may transmit a PDTCH block or a PACCH block on any of the uplink blocks allocated to the MS. The occurrence of the PACCH/U associated to a PDTCH/D shall be indicated by the network by polling the MS (see 3GPP TS 44.060).

##### 6.3.2.2.2	Mapping of the Packet Timing Advance Control Channel (PTCCH/U)

The PDCH carrying the PTCCH/U of one MS is defined in the resource assignment message (see 3GPP TS 44.060). PTCCH/U shall be mapped to one of the time slots where PDTCH(s) are allocated to the MS. PTCCH/U shall be allocated respecting the resources assigned to the MS and the MS multislot class. An MS shall be allocated a sub-channel of the PTCCH/U (0...15) as defined in table 6 of clause 7, where the sub-channel number is equal to the Timing Advance Index (TAI) indicated in the resource allocation message (see 3GPP TS 44.060).

In a dual carrier or a DLMC configuration, an MS shall be assigned a PTCCH/U sub-channel on a physical channel having one frequency domain description (see subclause 5.4) only.

In RTTI configuration, an MS shall be assigned a PTCCH/U on only one of the physical channels comprising an uplink PDCH-pair (see subclause 6.3.2.1).

##### 6.3.2.2.3	Mapping of the uplink PCCCH i.e. PRACH

The mapping of the PRACH is defined in table 6 of clause 7, where the possible blocks are indicated. The PRACH is dynamically allocated in groups of four PRACH blocks By (y=4x+i, i=0 ,.., 3) corresponding to one PDCH block Bx (x=0,...,11), indicated by USF=FREE in the same way as defined for PDTCH/U (see subclause 6.3.2.2.1).

Optionally, a subset of the blocks Bx can be allocated to PRACH in a fixed way. The number of allocated blocks is indicated by the parameter BS_PRACH_BLKS broadcast on the PBCCH, where BS_PRACH_BLKS=0...12. The blocks are allocated according to the ordered list defined in subclause 6.3.2.1. The blocks shall also be indicated by the USF=FREE. The MS may choose to use the BS_PRACH_BLKS or USF to determine the fixed allocated part of PRACH.

##### 6.3.2.2.3a	Mapping of the COMPACT uplink CPCCCH i.e. CPRACH

The CPRACH is dynamically or fixed allocated in the same way as defined for PRACH (see subclause 6.3.2.2.3. USF should be set equal to FREE for downlink block B0 on a serving time group when 4 time groups are assigned. Uplink blocks (other than block B1 on a serving time group) that are preceded by CPBCCH and CPCCCH blocks should be prioritized for use as CPRACH.

See Annex D for examples based on sixteen prioritized CPRACH blocks.

##### 6.3.2.2.4	Mapping of the MBMS uplink MPRACH

The mapping of the MPRACH is defined in table 6 of clause 7, where the possible blocks are indicated. The MPRACH is dynamically allocated in groups of four MPRACH blocks By (y=4x+i, i=0 ,.., 3) corresponding to one PDCH block Bx (x=0,...,11), indicated by a value of the USF, in the same way as defined for PDTCH/U (see subclause 6.3.2.2.1). The value of the USF is signalled in the MBMS notification message (see 3GPP TS 44.060).

##### 6.3.2.2.5	Mapping of the uplink EC-CCCH, i.e. EC-RACH

The mapping of the EC-RACH is defined in table 6a of clause 7, where the possible blocks are indicated for each uplink Coverage Class. Furthermore, two different EC-RACH mappings exist. The mapping to be used is signalled on cell level in EC-SI, see 3GPP TS 44.018. The EC-RACH is either mapped onto a single TS, or over 2 consecutive TS for CC2, CC3 and CC4. CC1 EC-RACH is always mapped onto 1 TS.

The EC-RACH is fixed allocated in the full 51-multiframe. The EC-RACH blocks constitute different number of EC-RACH bursts depending on the uplink Coverage Class used, see table 6a. One, four, sixteen or fourty-eight bursts are used for CC1, CC2, CC3 and CC4 EC-RACH blocks respectively. In case of CC4, the bursts are mapped over two 51-multiframes, and for other Coverage Classes each block is contained in a single 51-multiframe.

#### 6.3.2.3	Mapping of the downlink channels

##### 6.3.2.3.1	Mapping of the ((EC-)PDTCH/D) and (EC-)PACCH/D

The PDCH(s) in BTTI configuration or the PDCH-pair(s) in RTTI configuration where the MS may expect occurrence of its (EC-)PDTCH/D(s) for a mobile terminated transfer or its (EC-)PACCH/D, for both mobile originated and mobile terminated transfer, are indicated in resource assignment messages (see 3GPP TS 44.060). (EC-)PDTCH/D and (EC-)PACCH/D can be mapped dynamically on all blocks except those used for PBCCH (see subclause 6.3.2.3.3). The logical channel type shall be indicated in the block header for GPRS, it shall be indicated by Stealing Flags for EGPRS and EGPRS2, and it shall be detected by the MS in case of EC-EGPRS operation. The mobile owner of the (EC-)PDTCH/D or (EC-)PACCH/D shall be indicated by the TFI (Temporary Flow Identity) (see 3GPP TS 44.060).

If PDTCH/D is mapped on blocks, which may be used for PCCCH and where paging may appear, the network shall only use coding schemes CS-1 to CS-4.

NOTE: 	This restriction is needed to avoid the expiry of the downlink signalling counter (DSC) for non-EGPRS capable mobile stations in case the network uses MCS-1 to MCS-9. CS-1 should be favoured, as it provides the strongest error protection.

##### 6.3.2.3.2	Mapping of the PTCCH/D

The PTCCH/D is mapped as defined in table 6 of clause 7. The PTCCH/D carries signalling messages including timing advance information for MSs sharing the PTCCH/U on the same PDCH.

In a dual carrier or a DLMC configuration, an MS shall be assigned a PTCCH/D channel on a physical channel having one frequency domain description (see subclause 5.4) only.

In RTTI configuration, an MS shall be assigned a PTCCH/D on only one of the physical channels comprising a downlink PDCH-pair (see subclause 6.3.2.1). When only an uplink PDTCH is assigned to the MS, the PTCCH shall be assigned on one of the PDCHs comprising the “corresponding downlink PDCH-pair” for the uplink PDTCH (see subclause 6.3.2.2.1.2).

##### 6.3.2.3.3	Mapping of the PBCCH

The PBCCH is mapped onto one PDCH only, indicated in the BCCH. The PBCCH is mapped on BS_PBCCH_BLKS blocks (where 1BS_PBCCH_BLKS4) per multiframe, according to the ordered list described in subclause 6.3.2.1. The blocks allocated are specified in table 6 of clause 7. The parameter BS_PBCCH_BLKS is broadcast on PBCCH in block B0 (see subclause 3.3.2.4).

##### 6.3.2.3.3a	Mapping of the COMPACT CPBCCH

The CPBCCH is mapped onto a primary COMPACT carrier on the time group indicated by TG on CSCH (see subclause 3.3.2.2). This time group is known as the serving time group and rotates over odd timeslot numbers (see subclause 6.3.2.1). The CPBCCH is mapped on BS_PBCCH_BLKS blocks (where 1BS_PBCCH_BLKS4) per multiframe, according to the ordered list described in subclause 6.3.2.1. The blocks allocated are specified in table 9 of clause 7. The parameters BS_PBCCH_BLKS is broadcast on CPBCCH in block B0 (see subclause 3.3.2.4).

See Annex D for examples based on one CPBCCH block.

When USF=FREE in downlink block B0 on a serving time group, the CPRACH is allocated in uplink block B1 after timeslot rotation. When USF has any other value in downlink block B0 on a serving time group, the uplink allocation of B1 is valid for  the same timeslot, irrespective of timeslot rotation.

##### 6.3.2.3.3b	Mapping of the Extended Coverage BCCH (EC-BCCH)

The EC-BCCH is mapped onto the BCCH carrier onto one or several PDCHs as defined in table 6a of clause 7.

##### 6.3.2.3.4	Mapping of the PCCCH

The PCCCH and its different logical channels (PAGCH, PPCH) can be mapped dynamically and are identified by the message header. The configuration is partly fixed by some parameters broadcast by the PBCCH and defined in subclause 3.3.2.4:

a)	BS_PBCCH_BLKS, that defines the number of PBCCH blocks per multiframe, according to the ordered list described in subclause 6.3.2.1, on the PDCH that carries PBCCH;

b)	BS_PAG_BLKS_RES, that defines the number of blocks in addition to BS_PBCCH_BLKS, according to the ordered list described in subclause 6.3.2.1, where PPCH shall not occur on every PDCH that carries PCCCH.

PCCCH (except PPCH) can be mapped on all blocks except those used for PBCCH.

If PBCCH is allocated on timeslot k, PCCCHs shall be allocated only on timeslots n where n>k-4 and 0£n£7 in order to provide time for the MS to switch from PBCCH to PCCCH.

##### 6.3.2.3.4a	Mapping of the COMPACT CPCCCH

The CPCCCH and its different logical channels (CPAGCH, CPPCH) can be mapped dynamically and are identified by the message header. The configuration is partly fixed by some parameters broadcast by the CPBCCH and defined in subclause 3.3.2.4:

a)	BS_PBCCH_BLKS, that defines the number of CPBCCH blocks per multiframe, according to the ordered list described in subclause 6.3.2.1, on the radio frequency channel that carries CPBCCH;

b)	BS_PAG_BLKS_RES, that defines the number of blocks in addition to BS_PBCCH_BLKS, where CPPCH shall not occur on every radio frequency channel that carries CPCCCH. These blocks without CPPCH are allocated after CPPCH blocks according to the ordered list described in subclause 6.3.2.1.

CPCCCH (except CPPCH) can be mapped on all blocks except those used for CPBCCH.

For primary COMPACT carriers, CPCCCHs shall be allocated on the same time group as CPBCCH. CPCCCHs on secondary COMPACT carrier(s) shall be allocated on same time group as for primary COMPACT carrier.

See Annex D for examples based on three CPCCCH blocks.

##### 6.3.2.3.4b	Mapping of the Extended Coverage CCCH (EC-CCCH)

The EC-CCCH and its different logical channels (EC-AGCH, EC-PCH) can be mapped dynamically and are identified by the message header. The mapping is as defined in table 6a of clause 7.

#### 6.3.2.4	Mapping of PBCCH data

In order to facilitate the MS operation, the network is required to transmit certain types of Packet System Information (PSI) messages in specific multiframes and specific PBCCH blocks within the multiframes. The occurrence of the PSI1 message is defined by TC = (FN DIV 52) mod PSI1_REPEAT_PERIOD, where PSI1_REPEAT_PERIOD (range 1 - 16) is indicated in the SI13 message on BCCH, the PSI 1 message on PBCCH and, if present, in the Neighbour Cell parameters in PSI3 and PSI3bis messages sent on serving cell PBCCH.

The PSI1 message is transmitted at TC = 0 according to rule i) and ii) below.

The PSI messages other than the PSI1 message are divided into two groups of PSI messages. One group of PSI messages is transmitted with a low repetition rate and a second group is transmitted with a high repetition rate.

The number of PSI message instances sent with high repetition rate is indicated by the parameter PSI_COUNT_HR (range 0 to 16) in the PSI1 message. The PSI messages in this group are sent  according to rule iii) below.

The number of PSI message instances sent with low repetition rate is indicated by the parameter PSI_COUNT_LR (range 0 to 63) in the PSI1 message. The PSI messages in this group are sent according to rule iv) below.

The following rules apply:

i)	PSI1 shall be sent in block B0 when TC = 0;

ii)	if the value of the parameter BS_PBCCH_BLKS is greater than 1, the PSI1 shall also be sent in block B6  when TC = 0;

iii)	the PSI messages in the group sent with high repetition rate shall be sent in a sequence determined by the network and starting at TC = 0, using the PBCCH blocks within each multiframe, in the order of occurrence, which are not occupied according to rule i) or ii). The sequence of these PSI messages shall be repeated starting at each occurrence of TC = 0;

iv)	the PSI messages in the group sent with low repetition rate shall be sent in a sequence determined by the network and continuously repeated, using the PBCCH blocks within each multiframe, in the order of occurrence, which are not occupied according to rules i) to iii) . The sequence of these PSI messages shall be restarting at FN = 0.

If there are multiple instances of a particular type of PSI message (see 3GPP TS 44.060), they shall all be sent within same group of PSI messages according to either rule iii) or iv) above. They shall be sent in a single sequence in the ascending order of the message instance number of that type of PSI message.

The same PSI message shall not occur twice within the lists defined by PSI_COUNT_LR and PSI_COUNT_HR

A full set of Packet System Information messages contains one consistent set of the messages included in PSI_COUNT_LR and one consistent set of the messages included in PSI_COUNT_HR plus the PSI1 message.

NOTE:	The parameters BS_PBCCH_BLKS and PSI1_REPEAT_PERIOD_shall be selected by the network such that all PSI message present in the cell can be sent according to rules i) to iv) above. It is the responsibility of the network to optimise the broadcast of the PSI messages so that the MS can find the important parameters for cell re-selection and access as fast as possible without unnecessary power consumption. The PSI mapping scheme information can be utilised by the MS to estimate the actual minimum cell reselection time.

#### 6.3.2.4a	Mapping of COMPACT CPBCCH data

See subclause 6.3.2.4, with the exception that the CPBCCH is a stand-alone packet control channel for COMPACT.

#### 6.3.2.4b	Mapping of EC-BCCH data

In order to facilitate the MS operation, the network is required to transmit EC-System Information (EC-SI) messages on two EC-BCCH blocks per 51-multiframe. An EC-SI message may consist of one or more message instances (see 3GPP TS 44.018). Each EC-SI message instance is repeated over 16 consecutive EC-BCCH blocks in the two EC-BCCH blocks over 8 consecutive 51-multiframes.

The following rules apply:

i)    An EC-SI message instance is started in EC-BCCH block B0 when TC = (FN DIV 51) mod 8 = 0 and is repeated over B1 to B15 in 8 consecutive 51-multiframes, see table 6a.

ii)   The network determines when to send any given EC-SI message.

iii)  When there are multiple EC-SI message instances of an EC-SI message they shall be sent in ascending order (see 3GPP TS 44.018).

2nd modification

### 6.4.1	Permitted channel combinations onto a basic physical channel

The following are the permitted ways, as defined by 3GPP TS 44.003, in which channels can be combined onto basic physical channels for one or several MSs.

The following definitions are used in the list of combinations below.

| Combination designation | Channel combination |
| --- | --- |
| SUB_TA | TCH/H + FACCH/H + SACCH/TH |
| SUB_T | TCH/H |
| SUB_PA | PDTCH/H + PACCH/H |
| SUB_TE | TCH/H + FACCH/H + SACCH/TPH + EPCCH/H |
| SUB_OTA | O-TCH/H + O-FACCH/H + SACCH/TH |
| SUB_OT | O-TCH/H |
| SUB_OTE | O-TCH/H + O-FACCH/H + SACCH/TPH + EPCCH/H |

Numbers appearing in parenthesis after channel designations indicate sub-channel numbers. (0..n) shall be interpreted as subchannel 0, 1,…,n-1 and n. Channels and sub-channels need not necessarily be assigned.

- TCH/F + FACCH/F + SACCH/TF

- O-TCH/F + O-FACCH/F + SACCH/TF

- FCCH + SCH + BCCH + CCCH

- FCCH + SCH + BCCH + CCCH + SDCCH/4(0..3) + SACCH/C4(0..3)

- BCCH + CCCH

- SDCCH/8(0 .7) + SACCH/C8(0 . 7)

- TCH/F + FACCH/F + SACCH/M

- TCH/F + SACCH/M

- TCH/FD + SACCH/MD

- PBCCH + PCCCH + PDTCH/F + PACCH/F + PTCCH/F

- PCCCH + PDTCH/F + PACCH/F + PTCCH/F

- PDTCH/F + PACCH/F + PTCCH/F

- CTSBCH + CTSPCH + CTSARCH + CTSAGCH

- CTSPCH + CTSARCH + CTSAGCH

- CTSBCH

- CTSBCH + TCH/F + FACCH/F + SACCH/CTS

- E-TCH/F + E-IACCH/F + E-FACCH/F + SACCH/TF

- E-TCH/F + E-IACCH/F + E-FACCH/F + SACCH/M

- E-TCH/F + E-IACCH/F + SACCH/M

- E-TCH/FD + E-IACCH/F + SACCH/MD

- CFCCH + CSCH + CPBCCH + CPCCCH + PDTCH/F + PACCH/F + PTCCH/F

- CPCCCH + PDTCH/F + PACCH/F + PTCCH/F

- O-TCH/F + O-FACCH/F + SACCH/TPF + EPCCH/F

- TCH/F + FACCH/F + SACCH/TPF + EPCCH/F

- TCH/F + FACCH/F + SACCH/MP + EPCCH/M

- TCH/F + SACCH/MP + EPCCH/M

- TCH/FD + SACCH/MPD + EPCCH/MD

xxix)			PDTCH/F + PACCH/F + SACCH/TF

- PDTCH/F + PACCH/F + SACCH/TPF + EPCCH/F

xxxi)		PDTCH/F + PACCH/F + SACCH/M

xxxii)		PDTCH/FD + PACCH/FD + SACCH/MD

xxxiii)		PDTCH + PACCH/F + SACCH/M + EPCCH/M

xxxiv)		PDTCH + PACCH/F + SACCH/M + EPCCH/MD

xxxv)		PRACH + PDTCH/U + PACCH/U + PTCCH/U + MPRACH

xxxvi)		PDTCH/U + PACCH/U + PTCCH/U + MPRACH

xxxvii)		EC-SCH + EC-BCCH + EC-CCCH

xxxviii)	EC-CCCH

xxxix)		EC-PDTCH/F + EC-PACCH/F

The following combinations of half rate channels are allowed on a basic physical channel for a single mobile, where the second half rate channel need not be assigned:

a1)	SUB_TA + SUB_T	(Lm + Lm configuration)

a2)	SUB_TA + SUB_OT	(Lm + Lm configuration)

a3)	SUB_TA + SUB_PA	(DTM single slot)

a4)	SUB_TE + SUB_T	(Lm + Lm configuration)

a5)	SUB_TE + SUB_OT	(Lm + Lm configuration)

a6)	SUB_TE + SUB_PA	(DTM single slot)

a7)	SUB_OTA + SUB_OT	(Lm + Lm configuration)

a8)	SUB_OTA + SUB_T	(Lm + Lm configuration)

a9)	SUB_OTA + SUB_PA	(DTM single slot)

a10)	SUB_OTE + SUB_OT	(Lm + Lm configuration)

a11)	SUB_OTE + SUB_T	(Lm + Lm configuration)

a12)	SUB_OTE + SUB_PA	(DTM single slot)

The following combinations of half rate channels are allowed on a basic physical channel for two mobiles:

b1)	SUB_TA + SUB_TA

b2)	SUB_TA + SUB_TE

b3)	SUB_TA + SUB_OTA

b4)	SUB_TA + SUB_OTE

b5)	SUB_TE + SUB_TE

b6)	SUB_TE + SUB_OTA

b7)	SUB_TE + SUB_OTE

b8)	SUB_OTA + SUB_OTA

b9)	SUB_OTA + SUB_OTE

b10)	SUB_OTE + SUB_OTE

The following combinations of full rate channels are allowed on a basic physical channel capable of VAMOS for two mobiles in VAMOS mode (where the 2 full rate channels constitute a VAMOS pair):

c11)	i) + i)

c12)	i) + xxv)

c13)	xxv) + xxv)

The following combinations of full rate and half rate channels are allowed on a basic physical channel capable of VAMOS for three mobiles in VAMOS mode (where the half rate channel on sub-channel number 0 and the full rate channel constitute one VAMOS pair whilst the half rate channel on sub-channel number 1 and the full rate channel constitute a different VAMOS pair on the same basic physical channel capable of VAMOS)

d1)	SUB_TA + SUB_TA + i)

d2)	SUB_TA + SUB_TA + xxv)

d3)	SUB_TA + SUB_TE + i)

d4)	SUB_TA + SUB_TE + xxv)

d5)	SUB_TE + SUB_TE + i)

d6)	SUB_TE + SUB_TE + xxv)

The following combinations of half rate channels are allowed on a basic physical channel capable of VAMOS for 3 mobiles of which 2 are in VAMOS mode (where the pair of half rate channels sharing the same sub-channel number constitutes the VAMOS pair whilst the other half rate channel is not in VAMOS mode)

e1)	SUB_TA + SUB_TA + SUB_TA

e2)	SUB_TA + SUB_TA + SUB_TE

e3)	SUB_TE + SUB_TE + SUB_TA

e4)	SUB_TE + SUB_TE + SUB_TE

The following combinations of half rate channels are allowed on a basic physical channel capable of VAMOS for four mobiles in VAMOS mode (where the pair of half rate channels on the sub-channel number 0 constitutes one VAMOS pair and the other pair of half rate channels on the sub-channel number 1 constitutes the other VAMOS pair):

f1)	SUB_TA + SUB_TA + SUB_TA + SUB_TA

f2)	SUB_TA + SUB_TA + SUB_TA + SUB_TE

f3)	SUB_TA + SUB_TA + SUB_TE + SUB_TE

f4)	SUB_TA + SUB_TE + SUB_TE + SUB_TE

f5)	SUB_TE + SUB_TE + SUB_TE + SUB_TE

NOTE 0: 	CCCH = PCH+ RACH + AGCH + NCH.

EC-CCCH = EC-PCH+EC-RACH+EC-AGCH.

PCCCH = PPCH+PRACH+PAGCH

CPCCCH = CPPCH + CPRACH + CPAGCH

NOTE 1:	Where the SMSCB is supported, the CBCH replaces SDCCH number 2 in cases v) and vii) above.

NOTE 2:	A combined CCCH/SDCCH allocation (case v) above) may only be used when no other CCCH channel is allocated.

NOTE 3:	Combinations viii), ix), x), xix), xx), xxi), xxix), xxx), xxxi) and xxxii) are used without EPC in multislot configurations as defined in subclause 6.4.2.

NOTE 4:	Combinations xiv), xv), xvi) and xvii) shall be used in CTS; combinations xiv), xvi) and xvii) shall be mutually exclusive; combinations xiv) and xv) shall also be mutually exclusive.

NOTE 5:	Combinations xxii) and xxiii) shall be used for COMPACT on serving time groups.

NOTE 6:	Combinations i), ii), xiii), xxv), xxiv) or any of a1) to a12) shall be used for single timeslot operation in DTM.

NOTE 7:	A unidirectional TCH combination i), viii), ix) or x) may be combined with the corresponding E-TCH combination xviii), xix), xx) or xxi) respectively in the other direction.

NOTE 8:	Combinations xxvi), xxvii), xxviii) , xxxii), xxxiii) and xxxiv) are used with EPC in multislot configurations as defined in subclause 6.4.2.

NOTE 9:	The basic physical channel onto which channels can be combined according to combinations i), ii), viii), ix), x), xviii), xix), xx), xxi), xxiv), xxv), xxvi), xxvii), xxviii), xxix), xxx), xxxi), xxxii), xxxiii) and xxxiv) is referred to as dedicated basic physical subchannel full rate (DBPSCH/F) in Iu mode.

NOTE 10:	The basic physical channel onto which channels can be combined according to combinations xi), xii) and xiii) is referred to as shared basic physical subchannel full rate (SBPSCH/F) in Iu mode.

NOTE 11:	The part of the basic physical channel onto which channels can be combined according to combinations SUB_TA, SUB_T, SUB_PA, SUB_TE, SUB_OTA, SUB_OT, SUB_OTE is referred to as dedicated basic physical subchannel half rate (DBPSCH/H) in Iu mode.

NOTE 12:	The part of the basic physical channel onto which channels can be combined according to combination SUB_PA is referred to as shared basic physical subchannel half rate (SBPSCH/H) in Iu mode.

NOTE 13:	The parts of the basic physical channel onto which SDCCH and SACCH can be combined according to combination v) are referred to as dedicated basic physical subchannels for SDCCH/4 and SACCH/C4 (DBPSCH/S4) in Iu mode.

NOTE 14:	The parts of the basic physical channel onto which SDCCH and SACCH can be combined according to combination vii) are referred to as dedicated basic physical subchannels for SDCCH/8 and SACCH/C8 (DBPSCH/S8) in Iu mode.

NOTE 15:	Combinations xxxv) and xxxvi) are used only for MBMS.

NOTE 16: In RTTI configuration only combination xiii) shall be used.

3rd modification

### 6.5.1	General

i)	A base transceiver station must transmit a burst in every timeslot of every TDMA frame in the downlink of radio frequency channel C0 of the cell allocation (to allow mobiles to make power measurements of the radio frequency channels supporting the BCCH, see 3GPP TS 45.008). In order to achieve this requirement a dummy burst is defined in subclause 5.2.6 which shall be transmitted by the base transceiver station on all timeslots of all TDMA frames of radio frequency channel C0 for which no other channel requires a burst to be transmitted.

ii)	Timeslot number 0 of radio frequency channel C0 of the cell allocation must support either channel combinations iv) or v) in subclause 6.4.1. No other timeslot or allocated channel from the cell allocation is allowed to support channel combinations iv) or v) in subclause 6.4.1.

iii)	The parameter BS_CC_CHANS in the BCCH defines the number of basic physical channels supporting common control channels (CCCHs). All shall use timeslots on radio frequency channel C0 of the cell allocation. The first CCCH shall use timeslot number 0, the second timeslot number 2, the third timeslot number 4 and the fourth timeslot number 6. Each CCCH carries its own CCCH_GROUP of mobiles in idle mode. Mobiles in a specific CCCH_GROUP will listen for paging messages and make random accesses only on the specific CCCH to which the CCCH_GROUP belongs. The method by which a mobile determines the CCCH_GROUP to which it belongs is defined in subclause 6.5.2.

iv)	The parameter BS_CCCH_SDCCH_COMB in the BCCH (see subclause 3.3.2) defines whether the common control channels defined are combined with SDCCH/4(0.3) + SACCH/C4(0.3) onto the same basic physical channel. If they are combined then the number of available random access channel blocks (access grant channel blocks and paging channel blocks; see following), are reduced as defined in table 5 of clause 7.

v)	The PCH, AGCH, NCH and BCCH Ext may share the same TDMA frame mapping (considered modulo 51) when combined onto a basic physical channel. The channels are shared on a block by block basis, and information within each block, when de-interleaved and decoded allows a mobile to determine whether the block contains paging messages, notification message, system information messages or access grants. However, to ensure a mobile satisfactory access to the system a variable number of the available blocks in each 51-multiframe can be reserved for access grants and system information messages, only. The number of blocks not used for paging (BS_AG_BLKS_RES) starting from, and including block number 0 is broadcast in the BCCH (see subclause 3.3.2). As above the number of paging blocks per 51-multiframe considered to be "available" shall be reduced by the number of blocks reserved for access grant messages.

The number of paging blocks per 51-multiframe shall be the same for all cells in a routing area where eDRX is supported to ensure the nominal paging group of a MS that uses eDRX (see 3GPP TS 44.018 [10]) occurs at the same location (i.e. in the same paging block) within the set of 51-multiframes comprising its eDRX cycle in all cells in that routing area.

If system information messages are sent on BCCH Ext, BS_AG_BLKS_RES shall be set to a value greater than zero.

Table 5 of clause 7 defines the access grant blocks and paging blocks available per 51-multiframe.

vi)	Another parameter in the BCCH, BS_PA_MFRMS indicates the number of 51-multiframes between transmissions of paging messages to mobiles in idle mode of the same paging group. The "available" paging blocks per CCCH are then those "available" per 51-multiframe on that CCCH (determined by the two above parameters) multiplied by BS_PA_MFRMS. An exception case is where eDRX is supported in a given routing area in which case all cells therein shall have the same number of paging blocks per 51-multiframe (see 3GPP TS 44.018 [10]). In this exception case the "available" paging blocks per eDRX cycle on a CCCH is determined by multiplying the number of paging blocks per 51-multiframe by the number of 51-multiframes per eDRX cycle (BS_ePA_MFRMS) (see Table 6.5.6a-1). Mobiles are normally only required to monitor every Nth block of their paging channel, where N equals the number of "available" blocks in total (determined by the above BCCH parameters) on the paging channel of the specific CCCH which their CCCH_GROUP is required to monitor. Note that when eDRX is used then N is determined using BS_ePA_MFRMS which is not a parameter sent on the BCCH but is derived directly from the eDRX cycle length negotiated by the mobile station with the network (see sub-clause 6.5.2a and Table 6.5.6a-1). Other paging modes (e.g. page reorganize or paging overload conditions described in 3GPP TS 44.018) may require the mobile to monitor paging blocks more frequently than this. All the mobiles listening to a particular paging block are defined as being in the same PAGING_GROUP. The method by which a particular mobile determines to which particular PAGING_GROUP it belongs and hence which particular block of the available blocks on the paging channel is to be monitored is defined in subclause 6.5.2 and 6.5.2a.

vii)	An MS which has its membership of at least one voice group or voice broadcast call group set to the active state shall, in addition to monitoring the paging blocks as described above, monitor the notification channel, NCH. This logical channel is always mapped onto contiguous blocks reserved for access grants, in a position and number as given by the parameter NCP, defined in 3GPP TS 44.018, broadcast on the BCCH. The channel may be present when a cell supports voice group or voice broadcast calls. The coding of the various structural parameters described above in this subclause is not changed. Information within a block, when deinterleaved and decoded, allows the MS to determine whether the block contains access grant messages or notification messages.

viii)	In presence of PCCCH, the parameter BS_PCC_CHANS in the PBCCH defines the number of physical channels for packet data (PDCH) carrying PCCCH. The (P)BCCH shall in addition indicate the physical description of those channels. Each PCCCH carries its own PCCCH_GROUP of MSs in GPRS attached mode. MS in a specific PCCCH_GROUP will listen for paging messages and make random accesses only on the specific PCCCH to which the PCCCH_GROUP belongs. The method by which an MS determines the PCCCH_GROUP to which it belongs is defined in subclause 6.5.6.

ix)	In CTS, the CTSBCH (CTSBCH-SB and CTSBCH-FB) shall always be transmitted by the CTS-FP according to the rules defined in Clause 6 and table 8 of clause 7.

In CTS idle mode, a CTS-MS shall be assigned a CTS_PAGING_GROUP, as specified in subclause 6.5.7. Several CTS-MS can be assigned the same CTS_PAGING_GROUP. The CTS-MS shall determine the specific 52-multiframe where a paging block may be sent to it according to the rule defined in subclause 6.5.7, and shall listen to the CTSBCH of the previous 52-multiframe. In this 52-multiframe, the CTS-MS shall decode the CTSBCH-SB information bits : if the flag indicating the presence of a CTSPCH in the next 52-multiframe is properly set (see 3GPP TS 44.056), the CTS-MS shall listen to the next CTSPCH and read the paging block. With this method, it is not necessary to maintain on the physical channel the CTSPCH : the CTSPCH shall only be transmitted when a paging message shall be addressed to one or several CTS-MS in a CTS paging group.

When using the CTSARCH, the CTS-MS shall send two bursts on the CTSARCH: these two bursts shall be sent on two successive frames and shall fulfil the mapping defined in table 8 of clause 7, with the requirement of the first burst being sent in a TDMA frame with even FN. They shall contain the same access request message, which is specified in 3GPP TS 44.056. The first sent burst can be used by the CTS-FP to assess the path loss between the CTS-MS and itself, in order to effectively decode the second burst.

x)	For COMPACT, the base transceiver station shall transmit a burst in a PDCH allocated to carry CPBCCH, in all TDMA Frames where CPBCCH, CFCCH, CSCH is allocated or where CPPCH can appear. In TDMA Frames where CPPCH can appear on the physical channel where CPBCCH is allocated, the base transceiver station shall transmit a dummy block in case no block is required to be transmitted.

xi)	For COMPACT, a base station does not transmit a burst in every timeslot of every TDMA frame in the downlink of the COMPACT control carrier (i.e., discontinuous transmission is used).

xii)	For COMPACT, inter base station time synchronization is required. Timeslot number (TN) = i (i = 0 to 7) and frame number (FN) with FN mod 208 =0  shall occur at the same time in all cells.

xiii)	For the primary COMPACT carrier, timeslot numbers (TN) 1, 3, 5, and 7 shall support channel combination xxii) in subclause 6.4.1. TNs 0, 2, 4, and 6 shall support channel combination xiii).

xiv)	For the secondary COMPACT carrier(s) carrying CPCCCH, timeslot numbers (TN) 1, 3, 5, and 7 shall support channel combination xxiii) in subclause 6.4.1. TNs 0, 2, 4, and 6 shall support channel combination xiii). CPCCCHs on secondary COMPACT carrier(s) shall be allocated on same time group as for primary COMPACT carrier.

xv)	For the secondary COMPACT carrier(s) not carrying CPCCCH, timeslot numbers (TN) 0 through 7 shall support channel combination xiii) in subclause 6.4.1.

xvi)	For COMPACT, BS_PAG_BLKS_RES shall be less than or equal to 8 and less than or equal to 10-BS_PBCCH_BLKS.

xvii)	For COMPACT, CFCCH, CSCH, CPBCCH, and CPCCCH are rotated as described in subclause 6.3.2.1. PDTCH, PACCH, and PTCCH do not rotate.

xviii)	For COMPACT, the parameters NIB_CCCH_0, NIB_CCCH_1, NIB_CCCH_2, and NIB_CCCH_3 shall not be broadcast for a serving time group.

xix)	For the COMPACT, NIB_CCCH_0, NIB_CCCH_1, NIB_CCCH_2, and NIB_CCCH_3 blocks shall be idle for non-serving time groups and rotate in accordance with the non-serving time groups.

The downlink position of the NIB_CCCH idle blocks is based on the ordered list as defined in subclause 6.3.2.1. The MS shall ignore these downlink idle blocks and shall interpret this action as not having detected an assigned USF value on an assigned PDCH.

xx)	For COMPACT large cells, NIB_CCCH_0, NIB_CCCH_1, NIB_CCCH_2, and NIB_CCCH_3 blocks shall be idle on timeslots immediately preceding and succeeding non-serving time groups and rotate in accordance with the non-serving time groups. The MS shall ignore these downlink idle blocks and shall interpret this action as not having detected an assigned USF value on an assigned PDCH.

The downlink position of the NIB_CCCH idle blocks is based on the ordered list as defined in subclause 6.3.2.1.

xxi)	For COMPACT, the MS attempts uplink random access on its designated serving time group (TG) by monitoring for USF=FREE in every downlink block.

For dynamic allocation, while in the uplink transfer state, the MS monitors all of the downlink non-idle blocks of its assigned PDCH for uplink assignments. The MS shall ignore downlink idle blocks and shall interpret this action as not having detected an assigned USF value on an assigned PDCH.

USF should be set equal to FREE for downlink non-idle blocks B0 on timeslot numbers (TN) 1, 3, 5, and 7.

xxii)	While in broadcast/multicast receive mode (see 3GPP TS 45.008), the MS shall continue to monitor system information either on the BCCH or, if present, on the PBCCH unless the network has indicated that system information and paging messages are sent on the PACCH for the MBMS radio bearer the MS listens to. If the network has not indicated that system information and paging messages are sent on the PACCH, or if the MS does not have an MS_ID, the MS shall additionally read paging messages either from the CCCH or, if the PBCCH is present, from the PCCCH. The MS shall then monitor the same paging group as in packet idle mode, i.e. shall determine the paging blocks to monitor using the methods described in subclause 6.5.2 or subclause 6.5.6. If the location of the MBMS radio bearer with respect to the control channels does not allow the mobile station to satisfy this requirement, the mobile station shall not read those radio blocks of the MBMS radio bearer that would prevent the monitoring of its paging group on the paging channel(s).

xxiii)	For a mobile in DLMC configuration, fallback to reception of a single carrier, irrespective of the number of assigned carriers,  is performed with regular periodicity. The periodicity is based on BS_PA_MFRMS, which indicates the number of 52-multiframes between two fallback periods. A mobile belongs to a particular single carrier fallback group which identifies a specific basic radio block period, or in case of RTTI mode two consecutive reduced radio block periods, within the set of “available” single carrier fallback blocks, during which it performs single carrier fallback. The single carrier fallback applies to all assigned PDCHs (or PDCH pairs, in case of RTTI mode) on the carrier where single carrier fallback is performed.

The method by which a particular mobile determines to which single carrier fallback group it belongs is defined in subclause 6.5.8. Single carrier fallback is performed on the carrier where the PTCCH is assigned. In case no PTCCH is assigned the single carrier fallback is performed on the carrier with the lowest number (see 3GPP TS 44.060 for numbering of carriers). Irrespective of which carrier is used, the single carrier fallback applies to all assigned PDCHs (in BTTI configuration)/PDCH-pairs (in RTTI configuration) on that carrier. In case of inter-band reception, the single carrier fallback only applies in one frequency band.

xxiv)	For EC-EGPRS, timeslot number 1 of radio frequency channel C0 of the cell allocation must support channel combination xxxvii) in subclause 6.4.1. No other timeslot or allocated channel from the cell allocation is allowed to support channel combination xxxvii).

xxv)	For EC-EGPRS, the parameter EC_BS_CC_CHANS in the EC-SI on EC-BCCH defines the number of basic physical channels supporting extended coverage common control channels (EC-CCCHs). All shall use timeslots on radio frequency channel C0 of the cell allocation. The first EC-CCCH shall use timeslot number 1, the second timeslot number 3, the third timeslot number 5 and the fourth timeslot number 7. Each EC-CCCH carries its own EC_CCCH_GROUP of mobiles in idle mode. Mobiles in a specific EC_CCCH_GROUP will listen for paging messages and make random accesses only on the specific EC-CCCH to which the EC_CCCH_GROUP belongs. In case 2 TS EC-RACH mapping is signaled by the network in EC-SI (see 3GPP TS 44.018), the physical resources where the EC-RACH is mapped for CC2, CC3 and CC4 shall be the physical channel used by EC_CCCH_GROUP and one timeslot lower, see table 6a. In this case the network shall ensure that a CCCH_GROUP is mapped onto the lower timeslot. The method by which a mobile determines the EC_CCCH_GROUP to which it belongs is defined in subclause 6.5.2a.

xxvi)	For EC-EGPRS, the EC-AGCH, and EC-PCH may share the same TDMA frame mapping (considered modulo 51) when combined onto a basic physical channel. The channels are shared on a block by block basis, and information within each block, when de-interleaved and decoded allows a mobile to determine whether the block contains paging messages or access grants. Table 6a of clause 7 defines the access grant blocks and paging blocks available per 51-multiframe for each Coverage Class respectively. The same number of paging blocks are always available per 51-multiframe. The number of access grant blocks can differ depending if the basic physical channel for EC-CCCH is timeslot number 1 or not.

xxvii)	For EC-EGPRS, the "available" paging blocks per eDRX cycle on an EC-CCCH is Coverage Class specific. It is determined by BS_ePA_MFRMS, see Table 6.5.6a-1, and the downlink Coverage Class of the MS, see subclause 6.5.2b. A mobile station selects one of the "available" paging blocks per eDRX cycle as its PAGING_GROUP based on IMSI. All MSs belonging to the same Coverage Class, using the same eDRX cycle and that has selected the same paging block are defined as being in the same PAGING_GROUP. The method by which a particular mobile determines to which particular PAGING_GROUP it belongs and hence which particular block of the “available” paging blocks on the paging channel is to be monitored is defined in subclause 6.5.2b.

For information

### 6.5.2	Determination of CCCH_GROUP and PAGING_GROUP for MS in idle mode

This sub-clause applies to the case where a MS is not using extended DRX cycles.

CCCH_GROUP (0 .. BS_CC_CHANS-1) = ((IMSI mod 1000) mod (BS_CC_CHANS x N)) div N

PAGING_GROUP (0 .. N-1) = ((IMSI mod 1000) mod (BS_CC_CHANS x N)) mod N

where

N = number of paging blocks "available" on one CCCH = (number of paging blocks "available" in a 51-multiframe on one CCCH) x BS_PA_MFRMS.

IMSI = International Mobile Subscriber Identity, as defined in 3GPP TS 23.003.

mod = Modulo.

div = Integer division.

4th modification

### 6.5.2b	Determination of EC_CCCH_GROUP and PAGING_GROUP for MS in idle mode when using extended DRX cycles for EC-EGPRS

EC_CCCH_GROUP (0 … EC_BS_CC_CHANS-1) of the mobile station = mod (IMSI, EC_BS_CC_CHANS) where EC_BS_CC_CHANS ranges from 1 to 4.

The paging group for EC-EGPRS when using eDRX is determined by the eDRX value negotiated between the MS and the network, see Table 6.5.6a-1, and downlink Coverage Class 1 (CC1), irrespective of the downlink CC used by the MS according to:

PAGING_GROUP_CC1(0 … N-1)  = mod (IMSI, N)

where

N = number of paging groups for CC1 on one EC-CCCH within a given eDRX cycle = BS_ePA_MFRMS*16 (see Table 6.5.6a-1).

In case no eDRX value has been negotiated between the MS and the network, the lowest eDRX cycle shall be assumed, i.e. BS_ePA_MFRMS = 8.

The paging group for a MS belonging to another downlink coverage class than CC1 (i.e. CC2, CC3 or CC4) shall be derived assuming that the physical resource of PAGING_GROUP_CC1 is contained within the physical resource of PAGING_GROUP for the downlink CC of the MS and is derived per Coverage Class according to the procedures below.

For CC1:

PAGING_GROUP (0 .. M-1)  = PAGING_GROUP_CC1

For CC2:

PAGING_GROUP (0 .. M-1)  = (PAGING_GROUP_CC1 div 4) mod 4 + 2*(PAGING_GROUP_CC1 div 32)

For CC3:

PAGING_GROUP (0 .. M-1)  = (PAGING_GROUP_CC1 div 8) mod 2 + 2*(PAGING_GROUP_CC1 div 32)

For CC4:

PAGING_GROUP (0 .. M-1)  = (PAGING_GROUP_CC1 div 8) mod 2 + 2*(PAGING_GROUP_CC1 div 64)

where

M = N div CC_DIV

Table 6.5.2-1. CC_DIV.

| CC1 | CC2 | CC3 | CC4 |
| --- | --- | --- | --- |
| 1 | 8 | 16 | 32 |

For example, a MS that uses eDRX, where eDRX cycle value = “0000” (BS_ePA_MFRMS=8) has been negotiated between the MS and the network (see Table 6.5.6a-1), and that belongs to CC4, will first derive PAGING_GROUP_CC1 according to mod (IMSI, 16*8). As a second step the PAGING_GROUP for the applicable downlink coverage class (CC4) is derived as (mod (IMSI, 16*8) div 8) mod 2 + 2*( mod (IMSI, 16*8) div 64).

For information

### 6.5.3	Determination of specific paging multiframe and paging block index

This sub-clause applies to the case where a MS is not using extended DRX cycles.

The required 51-multiframe occurs when:

PAGING_GROUP div (N div BS_PA_MFRMS) = (FN div 51) mod (BS_PA_MFRMS)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod (N div BS_PA_MFRMS)

where the index is then used with the look-up table 5 of clause 7 to determine the actual paging channel interleaved block to be monitored.

In GPRS non-DRX mode, the MS shall listen to all blocks of the CCCH channel.

5th modification

### 6.5.3a 	Determination of specific paging multiframe and paging block index for EC-EGPRS

#### 6.5.3a.1	CC1

The 51-multiframe where the paging block is mapped for CC1 occurs when:

PAGING_GROUP div (M div BS_ePA_MFRMS) = (FN div 51) mod (BS_ePA_MFRMS)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod (M div BS_ePA_MFRMS)

where the index is then used with the look-up table 6a of clause 7 to determine the actual paging channel interleaved block to be monitored.

#### 6.5.3a.2	CC2

The two 51-multiframes where the paging block is mapped for CC2 occurs when:

PAGING_GROUP div 4 = (FN div 102) mod (BS_ePA_MFRMS div 2)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 4

where the index is then used with the look-up table 6a of clause 7 to determine the actual paging channel interleaved block to be monitored.

#### 6.5.3a.3	CC3

The two 51-multiframes where the paging block is mapped for CC3 occurs when:

PAGING_GROUP div 2 = (FN div 102) mod (BS_ePA_MFRMS div 2)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 2

where the index is then used with the look-up table 6a of clause 7 to determine the actual paging channel interleaved block to be monitored.

#### 6.5.3a.4	CC4

The four 51-multiframes where the paging block is mapped for CC4 occurs when:

PAGING_GROUP div 2 = (FN div 204) mod (BS_ePA_MFRMS div 4)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 2

where the index is then used with the look-up table 6a of clause 7 to determine the actual paging channel interleaved block to be monitored.

6th modification

### 6.5.6a	Determination of extended DRX cycle for MS in GPRS attached mode

A MS that uses eDRX (see 3GPP TS 44.018 [10]) indicates an eDRX cycle value from Table 6.5.6a-1 when negotiating eDRX with the network (see 3GPP TS 24.008 [21] and 3GPP TS 23.060 [22]).  After power on and cell selection but before negotiating an eDRX cycle with the network a MS is not required to monitor the PCH. After cell reselection from a cell where eDRX was used but before acquiring System Information in the new cell a MS shall monitor the PCH according to its last negotiated eDRX cycle.

Table 6.5.6a-1: Set of eDRX Cycles Supported

| eDRX cycle value | eDRX cycle length | Number of 51-MF per eDRX cycle (BS_ePA_MFRMS) | eDRX cycles per hyperframe |
| --- | --- | --- | --- |
| 0000 | ~1.9 seconds | 8 | 6656 |
| 0001 | ~3.8 seconds | 16 | 3328 |
| 0010 | ~7.5 seconds | 32 | 1664 |
| 0011 | ~12.2 seconds | 52 | 1024 |
| 0100 | ~24.5 seconds | 104 | 512 |
| 0101 | ~49 seconds | 208 | 256 |
| 0110 | ~1.63 minutes | 416 | 128 |
| 0111 | ~3.25 minutes | 832 | 64 |
| 1000 | ~6.5 minutes | 1664 | 32 |
| 1001 | ~13 minutes | 3328 | 16 |
| 1010 | ~26 minutes | 6656 | 8 |
| 1011 | ~52 minutes | 13312 | 4 |
| Note 1: 53248 51-multiframes occur with the TDMA FN space (2715648 TDMA frames) Note 2: All remaining eDRX Cycle Values are reserved |  |  |  |

7th modification

# 7	Mapping tables

Table 1 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

Channel	Sub-channel	Direction	Allowable time slot	Allowable RF channel	Burst	Repeat length	Interleaved block 
designation	number 		assignments	assignments	type	in TDMA frames	TDMA frame mapping

TCH/FS, TCH/EFS,
TCH/AFS, TCH/F2.4,
TCH/WFS & 
O-TCH/WFS		D&U2	0 ... 7	C0 ... Cn	NB1	13	B0(0...7),B1(4...11),B2(8...11,0...3)

TCH/HS, TCH/AHS,
O-TCH/AHS &
O-TCH/WHS	0	D&U2	0 ... 7	C0 ... Cn	NB1	13	B0(0,2,4,6),B1(4,6,8,10),B2(8,10,0,2)
	1						B0(1,3,5,7),B1(5,7,9,11),B2(9,11,1,3)

FACCH/F
& O-FACCH/F		D&U	0 ... 7	C0 ... Cn	NB1	13	B0(0...7),B1(4...11),B2(8...11,0...3)

FACCH/H 
& O-FACCH/H	0	U	0 ... 7	C0 ... Cn	NB1	26	B0(0,2,4,6,8,10),B1(8,10,13,15,17,19),B2(17,19,21,23,0,2)
FACCH/H 
& O-FACCH/H	0	D	0 ... 7	C0 ... Cn	NB1	26	B0(4,6,8,10,13,15),B1(13,15,17,19,21,23),B2(21,23,0,2,4,6)
FACCH/H 
& O-FACCH/H	1	U	0 ... 7	C0 ... Cn	NB1	26	B0(1,3,5,7,9,11),B1(9,11,14,16,18,20),B2(18,20,22,24,1,3)
FACCH/H 
& O-FACCH/H	1	D	0 ... 7	C0 ... Cn	NB1	26	B0(5,7,9,11,14,16),B1(14,16,18,20,22,24),B2(22,24,1,3,5,7)

E-FACCH/F		D&U	0 … 7	C0 … Cn	NB1	13	B0(0...3),B1(4...7),B2(8...11)

E-IACCH/F		D&U	0 ... 7	C0 ... Cn		26	B0(0 ... 3)B1(4 ... 7)B2(8 ... 11)B3(13 ... 16)

B4(17 ... 20)B5(21 ... 24)

SACCH/TF & 
SACCH/TPF		D&U2	0	C0 ... Cn	NB3	104	B(12, 38, 64, 90)	NOTE 1:
SACCH/TF & 
SACCH/TPF		D&U2	1	C0 ... Cn	NB3	104	B(25, 51, 77, 103)	An Access Burst (AB) is used
SACCH/TF & 
SACCH/TPF		D&U2	2	C0 ... Cn	NB3	104	B(38, 64, 90, 12)	on the uplink during handover
SACCH/TF & 
SACCH/TPF		D&U2	3	C0 ... Cn	NB3	104	B(51, 77, 103, 25)	and on channels used for voice
SACCH/TF & 
SACCH/TPF		D&U2	4	C0 ... Cn	NB3	104	B(64, 90, 12, 38)	group calls when a request to
SACCH/TF & 
SACCH/TPF		D&U2	5	C0 ... Cn	NB3	104	B(77, 103, 25, 51)	talk is made.
SACCH/TF & 
SACCH/TPF		D&U2	6	C0 ... Cn	NB3	104	B(90, 12, 38, 64)
SACCH/TF & 
SACCH/TPF		D&U2	7	C0 ... Cn	NB3	104	B(103, 25, 51, 77)

SACCH/M & 
SACCH/MP		D&U2	0 ... 7	C0 ... Cn	NB3	104	B(12, 38, 64, 90)

SACCH/CTS		D&U	0, 1	C0 ... Cn	NB	104	B(12, 38, 64, 90)

SACCH/CTS		D&U	2, 3	C0 ... Cn	NB	104	B(38, 64, 90, 12)

SACCH/CTS		D&U	4, 5	C0 ... Cn	NB	104	B(64, 90, 12, 38)

SACCH/CTS		D&U	6, 7	C0 ... Cn	NB	104	B(90, 12, 38, 64)

SACCH/TH &	0	D&U2	0	C0 ... Cn	NB3	104	B(12, 38, 64, 90)	NOTE 2:
SACCH/TPH	1						B(25, 51, 77, 103)	The uplink of a channel 
SACCH/TH &	0	D&U2	1	C0 ... Cn	NB3	104	B(12, 38, 64, 90)	used for voice broadcast
SACCH/TPH	1						B(25, 51, 77, 103)	or a voice group call may
SACCH/TH &	0	D&U2	2	C0 ... Cn	NB3	104	B(38, 64, 90, 12)	actually not be used.
SACCH/TPH	1						B(51, 77, 103, 25)
SACCH/TH &	0	D&U2	3	C0 ... Cn	NB3	104	B(38, 64, 90, 12)
SACCH/TPH	1						B(51, 77, 103, 25)
SACCH/TH &	0	D&U2	4	C0 ... Cn	NB3	104	B(64, 90, 12, 38)
SACCH/TPH	1						B(77, 103, 25, 51)	NOTE 3:
SACCH/TH &	0	D&U2	5	C0 ... Cn	NB3	104	B(64, 90, 12, 38)	An Access Burst (AB) may be 
SACCH/TPH	1						B(77, 103, 25, 51)	used on the uplink during 
SACCH/TH &	0	D&U2	6	C0 ... Cn	NB3	104	B(90, 12, 38, 64)	handover.
SACCH/TPH	1						B(103, 25, 51, 77)
SACCH/TH &	0	D&U2	7	C0 ... Cn	NB3	104	B(90, 12, 38, 64)
SACCH/TPH	1						B(103, 25, 51, 77)

EPCCH/F		D&U2	0,2,4,6	C0 ... Cn	NB3	26	B0(12) 
EPCCH/F		D&U2	1,3,5,7	C0 ... Cn	NB3	26	B0(25) 
EPCCH/M		D&U2	0 … 7	C0 ... Cn	NB3	26	B0(12)

EPCCH/H	0	D&U2	0 … 7	C0 ... Cn	NB3	26	B0(12) 
	1						B0(25)

Table 1a: Mapping of logical channels onto physical channels (see subclause 6.3.1.1)

Channel	Sub-channel	Direction	Allowable time slot	Allowable RF channel	Burst	Repeat length	Interleaved block 
designation	number 		assignments	assignments	type	in TDMA frames	TDMA frame mapping

TCH/FS, TCH/EFS,
TCH/AFS,
TCH/WFS		D&U2	0, 2, 4, 6	C0 ... Cn	NB1	26	B0(0...7),B1(4...11),B2(8...12,14...16),B3(12,14…20),
							B4(17…24),B5(21..24,0..3)

TCH/FS, TCH/EFS,
TCH/AFS,
TCH/WFS		D&U2	 1, 3 , 5, 7	C0 ... Cn	NB1	26	B0(0...7),B1(4...11),B2(8...11,13...16),B3(13…20),
							B4(17…23,25),B5(21..23,25,0..3)

TCH/HS, TCH/AHS	0	D&U2	0 ... 7	C0 ... Cn	NB1	26	B0(0, 2, 4, 6),B1(4, 6, 8, 10),B2(8, 10, 12, 15),
							B3(12, 15, 17,19),B4(17, 19, 21, 23),B5(21, 23, 0, 2)
	1						B0(1, 3, 5, 7),B1(5, 7, 9, 11),B2(9, 11, 14, 16),
							B3(14, 16, 18, 20),B4(18, 20, 22, 25),B5(22, 25, 1, 3)

FACCH/F		D&U	 0,2,4,6	C0 ... Cn	NB1	26	B0(0...7),B1(4...11),B2(8...12,14...16),B3(12,14…20),
							B4(17…24) ,B5(21..24,0..3)

FACCH/F		D&U	1,3,5,7	C0 ... Cn	NB1	26	B0(0...7),B1(4...11),B2(8...11,13...16),B3(13…20),
							B4(17…23,25) ,B5(21..23,25,0..3)

FACCH/H 	0	U	0 ... 7	C0 ... Cn	NB1	26	B0(0,2,4,6,8,10),B1(8,10,12, 15,17,19),B2(17,19,21,23,0,2)
FACCH/H 	0	D	0 ... 7	C0 ... Cn	NB1	26	B0(4,6,8,10,12, 15),B1(12,15,17,19,21,23),B2(21,23,0,2,4,6)
FACCH/H 	1	U	0 ... 7	C0 ... Cn	NB1	26	B0(1,3,5,7,9,11),B1(9,11,14,16,18,20),B2(18,20,22,25,1,3)
FACCH/H 	1	D	0 ... 7	C0 ... Cn	NB1	26	B0(5,7,9,11,14,16),B1(14,16,18,20,22,25),B2(22,25,1,3,5,7)

SACCH/TF & 
SACCH/TPF		D&U2	0	C0 ... Cn	NB3	104	B(13, 39, 65, 91) 
SACCH/TF & 
SACCH/TPF		D&U2	1	C0 ... Cn	NB3	104	B(24, 50, 76, 102) 
SACCH/TF & 
SACCH/TPF		D&U2	2	C0 ... Cn	NB3	104	B(39, 65, 91, 13) 
SACCH/TF & 
SACCH/TPF		D&U2	3	C0 ... Cn	NB3	104	B(50, 76, 102, 24) 
SACCH/TF & 
SACCH/TPF		D&U2	4	C0 ... Cn	NB3	104	B(65, 91, 13, 39) 
SACCH/TF & 
SACCH/TPF		D&U2	5	C0 ... Cn	NB3	104	B(76, 102, 24, 50) 
SACCH/TF & 
SACCH/TPF		D&U2	6	C0 ... Cn	NB3	104	B(91, 13, 39, 65)
SACCH/TF & 
SACCH/TPF		D&U2	7	C0 ... Cn	NB3	104	B(102, 24, 50, 76)

SACCH/M & 
SACCH/MP		D&U2	0 ... 7	C0 ... Cn	NB3	104	B(13, 39, 65, 91)

SACCH/CTS		D&U	0, 1	C0 ... Cn	NB	104	B(13, 39, 65, 91)

SACCH/CTS		D&U	2, 3	C0 ... Cn	NB	104	B(39, 65, 91, 13)

SACCH/CTS		D&U	4, 5	C0 ... Cn	NB	104	B(65, 91, 13, 39)

SACCH/CTS		D&U	6, 7	C0 ... Cn	NB	104	B(91, 13, 39, 65)

SACCH/TH &	0	D&U2	0	C0 ... Cn	NB3	104	B(13, 39, 65, 91) 
SACCH/TPH	1						B(24, 50, 76, 102) 
SACCH/TH &	0	D&U2	1	C0 ... Cn	NB3	104	B(13, 39, 65, 91)
SACCH/TPH	1						B(24, 50, 76, 102) 
SACCH/TH &	0	D&U2	2	C0 ... Cn	NB3	104	B(39, 65, 91, 13)
SACCH/TPH	1						B(50, 76, 102, 24)
SACCH/TH &	0	D&U2	3	C0 ... Cn	NB3	104	B(39, 65, 91, 13)
SACCH/TPH	1						B(50, 76, 102, 24)
SACCH/TH &	0	D&U2	4	C0 ... Cn	NB3	104	B(65, 91, 13, 39)
SACCH/TPH	1						B(76, 102, 24, 50) 
SACCH/TH &	0	D&U2	5	C0 ... Cn	NB3	104	B(65, 91, 13, 39)
SACCH/TPH	1						B(76, 102, 24, 50) 
SACCH/TH &	0	D&U2	6	C0 ... Cn	NB3	104	B(91, 13, 39, 65)
SACCH/TPH	1						B(102, 24, 50, 76)
SACCH/TH &	0	D&U2	7	C0 ... Cn	NB3	104	B(91, 13, 39, 65)
SACCH/TPH	1						B(102, 24, 50, 76)

EPCCH/F		D&U2	0,2,4,6	C0 ... Cn	NB3	26	B0(13) 
EPCCH/F		D&U2	1,3,5,7	C0 ... Cn	NB3	26	B0(24) 
EPCCH/M		D&U2	0 … 7	C0 ... Cn	NB3	26	B0(13)

EPCCH/H	0	D&U2	0 … 7	C0 ... Cn	NB3	26	B0(13) 
	1						B0(24)

NOTE 1: An Access Burst (AB) is used on the uplink during handover and on channels used for voice group calls when a request to talk is made.

NOTE 2: The uplink of a channel used for voice broadcast or a voice group call may actually not be used.

NOTE 3: An Access Burst (AB) may be used on the uplink during handover.

Table 2 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

Channel	Sub-	Direction	Allowable	Allowable	Burst	Repeat	Interleaved block
designation	channel		timeslot	RF channel	type	length in	TDMA frame
	number		assignments	assignments		TDMA frames	mapping

TCH/F4.8		D&U	0 ... 7	C0 ... Cn	NB1	26	B0(0 ... 11, 13 ... 22)
TCH/F9.6							B1(4 ... 11, 13 ... 24, 0, 1)
TCH/F14.4							B2(8 ... 11, 13 ... 24, 0 ... 5)
							B3(13 ... 24, 0 ... 9)
							B4(17 ... 24, 0 ... 11, 13, 14)
							B5(21 ... 24, 0 ... 11, 13 ... 18)

E-TCH/F28.8		D&U	0 ... 7	C0 ... Cn	NB (8PSK)	26	B0(0 ... 11, 13 ... 22)
E-TCH/F32.0							B1(4 ... 11, 13 ... 24, 0, 1)
E-TCH/F43.2							B2(8 ... 11, 13 ... 24, 0 ... 5)
							B3(13 ... 24, 0 ... 9)
							B4(17 ... 24, 0 ... 11, 13, 14)
							B5(21 ... 24, 0 ... 11, 13 ... 18)

TCH/H2.4	0	D&U	0 ... 7	C0 ... Cn	NB1	26	B0(0,2,4,6,8,10,13,15,17,19,21,
TCH/H4.8							23,0,2,4,6,8,10,13,15,17,19)
							B1(8,10,13,15,17,19,21,23,0,2,4,
							6,8,10,13,15,17,19,21,23,0,2),
							B2(17,19,21,23,0,2,4,6,8,10,13,
							15,17,19,21,23,0,2,4,6,8,10)

	1						B0(1,3,5,7,9,11,14,16,18,20,22,
							24,1,3,5,7,9,11,14,16,18,20),
							B1(9,11,14,16,18,20,22,24,1,3,5,
							7,9,11,14,16,18,20,22,24,1,3),
							B2(18,20,22,24,1,3,5,7,9,11,14,
							16,18,20,22,24,1,3,5,7,9,11)

NOTE 1:	An Access Burst (AB) is used on the uplink during handover.

Table 3 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

Channel	Sub-	Direction	Allowable	Allowable	Burst	Repeat	Interleaved block
designation	channel		timeslot	RF channel	type	length in	TDMA frame
	number		assignments	assignments		TDMA frames	mapping

FCCH		D	0	C0	FB	51	B0(0),B1(10),B2(20),B3(30),B4(40)

SCH		D	0	C0	SB	51	B0(1),B1(11),B2(21),B3(31),B4(41)

BCCH Norm		D	0,2,4,6	C0	NB	51	B(2..5)

BCCH Ext		D	0,2,4,6	C0	NB	51	B(6...9)

PCH		D	0,2,4,6	C0	NB	51	B0(6..9),B1(12..15),B2(16..19)
AGCH							B3(22..25),B4(26..29),B5(32..35),
							B6(36..39),B7(42..45),B8(46..49)

NCH		D	0	C0	NB	51	B0(6..9),B1(12..15),B2(16..19)
							B3(22..25),B4(26..29),B5(32..35),
							B6(36..39)

RACH		U	0,2,4,6	C0	AB	51	B0(0),B1(1)..B50(50)

CBCH(SDCCH/4)		D	0	C0	NB	51	B(32..35)

CBCH(SDCCH/8)		D	0 ... 3	C0 ... Cn	NB	51	B(8..11)

SDCCH/4	0	D	0	C0	NB1	51	B(22..25)
		U					B(37..40)
	1	D					B(26..29)
		U					B(41..44)
	2	D					B(32..35)
		U					B(47..50)
	3	D					B(36..39)
		U					B(0..3)

SACCH/C4	0	D	0	C0	NB3	102	B(42..45)
		U					B(57..60)
	1	D					B(46..49)
		U					B(61..64)
	2	D					B(93..96)
		U					B(6..9)
	3	D					B(97..100)
		U					B(10..13)

NOTE 1:	An Access Burst (AB) is used on the uplink during handover.

NOTE 3:	An Access Burst (AB) may be used on the uplink during handover.

Table 4 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

Channel	Sub-	Direction	Allowable	Allowable	Burst	Repeat	Interleaved block
designation	channel		timeslot	RF channel	type	length in	TDMA frame
	number		assignments	assignments		TDMA frames	mapping

SDCCH/8	0	D	0 ... 7	C0 ... Cn	NB1	51	B (0 ... 3)
		U					B (15 ... 18)
	1	D					B (4 ... 7)
		U					B (19 ... 22)
	2	D					B (8 ... 11)
		U					B (23 ... 26)
	3	D					B (12 ... 15)
		U					B (27 ... 30)
	4	D					B (16 ... 19)
		U					B (31 ... 34)
	5	D					B (20 ... 23)
		U					B (35 ... 38)
	6	D					B (24 ... 27)
		U					B (39 ... 42)
	7	D					B (28 ... 31)
		U					B (43 ... 46)

SACCH/C8	0	D	0 ... 7	C0 ... Cn	NB3	102	B (32 ... 35)
		U					B (47 ... 50)
	1	D					B (36 ... 39)
		U					B (51 ... 54)
	2	D					B (40 ... 43)
		U					B (55 ... 58)
	3	D					B (44 ... 47)
		U					B (59 ... 62)
	4	D					B (83 ... 86)
		U					B (98 ... 101)
	5	D					B (87 ... 90)
		U					B (0 ... 3)
	6	D					B (91 ... 94)
		U					B (4 ... 7)
	7	D					B (95 ... 98)
		U					B (8 ... 11)

NOTE 1:	An Access Burst (AB) is used on the uplink during handover.

NOTE 3:	An Access Burst (AB) may be used on the uplink during handover.

Table 5 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

BS_CCCH_SDCCH_COMB

Random access channel blocks available

Access grant blocks available (NOTE: Some access grant blocks may also be used for the NCH)

BS_AG_BLKS_RES

Number of paging blocks available per 51-multiframe

Paging channel blocks available

(Paging block index = 0, 1, 2, 3, 4, 5, 6, 7, 8)

False	B0, B1 ... B50	B0, B1 ... B8	0	9	B0, B1, B2, B3, B4, B5, B6, B7, B8

False			1	8	B1, B2, B3, B4, B5, B6, B7, B8

False			2	7	B2, B3, B4, B5, B6, B7, B8

False			3	6	B3, B4, B5, B6, B7, B8

False			4	5	B4, B5, B6, B7, B8

False			5	4	B5, B6, B7, B8

False			6	3	B6, B7, B8

False			7	2	B7, B8

True	B4, B5, B14, B15 ... B36, B45, B46	B0, B1, B2	0	3	B0, B1, B2

True			1	2	B1, B2

True			2	1	B2

Table 6 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5) -

| Channel designation | Sub-channel number | Direction | Allowable time-slot assignment | Allowable RF channel assignment | Burst type | Repeat length in TDMA frames | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PDTCH/F, PACCH/F |  | D&U | 0...7 | C0...Cn | NB1 | 52 | BTTI configuration: B0(0...3), B1(4...7), B2(8...11), B3(13...16), B4(17...20), B5(21..24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) RTTI configuration: B0a(0,0’,1,1’), B0b(2,2’,3,3’), B1a(4,4’,5,5’), B1b(6,6’,7,7’), B2a(8,8’,9,9’), B2b(10,10’,11,11’), B3a(13,13’,14,14’), B3b(15,15’,16,16’), B4a(17,17’,18,18’), B4b(19,19’,20,20’), B5a(21,21’,22,22’), B5b(23,23’,24,24’), B6a(26,26’,27,27’), B6b(28,28’,29,29’), B7a(30,30’,31,31’), B7b(32,32’,33,33’), B8a(34,34’,35,35’), B8b(36,36’,37,37’), B9a(39,39’,40,40’), B9b(41,41’,42,42’), B10a(43,43’,44,44’), B10b(45,45’,46,46’), B11a(47,47’,48,48’), B11b(49,49’,50,50’) Note : In RTTI configuration, a PDTCH is mapped onto two PDCHs, a PDCH-pair. Number n indicates mapping on the PDCH with the lowest timeslot number in TDMA frame n, whereas number n’ indicates mapping on the PDCH with the highest timeslot number in TDMA frame n. |
| PDTCH/H, PACCH/H | 0 1 | D&U D&U | 0...7 0…7 | C0... Cn C0…Cn | NB1 NB1 | 52 52 | B0(0,2,4,6), B1(8,10,13,15), B2(17,19,21,23), B3(26,28,30,32), B4(34,36,39,41), B5(43,45,47,49) B0(1,3,5,7), B1(9,11,14,16), B2(18,20,22,24), B3(27,29,31,33), B4(35,37,40,42), B5(44,46,48,50) |
| PBCCH |  | D | 0...7 | C0...Cn | NB | 52 | B0(0... 3), B3(13...16), B6(26...29), B9(39...42) |
| PRACH |  | U | 0...7 | C0...Cn | AB | 52 | B0(0)...B11(11), B12(13)...B23(24), B24(26)... B35(37), B36(39)...B47(50) |
| MPRACH |  | U | 0...7 | C0...Cn | AB | 52 | B0(0)...B11(11), B12(13)...B23(24), B24(26)... B35(37), B36(39)...B47(50) |
| PPCH |  | D | 0...7 | C0...Cn | NB | 52 | B1(4 ... 7), B2(8...11), B3(13...16), B4(17...20), B5(21..24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
| PAGCH |  | D | 0...7 | C0...Cn | NB | 52 | B0(0...3), B1(4 ... 7), B2(8...11), B3(13...16), B4(17...20), B5(21..24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
| PTCCH/D |  | D | 0...7 | C0...Cn | NB | 416 | B0(12,38,64,90), B1(116,142,168,194), B2(220,246,272,298), B3(324,350,376,402) |
| PTCCH/U | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 | U | 0...7 | C0...Cn | AB | 416 | B0(12) B0(38) B0(64) B0(90) B0(116) B0(142) B0(168) B0(194) B0(220) B0(246) B0(272) B0(298) B0(324) B0(350) B0(376) B0(402) |

NOTE 1:	An Access Burst (AB) may be used on the uplink as polling response.

Table 6a : Mapping of Extended Coverage logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

| Channel designation | Dir. | Allowable timeslot assignment | Allowable RF channel assignment | Burst type | Repeat length TDMA frames | Coverage class (CC) | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC-SCH | D | 1 | C0 | SB | 204 | - | B0([0,1,2,3,4,5,6] + 51 N), N=0,1,2,3 |
| EC-BCCH | D | 1,3,5,7 | C0 | NB | 408 | - | B0([7,8,9,10,11,12,13,14]+51N), N=0,…,7 |
| EC-PCH | D | 1,3,5,7 | C0 | NB | 51 | 1 | B0(19,20),B1(21,22),…,B15(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,1 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,3 |
| EC-AGCH | D | 1 | C0 | NB | 51 | 1 | B0(11,12),B1(13,14),…,B19(49,50) |
|  |  |  |  |  | 102 | 2 | B0([11,…,18] + 51N),B1([19,…,26] + 51N),…,B4([43,…,50] + 51N) |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,3 |
|  |  | 3,5,7 |  |  | 51 | 1 | B0(1,2),B1(3,4),…,B24(49,50) |
|  |  |  |  |  | 102 | 2 | B0([3,…,10] + 51N),B1([11,…,18] + 51N),…,B5([43,…,50] + 51N) |
|  |  |  |  |  | 102 | 3 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,1 |
|  |  |  |  |  | 204 | 4 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,1,2,3 |
| EC-RACH, 1 TS | U | 1,3,5,7 | C0 | AB | 51 | 1 | B0(0),B1(1),..B50(50) |
|  |  |  |  |  |  | 2 | B0(1,…,4), B1(5,…,8),...B11(45,…,48) |
|  |  |  |  |  |  | 3 | B0(2,…, 17),B1(18,…,33)…, B2(34,…,49) |
|  |  |  |  |  | 102 | 4 | B0([3,…,26] + 51N),B1([27,…,50] + 51N), N=0,1 |
| EC-RACH, 2 TS | U | 0,1, or, 2,3, or, 4,5, or, 6,7 | C0 | AB | 51 | 2 | B0(1,2,1’,2’), B1(3,4,3’,4’),...B24(49,50,49’,50’) |
|  |  |  |  |  |  | 3 | B0(2,…,9,1’,…,9’), B1(10,…,17,10’,…,17’),...,B5(42,49,42’,49’) |
|  |  |  |  |  | 102 | 4 | B0([3,…,14,3’,…,14’] + 51N),…,B3([39,…,50,39’,…,50’] + 51N), N=0,1 |
| EC-PACCH, EC-PDTCH | D&U | 0…7 | C0…Cn | NB2 | 52 | 1 | B0(0...3), B1(4...7), B2(8...11), B3(13...16), B4(17...20), B5(21…24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
|  |  |  |  |  |  | 2 | B0(0...3,0'...3',0''...3'',0'''...3'''),B1(4...7,4'...7',4''...7'',4'''...7'''), B2(8...11, 8'...11',8''...11'', 8'''...11'''), B3(13...16,13'...16',13''...16'',13'''...16'''), B4(17...20,17'...20',17''...20'',17'''...20'''), B5(21..24,21'...24',21''...24'',21'''...24'''), B6(26...29,26'...29',26''...29'',26'''...29'''), B7(30...33,30'...33',30''...33'',30'''...33'''), B8(34...37,34'...37',34''...37'',34'''...37'''), B9(39...42,39'...42',39''...42'',39'''...42'''), B10(43...46,43'...46',43''...46'',43'''...46'''), B11(47...50,47'...50',47''...50'',47'''...50''')1 |
|  |  |  |  |  |  | 3 | B0(0...7,0'...7',0''...7'',0'''...7'''), B1(8...11,13…16,8'...11',13'…16',8''...11'',13''…16'',8'''...11''',13''',…,16'''), B2(17...24, 17'...24',17''...24'', 17'''...24'''),B3(26...33,26'...33',26''...33'',26'''...33'''), B4(34...37,39…42,34'...37',39'…42',34''...37'',39''…42'',34'''...37''',39'''…42'''), B5(43...50,43'...50',43''...50'',43'''...50''')1 |
|  |  |  |  |  |  | 4 | B0(0...11,13…16,0'...11',13'…16',0''...11'',13''…16'',0'''...11''',13'''…16'''), B1(17...24,26…33,17'...24',26'…33',17''...24'',26''…33'',17'''...24''',26'''…33'''), B2(34...37,39...50,34'...37',39'...50',34’’...37’’,39'’...50'’,34'’'...37'’',39'''...50''')1 |
| NOTE 1: A EC-PDTCH is mapped onto four PDCHs. Number n indicates mapping on the PDCH with the lowest timeslot number in TDMA frame n, whereas number n', n'', and n''' indicates mapping on the PDCH with the second lowest, second highest, and highest timeslot number in TDMA frame n respectively. The four timeslots shall be consecutive. NOTE 2: Access Bursts (AB) instead of Normal Bursts (NB) may be used on the uplink as polling response on EC-PACCH/U. The same resources are used by the Access Burst. |  |  |  |  |  |  |  |

Table 7 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

Non-COMPACT :

BS_PAG_BLKS_RES + BS_PBCCH_BLKS

Number of paging blocks available per 52-multiframe

Paging channel blocks available for 52-multiframe

(Paging block index = 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10)

1	11	B1, B2, B3, B4, B5, B6, B7, B8, B9, B10, B11

2	10	B1, B2, B3, B4, B5, B7, B8, B9, B10, B11

3	9	B1, B2, B4, B5, B7, B8, B9, B10, B11

4	8	B1, B2, B4, B5, B7, B8, B10, B11

5	7	B2, B4, B5, B7, B8, B10, B11

6	6	B2, B4, B5, B8, B10, B11

7	5	B2, B5, B8, B10, B11

8	4	B2, B5, B8, B11

9	3	B5, B8, B11

10	2	B5, B11

11	1	B11

Table 7a : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

COMPACT :

|  | BS_PBCCH_BLKS = 1 | BS_PBCCH_BLKS = 2 |
| --- | --- | --- |
| BS_PAG_ BLKS_RES | Paging channel blocks available for 52-multiframe (Paging block index = 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10) | Paging channel blocks available for 52-multiframe (Paging block index = 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10) |
| 0 1 2 3 4 5 6 7 8 | B1, B2, B3, B4, B5, B6, B7, B8, B9, B10, B11 B1, B2, B3, B4, B5, B6, B7, B8, B9, B10 B1, B2, B3, B4, B6, B7, B8, B9, B10 B1, B2, B3, B4, B6, B7, B9, B10 B1, B3, B4, B6, B7, B9, B10 B1, B3, B4, B6, B7, B9 B1, B3, B6, B7, B9 B1, B3, B6, B9 B3, B6, B9 | B1, B2, B3, B4, B5, B7, B8, B9, B10, B11 B1, B2, B3, B4, B5, B7, B8, B9, B10 B1, B2, B3, B4, B7, B8, B9, B10 B1, B2, B3, B4, B7, B9, B10 B1, B3, B4, B7, B9, B10 B1, B3, B4, B7, B9 B1, B3, B7, B9 B1, B3, B9 B3, B9 |
|  | BS_PBCCH_BLKS = 3 | BS_PBCCH_BLKS = 4 |
| BS_PAG_ BLKS_RES | Paging channel blocks available for 52-multiframe (Paging block index = 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10) | Paging channel blocks available for 52-multiframe (Paging block index = 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10) |
| 0 1 2 3 4 5 6 7 8 | B1, B2, B4, B5, B7, B8, B9, B10, B11 B1, B2, B4, B5, B7, B8, B9, B10 B1, B2, B4, B7, B8, B9, B10 B1, B2, B4, B7, B9, B10 B1, B4, B7, B9, B10 B1, B4, B7, B9 B1, B7, B9 B1, B9 - | B1, B2, B4, B5, B7, B8, B10, B11 B1, B2, B4, B5, B7, B8, B10 B1, B2, B4, B7, B8, B10 B1, B2, B4, B7, B10 B1, B4, B7, B10 B1, B4, B7 B1, B7 - - |

NOTE: In COMPACT, BS_PAG_BLKS_RES shall be less than or equal to 8 and less than or equal to 10 BS_PBCCH_BLKS.

Table 8 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

| Channel designation | Sub-channel number | Direction | Allowable time-slot assignment | Allowable RF channel assignment | Burst type | Repeat length in TDMA frames | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CTSBCH-FB |  | D | 0...7 | C0 | FB | 52 | B(25) |
| CTSBCH-SB |  | D | 0...7 | C0 | SB | 52 | B(51) |
| CTSPCH |  | D | 0...7 | C0…Cn | NB | 52 | B(2...5) |
| CTSARCH |  | U | 0...7 | C0 C0…Cn | SB | 52 | B0(2)…B7(9) B8(10)…B13(15) |
| CTSAGCH |  | D | 0...7 | C0 C0…Cn | NB | 52 | B0(16..19) B1(20..23) |

NOTE 1: In CTS, C0 shall be interpreted as CTSBCH RF channel and C0…Cn as the RF channels of the TFH carrier list (refer to 3GPP TS 45.056).

NOTE 2: For the precise specification of the timeslot assignment, refer to subclause 6.3.3.1 and 6.3.3.2.

Table 9 : Mapping of COMPACT logical channels onto physical channels (see subclauses 6.3, 6.4, and 6.5)

| Channel Designation | Sub- Channel Number | Direction | Allowable Timeslot Alignment | Allowable RF Channel Assignment | Burst Type | Repeat Length in TDMA Frames | Interleaved Block TDMA Frame Mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CFCCH |  | D | 1, 3, 5, 7 | C0 … Cn | FB | 52 | B0 (25) |
| CSCH |  | D | 1, 3, 5, 7 | C0 … Cn | SB | 52 | B0 (51) |
| CPBCCH |  | D | 1, 3, 5, 7 | C0 … Cn | NB | 52 | B0 (0 … 3), B6 (26 … 29), B3 (13 … 16), B9 (39 … 42) |
| CPRACH |  | U | 1, 3, 5, 7 | C0 … Cn | AB | 52 | B0 (0) … B11 (11), B12 (13) … B23 (24), B24 (26) … B35 (37), B36 (39) … B47 (50) |
| CPAGCH, CPPCH |  | D | 1, 3, 5, 7 | C0 … Cn | NB | 52 | B1 (4 … 7), B2 (8 … 11), B3 (13 … 16), B4 (17 … 20), B5 (21 … 24), B6 (26 … 29), B7 (30 … 33), B8 (34 … 37), B9 (39 … 42), B10 (43 … 46), B11 (47 … 50) |

Figure 1: Mapping of logical channels onto physical channels based on the physical resource

Figure 2: The structure imposed on the physical resource: Timeslots, TDMA Frames and Radio Frequency channels
(in this example the cell has an allocation of 4 RF Channels pairs

Figure 3: Timeslot and format of bursts

(This example of a physical channel is non-hopping using timeslot 0 of every TDMA frame)

Figure 4: Mobile Station usage of physical channel timeslots (For a full-rate hopping traffic channel assigned timeslot 3)

(This example of a physical channel is hopping using 3 of every other TDMA frame)

Figure 5: Example of two different physical channels

Figure 6: Block diagram of the frequency hopping algorithm when HSN  0

Figure 6a: General structure of hopping sequence generation for CTS

NOTE:	Example with vector a = (a0, a1, a2, a3) = (5, 8, 2, 11).

Figure 7b: TDMA frame mapping for TCH/HS + SACCH/HS sub-channels 0 and 1

Figure 7c: TDMA frame mapping for TCH/FS + SACCH/FS for VAMOS II and VAMOS III MS when assigned a training sequence in TSC Set 2 or Set 4

Figure 7d: TDMA frame mapping for TCH/HS + SACCH/HS sub-channels 0 and 1 for VAMOS II and VAMOS III MS when assigned a training sequence in TSC Set 2 or Set 4

Figure 7: Example of a TDMA frame mapping for traffic channels

Figure 8c: TDMA Frame mapping for EC-SCH+EC-BCCH+EC-CCCH

Figure 8: Example of TDMA Frame mapping for control channels

52 TDMA Frames

X = Idle frame
T = Frame used for PTCCH
B0 - B11 = Radio blocks

Figure 9: 52- multiframe for PDCHs in BTTI configuration

Figure 9a: 52- multiframe for PDCH-pairs in RTTI configuration

Figure 9b: 52-multiframe for EC-EGPRS for different Coverage Classes (CC)

52 TDMA Frames

A : 	TDMA frame for CTSARCH

B : 	TDMA frame for CTSBCH

P : 		TDMA frame for CTSPCH

G :		TDMA frame for CTSAGCH

X :		Idle frame

Figure 10: 52-multiframe for CTS

End of modifications