3GPP TSG-GERAN Meeting #69	GP-160069

# St.Julian's, Malta, 15th – 19th February, 2016							(Revision of GP-151117)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.060 | CR | 1615 | rev | 1 | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EC-EGPRS - RLC/MAC block structure (Rel-13) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G2 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2016-02-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of EC-EGPRS - RLC/MAC block structures. The RLC/MAC block structures and headers have been updated for EC-EGPRS operation for both the data and control plane as well as for uplink and downlink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RLC/MAC block structures and headers have been updated for EC-EGPRS operation where new headers and header fields have been created for EC-EGPRS operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EC-EGPRS operation will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.0a, 10.0a2, 10.0a3 10.0b.2, 10.0b.2.2, 10.0b.3, 10.0b.3.2, 10.3.1.3(New), 10.3.2, 10.3a, 10.3a.0, 10.3a.1, 10.3a.2, 10.3a.3, 10.3a.3.1, 10.3a.3.2, 10.3a.3.3, 10.3a.4, 10.3a.4.1, 10.3a.4.2, 10.3a.4.3, 10.4.2, 10.4.3, 10.4.4a, 10.4.4c(New), 10.4.5, 10.4.6, 10.4.6a(New), 10.4.7, 10.4.8a, 10.4.8a.1, 10.4.8a.2, 10.4.8a.3, 10.4.8b, 10.4.9f(New), 10.4.9g(New), 10.4.10a, 10.4.10b(New),10.4.12, 10.4.14a,10.4.20, 10.4.27, 10.4.28(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1612, TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.010 CR 0065 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR TS 51.021 CR |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

================ first change ================

# 10	RLC/MAC block structure

## 10.0a	RLC/MAC block structure

Different RLC/MAC block structures are defined for data transfers and for control message transfers. The RLC/MAC block structures for data transfers are different for GPRS and (EC-)EGPRS, whereas the same RLC/MAC block structure is used for control message transfers.

### 10.0a.1	GPRS RLC/MAC block for data transfer

The RLC/MAC block for GPRS data transfer consists of a MAC header and an RLC data block. The RLC data block consists of an RLC header, an RLC data unit and spare bits.

| RLC/MAC block |  |  |  |
| --- | --- | --- | --- |
| MAC header | RLC data block |  |  |
|  | RLC header | RLC data unit | Spare bits |

Figure 10.0a.1.1: RLC/MAC block structure for data transfer for GPRS

The RLC data unit contains octets from one or more upper layer PDUs.

### 10.0a.2	(EC-)EGPRS RLC/MAC block for data transfer

The RLC/MAC block for EGPRS, EC-EGPRS and EGPRS2 data transfer is shown in figure 10.0a.2.1 and consists of:

-	 a combined RLC/MAC header,

-	one or two RLC data blocks for (EC-)EGPRS, or up to four RLC data blocks for EGPRS2,

-	an optional PAN field which is included in case FANR is activated,

-	and an optional eTFI field which is included for the case where a mobile station in a DLMC configuration is assigned an eTFI for a downlink TBF. If an eTFI is assigned for a downlink TBF then an eTFI is included in an uplink RLC/MAC block for data transfer only if a PAN field corresponding to that downlink TBF is also included.

| RLC/MAC block |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RLC/MAC header | RLC data block 1 | RLC data block 2 (conditional) | RLC data block 3 (conditional) | RLC data block 4 (conditional) | PAN (optional) | eTFI (optional) |

Figure 10.0a.2.1: RLC/MAC block structure for EGPRS, EC-EGPRS and EGPRS2 data transfer

Each RLC data blocks contain octets from one or more upper layer PDUs.

The PAN field may only be included within an EGPRS RLC/MAC block for data transfer within a TBF with FANR activated.

In (EC-)EGPRS, depending on the modulation and coding scheme (see 3GPP TS 44.004 and 3GPP TS 45.003) one or two RLC data blocks are contained in one RLC/MAC block. For MCS-1, MCS-2, MCS-3, MCS-4, MCS-5 and MCS-6 there is one RLC data block, whereas for MCS-7, MCS-8 and MCS-9 there are two RLC data blocks in the RLC/MAC block.

In (EC-)EGPRS, in each transfer direction, uplink and downlink, three different header types are defined. Which header type that is used depends on the modulation and coding scheme (MCS):

Header type 1	is used with modulation and coding scheme MCS-7, MCS-8 and MCS-9.

Header type 2	is used with modulation and coding scheme MCS-5 and MCS-6.

Header type 3	is used with modulation and coding scheme MCS-1, MCS-2, MCS-3 and MCS-4.

In EGPRS2, depending on the modulation and coding scheme (see 3GPP TS 44.004 and 3GPP TS 45.003) one to four RLC data blocks are contained in one RLC/MAC block as follows:

-	One RLC data block per RLC/MAC block: MCS-1, MCS-2, MCS-3, MCS-4, MCS-5, MCS-6, DAS-5, DAS-6, DAS-7, DBS-5, DBS-6, UBS-5 and UBS-6;

-	Two RLC data blocks per RLC/MAC block: MCS-7, MCS-8, MCS-9, DAS-8, DAS-9, DAS-10, DBS-7, DBS-8, UAS-7, UAS-8, UAS-9, UBS-7 and UBS-8;

-	Three RLC data blocks per RLC/MAC block: DAS-11, DAS-12, DBS-9, DBS-10, UAS-10, UAS-11, UBS-9 and UBS-10;

-	Four RLC data blocks per RLC/MAC block: DBS-11, DBS-12, UBS-11 and UBS-12.

In EGPRS2, ten header types are used in the downlink direction. Which header type is used depends on the modulation and coding scheme:

Header type 1	is used with modulation and coding scheme MCS-7, MCS-8 and MCS-9

Header type 2	is used with modulation and coding scheme DAS-5, DAS-6 and DAS-7.

Header type 3	is used with modulation and coding scheme MCS-1, MCS-2, MCS-3 and MCS-4.

Header type 4	is used with modulation and coding scheme DAS-8 and DAS-9.

Header type 5	is used with modulation and coding scheme DAS-11 and DAS-12.

Header type 6	is used with modulation and coding scheme DBS-5 and DBS-6.

Header type 7	is used with modulation and coding scheme DBS-7 and DBS-8.

Header type 8	is used with modulation and coding scheme DBS-9 and DBS-10.

Header type 9	is used with modulation and coding scheme DBS-11 and DBS-12.

Header type 10	is used with modulation and coding scheme DAS-10.

In EGPRS2, eight header types are used in the uplink direction. Which header type is used depends on the modulation and coding scheme:

Header type 2	is used with modulation and coding scheme MCS-5 and MCS-6.

Header type 3	is used with modulation and coding scheme MCS-1, MCS-2, MCS-3 and MCS-4.

Header type 4	is used with modulation and coding scheme UAS-7, UAS-8 and UAS-9.

Header type 5	is used with modulation and coding scheme UAS-10 and UAS-11.

Header type 6	is used with modulation and coding scheme UBS-5 and UBS-6.

Header type 7	is used with modulation and coding scheme UBS-7 and UBS-8.

Header type 8	is used with modulation and coding scheme UBS-9 and UBS-10.

Header type 9	is used with modulation and coding scheme UBS-11 and UBS-12.

================ next change ================

### 10.0b.2	Assembling conventions

Different assembling conventions apply for GPRS RLC data blocks, RLC/MAC control blocks, 11-bit and 8-bit control messages and (EC-)EGPRS RLC data blocks.

================ next change ================

#### 10.0b.2.2	Assembling convention for (EC-)EGPRS RLC data blocks

The different components of the RLC/MAC block carrying an  (EC-)EGPRS RLC data block shall be assembled sequentially. A component may consist of a non-integer number of octets. Each octet shall be treated as a separate field when mapped into the physical block. The lowest numbered bit represents the lowest order value.

The assembling of components shall be performed progressively, starting with octet number 1 of the physical block. If the boundary between two components falls within an octet of the physical block, the components, or parts thereof, that are contained in that octet shall be assembled progressively, starting with bit number 1 of the octet. (i.e. going from bit number 1 to bit number 8, except in octet number N1, where components are assembled going from bit number 1 to bit number n).

================ next change ================

### 10.0b.3	Field mapping conventions

Different field mapping conventions apply for GPRS RLC data blocks, RLC/MAC control blocks, 11-bit and 8-bit control messages and (EC-)EGPRS RLC data blocks.

#### 10.0b.3.1	Field mapping convention for GPRS RLC data blocks, CS-1 or CS-3 encoded RLC/MAC control blocks, EC-PACCH/D and EC-PACCH/U, 11-bit and 8-bit control messages

When a field within a GPRS RLC data block or a CS-1 encoded RLC/MAC control block, or a CS-3 encoded RLC/MAC control block, or EC-PACCH/D or EC-PACCH/U encoded RLC/MAC control block or an 11-bit or an 8-bit control message is contained within a single octet of the physical block, the lowest numbered bit of the field represents the lowest order value.

When a field spans more than one octet of the physical block, the order of bit values within each octet progressively decreases as the octet number increases. In that part of a field contained in a given octet, the lowest numbered bit represents the lowest order value.

#### 10.0b.3.2	Field mapping convention for (EC-)EGPRS RLC data blocks and MCS-0 encoded RLC/MAC control blocks

When a field within an (EC-)EGPRS RLC data block is contained within a single octet of the physical block, the lowest numbered bit of the field represents the lowest order value.

When a field spans more than one octet of the physical block, the order of bit values within each octet progressively increases as the octet number increases. In that part of a field contained in a given octet, the lowest numbered bit represents the lowest order value.

================ next change ================

### 10.3.1	Downlink RLC/MAC control block

#### 10.3.1.1	Blocks encoded using CS-1

The Downlink RLC/MAC control block together with its MAC header is formatted as shown in figure 10.3.1.1.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| Payload Type |  | RRBP |  | S/P |  | USF |  | MAC header |
| RBSN | RTI |  |  |  |  | FS | AC | Octet 1 (optional) |
| PR |  | TFI |  |  |  |  | D | Octet 2 (optional) |
| RBSNe |  |  | FSe | Spare |  |  |  | Octet 2/3 (optional) see note |
|  |  |  |  |  |  |  |  | Octet M |
| Control Message Contents |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet 21 |
|  |  |  |  |  |  |  |  | Octet 22 |
| NOTE: This optional octet is included in case of extended RLC/MAC control message segmentation. Its presence is indicated through the combination of RBSN bit and FS bit equal to (RBSN=’1’ and FS=’0’) |  |  |  |  |  |  |  |  |

Figure 10.3.1.1.1: Downlink RLC/MAC control block together with its MAC header

NOTE:	The header octets and the control message contents octets shall be encoded following the field mapping convention defined in sub-clause 10.0b.3.1.

#### 10.3.1.2	Blocks encoded using MCS-0

The downlink RLC/MAC control block header shall use the Header type 3 as shown in figure 10.3a.3.3.3 of subclause 10.3a.3.3. The RLC/MAC control block message contents are formatted as shown in figure 10.3.1.2.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| RBSN | RTI |  |  |  |  | FS | AC | Octet 1 (optional) |
| PR |  | TFI |  |  |  |  | D | Octet 2 (optional) |
| RBSNe |  |  | FSe | Spare |  |  |  | Octet 2/3 (optional) see note |
|  |  |  |  |  |  |  |  | Octet M |
| Control Message Contents |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet 21 |
|  |  |  |  |  |  |  |  | Octet 22 |
| NOTE: This optional octet is included in case of extended RLC/MAC control message segmentation. Its presence is indicated through the combination of RBSN bit and FS bit equal to (RBSN=’1’ and FS=’0’) |  |  |  |  |  |  |  |  |

Figure 10.3.1.2.1: Downlink RLC/MAC control block message contents

NOTE:	The optional header octets and the control message contents octets shall be encoded following the field mapping convention defined in sub-clause 10.0b.3.2.

#### 10.3.1.3	Blocks encoded for EC-PACCH/D

A coding block payload content of up to 80 bits has been defined for use on EC-PACCH/D. The payload is coded to generate punctured bits for EC-PACCH/D and designed to provide compatibility with GPRS and EGPRS. The EC-PACCH/D RLC/MAC control block for EC-EGPRS together with its MAC header is formatted as shown in figure 10.3.1.3.1.

|  |  |  |  | Bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
| PR | Payload Type |  | RRBP |  |  | S/P |  | USF |  | MAC header |
| RBSN | FS |  | TFI |  |  |  |  |  | PRe | Octet 1 (optional) |
|  |  |  |  |  |  |  |  |  |  | Octet 2 |
| Control Message Contents |  |  |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  |  |  | Octet 10 |

Figure 10.3.1.3.1: EC-EGPRS Downlink RLC/MAC control block together with its MAC header

### 10.3.2	Uplink RLC/MAC control block

The Uplink RLC/MAC control block together with its MAC header is formatted as shown in figure 10.3.2.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| Payload Type |  |  |  | spare |  |  | R | MAC header |
|  |  |  |  |  |  |  |  | Octet 1 |
|  |  |  |  |  |  |  |  | Octet 2 |
|  |  |  |  |  |  |  |  | Octet 3 |
| Control Message Contents |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet 21 |
|  |  |  |  |  |  |  |  | Octet 22 |

Figure 10.3.2.1: Uplink RLC/MAC control block together with its MAC header

A coding block payload content of 64 bits has been defined for use on EC-PACCH/U. The payload is coded to generate punctured bits for EC-PACCH/U and designed to provide compatibility with GPRS and EGPRS. The EC-PACCH/U RLC/MAC control block for EC-EGPRS together with its MAC header is formatted as shown in figure 10.3.2.2.

|  |  |  |  |  |  |  |  |  |  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  | R |  | MAC header |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 1 |  |
| Control Message Contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  | … |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 8 |  |

Figure 10.3.2.2: EC-EGPRS Uplink RLC/MAC control block together with its MAC header.

================ next change ================

## 10.3a	(EC-)EGPRS RLC data blocks and RLC/MAC headers

### 10.3a.0	General

The (EC-)EGPRS RLC data block consists of a FBI (downlink) or TI (uplink) field and an E field followed by an EGPRS RLC data unit. The EGPRS RLC data unit is a sequence of N2 octets that are numbered from 1 to N2.

NOTE:	The octets of an EGPRS RLC data unit are not necessarily aligned with the octets of the RLC/MAC block. An octet of the EGPRS RLC data unit may thus span across the boundary between two consecutive octets of the RLC/MAC block.

The RLC/MAC block format convention of sub-clause 10.0b for (EC-)EGPRS applies when the components of the (EC-)EGPRS RLC data block are assembled into the RLC/MAC block.

| E | FBI/TI | EGPRS RLC data unit |
| --- | --- | --- |

Figure 10.3a.0.1: Components of the EGPRS and EC-EGPRS RLC data block

The size of the EGPRS RLC data unit for each of the channel coding schemes is shown in table 10.3a.0.1.

Table 10.3a.0.1: EGPRS RLC data unit size

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 31 | A padding |
| MCS-3 | 37 | A padding / A |
| MCS-4 | 44 | C |
| MCS-5 | 56 | B |
| MCS-6 with padding | 68 | A padding |
| MCS-6 | 74 | A |
| MCS-7 | 2x56 | B |
| MCS-8 | 2x68 | A padding |
| MCS-9 | 2x74 | A |
| NOTE 1: The four families of EGPRS RLC data blocks C, B, A and A padding based on a common size basis (22, 28, 37 and 68 octets respectively) enable link adaptation retransmission as described in sub-clause 9. NOTE 2: Modulation and coding schemes of family A padding are compatible with Family A padding6 defined for EGPRS2. |  |  |

The size of the RLC data unit for each of the channel coding schemes used in EGPRS2 is shown in tables 10.3a.0.3, 10.3a.0.4, 10.3a.0.5, and 10.3a.0.6.

Table 10.3a.0.3: RLC data unit size (EGPRS2-A downlink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 with padding | 26 | B padding2 |
| MCS-2 | 28 | B padding2 / B |
| MCS-3 with padding | 31 | A padding6 |
| MCS-3 | 37 | A padding6 |
| MCS-4 | 44 | C |
| DAS-5 | 56 | B |
| DAS-6 | 68 | A padding6 |
| MCS-6 | 74 | NOTE 2 |
| DAS-7 | 82 | B padding2 |
| MCS-7 | 2x56 | B |
| DAS-8 | 2x56 | B |
| MCS-8 | 2x68 | A padding6 |
| DAS-9 | 2x68 | A padding6 |
| DAS-10 | 2x82 | B padding2 |
| DAS-11 | 3x68 | A padding6 |
| DAS-12 | 3x82 | B padding2 |
| NOTE 1: The four families of RLC data blocks (C, B, A padding6, and B padding2) based on a common size basis (22, 28, 68 and 82 octets respectively) enable link adaptation retransmission as described in sub-clause 9. NOTE 2: MCS-6 is also used for EGPRS2-A downlink (see sub-clause 5.2.1). |  |  |

Table 10.3a.0.4: RLC data unit size (EGPRS2-B downlink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 31 | A padding6 |
| MCS-3 | 37 | A padding6 / A |
| MCS-4 | 44 | C |
| DAS-5 | 56 | B |
| DBS-5 | 56 | B |
| DAS-6 | 68 | A padding6 |
| MCS-6 | 74 | A |
| DBS-6 | 74 | A |
| MCS-7 | 2x56 | B |
| DAS-8 | 2x56 | B |
| DBS-7 | 2x56 | B |
| MCS-8 | 2x68 | A padding6 |
| DAS-9 | 2x68 | A padding6 |
| MCS-9 | 2x74 | A |
| DAS-10 with padding | 2x74 | A padding8 |
| DBS-8 | 2x74 | A |
| DBS-9 | 3x56 | B |
| DAS-11 | 3x68 | A padding6 |
| DAS-12 with padding | 3x74 | A padding8 |
| DBS-10 | 3x74 | A |
| DBS-11 | 4x68 | A padding6 |
| DBS-12 | 4x74 | A |
| NOTE 1: The five families of RLC data blocks (C, B, A, A padding6 and A padding8) based on a common size basis (22, 28, 37, 68 and 74 octets respectively) enable link adaptation retransmission as described in sub-clause 9. |  |  |

Table 10.3a.0.5: RLC data unit size (EGPRS2-A uplink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 27 | A padding10 |
| MCS-3 | 37 | A padding10 / A |
| MCS-4 | 44 | C |
| MCS-5 | 56 | B |
| MCS-6 with padding | 64 | A padding10 |
| MCS-6 | 74 | A |
| UAS-7 | 2x56 | B |
| UAS-8 | 2x64 | A padding10 |
| UAS-9 | 2x74 | A |
| UAS-10 | 3x56 | B |
| UAS-11 | 3x64 | A padding10 |
| NOTE 1: The four families of RLC data blocks (C, B, A, and A padding10) based on a common size basis (22, 28, 37 and 64 octets respectively) enable link adaptation retransmission as described in sub-clause 9. |  |  |

Table 10.3a.0.6: RLC data unit size (EGPRS2-B uplink)

| Channel Coding Scheme | EGPRS RLC data unit size (N2) (octets) | Family |
| --- | --- | --- |
| MCS-1 | 22 | C |
| MCS-2 | 28 | B |
| MCS-3 with padding | 31 | A padding6 |
| MCS-3 | 37 | A padding6 / A |
| MCS-4 | 44 | C |
| UBS-5 | 56 | B |
| UBS-6 with padding | 68 | A padding6 |
| UBS-6 | 74 | A |
| UBS-7 | 2x56 | B |
| UBS-8 with padding | 2x68 | A padding6 |
| UBS-8 | 2x74 | A |
| UBS-9 | 3x56 | B |
| UBS-10 with padding | 3x68 | A padding6 |
| UBS-10 | 3x74 | A |
| UBS-11 | 4x68 | A padding6 |
| UBS-12 | 4x74 | A |
| NOTE 1: The four families of RLC data blocks (C, B, A and A padding6) based on a common size basis (22, 28, 37 and 68 octets respectively) enable link adaptation retransmission as described in sub-clause 9. |  |  |

### 10.3a.1	(EC-)EGPRS downlink RLC data block

The EGPRS downlink RLC data blocks are formatted according to figure 10.3a.1.1.

|  |  |  | Bit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |
|  |  |  | FBI | E |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  | E | Octet 1 (note) (optional) |
| . . . |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  | Octet M+1 |
| RLC data |  |  |  |  |  |  |  | . . Octet K-1 |
| spare |  | DTR Blks |  | CI | TN/PDCH-pair |  |  | Octet K (optional) |
|  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  | Octet N2 |

NOTE:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

NOTE:	In EC-EGPRS DTR will not be supported.

Figure 10.3a.1.1: (EC-)EGPRS downlink RLC data block

### 10.3a.2	(EC-)EGPRS Uplink RLC data block

The (EC-)EGPRS uplink RLC data block are formatted according to figure 10.3a.2.1.

|  |  |  | Bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |  |
|  |  |  | TI | E |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  |  | E | Octet 1 (note 1) (optional) |
| spare |  |  |  |  |  | Selected PLMN Index |  |  | Octet 2 (note 3) (optional) |
| . . . |  |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+1 \ |
|  |  |  | TLLI |  |  |  |  |  | Octet M+2 } (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+3 / |
|  |  |  |  |  |  |  |  |  | Octet M+4 / |
| PFI |  |  |  |  |  |  |  | E | Octet M+5 / |  |
|  |  |  |  |  |  |  |  |  | Octet M+6 |
| RLC data |  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  |  | Octet N2 |

NOTE 1:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

NOTE 2:	The field mapping convention for (EC-)EGPRS (sub-clause 10.0b.3.2) applies. According to that, in particular regarding the TLLI field, the least significant byte of the TLLI value shall be mapped on octet M+1 and the most significant byte of the TLLI value shall be mapped on octet M+4 of the uplink (EC-)EGPRS RLC data block.

NOTE 3:	This octet is only included if the value of the first instance of the Length indicator in the RLC data block is 123.

NOTE 4:	In EC-EGPRS PFI will not be supported.

Figure 10.3a.2.1: Uplink (EC-)EGPRS RLC data block

### 10.3a.3	(EC-)EGPRS Downlink RLC/MAC header

#### 10.3a.3.1	Header type 1: header for MCS-7, MCS-8 and MCS-9

For an EGPRS TBF without FANR activated, the EGPRS combined downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.3.1.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | RRBP |  | ES/P |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.1: EGPRS downlink RLC data block header (FANR not activated)
for MCS-7, MCS-8 and MCS-9

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF (see sub-clause 5.2.1), the downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 shall be formatted according to figure 10.3a.3.1.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.2: EGPRS (FANR activated) / EGPRS2 downlink RLC data block header

For an EGPRS TBF without FANR activated in a DLMC configuration using an SNS of 8192, the EGPRS combined downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.3.1.3.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | BSN2 | RRBP | ES/P |  | USF |  |  | 1 |
| BSN1 |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.3: DLMC EGPRS downlink RLC data block header (FANR not activated) 
for MCS-7, MCS-8 and MCS-9

For an EGPRS TBF with FANR activated in a DLMC configuration using an SNS of 8192 and for an EGPRS2 TBF in a DLMC configuration using an SNS of 8192, the downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 shall be formatted according to figure 10.3a.3.1.4.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | BSN2 | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | BSN1 | 4 |
| CPS |  |  |  |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.1.4: DLMC EGPRS (FANR activated) / DLMC EGPRS2 downlink RLC data block header for MCS-7, MCS-8 and MCS-9

For an EC-EGPRS TBF, the combined downlink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.3.1.5.

| Bit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| RRBP |  |  |  |  | ECS/P |  | USF |  |  | 1 |
| BSN1 | PR |  |  |  | TFI |  |  |  |  | 2 |
| BSN2 |  |  |  |  |  | BSN1 |  |  |  | 3 |
| CC |  |  | CPS |  |  |  |  |  | Spare | 4 |
| Spare |  |  |  |  |  |  |  |  |  | 5 |

Figure 10.3a.3.1.5: EC-EGPRS downlink RLC data block header for MCS-7, MCS-8 and MCS-9

#### 10.3a.3.2	Header type 2: header for MCS-6, MCS-5, DAS-5, DAS-6 and DAS-7

For an EGPRS TBF without FANR activated, the EGPRS combined downlink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.3.2.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | RRBP |  | ES/P |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
|  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.1: EGPRS downlink RLC data block header (FANR not activated)
for MCS-5 and MCS-6

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF (see sub-clause 5.2.1), the downlink RLC/MAC header for MCS-5 and MCS-6 shall be formatted according to figure 10.3a.3.2.2, whereas the RLC/MAC header for DAS-5, DAS-6 and DAS-7 shall be formatted as shown on figure 10.3a.3.2.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
|  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.2: EGPRS (FANR activated) / EGPRS2 downlink RLC data block header
for MCS-5, MCS-6, DAS-5, DAS-6 and DAS-7

For an EGPRS TBF without FANR activated in a DLMC configuration using an SNS of 8192, the EGPRS combined downlink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.3.2.3.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | spare | RRBP | ES/P |  | USF |  |  | 1 |
| BSN1 |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  | 3 |
|  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.3: DLMC EGPRS downlink RLC data block header (FANR not activated)
for MCS-5 and MCS-6

For an EGPRS TBF with FANR activated in a DLMC configuration using an SNS of 8192 and for an EGPRS2 TBF in a DLMC configuration using an SNS of 8192, the downlink RLC/MAC header for MCS-5, MCS-6, DAS-5, DAS-6 and DAS-7  shall be formatted according to figure 10.3a.3.2.4.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI | spare | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  | 3 |
|  |  |  |  |  | CPS |  |  | BSN1 | 4 |

Figure 10.3a.3.2.4: DLMC EGPRS (FANR activated) / DLMC EGPRS2 downlink RLC data block header
for MCS-5, MCS-6, DAS-5, DAS-6 and DAS-7

For an EC-EGPRS TBF, the EGPRS combined downlink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.3.2.5.

| Bit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| RRBP |  |  |  | ECS/P |  | USF |  |  |  | 1 |
| BSN1 | PR |  |  | TFI |  |  |  |  |  | 2 |
| CC | CPS |  |  |  | BSN1 |  |  |  |  | 3 |
|  |  |  |  |  | Spare |  |  | CC |  | 4 |

Figure 10.3a.3.2.5: EC-EGPRS downlink RLC data block header for MCS-5 and MCS-6

#### 10.3a.3.3	Header type 3: header for MCS-4, MCS-3, MCS-2, MCS-1 and MCS-0 case

For an EGPRS TBF without FANR activated, the EGPRS combined downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.3.3.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | RRBP |  |  |  |  | ES/P |  |  | USF |  |  |  | 1 |
| BSN1 |  |  | PR |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.1: EGPRS downlink RLC data block header (FANR not activated)
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF (see sub-clause 5.2.1), the downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 shall be formatted according to figure 10.3a.3.3.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | PANI |  | CES/P |  |  |  |  |  | USF |  |  |  | 1 |
| BSN1 |  |  | PR |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.2: EGPRS (FANR activated) / EGPRS2 downlink RLC data block header
for MCS-1, MCS-2, MCS-3 and MCS-4

For a TBF in RTTI configuration, the downlink RLC/MAC control block header for MCS-0 shall be formatted as defined in figure 10.3a.3.3.3.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| Payload type | Spare |  | RRBP | S/P | USF |  |  | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 |
|  | 0 | 0 | CPS |  |  |  | Spare | 4 |
| NOTE2: Field indicated as ‘0’ will be replaced by an 18 bit CRC during the channel coding, see sub-clause 5.1.4a.1.4 in 3GPP TS 45.003. |  |  |  |  |  |  |  |  |

Figure 10.3a.3.3.3: Downlink RLC/MAC control block header
for MCS-0.

For an EGPRS TBF without FANR activated in a DLMC configuration using an SNS of 8192, the EGPRS combined downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.3.3.4.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | spare |  | RRBP |  | ES/P |  |  | USF |  |  |  | 1 |
| BSN1 |  |  |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.4: DLMC EGPRS downlink RLC data block header (FANR not activated)
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EGPRS TBF with FANR activated in a DLMC configuration using an SNS of 8192 and for an EGPRS2 TBF in a DLMC configuration using an SNS of 8192, the downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 shall be formatted according to figure 10.3a.3.3.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI | PANI |  | spare |  | CES/P |  |  |  | USF |  |  |  | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
|  |  | SPB |  |  | CPS |  |  |  |  |  | BSN1 |  | 4 |

Figure 10.3a.3.3.5: DLMC EGPRS (FANR activated) / DLMC EGPRS2 downlink RLC data block header
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EC-EGPRS TBF, the EGPRS combined downlink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.3.3.6.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 | 5 | 4 |  | 3 | 2 | 1 | Octet |
| RRBP |  |  |  |  | ECS/P |  |  | USF |  |  | 1 |
| BSN1 | PR |  |  |  | TFI |  |  |  |  |  | 2 |
| CC |  |  | SPB |  |  | BSN1 |  |  |  |  | 3 |
|  | Spare |  |  |  |  |  | CPS |  |  |  | 4 |

Figure 10.3a.3.3.6: EC-EGPRS downlink RLC data block header for MCS-1, MCS-2, MCS-3 and MCS-4

#### 10.3a.3.4	Header type 4: header for DAS-8 and DAS-9

The combined downlink RLC/MAC header for DAS-8 and DAS-9 (header type 4) shall be formatted according to figure 10.3a.3.4.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | PANI |  | CES/P |  |  |  | USF |  |  | 1 |
| BSN1 |  |  |  | PR |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
| Spare | CPS |  |  |  |  |  |  | BSN2 |  |  | 5 |
|  |  |  |  |  |  |  |  |  |  | Spare | 6 |

Figure 10.3a.3.4.1: Combined downlink RLC data block header
for DAS-8 and DAS-9.

For an EGPRS2 TBF in a DLMC configuration using an SNS of 8192 the combined downlink RLC/MAC header for DAS-8 and DAS-9 (header type 4) shall be formatted according to figure 10.3a.3.4.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | PANI |  | BSN2 |  | CES/P |  | USF |  |  | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
| BSN2 | CPS |  |  |  |  |  |  | BSN2 |  |  | 5 |
|  |  |  |  |  |  |  |  |  |  | Spare | 6 |

Figure 10.3a.3.4.2: DLMC Combined downlink RLC data block header
for DAS-8 and DAS-9.

#### 10.3a.3.5	Header type 5: header for DAS-11 and DAS-12

The combined downlink RLC/MAC header for DAS-11 and DAS-12 (header type 5) shall be formatted according to figure 10.3a.3.5.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 | 4 |  | 3 | 2 | 1 | Octet |
| TFI |  | PANI |  | CES/P |  |  |  |  | USF |  |  | 1 |
| BSN1 |  |  |  | PR |  |  | TFI |  |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
| BSN3 |  |  |  |  |  |  |  |  | BSN2 |  |  | 5 |
| CPS |  |  |  |  | BSN3 |  |  |  |  |  |  | 6 |
|  |  |  |  |  | Spare |  |  | CPS |  |  |  | 7 |

Figure 10.3a.3.5.1: Combined downlink RLC data block header
for DAS-11 and DAS-12.

For an EGPRS2 TBF in a DLMC configuration using an SNS of 8192 the combined downlink RLC/MAC header for DAS-11 and DAS-12 (header type 5) shall be formatted according to figure 10.3a.3.5.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | PANI |  | BSN2 | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
| BSN3 |  |  |  |  |  |  |  | BSN2 |  |  | 5 |
| CPS |  |  |  |  | BSN3 |  |  |  |  |  | 6 |
|  |  |  |  |  | BSN2 |  | BSN3 | CPS |  |  | 7 |

Figure 10.3a.3.5.2: DLMC Combined downlink RLC data block header
for DAS-11 and DAS-12.

#### 10.3a.3.6	Header type 6: header for DBS-5 and DBS-6

The combined downlink RLC/MAC header for DBS-5 and DBS-6 (header type 6) shall be formatted according to figure 10.3a.3.6.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 |  | Octet |  |
| TFI | PANI |  | CES/P |  |  | USF |  |  |  | 1 |  |
| BSN |  | PR |  |  | TFI |  |  |  |  | 2 |  |
| BSN |  |  |  |  |  |  |  |  |  | 3 |  |
|  |  | Spare |  |  | CPS |  |  | BSN |  | 4 |  |

Figure 10.3a.3.6.1: Combined downlink RLC data block header
for DBS-5 and DBS-6.

#### 10.3a.3.7	Header type 7: header for DBS-7 and DBS-8

The combined downlink RLC/MAC header for DBS-7 and DBS-8 (header type 7) shall be formatted according to figure 10.3a.3.7.1.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI | PANI |  | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  | PR |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | BSN1 | 4 |
| Spare | CPS |  |  |  |  | BSN2 |  |  | 5 |
|  |  |  |  |  |  |  |  | Spare | 6 |

Figure 10.3a.3.7.1: Combined downlink RLC data block header
for DBS-7 and DBS-8.

#### 10.3a.3.8	Header type 8: header for DBS-9 and DBS-10

The combined downlink RLC/MAC header for DBS-9 and DBS-10 (header type 8) shall be formatted according to figure 10.3a.3.8.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  |  |  | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  |  | PANI |  |  | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  | PR |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
| BSN 3 |  |  |  |  |  |  |  |  | BSN2 |  |  | 5 |
| CPS |  |  |  |  |  |  | BSN3 |  |  |  |  | 6 |
|  |  |  |  |  |  |  | Spare |  | CPS |  |  | 7 |

Figure 10.3a.3.8.1: Combined downlink RLC data block header
for DBS-9 and DBS-10.

#### 10.3a.3.9	Header type 9: header for DBS-11 and DBS-12

The combined downlink RLC/MAC header for DBS-11 and DBS-12 (header type 9) shall be formatted according to figure 10.3a.3.9.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  |  |  | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  |  | PANI |  |  | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  | PR |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
| BSN 3 |  |  |  |  |  |  |  |  | BSN2 |  |  | 5 |
| BSN4 |  |  |  |  |  |  | BSN3 |  |  |  |  | 6 |
| CPS |  | BSN4 |  |  |  |  |  |  |  |  |  | 7 |
| Spare |  | CPS |  |  |  |  |  |  |  |  |  | 8 |
|  |  |  |  |  |  |  |  |  |  |  | Spare | 9 |

Figure 10.3a.3.9.1: Combined downlink RLC data block header
for DBS-11 and DBS-12.

#### 10.3a.3.10	Header type 10: header for DAS-10

The combined downlink RLC/MAC header for DAS-10 (header type 10) shall be formatted according to figure 10.3a.3.10.1.

| Bit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | PANI | CES/P |  |  |  | USF |  |  | 1 |
| BSN1 |  |  | PR |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  | BSN1 | 4 |
|  | Spare |  |  | CPS |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.10.1: Combined downlink RLC data block header
for DAS-10.

For an EGPRS2 TBF in a DLMC configuration using an SNS of 8192 the combined downlink RLC/MAC header for DAS-10 (header type 10) shall be formatted according to figure 10.3a.3.10.2.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | PANI |  | BSN2 | CES/P |  |  | USF |  |  | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  |  | 2 |
| BSN1 |  |  |  |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  | BSN1 | 4 |
|  | BSN2 |  | Spare |  | CPS |  |  | BSN2 |  |  | 5 |

Figure 10.3a.3.10.2: DLMC Combined downlink RLC data block header
for DAS-10.

### 10.3a.4	(EC-)EGPRS Uplink RLC/MAC header

#### 10.3a.4.1	Header type 1: header for MCS-7, MCS-8 and MCS-9

The EGPRS combined uplink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.4.1.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | 4 |
| Spare | PI | RSB | CPS |  |  |  |  | 5 |
|  |  | Spare |  |  |  |  |  | 6 |

Figure 10.3a.4.1.1: EGPRS uplink RLC data block header (FANR not activated)
for MCS-7, MCS-8 and MCS-9.

For a EGPRS TBF with FANR activated, the uplink RLC/MAC header for MCS-7, MCS-8 and MCS-9 shall be formatted according to figure 10.3a.4.1.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | 4 |
| PANI | PI | RSB | CPS |  |  |  |  | 5 |
|  |  | Spare |  |  |  |  |  | 6 |

Figure 10.3a.4.1.2: EGPRS uplink RLC data block header (FANR activated)
for MCS-7, MCS-8 and MCS-9.

The EC-EGPRS combined uplink RLC/MAC header for MCS-7, MCS-8 and MCS-9 (header type 1) shall be formatted according to figure 10.3a.4.1.3.

| Bit |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 |  | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  |  | Countdown Value |  |  |  |  |  | FOI |  | R | 1 |
| BSN1 |  |  |  |  |  |  |  | TFI |  |  |  | 2 |
| Spare |  |  |  |  |  | BSN2 |  |  |  |  |  | 3 |
| SI | Spare | RSB |  | CPS |  |  |  |  |  |  |  | 4 |
| Spare |  |  |  | rTLLI |  |  |  |  |  | RI |  | 5 |
|  |  | Spare |  |  |  |  |  |  |  |  |  | 6 |

Figure 10.3a.4.1.3: EC-EGPRS uplink RLC data block header for MCS-7, MCS-8 and MCS-9.

#### 10.3a.4.2	Header type 2: header for MCS-6 and MCS-5

The EGPRS combined uplink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.4.2.1.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI |  | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  |  | 3 |
| Spare |  |  |  |  | PI | RSB | CPS |  | 4 |
|  |  |  | Spare |  |  |  |  |  | 5 |

Figure 10.3a.4.2.1: EGPRS uplink RLC data block header (FANR not activated)
for MCS-5 and MCS-6

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF, the uplink RLC/MAC header for MCS-5 and MCS-6 shall be formatted according to figure 10.3a.4.2.2.

| Bit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  | Countdown Value |  |  |  |  | SI |  | R | 1 |
| BSN1 |  |  |  |  |  | TFI |  |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  |  |  | 3 |
| Spare |  |  |  | PANI |  | PI | RSB | CPS |  | 4 |
|  |  |  | Spare |  |  |  |  |  |  | 5 |

Figure 10.3a.4.2.2: EGPRS (FANR activated) / EGPRS2 uplink RLC data block header 
for MCS-5 and MCS-6

The EC-EGPRS combined uplink RLC/MAC header for MCS-5 and MCS-6 (header type 2) shall be formatted according to figure 10.3a.4.2.3.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| TFI |  |  |  | Countdown Value |  |  |  | FOI |  | R | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  |  | 2 |
| SI | Spare |  | RSB |  | Spare |  | CPS |  |  |  | 3 |
| Spare |  |  |  |  | rTLLI |  |  |  | RI |  | 4 |
|  |  |  |  |  | Spare |  |  |  |  |  | 5 |

Figure 10.3a.4.2.3: EC-EGPRS uplink RLC data block header for MCS-5 and MCS-6

#### 10.3a.4.3	Header type 3 : header for MCS-4, MCS-3, MCS-2 and MCS-1

The EGPRS combined uplink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.4.3.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  | 3 |
|  | Spare | PI | RSB | SPB |  | CPS |  | 4 |

Figure 10.3a.4.3.1: EGPRS uplink RLC data block header (FANR not activated)
for MCS-1, MCS-2, MCS-3 and MCS-4

For an EGPRS TBF with FANR activated and for an EGPRS2 TBF, the uplink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 shall be formatted according to figure 10.3a.4.3.2.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| CPS |  | BSN1 |  |  |  |  |  | 3 |
|  | PANI | PI | RSB | SPB |  | CPS |  | 4 |

Figure 10.3a.4.3.2: EGPRS (FANR activated)/EGPRS2 uplink RLC data block header
for MCS-1, MCS-2, MCS-3 and MCS-4

The EC-EGPRS combined uplink RLC/MAC header for MCS-1, MCS-2, MCS-3 and MCS-4 (header type 3) shall be formatted according to figure 10.3a.4.3.3.

|  |  |  |  | Bit |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |  |
| TFI |  | Countdown Value |  |  |  | FOI | R | 1 |  |
| BSN1 |  |  |  |  | TFI |  |  | 2 |  |
| SI | Spare | RSB | SPB |  | CPS |  |  | 3 |  |
|  | Spare |  | rTLLI |  |  |  | RI | 4 |  |

Figure 10.3a.4.3.3: EC-EGPRS uplink RLC data block header for MCS-1, MCS-2, MCS-3 and MCS-4

#### 10.3a.4.4	Header type 4: header for UAS-7, UAS-8 and UAS-9

The combined uplink RLC/MAC header for UAS-7, UAS-8 and UAS-9 (header type 4) shall be formatted according to figure 10.3a.4.4.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | 4 |
| Spare | PANI | PI | CPS |  |  |  |  | 5 |
|  |  |  |  |  |  |  | Spare | 6 |

Figure 10.3a.4.4.1: Combined uplink RLC data block header
for UAS-7, UAS-8 and UAS-9.

#### 10.3a.4.5	Header type 5: header for UAS-10 and UAS-11

The combined uplink RLC/MAC header for UAS-10 and UAS-11 (header type 5) shall be formatted according to figure 10.3a.4.5.1.

| Bit |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  | 4 |
| BSN3 |  |  |  |  |  |  |  | 5 |
| CPS |  |  |  |  |  | BSN3 |  | 6 |
|  |  |  |  | Spare |  | PANI | PI | 7 |

Figure 10.3a.4.5.1: Combined uplink RLC data block header
for UAS-10, UAS-11.

#### 10.3a.4.6	Header type 6: header for UBS-5 and UBS-6

The combined uplink RLC/MAC header for UBS-5 and UBS-6 (header type 6) shall be formatted according to figure 10.3a.4.6.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 | 6 |  |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  |  | Countdown Value |  |  |  |  |  | SI | R | 1 |
| BSN |  |  |  |  |  |  |  | TFI |  |  | 2 |
| CPS |  |  | BSN |  |  |  |  |  |  |  | 3 |
|  |  |  |  |  | Spare |  |  | PANI | PI | CPS | 4 |

Figure 10.3a.4.6.1: Combined uplink RLC data block header
for UBS-5, UBS-6.

#### 10.3a.4.7	Header type 7: header for UBS-7 and UBS-8

The combined uplink RLC/MAC header for UBS-7 and UBS-8 (header type 7) shall be formatted according to figure 10.3a.4.7.1.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  |  | BSN1 |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  | 4 |
| Spare |  | PANI |  | PI | CPS |  |  |  | 5 |

Figure 10.3a.4.7.1: Combined uplink RLC data block header
for UBS-7, UBS-8.

#### 10.3a.4.8	Header type 8: header for UBS-9 and UBS-10

The combined uplink RLC/MAC header for UBS-9 and UBS-10 (header type 8) shall be formatted according to figure 10.3a.4.8.1.

| Bit |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 | 4 | 3 | 2 | 1 | Octet |
| TFI |  |  |  | Countdown Value |  |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  |  |  | BSN1 |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  |  | 4 |
| BSN3 |  |  |  |  |  |  |  |  |  |  | 5 |
| CPS |  |  |  |  |  |  |  |  | BSN3 |  | 6 |
|  |  |  |  |  |  |  | Spare |  | PANI | PI | 7 |

Figure 10.3a.4.8.1: Combined uplink RLC data block header
for UBS-9, UBS-10.

#### 10.3a.4.9	Header type 9: header for UBS-11 and UBS-12

The combined uplink RLC/MAC header for UBS-11 and UBS-12 (header type 9) shall be formatted according to figure 10.3a.4.9.1.

| Bit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 |  | 5 |  | 4 | 3 | 2 | 1 | Octet |
| TFI |  | Countdown Value |  |  |  |  |  | SI | R | 1 |
| BSN1 |  |  |  |  |  |  | TFI |  |  | 2 |
| BSN2 |  | BSN1 |  |  |  |  |  |  |  | 3 |
| BSN2 |  |  |  |  |  |  |  |  |  | 4 |
| BSN3 |  |  |  |  |  |  |  |  |  | 5 |
| BSN4 |  |  |  |  |  |  |  | BSN3 |  | 6 |
| CPS |  |  |  |  |  | BSN4 |  |  |  | 7 |
| Spare |  |  | PANI |  | PI | CPS |  |  |  | 8 |

Figure 10.3a.4.9.1: Combined uplink RLC data block header
for UBS-11, UBS-12.

================ next change ================

## 10.4	Header fields

### 10.4.1	Uplink state flag (USF) field

The USF field is sent in all downlink RLC/MAC blocks and indicates the owner or use of the next uplink radio block on the same timeslot (see 3GPP TS 45.002). The USF field is three bits in length and eight different USF values can be assigned, except on PCCCH, where the value '111' (USF=FREE) indicates that the corresponding uplink radio block contains PRACH.

### 10.4.2	Retry (R) bit

The Retry (R) bit shall indicate whether the mobile station transmitted the CHANNEL REQUEST message (see 3GPP TS 44.018), PACKET CHANNEL REQUEST message, EGPRS PACKET CHANNEL REQUEST message, EC-EGPRS PACKET CHANNEL REQUEST message or MPRACH PACKET CHANNEL REQUEST message one time or more than one time during its most recent channel access. The mobile station shall send the same value for the R bit in each uplink RLC/MAC block of the TBF.

Table 10.4.2.1: Retry (R) bit

| bit 1 | Retry (R) bit |
| --- | --- |
| 0 | MS sent channel request message once |
| 1 | MS sent channel request message twice or more |

### 10.4.3	Stall indicator (SI) bit

The Stall indicator (SI) bit indicates whether the mobile's RLC transmit window can advance (i.e. is not stalled) or can not advance (i.e. is stalled). The mobile station shall set the SI bit in all uplink RLC data blocks.

Table 10.4.3.1: Stall indicator bit

| bit 1 | Stall indicator |
| --- | --- |
| 0 | MS RLC transmit window is not stalled |
| 1 | MS RLC transmit window is stalled |
| NOTE 1: The bit numbering is relative to the field position. |  |

### 10.4.4	Supplementary/Polling (S/P) Bit

The S/P bit is used to indicate whether the RRBP field is valid or not valid in downlink GPRS RLC data blocks and downlink RLC/MAC control blocks using the coding scheme of CS-1and in downlink EC-EGPRS RLC/MAC control blocks on EC-PACCH/D.

Table 10.4.4.1: Supplementary/Polling (S/P) bit - GPRS case and RLC/MAC control

| bit 4 | S/P |
| --- | --- |
| 0 | RRBP field is not valid |
| 1 | RRBP field is valid |

### 10.4.4a	EGPRS Supplementary/Polling (ES/P) Field

The ES/P field is used to indicate whether the RRBP field is valid or not valid, and what fields the next uplink control block shall contain (see further clause 9).

NOTE: 	The type of Ack/Nack bitmap requested by this field is applicable only when the next uplink control block is used to send an EGPRS PACKET DOWNLINK ACK/NACK, EGPRS PACKET DOWNLINK ACK/NACK TYPE 2, EGPRS PACKET DOWNLINK ACK/NACK TYPE 3, EGPRS PACKET DOWNLINK ACK/NACK DLMC or MBMS DOWNLINK ACK/NACK message.

Table 10.4.4a.1: EGPRS Supplementary/Polling (ES/P) field (non-MBMS only)

| bits 5 4 | ES/P |
| --- | --- |
| 0 0 | RRBP field is not valid (no Polling) |
| 0 1 | RRBP field is valid - Extended Ack/Nack bitmap type FPB |
| 1 0 | RRBP field is valid - Extended Ack/Nack bitmap type NPB |
| 1 1 | RRBP field is valid - Extended Ack/Nack bitmap type NPB, measurement report included |

Table 10.4.4a.2: EGPRS Supplementary/Polling (ES/P) field (MBMS only)

| bits 5 4 | ES/P |
| --- | --- |
| 0 0 | RRBP field is not valid (no Polling) |
| 0 1 | RRBP field is valid - Extended Ack/Nack bitmap type FPB |
| 1 0 | RRBP field is valid - Extended Ack/Nack bitmap type NPB |
| 1 1 | RRBP field is valid - Extended Ack/Nack bitmap type NPB, neighbouring cell measurement reports included |

### 10.4.4b	Combined EGPRS Supplementary/Polling (CES/P) Field

The CES/P field is used to indicate what fields the next uplink radio block reserved by this field shall contain (see further clause 9). The single uplink block is defined by a delay relative to the first TDMA frame (N) of the downlink block containing the CES/P value. The procedures defined for transmission of a PACCH block to the network as described in sub-clause 10.4.5 shall apply.

If the mobile station is polled for a Piggy-backed Ack/Nack, in case EFTA is not used at the time the CES/P field is received, the mobile station shall transmit the uplink radio block on the same timeslot as the block where the CES/P field was received (in case of a BTTI configuration) or on the uplink PDCH pair corresponding to the downlink PDCH pair where the block containing the CES/P field was received (in case of an RTTI configuration).  In case EFTA is used at the time the CES/P field is received, the mobile station shall transmit the uplink radio block according to the uplink radio block transmission order as described in Annex N regardless of the timeslot or PDCH pair where the block containing the CES/P field was received. The mobile station need not monitor the USF in the associated downlink RLC/MAC block appearing just before the uplink block it shall transmit. However, when Extended Dynamic Allocation or Shifted USF operation is used, the corresponding USF monitoring procedure shall apply as described in sub-clause 8.1.1.2.1 and sub-clause 8.1.1.2.4 respectively.

Table 10.4.4b.1: Combined EGPRS Supplementary/Polling (CES/P) field

| bits 6 5 4 | CES/P |  |  |
| --- | --- | --- | --- |
|  | Feedback | Reserved Block |  |
|  |  | BTTI | RTTI |
| 0 0 0 | no Polling | - | - |
| 0 0 1 | Extended Ack/Nack bitmap type FPB, and if there is enough room left in RLC/MAC block, channel quality report(s) | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| 0 1 0 | Extended Ack/Nack bitmap type FPB, and if there is enough room left in RLC/MAC block, channel quality report(s) | (N+13) mod 2715648 | (N+8 or N+9) mod 2715648 |
| 0 1 1 | Piggy-backed Ack/Nack bitmap type FPB (see sub-clause 8.1.2.2) see note | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| 1 0 0 | Piggy-backed Ack/Nack bitmap type FPB (see sub-clause 8.1.2.2) see note | (N+13) mod 2715648 | (N+8 or N+9) mod 2715648 |
| 1 0 1 | Channel quality report(s), and if there is enough room left in RLC/MAC block, Extended Ack/Nack bitmap type NPB | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| 1 1 0 | Channel quality report(s), and if there is enough room left in RLC/MAC block, Extended Ack/Nack bitmap type NPB | (N+13) mod 2715648 | (N+8 or N+9) mod 2715648 |
| 1 1 1 | Extended Ack/Nack bitmap type NPB, and if there is enough room left in RLC/MAC block, channel quality report(s) | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| NOTE: In case where a PAN cannot be sent, the Extended Ack/Nack bitmap type NPB with measurement report included shall be transmitted on the allocated reserved block instead (see sub-clause 9.1.14.2). |  |  |  |

In case of a DLMC configuration using an SNS of 8192 the CES/P field within the RLC data block header (see sub-clause 10.3a.3.1, 10.3a.3.2, 10.3a.3.3, 10.3a.3.4, 10.3a.3.5 and 10.3a.3.10) is coded as shown in Table 10.4.4b.2.

Table 10.4.4b.2: Combined EGPRS Supplementary/Polling (CES/P) field - DLMC Configuration

| bits 5 4 | CES/P |  |  |
| --- | --- | --- | --- |
|  | Feedback | Reserved Block |  |
|  |  | BTTI | RTTI |
| 0 0 | no Polling | - | - |
| 0 1 | Extended Ack/Nack bitmap type FPB, and if there is enough room left in the RLC/MAC block, channel quality report(s) | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| 1 0 | Channel quality report(s), and if there is enough room left in the RLC/MAC block, Extended Ack/Nack bitmap type NPB | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| 1 1 | Piggy-backed Ack/Nack bitmap type FPB (see sub-clause 8.1.2.2) - see note | (N+8 or N+9) mod 2715648 | (N+6 or N+7) mod 2715648 |
| NOTE: In case where a PAN cannot be sent, the Extended Ack/Nack bitmap type NPB with channel quality report(s) included shall be transmitted on the allocated reserved block instead (see sub-clause 9.1.14.2). |  |  |  |

### 10.4.4c	EC-EGPRS Supplementary/Polling (ECS/P) Field

The ECS/P field is used to indicate whether the RRBP field is valid or not valid and, if valid, if a quality report is to be included in the next uplink control block. The quality report to be included contains the mean BEP and the downlink coverage class estimation.

Table 10.4.4c.1: EC-EGPRS Supplementary/Polling (ECS/P) field

| bits 5 4 | ECS/P |
| --- | --- |
| 0 0 | RRBP field is not valid (no Polling) |
| 0 1 | RRBP field is valid, Ack/Nack report to be included |
| 1 0 | RRBP field is valid, Ack/Nack report to be included. If there is enough room in the RLC/MAC block, a channel quality report shall also be included.. |
| 1 1 | Reserved |

### 10.4.5	Relative Reserved Block Period (RRBP) field

The RRBP value specifies a single uplink block in which the mobile station shall transmit either a PACKET CONTROL ACKNOWLEDGEMENT message, an EC-EGPRS PACKET CONTROL ACKNOWLEDGEMENT message a PACCH block or and EC-PACCH block to the network. If the RRBP field is received as part of an RLC/MAC block containing an RLC/MAC control block, the mobile station shall transmit a PACKET CONTROL ACKNOWLEDGEMENT or EC-EGPRS PACKET CONTROL ACKNOWLEDGEMENT message in the uplink radio block specified, except if:

-	The received message is a Packet Paging Request, Packet Access Reject, or Packet Queuing Notification message, or

-	It is specified elsewhere that the mobile station shall not respond to the polling request.

If the RRBP field is received as part of an RLC/MAC block containing an RLC/MAC control block containing a Packet Paging Request, Packet Access Reject, or Packet Queuing Notification message, or it is specified elsewhere that the mobile station shall not respond to the polling request, the mobile station shall ignore this RRBP field. The mobile station shall only react on RLC/MAC control blocks containing a valid RRBP field if the mobile station is addressed either in the downlink RLC/MAC control block header or in the control message itself. If the control message is segmented into more than one downlink RLC/MAC control blocks the mobile station shall react only on RLC/MAC control blocks containing a valid RRBP field if the mobile station is addressed in the downlink RLC/MAC control block header.

If the mobile station receives two or more RLC/MAC blocks containing an RLC/MAC control message with different RRBP values such that they specify the same uplink block, the mobile station shall transmit one PACKET CONTROL ACKNOWLEDGEMENT or EC-EGPRS PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block.

If the RRBP field is received as part of a RLC/MAC block containing an RLC data block, the mobile station shall transmit a PACCH block or an EC-PACCH block in the specified uplink radio block. If the mobile station receives two or more RLC/MAC blocks containing an RLC data block with different RRBP values such they specify the same uplink radio block, the mobile station shall transmit one (EC-)PACCH block in the specified uplink radio block.

If the mobile station receives an RLC data block and an RLC/MAC control block with different RRBP values such that they specify the same uplink radio block, the mobile station shall transmit a PACKET CONTROL ACKNOWLEDGEMENT or an EC-EGPRS PACKET CONTROL ACKNOWLEDGEMENT message in the specified uplink radio block.

If no uplink control timeslot is assigned to the mobile station and EFTA is not used at the time the RRBP field is received, the mobile station shall transmit the uplink radio block on the same timeslot as the block where the RRBP was received (in case of a BTTI configuration) or on the corresponding uplink PDCH pair to the downlink PDCH pair where the block containing the RRBP was received (in case of a RTTI configuration). If EFTA is used at the time the RRBP field is received however, the mobile station shall instead transmit the uplink radio block according to the uplink radio block transmission order as described in Annex N regardless of the timeslot or PDCH-pair where the block containing the RRBP was received.

If an uplink control timeslot is assigned to the mobile station, the mobile station shall transmit the uplink radio block on this uplink control timeslot (in case of a BTTI configuration) or on the uplink PDCH pair the uplink control timeslot belongs to (in case of an RTTI configuration).

NOTE:	In case of an RTTI configuration, the network should not poll the mobile station on a downlink PDCH pair for which no corresponding uplink PDCH pair exists.

After receiving an RLC/MAC block containing a valid RRBP field the mobile station need not monitor the USF in the associated downlink RLC/MAC block appearing just before the uplink block it shall transmit. However, when Extended Dynamic Allocation or Shifted USF operation is used, the corresponding USF monitoring procedure shall apply as described in sub-clause 8.1.1.2.1 and sub-clause 8.1.1.2.4 respectively.

After receiving an RLC/MAC block containing a valid RRBP field for an EC-EGPRS TBF, the mobile station need not monitor the associated downlink RLC/MAC blocks during the radio block periods where it shall transmit the uplink response.

A polled control message shall always be sent in the uplink block specified by the corresponding valid RRBP field of a downlink RLC/MAC control block, and not in any other uplink block that may be allocated to the mobile station.

The network should use the RRBP field to schedule the transmission of a (EC-EGPRS) PACKET CONTROL ACKNOWLEDGEMENT message or an uplink (EC-)PACCH block as follows:

-	For BTTI configuration, no later than the second last BTTI radio block, B(x-2) mod 12, before the first BTTI radio block, B(x), where the mobile station shall be ready to transmit and receive using a new assignment.

-	For RTTI configuration, no later than the second last RTTI radio block, B(x-2)b, before the first RTTI radio block, B(x)a, where the mobile station shall be ready to transmit and receive using a new assignment or no later than the second last RTTI radio block, B(x-1)a, before the first RTTI radio block, B(x)b, where the mobile station shall be ready to transmit and receive using a new assignment.

A mobile station that is scheduled an uplink block later than these times, may omit responding to the polling request.  If a new assignment specifies an uplink TBF, and the TTI configuration of the uplink TBF specified is the same as the TTI configuration of the ongoing uplink TBF that this assignment modifies, or if the assignment does not specify any uplink TBF, then the mobile station may delay the access using the new assignment in order to respond to the polling request.

The network should not use the RRBP field to schedule the transmission of PACKET CONTROL ACKNOWLEDGEMENT, EC-EGPRS PACKET CONTROL ACKNOWLEDGEMENT messages or uplink (EC-) PACCH blocks, in such way, that a mobile station has more than three such uplink blocks pending for transmission at any instant. A mobile station, that is scheduled such uplink blocks more frequent than that, may omit responding to the excessive polling requests.

The RRBP field shall be interpreted according to the TTI configuration in use for the mobile station on the PDCH (or on the PDCH pair) where the block containing the RRBP field is received at the time it is received, and, for a BTTI configuration, according to whether FANR is activated or not for the mobile station.

For non EC-EGPRS TBFs with FANR not activated, Table 10.4.5.1 specifies the coding of the RRBP field indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block. The delay is relative to the first TDMA frame (N) of the downlink block containing the RRBP value. For definition of TDMA frame numbering, see 3GPP TS 45.002.

Table 10.4.5.1: Relative Reserved Block Period (RRBP) field (FANR not activated)

| bit 6-5/7-6 | Full-rate PDCH uplink block with TDMA frame number | Full-rate PDCH uplink block with TDMA frame number (DLMC configuration with SNS 2048) | Half-rate PDCH uplink block with TDMA frame number |
| --- | --- | --- | --- |
| 0 0 | (N+13) mod 2715648 | (N+13) mod 2715648 | Reserved |
| 0 1 | (N+17 or N+18) mod 2715648 | (N+17 or N+18) mod 2715648 | (N+17 or N+18) mod 2715648 |
| 1 0 | (N+21 or N+22) mod 2715648 | (N+21 or N+22) mod 2715648 | Reserved |
| 1 1 | (N+26) mod 2715648 | (N+26) mod 2715648 | (N+26) mod 2715648 |

If the mobile station is operating on a half-rate PDCH and it receives an RLC/MAC block with a reserved RRBP value, it shall regard the RRBP field as not valid and shall ignore the polling.

For TBFs with FANR activated, Table 10.4.5.2 specifies the coding of the RRBP field indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block.

Table 10.4.5.2: Relative Reserved Block Period (RRBP) field (FANR activated)

| bit 6-5 | PDCH uplink block with TDMA frame number (BTTI) | PDCH uplink block with TDMA frame number (RTTI) |
| --- | --- | --- |
| 0 0 | (N+13) mod 2715648 | Reserved |
| 0 1 | (N+8 or N+9) mod 2715648 | (N+8 or N+9) mod 2715648 |
| 1 0 | Reserved | (N+6 or N+7) mod 2715648 |
| 1 1 | Reserved | Reserved |
| NOTE: Table 10.4.5.2 only applies to RLC/MAC control blocks encoded using CS-1, while the CES/P field is used in case of polling in RLC data blocks (see sub-clause 10.4.4b). |  |  |

In downlink RLC/MAC Control blocks encoded using MSC-0, a 1-bit RRBP is defined (see subclause 10.3a.3.3). In this case the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block is indicated in Table 10.4.5.3.

Table 10.4.5.3: Relative Reserved Block Period (RRBP) field – 1 bit field

| bit 5 | PDCH uplink block with TDMA frame number |
| --- | --- |
| 0 | (N+6 or N+7) mod 2715648 |
| 1 | (N+8 or N+9) mod 2715648 |

In a DLMC configuration using an SNS of 8192 for a downlink TBF where FANR is not activated, Table 10.4.5.4 specifies the RRBP field in the RLC data block header (see sub-clause 10.3a.3.1, 10.3a.3.2 and 10.3a.3.3) by indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block. The delay is relative to the first TDMA frame (N) of the downlink block containing the RRBP value. For definition of TDMA frame numbering, see 3GPP TS 45.002.

Table 10.4.5.4: Relative Reserved Block Period (RRBP) field (FANR not activated) – DLMC Configuration

| bit 6 | PDCH uplink block with TDMA frame number |
| --- | --- |
| 0 | (N+6 or N+7) mod 2715648 |
| 1 | (N+8 or N+9) mod 2715648 |

For EC-EGPRS TBFs, Table 10.4.5.5 specifies the coding of the RRBP field if it is received as part of an RLC/MAC block containing an RLC/MAC data block indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block according to its uplink Coverage Class. The delay is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of the EC-PACCH or EC-PDTCH message, according to the USED_DL_COVERAGE_CLASS field. For definition of TDMA frame numbering, see 3GPP TS 45.002.

Table 10.4.5.5: Relative Reserved Block Period (RRBP) field (EC-EGPRS TBF mode) if received as part of an RLC/MAC block containing an RLC/MAC data block

| bit 8 7 6 | Full-rate PDCH uplink block with TDMA frame number given UL CC1 or CC2 | Full-rate PDCH uplink block with TDMA frame number given UL CC3 | Full-rate PDCH uplink block with TDMA frame number given UL CC4 |
| --- | --- | --- | --- |
| 0 0 0 | (N+13) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 |
| 0 0 1 | (N+17 or N+18) mod 2715648 | (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 |
| 0 1 0 | (N+21 or N+22) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35 ) mod 2715648 | (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 |
| 0 1 1 | (N+26) mod 2715648 | (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 | (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 |
| 1 0 0 | (N+30 or N+31) mod 2715648 | (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 | (N+82 or N+83) mod 2715648 or (N+86 or N+87) mod 2715648 or (N+91) mod 2715648 or (N+95 or N+96) mod 2715648 |
| 1 0 1 | (N+34 or N+35) mod 2715648 | (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 | (N+99 or N+100) mod 2715648 or (N+104) mod 2715648 or (N+108 or N+109) mod 2715648 or (N+112 or N+113) mod 2715648 |
| 1 1 0 | (N+39) mod 2715648 | (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 | (N+117) mod 2715648 or (N+121 or N+122) mod 2715648 or (N+125 or N+126) mod 2715648 or (N+130) mod 2715648 |
| 1 1 1 | (N+43 or N+44) mod 2715648 | (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 | (N+134 or N+135) mod 2715648 or (N+138 or N+139) mod 2715648 or (N+143) mod 2715648 or (N+147 or N+148) mod 2715648 |
| NOTE: Each RRBP value in Table 10.4.5.5 will assign only one valid uplink block given the downlink coverage class used, i.e. when the poll is received, and the uplink coverage class used. See 3GPP TS 45.002 for the coverage class dependent channel mappings. |  |  |  |

For EC-EGPRS TBFs, Table 10.4.5.6 specifies the coding of the RRBP field if it is received as part of an RLC/MAC block containing an RLC/MAC control block indicating the number of TDMA frames the mobile station shall wait before transmitting the uplink RLC/MAC block according to its uplink Coverage Class. The delay is relative to the first TDMA frame (N) of the downlink block containing the last blind physical layer transmission of the EC-PACCH or EC-PDTCH message, according to the USED_DL_COVERAGE_CLASS field. For definition of TDMA frame numbering, see 3GPP TS 45.002.

Table 10.4.5.6: Relative Reserved Block Period (RRBP) field (EC-EGPRS TBF mode) if received as part of an RLC/MAC block containing an RLC/MAC control block

| bit 7-6 | Full-rate PDCH uplink block with TDMA frame number given UL CC1 or CC2 | Full-rate PDCH uplink block with TDMA frame number given UL CC3 | Full-rate PDCH uplink block with TDMA frame number given UL CC4 |
| --- | --- | --- | --- |
| 0 0 | (N+13) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 | (N+13) mod 2715648 or (N+17 or N+18) mod 2715648 or (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 |
| 0 1 | (N+17 or N+18) mod 2715648 | (N+21 or N+22) mod 2715648 or (N+26) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35) mod 2715648 or (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 |
| 1 0 | (N+21 or N+22) mod 2715648 | (N+30 or N+31) mod 2715648 or (N+34 or N+35 ) mod 2715648 | (N+47 or N+48) mod 2715648 or (N+52) mod 2715648 or (N+56 or N+57) mod 2715648 or (N+60 or N+61) mod 2715648 |
| 1 1 | (N+26) mod 2715648 | (N+39) mod 2715648 or (N+43 or N+44) mod 2715648 | (N+65) mod 2715648 or (N+69 or N+70) mod 2715648 or (N+73 or N+74) mod 2715648 or (N+78) mod 2715648 |
| NOTE: Each RRBP value in Table 10.4.5.6 will assign only one valid uplink block given the downlink coverage class used, i.e. when the poll is received, and the uplink coverage class used. See 3GPP TS 45.002 for the coverage class dependent channel mappings. |  |  |  |

#### 10.4.5.1	Special requirements in dual transfer mode

If the mobile station in dual transfer mode is using PDCH/H, where the exclusive allocation is required, special requirements apply when the mobile station receives a valid RRBP field in a downlink RLC/MAC block:

-	The mobile station may disregard the actual value of a valid RRBP field. The mobile station shall respond to the polling request at the TDMA frame number specified by one of the allowed RRBP values, regardless of which value that was actually received.

-	If the mobile station receives more than one RLC/MAC block with a valid RRBP field, the mobile station shall respond to each one of the polling requests with a separate PACKET CONTROL ACKNOWLEDGEMENT message or PACCH block to the network.

-	When the mobile station responds with a PACKET CONTROL ACKNOWLEDGEMENT message to a valid RRBP field, the mobile station shall use the RLC/MAC control block format. That is regardless of the CONTROL_ACK_TYPE parameter received in the broadcast information of the cell or the TYPE_OF_ACK parameter received in a PACKET POLLING REQUEST message.

If the mobile station in dual transfer mode is not using PDCH/H, the normal requirements apply when the mobile station receives a valid RRBP field in a downlink RLC/MAC block.

### 10.4.6	Countdown Value (CV) field

The Countdown Value (CV) field is sent by the mobile station to allow the network to calculate the number of RLC data blocks remaining for the current uplink RLC entity. The CV value shall be calculated according to the process described in sub-clause 9.3.1. The CV field is 4 bits in length and is encoded as a binary number with range 0 to 15.

A mobile station with an uplink EC-EGPRS TBF is required to send the Countdown Value in an RLC uplink data block in order to inform the network that there are more remaining RLC data blocks for the uplink TBF than it has previously signalled to the network. When transmitting the last RLC data block, the CV equal to zero shall always be included and the Follow-On Indicator (FOI) shall indicate its presence.

### 10.4.6a	Follow-On Indicator field (FOI)

The Follow-On Indicator bit is sent from the mobile station to the network to indicate the presence of a valid Countdown Value field in the uplink RLC data block header during and uplink EC-EGPRS TBF.

Table 10.4.6a.1: Follow On Indicator field

| bit 2 | FOI |
| --- | --- |
| 0 | Countdown Value not present |
| 1 | Countdown Value present |

### 10.4.7	Payload Type field

The Payload Type field shall indicate the type of data contained in remainder of the RLC/MAC block. The encoding of the Payload Type field is shown in table 10.4.7.1.

Table 10.4.7.1: Payload Type field

| bit 8 7 | Payload Type |
| --- | --- |
| 0 0 | RLC/MAC block contains an RLC data block |
| 0 1 | RLC/MAC block contains an RLC/MAC control block that does not include the optional octets of the RLC/MAC control header |
| 10 | In the downlink direction, the RLC/MAC block contains an RLC/MAC control block that includes the optional first octet of the RLC/MAC control header. In the uplink direction, this value is reserved. |
| 1 1 | Reserved. In this version of the protocol, the mobile station shall ignore all fields of the RLC/MAC block except for the USF field |

In downlink RLC/MAC Control blocks encoded using MSC-0 or EC-PACCH/D, the encoding of the Payload Type field is shown in table 10.4.7.2.

Table 10.4.7.2: Payload Type field, MCS-0 and EC-PACCH/D

| bit 1 | Payload Type |
| --- | --- |
| 0 | RLC/MAC block contains an RLC/MAC control block that does not include the optional octets of the RLC/MAC control block message shown in figure 10.3.1.2 or figure 10.3.1.3 |
| 1 | RLC/MAC block contains an RLC/MAC control block that includes at least the first optional octet of the RLC/MAC control block message shown in figure 10.3.1.2 or figure 10.3.1.3. |
| NOTE 1: The bit numbering is relative to the field position. |  |

### 10.4.8	Final block indicator (FBI) bit

The Final block indicator (FBI) bit indicates that the downlink RLC data block is the last RLC data block of the downlink TBF.

Table 10.4.8.1: Final block indicator bit

| bit 1 | Final block indicator |
| --- | --- |
| 0 | Current block is not last RLC data block in TBF |
| 1 | Current block is last RLC data block in TBF |

================ next change ================

### 10.4.8a	Coding and Puncturing Scheme indicator field (CPS)

In (EC-)EGPRS header, the Coding and Puncturing Scheme indicator field is used to indicate the kind of channel coding and puncturing used for data blocks (see 3GPP TS 45.003).

#### 10.4.8a.1	Header type 1

Table 10.4.8a.1.1: Coding and Puncturing Scheme indicator field for Header type 1 in (EC-)EGPRS TBF or downlink EGPRS2 TBF

| bits 54321 | CPS |
| --- | --- |
| 00000 | (MCS-9/P1 ; MCS-9/P1) (see NOTE 2) |
| 00001 | (MCS-9/P1 ; MCS-9/P2) (see NOTE 2) |
| 00010 | (MCS-9/P1 ; MCS-9/P3) (see NOTE 2) |
| 00100 | (MCS-9/P2 ; MCS-9/P1) (see NOTE 2) |
| 00101 | (MCS-9/P2 ; MCS-9/P2) (see NOTE 2) |
| 00110 | (MCS-9/P2 ; MCS-9/P3) (see NOTE 2) |
| 01000 | (MCS-9/P3 ; MCS-9/P1) (see NOTE 2) |
| 01001 | (MCS-9/P3 ; MCS-9/P2) (see NOTE 2) |
| 01010 | (MCS-9/P3 ; MCS-9/P3) (see NOTE 2) |
| 01011 | (MCS-8/P1 ; MCS-8/P1) |
| 01100 | (MCS-8/P1 ; MCS-8/P2) |
| 01101 | (MCS-8/P1 ; MCS-8/P3) |
| 01110 | (MCS-8/P2 ; MCS-8/P1) |
| 01111 | (MCS-8/P2 ; MCS-8/P2) |
| 10000 | (MCS-8/P2 ; MCS-8/P3) |
| 10001 | (MCS-8/P3 ; MCS-8/P1) |
| 10010 | (MCS-8/P3 ; MCS-8/P2) |
| 10011 | (MCS-8/P3 ; MCS-8/P3) |
| 10100 | (MCS-7/P1 ; MCS-7/P1) |
| 10101 | (MCS-7/P1 ; MCS-7/P2) |
| 10110 | (MCS-7/P1 ; MCS-7/P3) |
| 10111 | (MCS-7/P2 ; MCS-7/P1) |
| 11000 | (MCS-7/P2 ; MCS-7/P2) |
| 11001 | (MCS-7/P2 ; MCS-7/P3) |
| 11010 | (MCS-7/P3 ; MCS-7/P1) |
| 11011 | (MCS-7/P3 ; MCS-7/P2) |
| 11100 | (MCS-7/P3 ; MCS-7/P3) |
|  | All the other values are reserved for future use |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: MCS-9 shall only be used for an (EC-)EGPRS TBF or for a downlink EGPRS2-B TBF. |  |

#### 10.4.8a.2	Header type 2

In (EC-)EGPRS and in EGPRS2-A uplink, the Coding and Puncturing Scheme indicator field indicates the kind of channel coding and puncturing used for data block as defined in table 10.4.8a.2.1.

Table 10.4.8a.2.1: Coding and Puncturing Scheme indicator field for Header type 2 in (EC-)EGPRS TBF or uplink EGPRS2-A TBF

| bits 321 | (first block) CPS |
| --- | --- |
| 000 | MCS-6/P1 |
| 001 | MCS-6/P2 |
| 010 | MCS-6/P1 with 6 octets padding (see NOTE 2) |
| 011 | MCS-6/P2 with 6 octets padding (see NOTE 2) |
| 100 | MCS-5/P1 |
| 101 | MCS-5/P2 |
| 110 | MCS-6/P1 with 10 octets padding (see NOTE 3) |
| 111 | MCS-6/P2 with 10 octets padding (see NOTE 3) |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: MCS-6 with 6 octets padding shall only be used for an EGPRS TBF or, in case of an uplink EGPRS2-A TBF, for retransmission of blocks originally transmitted using EGPRS. NOTE 3: MSC-6 with 10 octets padding shall only be used for an uplink EGPRS2-A TBF or, in case of an uplink EGPRS TBF, for retransmission of blocks originally transmitted using EGPRS2-A. |  |

In EGPRS2 downlink, if the Downlink EGPRS Level assigned (see Table 11.2.7.1 and Table 12.10f.1) is EGPRS2-A, the Coding and Puncturing Scheme indicator field indicates the kind of channel coding and puncturing used for data block as defined in table 10.4.8a.2.2.

Table 10.4.8a.2.2: Coding and Puncturing Scheme indicator field for 
Header type 2 in downlink EGPRS2-A TBF

| bits 321 | CPS |
| --- | --- |
| 000 | MCS-6/P1 (see NOTE 2) |
| 001 | MCS-6/P2 (see NOTE 2) |
| 010 | DAS-5/P1 |
| 011 | DAS-5/P2 |
| 100 | DAS-6/P1 |
| 101 | DAS-6/P2 |
| 110 | DAS-7/P1 |
| 111 | DAS-7/P2 |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: In EGPRS2-A downlink, MCS-6 shall be used only for retransmissions of blocks originally transmitted using EGPRS. |  |

In EGPRS2 downlink, if the Downlink EGPRS Level assigned (see Table 11.2.7.1 and Table 12.10f.1) is EGPRS2-B, the Coding and Puncturing Scheme indicator field indicates the kind of channel coding and puncturing used for data block as defined in table 10.4.8a.2.3.

Table 10.4.8a.2.3: Coding and Puncturing Scheme indicator field for 
Header type 2 in downlink EGPRS2-B TBF

| bits 321 | CPS |
| --- | --- |
| 000 | MCS-6/P1 |
| 001 | MCS-6/P2 |
| 010 | DAS-5/P1 |
| 011 | DAS-5/P2 |
| 100 | DAS-6/P1 |
| 101 | DAS-6/P2 |
|  | All the other values are reserved for future use |
| NOTE: The bit numbering is relative to the field position. |  |

#### 10.4.8a.3	Header type 3

Table 10.4.8a.3.1: Coding and Puncturing Scheme indicator field for Header type 3 (non EC-EGPRS TBF)

| bits 4321 | First block CPS |
| --- | --- |
| 0000 | MCS-4/P1 |
| 0001 | MCS-4/P2 |
| 0010 | MCS-4/P3 |
| 0011 | MCS-3/P1 |
| 0100 | MCS-3/P2 |
| 0101 | MCS-3/P3 |
| 0110 | MCS-3/P1 with padding |
| 0111 | MCS-3/P2 with padding |
| 1000 | MCS-3/P3 with padding |
| 1001 | MCS-2/P1 |
| 1010 | MCS-2/P2 |
| 1011 | MCS-1/P1 |
| 1100 | MCS-1/P2 |
| 1101 | MCS-2/P1 with padding (see NOTE 2) |
| 1110 | MCS-2/P2 with padding (see NOTE 2) |
| 1111 | MCS-0 (see NOTE 3) |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: MCS-2 with padding shall only be used for a downlink EGPRS2-A TBF or, in case of a downlink EGPRS TBF, for retransmissions of blocks originally transmitted using EGPRS2-A. NOTE 3: MCS-0 shall only be used for a downlink TBF. |  |

The number of padding octets for uplink is indicated by the SPB field (see sub clause 10.4.8b).

In EC-EGPRS, the CPS field for MCS-1-4 in Header type 3 indicates the kind of channel coding and puncturing used for data block as defined in Table 10.4.8a.3.2.

Table 10.4.8a.3.2. CPS field for Header type 3 in EC-EGPRS TBF.

| bits 3 2 1 | CPS |
| --- | --- |
| 0 0 0 | MCS-4/P1 |
| 0 0 1 | MCS-4/P2 |
| 0 1 0 | MCS-3/P1 |
| 0 1 1 | MCS-3/P2 |
| 1 0 0 | MCS-3/P1 with padding |
| 1 0 1 | MCS-3/P2 with padding |
| 1 1 0 | MCS-2/P1 |
| 1 1 1 | MCS-1/P1 |

================ next change ================

### 10.4.8b	Split Block indicator field (SPB)

In (EC-)EGPRS uplink and EGPRS2 uplink, the Split Block indicator is only used in header type 3 to indicate if some user data is retransmitted using 2 block resegmentation (see clause 9).

Table 10.4.8b.1: Split Block indicator field (uplink)

| bits 2 1 | SPB |
| --- | --- |
| 0 0 | No retransmission |
| 0 1 | Retransmission - first part of block with 10 octet padding |
| 1 0 | Retransmission - first part of block with no padding or 6 octet padding |
| 1 1 | Retransmission - second part of block |
| NOTE: The bit numbering is relative to the field position. |  |

In case of EGPRS, 10 octet padding shall be used only for retransmissions of blocks originally transmitted using EGPRS2-A. In case of EGPRS2-A, 6 octet padding shall be used only for retransmissions of blocks originally transmitted using EGPRS.

In EGPRS downlink and EGPRS2 downlink, the Split Block indicator is only used in header type 3 to indicate if some user data is retransmitted using 2 or 3 block resegmentation (see clause 9).

Table 10.4.8b.2: Split Block indicator field (downlink)

| bits 2 1 | SPB |
| --- | --- |
| 0 0 | No retransmission |
| 0 1 | Retransmission - third part of block (Note 2) |
| 1 0 | Retransmission - first part of block |
| 1 1 | Retransmission - second part of block |
| NOTE 1: The bit numbering is relative to the field position. NOTE 2: In EGPRS downlink this code point is only used when retransmiting blocks at EGPRS level change from EGPRS2-A to EGPRS. |  |

================ next change ================

### 10.4.9b	Final Segment (FS) bit

The Final Segment (FS) bit indicates that the downlink RLC/MAC control block contains the final segment of an RLC/MAC control message except when it is sent using extended RLC/MAC control message segmentation. In case of extended RLC/MAC control message segmentation, the final segment of an RLC/MAC control message is indicated with the FSe bit as defined in sub-clause 10.4.9e while the FS bit is set to ‘0’ (see sub-clauses 9.1.12a and 10.4.12b).

Table 10.4.9b.1: Final Segment (FS) bit

| bit 1 | Final Segment (FS) bit |
| --- | --- |
| 0 | Current block does not contain the final segment of an RLC/MAC control message |
| 1 | Current block contains the final segment of an RLC/MAC control message |
| NOTE: The bit numbering is relative to the field position. |  |

================ next change ================

### 10.4.9f	Reduced TLLI (rTLLI)

A mobile station in EC-EGPRS operation that has been indicated to use the enhanced access burst procedure shall only include the TLLI in the first RLC data block of the uplink TBF and the rTLLI in the RLC/MAC headers of the remaining RLC data blocks until the contention resolution is completed (see sub-clause 7a.2.1.2).

### 10.4.9g	Reduced TLLI Indicator (RI)

The RI bit is used to indicate whether the rTLLI field is valid or not valid.

Table 10.4.9g.1: Reduced TLLI Indicator (RI)

| bit 1 | RI |
| --- | --- |
| 0 | RI field is not valid |
| 1 | RI field is valid |

================ next change ================

### 10.4.10a	Power Reduction (PR) field

The Power Reduction (PR) field indicates the power level reduction of the current RLC block.

The coding of Power Reduction (PR) field is shown in Table 10.4.10a.1. There is one value of the PR field which indicates that the field shall be ignored by the MS.

If downlink power control is not used, the MS shall ignore the PR field.

If downlink power control is used and the PR field is not included in a downlink RLC/MAC control block, the MS shall act as if the block contained a usable PR field with value ‘0 0’.

Table 10.4.10a.1: Power Reduction (PR) field

| bit 8 7 | Power Reduction |
| --- | --- |
| 0 0 | 0 dB (included) to 3 dB (excluded) less than BCCH level - P0 |
| 0 1 | 3 dB (included) to 7 dB (excluded) less than BCCH level - P0 |
| 1 0 | 7 dB (included) to 10 dB (included) less than BCCH level - P0 |
| 1 1 | Not usable |

The Power Reduction (PR) field in a downlink EC-EGPRS control block indicates the power level reduction of the current RLC/MAC control block. The coding of Power Reduction (PR) field for a downlink EC-EGPRS control block is shown in Table 10.4.10a.2.

Table 10.4.10a.2: Power Reduction (PR) field for EC-EGPRS control blocks

| bit 8 | Power Reduction |
| --- | --- |
| 0 | 0 dB (included) to 3 dB (excluded) less than BCCH level - P0 |
| 1 | 3 dB (included) to 7 dB (excluded) less than BCCH level - P0 |

### 10.4.10b Power Reduction extension (PRe) field

The Power Reduction extension (PRe) field indicates the extended power level reduction of the current RLC/MAC control block. The PRe field is only included if the Payload Type, see figure 10.4.7.2, is set to 1.

When included the PRe field, together with the PR field, gives the coding of the power reduction for a downlink EC-EGPRS control block as shown in Table 10.4.10b.1.

Table 10.4.10b.1: PR bit, PRe bit

| PR | PRe | Power Reduction |
| --- | --- | --- |
| 0 | 0 | 0 dB (included) to 3 dB (excluded) less than BCCH level - P0 |
| 1 | 0 | 3 dB (included) to 7 dB (excluded) less than BCCH level - P0 |
| 0 | 1 | 7 dB (included) to 10 dB (included) less than BCCH level - P0 |
| 1 | 1 | Reserved |

================ next change ================

### 10.4.12	Block Sequence Number (BSN) field

The Block Sequence Number (BSN) field carries the sequence absolute Block Sequence Number (BSN') modulo Sequence Number Space (SNS) (32 in EC-EGPRS, 128 in GPRS, 2 048 in EGPRS and 8192 in a DLMC configuration) of each RLC data block within the TBF.

In EC-EGPRS, the BSN is 5 bits in length and is encoded as a binary number with range 0 to 31.

In GPRS, the BSN is 7 bits in length and is encoded as a binary number with range 0 to 127.

In EGPRS, or in a DLMC configuration where a SNS of 2048 is used (see sub-clause 5.13), the BSN is 11 bits in length and is encoded as a binary number with range 0 to 2 047.

In a DLMC configuration where a SNS of 8192 is used (see sub-clause 5.13), the BSN is 13 bits in length and is encoded as a binary number with range 0 to 8191.

In case two to four RLC data blocks are sent within a RLC/MAC block, BSN2 to BSN4 are relative to BSN1, provided the difference between the second to fourth block number and the first block modulo SNS is less than Window Size (WS).

For example,

Second block number = [BSN1 + BSN2] modulo SNS

(e.g. SNS = 2 048, WS = 512, Block A block number = 10 and Block B block number= 2 000 then:

[Block A - Block B] modulo SNS = 58 < 512;

[Block B - Block A] modulo SNS = 1 990 > 512;

Then: Block #1 = Block B and Block #2 = Block A, BSN1 = 2 000 and BSN2 = 58).

In a DLMC configuration there are restrictions regarding the selection of RLC data blocks for inclusion within a given RLC/MAC block:

-		In the RLC/MAC header for MCS-7, MCS-8 and MCS-9 13 bits for BSN1 and 11 bits for BSN2 are defined. For these MCSs the difference between the two RLC data blocks (Block A and Block B) shall be less than 2048.

-	i.e. [Block A - Block B] modulo SNS < 2048 or [Block B - Block A] modulo SNS < 2048

-	In the RLC/MAC header for DAS-11 and DAS-12 13 bits for BSN1, 12 bits for BSN2 and 11 bits for BSN3 are defined. For these MCSs the difference between two of the RLC data blocks (i.e. two out of Block A, B or C) shall be less than 2048.

-	i.e. [Block X - Block Y] modulo SNS < 2048 or [Block Y - Block X] modulo SNS < 2048 where X is any one of Block A, B or C, and Y is any of the two remaining blocks.

For example,

SNS = 8192, WS = 4096:

BSN1=2000 (Block B block number=2000)

BSN2=58 (Block A block number=2058)

-	[Block A – Block B] modulo 8192 = 58 < 4096

-	[Block B – Block A] modulo 8192 = 8133 > 4096

then Block C block number shall fulfill [Block C - Block B] modulo SNS < 2048.

For example Block C block number = 4003, and BSN3=2003

-	[Block C – Block B] modulo 8192 = 2003 < 2048

================ next change ================

### 10.4.14a	Length Indicator (LI) field in EGPRS TBF mode, EC-EGPRS TBF mode and TCH TBF mode (Iu mode)

The Length indicator is used to delimit Upper Layer PDUs within the RLC data block. Additionally for Iu mode, a Length Indicator of value 127 may be used to indicate the presence of filler information within the RLC data block. If the first length indicator in the RLC data block is set to 127, then the entire RLC data block contains filler information.

The first Length Indicator (respectively, second length Indicator, if the first is of value 123, as described below) shall indicate the number of octets of the RLC data field belonging to the first Upper Layer PDU, the second Length Indicator (respectively third Length Indicator, if the first is of value 123, as described below) shall indicate the number of octets of the RLC data field belonging to the second Upper Layer PDU, etc. Only the last segment of any Upper Layer PDU, including those with only one segment, shall be identified with a Length Indicator. The length indicator shall be placed in the RLC data block corresponding to the last segment of the Upper Layer PDU, unless the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely. In this singular case, the Length Indicator shall be placed as the first Length Indicator in the next in sequence RLC data block and take the value 0. The inclusion of a Length Indicator that indicates the presence of a Selected PLMN Index field (see sub-clause 10.4.27) will never correspond to an Upper Layer PDU and therefore will never result in this singular case.

If the Upper Layer PDU does not fill the current RLC data block, a Length Indicator with value 127 (111 1111) shall be included as the last Length Indicator of the current RLC data block, indicating that there is no following Upper Layer PDU in this RLC data block. If the Upper Layer PDU does not fill the RLC data block and there is only one octet left, then the Length Indicator corresponding to the Upper Layer PDU is the last Length Indicator field that shall be included in the RLC data block.

During RLC non-persistent mode of operation, or, during RLC unacknowledged mode of operation if the network has commanded the usage of "Indication of Upper Layer PDU Start for RLC UM" for the TBF, and if the current RLC data block starts with the first segment of a new Upper Layer PDU, then a Length Indicator taking the value 126 shall be placed as the first Length Indicator in the RLC data block unless a Length Indicator with the value 0, signalling that the last segment of the previous Upper Layer PDU precisely fills the previous in sequence RLC data block, shall be included.

If the network and the mobile station support dynamic timeslot reduction (see sub-clause 8.1.8), a Length Indicator with value 125 indicates the presence of dynamic timeslot reduction control information which shall be included after the last Upper Layer PDU.

If EMSR is enabled, the transmission of an Upper Layer PDU can be suspended to transmit a higher priority Upper Layer PDU by including an LI with a value of 124 as the first LI field within the next transmitted RLC data block where an octet containing the TFI value associated with the higher priority Upper Layer PDU immediately follows the LI field. The transmission of the suspended Upper Layer PDU is resumed by including an LI with a value of 124 followed by an octet containing the TFI (and eTFI if assigned, see sub-clause 5.13) associated with the Upper Layer PDU whose transmission is resumed after the LI associated with the last segment of the higher priority Upper Layer PDU whose transmission is completed. The transition from the last segment of an Upper Layer PDU to the first segment of a new Upper Layer PDU associated with a different PFC can also occur within an RLC data block (i.e. no suspend occurs) in which case the transition is indicated by including an LI with a value of 124 followed by an octet containing the TFI (and eTFI if assigned) associated with the new Upper Layer PDU after the LI associated with the last segment of the Upper Layer PDU whose transmission is completed. In addition:

-	If the last segment of the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely and the next in sequence RLC data block is in the same radio block, then a Length Indicator with the value 0 shall be placed as the first Length Indicator in the next in sequence RLC data block.

-	If the last segment of the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely and the next in sequence RLC data block is in a different radio block, then a Length Indicator with the value 0 shall be placed as the first Length Indicator in the next in sequence RLC data block and the TFI in the RLC/MAC header associated with the next in sequence RLC data block and, if eTFI is assigned, the eTFI indicated by the CRC of the  next in sequence RLC data block, shall correspond to the last segment of the Upper Layer PDU.

-	When a Length Indicator having the value 124 is included in the same RLC data block as the last segment of the Upper Layer PDU and fills the RLC data block precisely then the first octet of the RLC data field of the next in sequence RLC data block shall include the TFI (and eTFI if assigned) corresponding to the Upper Layer PDU whose transmission is resumed (if a suspend occurred) or to a new Upper Layer PDU (if no suspend occurred).

A Length Indicator of value 123 is used by a mobile station supporting network sharing to indicate the presence of the Selected PLMN Index field (see sub-clause 10.4.27) in the octet immediately following this Length Indicator. In this case  it shall always be the first Length Indicator in the RLC data block. The first Length Indicator used for the case of EMSR requires that at least one previous RLC data block has been sent for the uplink TBF whereas the first Length Indicator used to indicate the presence of the Selected PLMN Index field shall always occur within the very first RLC data block sent for the uplink TBF and as such they can never occur within the same RLC data block.

The final RLC data block of a TBF shall have a Length Indicator field corresponding to the final Upper Layer PDU unless the final Upper Layer PDU fills the RLC data block precisely. If the final Upper Layer PDU fills the final RLC data block precisely, the final Upper Layer PDU shall be sent without a corresponding Length Indicator field.

The Length Indicator field is 7 bits in length and shall be encoded as a binary number. The valid values are the values ranging from 0 to 74 for an (EC-)EGPRS TBF, an EGPRS2-A uplink TBF or an EGPRS2-B TBF, from 0 to 82 for an EGPRS2-A downlink TBF, from 0 to 103 in TCH TBF mode (Iu mode), and the values 124 to 127. All other values are reserved. A mobile station detecting a reserved Length Indicator value or an inconsistent encoding of the Length Indicator and E fields shall ignore the RLC data block.

The interpretation of the value contained in the length indicator with corresponding E bit is summarized in table 10.4.14a.1 and some examples are shown in annex B.

Table 10.4.14a.1: Interpretation of values of LI field and E bit

| Value of LI in a RLC data block | Value of E bit in the same octet | Interpretation |
| --- | --- | --- |
| k-th LI (k>0 integer): 0< value <75 ((EC-)EGPRS except EGPRS2-A downlink) 0< value <83 (EGPRS2-A downlink) 0< value <104 (TCH) |  | The value of the k-th LI is the number of octets of the k-th Upper Layer PDU, or the last segment of it, in the current RLC data block. |
|  | 0 | There is at least one Upper Layer PDU following the k-th Upper Layer PDU in the current RLC data block. |
|  | 1 | There is no more than one Upper Layer PDU following the k-th Upper Layer PDU in the current RLC data block. |
| 1st LI: value =0 | 0 | The last Upper Layer PDU of the previous in sequence RLC data block ends at the boundary of that RLC data block and it has no LI in the header of that RLC data block. Thus the current RLC data block contains the first segment of all included Upper Layer PDUs. |
| 1st LI: value = 126 | 0 | The current RLC data block contains the first segment of all included Upper Layer PDUs. |
| k-th LI (k>1 integer): 0< value <75 ((EC-)EGPRS except EGPRS2-A downlink) 0< value <83 (EGPRS2-A downlink) 0< value <104 (TCH) |  | The k-th LI contains the number of octets of the (k-1)-th Upper Layer PDU in the current RLC data block. |
|  | 0 | There is at least one Upper Layer PDU following the (k-1)-th Upper Layer PDU in the current RLC data block. |
|  | 1 | There is no more than one Upper Layer PDU following the (k-1)-th Upper Layer PDU in the current RLC data block. |
| k-th LI: value=127 | 1 | The octets between the end of the Upper Layer PDU indicated by the (k-1)-th LI and the end of the current RLC data block are filling octets. |
| 1st LI: value=0 | 1 | The previous RLC data block contains a Upper Layer PDU, or a part of it, that fills precisely the previous data block and for which there is no length indicator in that RLC data block. The current RLC data block contains a Upper Layer PDU that either fills the current RLC data block precisely or continues in the next RLC data block. |
| 1st LI: value = 126 | 1 | The current RLC data block contains the first segment of an Upper Layer PDU that either fills the current RLC data block precisely or continues in the next RLC data block. |
| 1st LI: value=127 (Iu mode only) | 1 | All octets of the RLC Data block contain filling information. |
| k-th LI: value=125 (this value is not supprted in EC-EGPRS operation) | 0/1 | The current RLC data block contains the dynamic timeslot reduction control information. |
| k-th LI:value=124 (this value is only allowed if EMSR is enabled) | 0 | An LI value of 124 serves as a TFI transition indicator and is immediately followed by an octet containing the TFI associated with the next Upper Layer PDU segment in bit positions 1 to 5 (where bit 5 is the most significant bit) and reserved bits in bit positions 6 to 8. If eTFI is assigned then it shall be indicated by bit positions 6 to 8 (where bit 8 is the most significant bit). |
| 1 st LI:value=123 (this value is only allowed if network sharing is supported both by the network and the mobile station) | 0/1 | An LI value of 123 serves as a Selected PLMN Index field indicator and is immediately followed by an octet that contains the Selected PLMN Index field (see sub-clause 10.4.27). |
| No LI field present | n.a. | The Upper Layer PDU that starts with the current RLC data block either fills the current RLC data block precisely or continues in the following in-sequence RLC data block |

================ next change ================

### 10.4.19	PFI Indicator (PI) bit

The PFI Indicator (PI) indicates the presence of the optional PFI field. The PI shall be ignored in Iu mode.

Table 10.4.19.1: PFI Indicator (PI) bit

| bit | PFI Indicator (PI) bit |
| --- | --- |
| 0 | PFI is not present |
| 1 | PFI is present if TI field indicates presence of TLLI |

### 10.4.20	Packet Flow Identifier (PFI) field

The PFI field contains a PFI value encoded as the contents of the PFI information element as defined in 3GPP TS 44.018. The use of PFI is not supported in EC-EGPRS operation.

================ next change ================

### 10.4.27	Selected PLMN Index field

This field contains the index of the PLMN selected by a mobile station supporting network sharing. The index value refers either to the identity of the Common PLMN broadcast in the SYSTEM INFORMATION TYPE 3/4 message, or EC-SYSTEM INFORMATION TYPE 2 message for EC-EGPRS, or to the identity of an Additional PLMN broadcast in the SYSTEM INFORMATION TYPE 22 message, or EC-SYSTEM INFORMATION TYPE 4 message for EC-EGPRS, as described in Table 10.4.27.1.

Table 10.4.27.1: Selected PLMN Index field

| bit 3 2 1 | Selected PLMN Index |
| --- | --- |
| 0 0 1 | PLMN identity of the Common PLMN broadcast in SYSTEM INFORMATION TYPE 3/4. For EC-EGPRS, the PLMN identity of the Common PLMN is broadcast in EC-SYSTEM INFORMATION TYPE 2. |
| 0 1 0 | PLMN identity of the first Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-EGPRS, the PLMN identity of the first Additional PLMN in the network sharing information is broadcast in EC-SYSTEM INFORMATION TYPE 4. |
| 0 1 1 | PLMN identity of the second Additional PLMN in the network sharing information broadcast broadcast in SYSTEM INFORMATION TYPE 22. For EC-EGPRS, the PLMN identity of the second Additional PLMN in the network sharing information is broadcast in EC-SYSTEM INFORMATION TYPE 4. |
| 1 0 0 | PLMN identity of the third Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-EGPRS, the PLMN identity of the third Additional PLMN in the network sharing information is broadcast in EC-SYSTEM INFORMATION TYPE 4. |
| 1 0 1 | PLMN identity of the fourth Additional PLMN in the network sharing information broadcast in SYSTEM INFORMATION TYPE 22. For EC-EGPRS, the PLMN identity of the fourth Additional PLMN in the network sharing information is broadcast in EC-SYSTEM INFORMATION TYPE 4. |
|  | All other values are reserved |

================ next change ================

### 10.4.28	Coverage Class field (CC)

In EC-EGPRS operation, a 2-bit field is included to signal the downlink Coverage Class used in the block. The Coverage Class indicates a predefined number of blind physical layer transmissions to be applied for each logical channel, where the use of blind physical layer transmissions can allow the mobile station to operate in extended coverage, see 3GPP TS 43.064. The CC field can be used by a mobile station in a lower coverage class than indicated by the CC field, monitoring the downlink EC-PDTCH or EC-PACCH, to realize that the ongoing downlink transmission is intended for another mobile station with a higher downlink Coverage Class and therefore stop reading until the next downlink opportunity. The next monitoring opportunity is then given by the CC field.

Table 10.4.8d.1: Coverage Class field

| bits 2 1 | CC |
| --- | --- |
| 0 0 | Coverage Class 1, CC1 |
| 0 1 | Coverage Class 2, CC2 |
| 1 0 | Coverage Class 3, CC3 |
| 1 1 | Coverage Class 4, CC4 |
| NOTE: The bit numbering is relative to the field position. |  |