# Impact on PDCH for GPRS/EGPRS in a reduced BCCH spectrum allocation
(update of GP-160039)

# Introduction

At GERAN#67 a new work item named Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things (WI code: CIoT_EC_GSM) was approved, see [1]. One of the objectives of the work item is the following:

“Support for extended coverage GSM deployment in a reduced BCCH spectrum allocation, provided it is shown to be feasible, from 4/12 (2.4 MHz) frequency re-use to 3/9 (1.8 MHz) or 1/3 (600 kHz) frequency re-use, where legacy CS users might not be supported, and add respective normative changes, if any.”

The BCCH layer serves many purposes, which may be negatively impacted by a tightened reuse, including the following:

- Frequency and time synchronization (FCCH, SCH)

- Cell identification (reading BSIC on SCH) and received signal level measurements e.g. for cell reselection purposes

- System information acquisition (BCCH)

- Common control signaling (RACH, PCH, AGCH)

- User data traffic

In this contribution, item 5 is investigated for legacy services for BCCH layer spectrum allocations using 1.8 MHz down to 600 kHz. Item 1 is investigated in [2].

The document is an update of GP-160039 and include more recent simulation settings and specifically simulations run using the agreed traffic model for legacy traffic in [7].

# Assumptions

## Traffic generation

MTC traffic is generated according to ‘Global traffic model for MTC traffic of legacy GPRS’ [7]. It could be noted that with the aggressive model approach chosen, the load in the network will increase compared to the IoT model previously used in the study by around 40 % on the UL..

## RACH interference

Interference from RACH has been modelled. The power reduction on RACH introduced in GERAN Rel-11 is assumed not to be supported by the MSs, and hence full power is used on the RACH channel.

## BCCH Power Savings

BCCH power savings can be used to reduce interference on the BCCH frequency layer. With tighter BCCH frequency re-use the importance of this functionality increases. BCCH power savings can be used with various levels of reduction and selections of what timeslots and channels it should be applied to. For simulator implementation, a simple implementation for BCCH power savings was used with a reduction of 6 dB for timeslots not used for PDTCH or PACCH. In case PDTCH or PACCH are used on the DL, no power regulation is used. Timeslots TS0 (carrying BCCH, FCCH, SCH, CCCH) and TS1 (carrying EC-BCCH, EC-CCCH, EC-SCH) are excluded from BCCH Power Savings.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [5] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [5]

| Parameter | Value |
| --- | --- |
| General |  |
| Simulation time | 100 s |
| System size | 108 cells (all frequency re-uses) |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | 12, 9 and 3 |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Cell radius | 577.33 m |
| Legacy GPRS MTC parameters |  |
| PDTCH timeslots per cell | 7 PDCH(note 1) |
| Legacy GPRS MTC arrival rate per cell and second | 5.4 (100%)(note 2) |
| Coding schemes | CS-1 |
| GPRS L2S model | Approximated by EGPRS L2S (MCS-1) without incremental redundancy, see [6] |
| Minimum delay between subsequent transmissions on PDTCH and PACCH | 1 radio block |
| Incremental Redundancy | Off |
| Power control | DL: - Off - Power savings 6 dB if nothing to transmit on BCCH TS2-TS7. UL: - On (3 re-use) / Off (9 and 12 re-use) - Closed-loop PC based on estimated power level on RACH with power regulation starting at a received signal level of -70 dBm, using a down-regulation of at most 16 dB |
| IP header compression | Off |
| Device output power | 33 dBm (100%) |
| BPL model(note 3) | No BPL applied |
| Device timeout | 20 seconds |
| NOTE 1: The system simulator uses a network wide timeslot alignment with a random timeslot offset between cells. NOTE 2: Aggregated total event intensity on UL and DL. The traffic model and packet sizes are implemented as suggested in [7]. 5.4 transfers per cell and second corresponds to sum of the 1.39 events/cell/s DL and 4.03 events/cell/s DL. NOTE 3: According to model in [5] |  |

### Cell selection and coding scheme selection

Cell selection was based on the calculated path gain and a N(0,2) dB measurement error. All devices are stationary in the simulations so there will be no cell re-selection.

In the simulations, no link adaptation was used. Instead, the coding scheme was always selected to CS-1 and remained the same throughout the duration of the TBF.

### Control signaling

Packet uplink ACK/NACK (PUAN) is sent on PACCH/D to (negatively) acknowledge data sent in the UL, as well as Packet downlink ACK/NACK (PDAN) sent on PACCH/U to (negatively) acknowledge data sent on the DL. In the simulations its performance is modeled with EGPRS MCS-1. If a PUAN/PDAN is unsuccessfully received, the negative acknowledged blocks will not be transmitted. In the UL this means that the allocated MS will not transmit anything, but the radio block resources are consumed, and, on the DL the BTS will not be able to schedule retransmissions.

### Simulated scenarios

Table 2 summarizes the simulated scenarios and clarifies the legends in the figures presented in section 3.2. No explicit frequency planning effort has been made and the simulations only use regular repeatable cluster re-use patterns..

Table 2: Simulated scenarios

| Legend text | BCCH Re-use | Frequency planning |
| --- | --- | --- |
| Re-use = 12 | 12 | 4/12 cluster re-use pattern |
| Re-use = 9 | 9 | 3/9 cluster re-use pattern |
| Re-use = 3 | 3 | 1/3 cluster re-use pattern |

## Results

The results presented are:

- Resource (TS) Usage (section 3.2.1)

  - This represents the average amount of PDTCH DL and UL TS resources required per cell in the system, for the different scenarios, see Table 3.

- Latency of Uplink Transmissions (section 3.2.2)

  - The latency includes time to transfer the message.

  - The results are presented as CDFs of the delay at the target traffic load (5.4 users per cell and second), see Figure 1.

  - Failed attempts are not included in the statistics (following the agreed methodology).

- Latency of Downlink Transmissions (section 3.2.3)

  - The latency includes time to transfer the message The results are presented as CDFs of the delay at the target traffic load (5.4 users per cell and second), see Figure 2.

- Failed attempts (section 3.2.4)

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds.

- Uplink capacity (section 3.2.5)

  - Uplink capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. Results are shown in Table 5.

### Time Slot Usage

The TS Usage is shown in Table 3 for the downlink and uplink respectively. On the downlink, the TS Usage increases with roughly 12% from 0.26 to 0.29 when the re-use is changed from 9 to 3, and on the UL with 2%.

Table 3. TS Usage for the downlink and uplink

| BCCH Re-use | TS usage DL [#TS] | TS usage UL [#TS] |
| --- | --- | --- |
| 12 | 0.26 | 0.89 |
| 9 | 0.26 | 0.89 |
| 3 | 0.29 | 0.91 |

For the uplink, the TS Utilization also increases only marginally from 0.89 to 0.91 when the re-use is changed from 9 to 3.

### Latency of Uplink Transmissions

The latency of Uplink transmissions is represented by the latency of the data transfer, i.e. the common control signaling delay, see [3], is not included. A few users will experience an increased delay as seen in Figure 1. The delays are increasing with tighter frequency re-use.

Figure 1: Uplink Transmission Delay

The “knees” in the distribution are due to the three different packet sizes used in the traffic model. In the figure only the impact from the two biggest packet sizes can be seen, but there is also a small “knee” just below 30% for the smallest packet size.

### Latency of Downlink Transmissions

A few users will experience an increased delay as seen in Figure 2. Also in this case, the delay is increased with tighter frequency re-use.

Figure 2: Downlink Transmission Delay

### Failed Attempts

At the traffic load 5.4 events per cell and second, the percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is found to be 0 % in the 12, 9 and 3 re-use scenarios. Failed attempts are shown in Table 4.

Table 4: Failed attempts

| BCCH Re-use | Failed attempts [%] |
| --- | --- |
| 12 | 0 |
| 9 | 0 |
| 3 | 0 |

### Capacity

In [3] the capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. This definition is made with a standalone CIoT system in mind. The system in this evaluation serves only one traffic type (MTC traffic), but the event intensities and packet sizes differ on the downlink and uplink. On the downlink all packet sizes are the same (45 bytes), and have the intensity of 1.4 reports per sector and second. On the uplink the packet sizes are ‘randomly’ picked from 40, 150 or 1200 bytes and have the intensity of 3 reports per sector and second. Due to the mix of packet sizes and different intensities on uplink and downlink the capacity definition may be less meaningful, but anyway an attempt has been made to present the capacity for the combined intensity of 5.4 reports per sector and second. It should be noted that the measure is not really a capacity measure since it does not reflect the capacity limit of the system but rather at an assumed fixed load.

Capacity is here calculated as

(#sent reports per sector per hour)*(1 - failed attempts)/reuse

The capacity is shown in Table 5 for the simulated scenarios.

Table 5: Capacity

| BCCH Re-use | Capacity [reports/200kHz/hour] |
| --- | --- |
| 12 | 1620 |
| 9 | 2160 |
| 3 | 6480 |

As can be seen from the table, the 3-reuse scenario has three times higher capacity than the 9-reuse scenario, as expected considering the change in re-use factor, and the fact that no reports fails to be delivered.

# Discussion and conclusions

This paper shows that legacy MTC services may be accommodated on as low BCCH spectrum allocations as 600 kHz with a very marginal increase in TS utilization compared to 1.8 MHz. The transmission delays are increased for some devices; the effect is however rather small compared to the 67% reduction of the required frequency spectrum, corresponding to three times the spectral efficiency. For the 600 kHz spectrum allocation, the network interference levels may need to be controlled by efficient GPRS/EGPRS MS power control settings and BCCH Power Savings.

# References

GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

GP-151124, ”GPRS synchronization performance in a tight frequency reuse scenario”, source Ericsson LM. GERAN#68.

3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things V13.0.0.

GP-151183, “Impact on legacy services in a reduced BCCH spectrum allocation”, source Ericsson LM. GERAN#68.

GP-151104, ”Intended scope for reduced spectrum allocation on BCCH evaluation (update of GPE150043)”, source Ericsson LM. GERAN#68.

GP-150762, “EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations (update of GPC150441)”, source Ericsson LM. GERAN#67.

GP-160060, “Traffic model for legacy GPRS MTC”, source Orange, Ericsson LM. GERAN#69.

In reality, the MS would likely send preemptive retransmissions, but this is not modelled in the simulator.

It should be noted that this is rather an increase in spectral efficiency than capacity since the same absolute number of reports are delivered in both scenarios.