3GPP TSG RAN Meeting #68	RP-150607

# Malmö, Sweden, June 15th – 18th, 2015

Source:	NEC

Title:	Performance evaluation set 1 results for possible additional TDD configuration(s)

Document for:	Discussion

Agenda Item:	12.4.4

## Introduction

In RAN #66 meeting, a new SI “Study on possible additional configuration of LTE TDD” was approved [1]. The objective of the study is to evaluate issues related to the potential introduction of the following additional configuration(s) for LTE TDD; 10:0:0 and 9:1:0 (DL:Sp:UL).

In RAN #67 meeting, evaluation scenarios for possible additional TDD configuration(s) are approved [2] and captured in TR36.825 [3].

In this contribution, we provide our simulation results of performance evaluation set 1 for possible additional TDD configuration(s) according to the agreed scenarios and simulation assumptions in [4].

## Discussions

In the mobile network, the load of downlink data traffic is much higher than uplink according to the traffic trend. In the current LTE spec, seven TDD UL-DL configurations have been supported to address different DL and UL traffic ratios. Configuration 5 supports the largest downlink resources, where 8 DL subframes and one DwPTS in a special subframe can be used for downlink transmission. However, about 11.4~20% of radio resource will be wasted when there is little UL traffic, which is a significant performance loss. In order to meet the great market demand and the traffic trend, the introduction of a new TDD UL-DL configuration, ‘10:0:0/9:1:0 configuration’, is quite necessary. This 10:0:0 /9:1:0 configuration can be configured as a Secondary Cell only when a UE is configured with CA which will result in facilitating the availability of TDD in the global market, the details can be found in an accompany contribution [5].

- Deployment scenario

The scenario for performance evaluation set 1 is captured in TR 36.825, where no adjacent channel interference is present. In our simulation, supplementary downlink (SDL with new TDD UL-DL configuration 10:0:0) and baseline are evaluated. For both two cases, when UE is configured with CA, macro cell (FDD) is configured as a Primary cell (Pcell). For SDL, small cell with UL-DL configuration 10:0:0 is configured as a Secondary Cell (Scell), for baseline, small cell with UL-DL configuration #5 (8:1:1) with special subframe configuration #4 (12 downlink symbols) is configured as a Scell.

The UL-DL configurations of Scell for baseline and SDL can be found in Figure 1-a, and 1-b.

Figure 1-a TDD UL-DL configuration #5(8:1:1) for baseline

Figure 1-b TDD new UL-DL configuration (10:0:0) for SDL

  - Performance estimation

As mentioned in section 2.1, for the non-CA UE, macro cell is the only serving cell, so the performance of this kind of UE is the same for baseline and SDL. For the CA capable UE, Pcell (FDD) is the same for both cases, the performance gain for SDL is provided from Scell for the additional downlink subframes.

For the baseline, the number of Pcell subframes is 10, and the number of Scell subframes is about 8.86 (8 downlink subframes and 12 downlink symbols in special subframe). For SDL case, the number of Pcell subframes is 10, and the number of Scell subframes is 10. The links between Pcell and Scell are different, we assume link capacity of Pcell and Scell is Pc and Sc respectively.

The gain of SDL compared with baseline can be estimated as:

10*Pc+10*Sc10*Pc+8.86*Sc-1

The upper bound is the traffic only on Scell, the gain is 10/8.86-1≈12.9%, the lower bound is the traffic only on Pcell, the gain is 0.

## Performance evaluation

In this contribution, we provide the performance evaluation of SDL and baseline. The details of simulation assumption can be found in the appendix. The simulation results of traffic arrival rate lambda=2, 3, 4 for baseline and SDL are shown in Table 1-a, 1-b and 1-c.

Table 1-a Traffic arrival rate lambda = 2 for baseline and SDL, FTP model 1

| Cases | RU-Pcell | RU-Scell | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Mean(Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | 65.88% | 10.59% | 0.081 | 9.908 | 30.30 | 11.78 |
| SDL | 64.28% | 9.67% | 0.084 (+3.70%) | 10.58(+6.87%) | 32.52(+7.33%) | 12.64(+7.30%) |

Table 1-b Traffic arrival rate lambda = 3 for baseline and SDL, FTP model 1

| Cases | RU-Pcell | RU-Scell | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Mean(Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | 84.99% | 19.83% | 0.0189 | 6.71 | 26.85 | 8.90 |
| SDL | 84.27% | 18.27% | 0.0195(+3.17%) | 7.18(+7.00%) | 29.20(+8.75%) | 9.60(+7.87%) |

Table 1-c Traffic arrival rate lambda = 4 for baseline and SDL, FTP model 1

| Cases | RU-Pcell | RU-Scell | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Mean(Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | 93.80% | 30.19% | 0.00880 | 4.62 | 23.39 | 6.92 |
| SDL | 93.54% | 29.18% | 0.00891(+1.25%) | 4.90(+6.06%) | 25.64(+9.62%) | 7.48(+8.09%) |

Take the traffic arrival rate lambda=3 for example, the CDF of Pcell and Scell throughput for SDL and Baseline is shown in Figure 2.

In SDL, more downlink subframes can be used for data transmission, which leads to that the performance of SDL-Scell is better than that of baseline. In addition, more traffic can be off-loaded to Scell, there are more resources available in Pcell, leading to the better performance in Pcell of SDL compared with Pcell of baseline, as shown in Figure 2.

From the simulation results, we can observe that the cell edge (5% UPT) UEs are mainly macro-only UE, while for 50% and 95% UPT UEs, most have Scell links. So the performance gain is mainly focused on 50% and 95% UEs.

Observation 1: SDL can provide performance gain of 6 to 10% with the additional downlink subframe, and the performance gain is mainly focused on the 50% and 95% UPT UEs.

Figure 2 CDF of Pcell and Scell for SDL and Baseline (Lambda=3 for example)

Figure 3 shows the traffic allocation of Pcell from the total throughput for UEs with different traffic arrival rates.

From Figure 3, we can observe that about 63% UEs can get the Scell activated. And among these Scell activated UEs, most have better Scell links than Pcell (The UEs with Pcell throughput ratio < 0.5 shown in Figure 3).

With traffic arrival rate increasing, as shown in Figure 3, the throughput provided from Pcell (Macro) is getting smaller, and the Pcell is becoming more resource-constrained, this will lead to the cell edge (5% UPT) UE throughput gain gets smaller. While for the Scell activated UEs, more traffic is off-loaded on Scell, especially for high traffic arrival rate, leading the larger gain of mean UPT. So the SDL is more suitable for hot spots scenarios with high traffic arrival rate.

Observation 2: SDL is more suitable for hot spots scenarios with high traffic arrival rate.

Figure 3 CDF of Pcell throughput ratio for SDL and Baseline

## Conclusions

In this contribution, we analysed the performance of SDL and baseline, and we provided the simulation results with different traffic arrival rates. We observe that,

Observation 1: SDL can provide performance gain of 6 to 10% with the additional downlink subframe, and the performance gain is mainly focused on the 50% and 95% UPT UEs.

Observation 2: SDL is more suitable for hot spots scenarios with high traffic arrival rate.

## References

- RP-142248, ‘Study on possible additional configuration for LTE TDD’, CATT, CATR, NTT DOCOMO, RAN#66.

- RP-150485, ‘Text proposal for Section 5 and 6 of TR36.825’, NEC.

- TR 36.825 V0.1.1 ‘Feasibility study on possible additional configuration for LTE TDD’, March 2015.

- ‘Email discussion summary on simulation assumptions for FS_LTE_TDD_add_config: Co-existence and performance set 2’, CATT, 3GPP RAN email reflector, 30-April-2015.

- RP-150140, ‘Considerations of introducing possible additional TDD configuration(s)’, NEC, RAN#67.

- TR 36.872 V12.1.0 ‘Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects’, December 2013.

## Appendix

Table 2 Simulation assumption

| Scenario | Small cell 2a [6] |
| --- | --- |
| Duplex Mode | Macro as Pcell: FDD; |
|  | Small cell as Scell: TDD |
| Traffic model | FTP model 1 |
| Bandwidth | 10MHz |
| Pcell central frequency | 2GHz |
| Scell central frequency | 3.5GHz |
| Scell configuration | Baseline: TDD UL-DL configuration #5 (8:1:1) with special subframe configuration #4 (12 downlink symbols) |
|  | SDL: New TDD UL-DL configuration (10:0:0) |
| Wrapping method | Geographical distance based |
| Channel estimation | Ideal |
| Transmission mode | TM2 |
| Feedback | PUCCH 1-0 |
| Number of BS transmit antennas | 2 |
| Number of UE receive antennas | 2 |
| Ratio of CA capable UEs | 1:0 |
| Others | [3][4][6] |

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"