3GPP TSG-RAN WG2 Meeting #90 	R2-152907

# Fukuoka, Japan, 25 – 29 May 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 1846 | rev | 1 | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IDC signalling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SPIA_IDC_LTE-Core, TEI12 |  |  |  |  |  |  | Date: |  | 2015-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Stage-3 specification for IDC signalling is not aligned with the intended behaviour of allowing UE to signal possible IDC problems on non-serving frequencies as noted in the stage-2 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sections concerning initiation of IDC indications are corrected to enable UE to signal IDC indications on non-serving cell frequencies for which measObjectEUTRA is configured when the UE is expecting IDC problems that it cannot solve by itself if that frequency becomes a serving cell frequency |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is unable to report possible IDC issues on non-Serving frequencies until these frequencies become serving frequencies. This may result in inclreased signalling and/or dropped calls if problematic frequency is configured as serving cell. Impact analysis Impacted functionality: Initiation of IDC indications from the UE Inter-operability: - If the network is implemented according to the CR and the UE is not, the network will only receive the IDC indications after the non-serving frequency becomes a serving frequency and hence won’t benefit from the proposed change - If the UE is implemented according to the CR and network is not, then network will receive IDC indications on non-Serving frequencies. Since network reaction to the IDC indications is not specified, no inter-operability issues are foreseen in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.6.9.2	Initiation

A UE capable of providing IDC indications may initiate the procedure when it is configured to provide IDC indications and upon change of IDC problem information.

Upon initiating the procedure, the UE shall:

1>	if configured to provide IDC indications:

2>	if the UE did not transmit an InDeviceCoexIndication message since it was configured to provide IDC indications:

3>	if on one or more frequencies for which a measObjectEUTRA is configured, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	else:

3>	if the set of frequencies, for which a measObjectEUTRA is configured and on which the UE is experiencing IDC problems that it cannot solve by itself, is different from the set indicated in the last transmitted InDeviceCoexIndication message; or

3>	if for one or more of the frequencies in the previously reported set of frequencies, the interferenceDirection is different from the value indicated in the last transmitted InDeviceCoexIndication message; or

3>	if the TDM assistance information is different from the assistance information included in the last transmitted InDeviceCoexIndication message:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

NOTE  1:	The term “IDC problems” refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected.

NOTE  2:	For the frequencies on which a serving cell is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds. 
For frequencies on which a SCell is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell would result in interference issues that the UE would not be able to solve by itself. 
For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency became a serving frequency then this would result in interference issues that the UE would not be able to solve by itself.