3GPP TSG-RAN WG2 Meeting #91 	R2-153874

# Beijing, China, 24 – 28 August 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | 5786 | rev |  | Current version: | 12.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Beacon RSSI Encoding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incoporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_LTE_WLAN_interw-Core |  |  |  |  |  |  | Date: |  | 2015-08-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The thresholds for Beacon RSSI are signaled for RAN-assisted WLAN interworking. However, there is a discrepancy between how the Beacon RSSI is defined by IEEE (and refered in 25.215, 25.225, and 25.304) and the RRC signaled thresholds. The Beacon RSSI is defined in IEEE P802.11-REVmcTM/D4.0 Section 10.45 where it is stated that “The Beacon RSSI is reported in dBm”. On the other hand, the thresholds for BeaconRSSI are signaled as integers in (0…255) in ASN.1. Therefore, the interpretation of this encoding and corresdonce to dBm scale should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In WLAN Offload Configuration, the descriptions of Beacon RSSI thresholds are updated by mapping the signaled integer value to the dBm value used in RAN rules for WLAN interworking. Since the practical values of Beacon RSSI are in the negative range, the dBm value is determined by subtracting 128 from the integer value. Impact Analysis: Impacted functionality: - Since the change impacts the interpretation of the IE, the RNC should take this into account in signaling the threshold according to the proposed derivation. - The UE will need to subtract 128 from the signaled value to set the beacon RSSI thresholds to be used in RAN rules. - The proposed change does not change ASN.1 by not modifying the signaled values. Inter-operability: - If the UE implements the threshold setting according to the CR but the NW does not, then the used threshold will be 128 dBm lower than the intended threshold by the NW. - If the NW implements the threshold setting according to the CR but the UE does not, then the used threshold will be 128 dBm higher than the intended threshold by the NW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unit of the Beacon RSSI thresholds for RAN-assisted WLAN interworking will not be well defined. If the signaled integer values of (0…255) are interpreted as dBm, they will not be very useful for traffic steering to WLAN since measured values will always be less than the thresholds. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.9b.1, 11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 10.3.9b	WLAN Information elements

#### 10.3.9b.1	WLAN Offload Configuration

This IE indicates information for RAN-assisted WLAN interworking.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Threshold Serving RSCP | OP |  |  |  | REL-12 |
| >ThreshservingOffloadWLAN, low | MP |  | Integer (-119..-25 by step of 2) | RSCP, dBm | REL-12 |
| >ThreshservingOffloadWLAN, high | MP |  | Integer (-119..-25 by step of 2) | RSCP, dBm | REL-12 |
| Threshold Serving Ec/N0 | OP |  |  |  | REL-12 |
| >ThreshservingOffloadWLAN, low2 | MP |  | Integer (-24..0) | Ec/N0, dB | REL-12 |
| >ThreshservingOffloadWLAN, high2 | MP |  | Integer (-24..0) | Ec/N0, dB | REL-12 |
| Threshold Channel Utilization | OP |  |  |  | REL-12 |
| >ThreshchUtilWLAN, low | MP |  | Integer (0..255) |  | REL-12 |
| >ThreshchUtilWLAN, high | MP |  | Integer (0..255) |  | REL-12 |
| Threshold Backhaul Bandwidth | OP |  |  |  | REL-12 |
| >ThreshbackhRateDLWLAN, low | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| >ThreshbackhRateDLWLAN, high | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| >ThreshbackhRateULWLAN, low | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| >ThreshbackhRateULWLAN, high | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| Threshold Beacon RSSI | OP |  |  |  | REL-12 |
| >ThreshBeaconRSSIWLAN, low | MP |  | Integer (0..255) | Actual value = IE value – 128 in dBm. | REL-12 |
| >ThreshBeaconRSSIWLAN, high | MP |  | Integer (0..255) | Actual value = IE value – 128 in dBm. | REL-12 |
| Offload Preference Indicator | OP |  | Bitstring (16) |  | REL-12 |
| TsteeringWLAN | OP |  | Integer (0..31) | Value in seconds | REL-12 |

<Text skipped>

## 11.3	Information element definitions

<Text skipped>

-- ***************************************************

--

--     WLAN Offload INFORMATION ELEMENTS (10.3.9b)

--

-- ***************************************************

WLANOffloadConfig	::=		SEQUENCE {

threshServingRSCP 				WLANThreshServingRSCP					OPTIONAL,

threshServingECNO 				WLANThreshServingECNO					OPTIONAL,

threshChannelUtilization		WLANThreshChannelUtilization			OPTIONAL,

threshBackhaulBandwidth			WLANThreshBackhaulBandwidth				OPTIONAL,

threshBeaconRSSI				WLANThreshBeaconRSSI					OPTIONAL,

offloadPreferenceIndicator		BIT STRING (SIZE (16))					OPTIONAL,

t-SteeringWLAN					T-Reselection-S							OPTIONAL

}

WLANThreshServingRSCP	::=	SEQUENCE {

-- Actual value = IE value * 2 + 1

threshServingLow				INTEGER (-60..-13),

-- Actual value = IE value * 2 + 1

threshServingHigh				INTEGER (-60..-13)

}

WLANThreshServingECNO	::=	SEQUENCE {

threshServingLow2				INTEGER (-24..0),

threshServingHigh2 				INTEGER (-24..0)

}

WLANThreshChannelUtilization	::=	SEQUENCE {

threshChannelUtilizationLow			INTEGER (0..255),

threshChannelUtilizationHigh		INTEGER (0..255)

}

WLANThreshBackhaulBandwidth	::=		SEQUENCE {

threshBackhaulDLBandwidthLow		WLANThreshBackhaulRate,

threshBackhaulDLBandwidthHigh 		WLANThreshBackhaulRate,

threshBackhaulULBandwidthLow		WLANThreshBackhaulRate,

threshBackhaulULBandwidthHigh		WLANThreshBackhaulRate

}

WLANThreshBeaconRSSI 		::=		SEQUENCE {

-- Actual value = IE value - 128

threshBeaconRSSILow					INTEGER (0..255),

-- Actual value = IE value - 128

threshBeaconRSSIHigh				INTEGER (0..255)

}