3GPP TR 36.825 V0.3.0 (2015-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Feasibility study on possible additional configuration
for LTE TDD

(Release 13)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<LTE>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Deployment scenarios of possible additional TDD configuration(s)	7

4.1	Scenario(s)	7

4.2	Use case(s)	7

5	Coexistence study with adjacent standalone TDD operations	8

5.1	Scenarios	8

5.2	Inter-operator coexistence aspects	8

5.3	Coexistence study results	9

5.3.1	 Deterministic analysis	9

5.3.2	 Monte Carlo simulation	11

5.4	Unsynchronized operation	16

6	Evaluation of possible additional TDD configuration(s)	16

6.1	Scenarios	16

6.2	Intra-operator synchronization aspects	18

6.3	Evaluation results of Performance Evaluation Set 1	18

6.4	Summary of performance set 2 evaluations	19

6.4.1	Scenario 1	19

6.4.2	Scenario 2	23

6.4.3	Observations from performance set 2	25

6.5	Possible solutions to mitigate potential drawbacks of the additional TDD configuration(s)	28

6.5.1	Co-existence	28

6.5.1.1	UE/eNB sensing	28

6.5.1.2	Antenna downtilt	29

7	Ecosystem aspects	30

7.1	Terminal eco-system considerations	30

8	Summary	30

Annex A: Simulation assumptions for co-existence and performance set 2	31

A.1	ACIR	31

A.2	Propagation model	31

A.3	UE parameters	32

A.4	Macro parameters	32

A.5	Pico parameters	33

A.6	Shadowing correlation parameters	34

A.7	Other parameters	34

A.8	Peformance metrics	36

Annex X: Change history	37

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

Seven TDD UL/DL configurations have been supported in LTE TDD since Rel-8 to address different DL and UL traffic ratios. The most DL heavy TDD UL/DL configuration in current LTE TDD is configuration 5, where 8 DL subframes and one DwPTS in a special subframe within a radio frame (i.e. 10 subframes) can be used for DL transmission. There has been a proposal to introduce additional configuration into LTE TDD, aiming to provide even more resources for DL transmission.

There has not been any consensus on whether additional TDD configuration shall be introduced for LTE TDD. To gain a better understanding on the issues related to the potential introduction of additional configuration for LTE TDD, this study item is proposed. This study item shall not provide any recommendation on any follow-up normative work. Upon completion of this study item, the follow-up actions shall be discussed on the PCG level.

TDD operation in unlicensed spectrum is out of the scope of this study.

1	Scope

The objective of the study is to evaluate issues related to the potential introduction of the following additional configuration(s) for LTE TDD; 10:0:0 and 9:1:0 (DL:Sp:UL). This includes:

-	Identify the scenario(s), frequency band(s), and use cases(s) of the possible additional TDD configuration(s)

-	Coexistence with adjacent standalone LTE TDD operations in the same band

-	To be compared with the coexistence between intra-band adjacent LTE TDD operations using different UL/DL configurations

-	Evaluation of the potential benefits and drawbacks of the additional TDD configuration(s) and possible solutions to mitigate the potential drawbacks

-	Any other relevant ecosystem aspect(s)

-	Compare the option of using 10:0:0 and/or 9:1:0 TDD configuration for a TDD spectrum against the alternative of converting the TDD spectrum to FDD supplemental DL spectrum

NOTE:	The study shall not provide any recommendation on next steps / future normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-142248, "Study on possible additional configuration for LTE TDD", CATT, CATR, NTT DOCOMO, RAN #66, Dec.14

[3]	RP-150950, “Coexistence evaluation results for intra-band adjacent LTE TDD operations using different TDD UL/DL configurations”, NEC, RAN#68, Jun.15

[4]	RP-150693, “Coexistence Simulation Results with Adjacent Standalone TDD Operations”, Ericsson, RAN#68, Jun.15

[5]	RP-150699, “Co-existence evaluations for possible additional TDD configuration(s)”, Huawei, HiSilicon, RAN#68, Jun.15

[6]	RP-150978, “Coexistence simulation results for additional LTE TDD configuration”, ZTE, RAN#68, Jun.15

[7]	RP-150927, “Co-existence evaluation results for FS_LTE_TDD_add_config”, CATT, RAN#68, Jun.15

[8]	RP-150742, “Deterministic analysis for additional LTE TDD configuration”, ZTE, RAN#68, Jun.15

[9]	RP-150776, “Simulation results on co-existence evaluation for possible additional TDD configuration”, LG Electronics, RAN#68, Jun.15

[10]		3GPP TS 36.300: “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

[11]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[12]	3GPP TR 36.872: “Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects”

[13]	3GPP TS23.122: “Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

Acceptable interference level

Distance between two equipments

Path loss

Path loss for line-of-sight

Path loss for non line-of-sight

Signal to interference ratio

Scaling factor for PL given by high layer

Angle between eNB and UE in horizontal dimension

User arrival rate

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

TDD	Time Division Duplex

FDD	Frequency Division Duplex

SCell	Secondary Cell

PCell	Primary Cell

CA	Carrier Aggregation

CDF	Cumulative Distribution Function

RU	Resource Utilization

TU	Typical Urban

PSS	Primary Synchronization Signal

SSS	Secondary Synchronization Signal

Avg	Average

ITU	International Telecommunication Union

UL	Uplink

DL	Downlink

BS	Base Station

ACIR	Adjacent Channel Interference Ratio

UE	User Equipment

MUE	Macro User Equipment

ACLR	Adjacent Channel Leakage Power Ratio

FTP	File Transfer Protocol

(F)eICIC	Further enhanced Inter-Cell Interference Coordination

NAICS	Network-Assisted Interference Cancellation and Suppression

MBSFN	Multimedia Broadcast Multicast service Single Frequency Network

RSRQ	Reference Signal Receiving Quality

CRS	Cell-specific Reference Signal

PDSCH	Physical Downlink Shared Channel

CSI	Channel State Information

CP	Cyclic Prefix

HARQ	Hybrid Automatic Repeat Request

PDCCH	Physical Downlink Control Channel

PUCCH	Physical Uplink Control Channel

OFDM	Orthogonal Frequency Division Multiplex

4	Deployment scenarios of possible additional TDD configuration(s)

4.1	Scenario(s)

The following scenarios are of interests for operators who want to deploy additional TDD configuration(s). In all of the following cases, the SCell is operating the additional TDD configuration(s), and the PCell is operating FDD or TDD using the existing TDD configuration(s).

-	Scenario 1a: Operator deploys macro cell operating as PCell on carrier frequency f0 and macro cell operating as SCell on carrier frequency f1 (co-located), e.g. CA scenario 2 or 3 in TS36.300 v12.4.0. The macro cell on f1 uses the additional TDD configuration(s). Both of the two carriers can be aggregated by UE.

-	Scenario 1b: Operator deploys macro cell operating as PCell on carrier frequency f0 and macro cell as SCell on carrier frequency f1 (non-collocated). Both of the two carriers can be aggregated by UE.

-	Scenario 2: Operator deploys macro cell operating as PCell on carrier frequency f0 and small cell operating as SCell on carrier frequency f1 (non-co-located), e.g., CA scenario 4 in TS36.300 v12.4.0. Both of the two carriers can be aggregated by UE.

-	Scenario 3: Operator deploys small cell operating as PCell on carrier frequency f0 and small cell as Scell on carrier frequency f1 (co-located). Both of the two carriers can be aggregated by UE.

Based on the current specifications, inter-band carrier aggregation would be a pre-requisite for a UE to receive data on the SCell operating the additional TDD configuration(s).

4.2	Use case(s)

5	Coexistence study with adjacent standalone TDD operations

5.1	Scenarios

The following scenarios are selected to evaluate coexistence on intra-band adjacent LTE TDD operations using different TDD UL/DL configurations.

A.	Scenarios:

i.	Scenario 1:

1.	Operator_A: small cell (outdoor pico)

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode: New TDD UL/DL configuration (10:0:0)

2.	Operator_B: Macro cell

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode: TDD UL/DL configuration 2 with special configuration 4

ii.	Scenario 2:

1.	Operator_A: Macro cell

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode: New TDD UL/DL configuration (10:0:0)

2.	Operator_B: Macro cell

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode: TDD UL/DL configuration 2 with special configuration 4

B.	Evaluation methodology: Deterministic and Monte Carlo simulation to evaluate impact to both Operator_A and Operator_B performance for both scenarios.

C.	Traffic model: Full buffer traffic

Note: 2.7GHz was chosen based on average of all TDD frequency bands of interest. For the purpose of this TR, results for this carrier frequency are deemed to be applicable to all TDD bands.

5.2	Inter-operator coexistence aspects

Co-existence issues between operators on adjacent carriers in the same geographical area are not expected at least for the following cases.

-	All operators on adjacent carriers use the same TDD UL-DL configurations and the radio frame and subframe are time synchronized between the operators.

-	All operators on adjacent carriers only use the downlink subframes and/or DwPTS, regardless of whether or not the radio frame and subframe are synchronized among the operators. Under this assumption, the following options for the TDD UL-DL configuration are considered for the operators:

-	10:0:0 TDD UL-DL configuration

-	Existing TDD UL-DL configuration or 9:1:0 TDD UL-DL configuration without any uplink transmission

-	Guard band is used between adjacent carriers of different operators.

NOTE:	Usage of guard band is detrimental to spectrum efficiency.

5.3	Coexistence study results

5.3.1		Deterministic analysis

The deterministic analysis calculates the minimum required separation distance between the aggressor eNB and the victim eNB.

A tight requirement is that the acceptable eNB-to-eNB interference level is 7dB below the thermal noise floor, i.e.

Note that the victim macro eNB noise figure is 5dB [Annex A].

A relax requirement is that the acceptable eNB-to-eNB interference level is set according to the dynamic range requirement. For system bandwidth of 10MHz, the acceptable eNB-to-eNB interference level is -79.5dBm with victim macro eNB.

Scenario 1: Outdoor Pico to Macro cell adjacent channel case:

Table 5.3.1-1: Deterministic analysis result of Scenario 1 [3]

| Operator A BS transmission power | 24 dBm |
| --- | --- |
| Operator A BS antenna gain | 5 dBi |
| Operator B BS antenna gain | 15 dBi |
| ACIR BS to BS | 43 dB |
| Operator B Received Interference | 24+5+15-PL-43 = (1-PL) dBm |
| Operator B BS acceptable interference | -106.5 dBm |
| PLLOS (R) =100.7+23.5log10(R)+2.6 PLNLOS(R) = 125.2+36.3log10(R)+2.6; R in km |  |
| Minimum separation distance (tight requirement) | 1509m for LOS 276m for NLOS |

Table 5.3.1-2: Required minimum separation distance (km) for macro to pico BS [5]

| Interference mechanism (Adjacent channel) | Pathloss - LOS |  | Pathloss - NLOS |  |
| --- | --- | --- | --- | --- |
|  | Tight requirement | Relaxed requirement | Tight requirement | Relaxed requirement |
| Pico->Macro | 1.502 | 0.107 | 0.275 | 0.050 |

Table 5.3.1-3: minimum required separation distance between aggressor and victim eNBs [7]

| Scenario | Agg. eNB Tx Power (dBm) | Victim eNB acceptable interference (dBm) @10MHz | Pathloss model (dB), R in km |  | (Agg., Victim) eNB antenna gain (dBi) | ACIR (dB) | Min required distance (km) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | Tight requirement: -106.46 | LOS | PLLOS(R) = 100.7+23.5log10(R)+2.6 | (5, 15) | 43 | 1.503 |
|  |  |  | NLOS | PLNLOS(R) = 125.2+36.3log10(R)+2.6 |  |  | 0.275 |
|  |  | Relaxed requirement: -79.5 | LOS | PLLOS(R) = 100.7+23.5log10(R)+2.6 |  |  | 0.107 |
|  |  |  | NLOS | PLNLOS(R) = 125.2+36.3log10(R)+2.6 |  |  | 0.050 |

Table 5.3.1-4: required minimum separation distance for Macro/outdoor pico [8]

| aggressor -> victim | used path loss model | minimum separation distance R (km) |
| --- | --- | --- |
| outdoor pico->Macro | PLLOS (R) =100.7+23.5log10(R)+2.6 | 1.51 |
|  | PLNLOS(R) = 125.2+36.3log10(R)+2.6 | 0.28 |

Table 5.3.1-5: Separation (km) analysis [9]

| Requirement | LOS | Separation |
| --- | --- | --- |
| Tight | Pathloss - LOS | 1.502 |
| Tight | Pathloss - NLOS | 0.275 |
| Relaxed | Pathloss - LOS | 0.107 |
| Relaxed | Pathloss - NLOS | 0.050 |

Scenario 2: Macro to Macro cell adjacent channel case:

Table 5.3.1-6: Deterministic analysis result of Scenario 2 [3]

| Operator A BS transmission power | 46 dBm |
| --- | --- |
| Operator A BS antenna gain | 15 dBi |
| Operator B BS antenna gain | 15 dBi |
| ACIR BS to BS | 43 dB |
| Operator B Received Interference | 46+15+15-PL-43 = (33-PL) dBm |
| Operator B BS acceptable interference | -106.5 dBm |
| PLLOS (R) =98.45+20log10(R)+2.6 ,R in km |  |
| Minimum separation distance (tight requirement) | 83.66km |

Table 5.3.1-7: Required minimum separation distance (km) for macro to macro BS [5]

| Interference mechanism ( Adjacent channel) | Pathloss - LOS |  |
| --- | --- | --- |
|  | Tight requirement | Relaxed requirement |
| Macro->Macro | 83.185 | 3.734 |

Table 5.3.1-8: minimum required separation distance between aggressor and victim eNBs [7]

| Scenario | Agg. eNB Tx Power (dBm) | Victim eNB acceptable interference (dBm) @10MHz | Pathloss model (dB), R in km | (Agg., Victim) eNB antenna gain (dBi) | ACIR (dB) | Min required distance (km) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 46 | Tight requirement: -106.46 | PL=98.45+20*log10(R)+2.6 | (15, 15) | 43 | 83.272 |
|  |  | Relaxed requirement: -79.5 |  |  |  | 3.737 |

Table 5.3.1-9: required minimum separation distance for Macro/Macro [8]

| aggressor -> victim | used path loss model | Minimum separation distance R (km) |
| --- | --- | --- |
| Macro->Macro | PL(R)=98.45+20*log10(R)+2.6 | 83.66 |

Observations:

- In Scenario 1 (pico-Macro), the minimum BS site separation distance under the relaxed requirement is about 50 meters and 107 meters in the case of NLOS and LOS, respectively.

- In Scenario 1 (pico-Macro), the minimum BS site separation distance under the tight requirement is about 275 meters and 1500 meters in the case of NLOS and LOS, respectively.

- In Scenario 2 (Macro-Macro), the minimum BS site separation distance is about 3.8 km and 83.5 km under the relaxed and tight requirements, respectively.

5.3.2		Monte Carlo simulation

For coexistence evaluation, the following metrics are provided. The details of simulation assumptions are listed in Annex A.

CDF of Operator A’s DL geometry, assuming Operator B is performing DL transmission

CDF of Operator A’s DL geometry, assuming Operator B is performing UL transmission

CDF of Operator B’s UL geometry, assuming Operator A is performing UL transmission

CDF of Operator B’s UL geometry, assuming Operator A is performing DL transmission

Scenario 1: Outdoor Pico to Macro cell adjacent channel case:

Figure 5.3.2-1. Downlink geometry of Operator A [3]	Figure 5.3.2-2. Uplink geometry of Operator B [3]

Figure 5.3.2-3. Scenario 1 – downlink SINR [4]	Figure 5.3.2-4. Scenario 1 – uplink SINR [4]

Figure 5.3.2-5. Pico downlink geometry [5]	Figure 5.3.2-6. Macro uplink geometry [5]

Table 5.3.2-1: Macro UL throughput loss for macro-pico scenario [5]

| UL/DL configuration | Macro UL average throughput loss | Macro UL edge throughput loss |
| --- | --- | --- |
| Macro UL, Pico DL (Baseline: Macro UL, Pico UL) | -51.18% | -100% |

Figure 5.3.2-7. Pico downlink geometry [6]	Figure 5.3.2-8. Macro uplink geometry [6]

Figure 5.3.2-9. Victim (Operator A) DL geometry [7]	Figure 5.3.2-10. Victim (Operator B) UL geometry [7]

Table 5.3.2-2: Operator B’s UL cell average and cell edge throughput loss compared to the baseline [7]

| Scenario | Victim cell average UL throughput loss | Victim cell edge (5%) UL throughput loss |
| --- | --- | --- |
| 1 | -51% | -100% |

Figure 5.3.2-11. Operator A’s DL geometry [9]	Figure 5.3.2-12. Operator B’s UL geometry [9]

Scenario 2: Macro to Macro cell adjacent channel case:

Figure 5.3.2-13. Downlink geometry of Operator A [3]	Figure 5.3.2-14. Uplink geometry of Operator B [3]

Figure 5.3.2-15. Scenario 2 – downlink SINR [4]	Figure 5.3.2-16. Scenario 2 – uplink SINR [4]

Figure 5.3.2-17. Macro downlink geometry [6]	Figure 5.3.2-18. Macro uplink geometry [6]

Figure 5.3.2-19. Victim (Operator A) DL geometry [7]	Figure 5.3.2-20. Victim (Operator B) UL geometry [7]

Table 5.3.2-3: Operator B’s UL cell average and cell edge throughput loss compared to the baseline [7]

| Scenario | Victim cell average UL throughput loss | Victim cell edge (5%) UL throughput loss |
| --- | --- | --- |
| 2 | -100% | -100% |

Observations for Scenario 1:

- The DL geometry of the pico network improves by about 3-4 dB if the Macro network is using UL, compared to the baseline in which the Macro network is using DL.

- The Macro network sees about 5 dB degradation in the UL geometry when the pico network is using DL.

Observations for Scenario 2:

- The DL geometry of Macro UEs of Operator A is almost the same irrespective of the Macro network of Operator B is using UL or DL.

- The UL geometry of Macro UEs of Operator B is significantly decreased if the Macro network of Operator A is using DL, compared to the baseline in which the Macro network of Operator A is using UL.

Conclusion:

- From the study results, it is observed there is a coexistence issue of deploying the new TDD UL/DL configuration 10:0:0/9:1:0 in a channel that is adjacent to another channel that uses different TDD UL/DL configuration without adopting any interference mitigation scheme.

-	In the evaluated scenarios, the observed BS-to-BS interference from pico cell in the Macro-pico adjacent channel scenario is less than the BS-to-BS interference from Macro cell in the Macro-Macro adjacent channel scenario.

5.4	Unsynchronized operation

The below is excerpted from 6.6.3.3.16 in TS 36.101.

--------------------------------------------------------Start of 6.6.3.3.16------------------------------------------------------------------

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")

When "NS 22" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.16-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.16-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 3400 ≤ f ≤ 3800 | -23 (Note 1, Note 3) | 5 MHz |
|  | -40 (Note 2) | 1 MHz |
| NOTE 1: This requirement applies within an offset between 5 MHz and 25 MHz from the lower and from the upper edge of the channel bandwidth. NOTE 2: This requirement applies from 3400 MHz up to 25 MHz below the lower E-UTRA channel edge and from 25 MHz above the upper E-UTRA channel edge up to 3800 MHz. NOTE 3: This emission limit might imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |

--------------------------------------------------------End of 6.6.3.3.16------------------------------------------------------------------

For Band 42 and Band 43, the current 3GPP specification of TS 36.101 says that when UEs supporting Band 42 and/or Band 43 receives the NS_22 from the eNB, they shall meet the above requirement with 5 MHz offset from the both edges of a channel bandwidth and general spectrum emission mask and UTRA ACLR 1 within the adjacent 5 MHz region. In addition, it is allowed for the UEs to meet the requirement to utilize up to 5 dB power back off (A-MPR) according to Table 6.2.4-17 specified in TS 36.101. Note that the requirement has not included intra band contiguous CA case so far.

The above requirement was introduced to address UE-UE co-existence for Band 42&43 under the network where adjacent operators networks are not synchronized and/or do not use the same UL/DL TD-LTE configuration each other.

From the above, time synchronization and/or the same UL/DL configuration usage between operators are essential for TD-LTE operation.

6	Evaluation of possible additional TDD configuration(s)

6.1	Scenarios

Performance Evaluation Set 1

The purpose for this performance study is to evaluate throughput gain from using the possible new LTE TDD configurations (10:0:0 and 9:1:0) in a scenario where no adjacent channel interference is present (e.g. operators sharing the same frequency band use same TDD UL/DL configuration).

A.	Scenario:

i.	Deployment scenario: SCE scenario 2a (from TR36.872)

ii.	Throughput measurement: Downlink packet throughput

iii.	PCell:

1.	Carrier frequency: 2.0GHz

2.	Channel bandwidth: 10MHz

3.	Duplex mode: FDD

iv.	SCell:

1.	Carrier frequency: 3.5GHz

2.	Channel bandwidth: 10MHz

3.	Case 1: TDD UL/DL configuration #5 (8:1:1) with special configuration 4 as baseline performance

4.	Case 2: New TDD UL/DL configuration (10:0:0)

B.	Traffic model: High traffic load case (FTP traffic model)

C.	Channel reciprocity and channel sounding error modelling are not applicable to Case 2.

D.	Ratio of CA and non-CA capable UEs: 1:1 and 1:0

E.	Performance metric:

i.	5%, 50% and 95%-tile of packet throughput (aggregated between PCell and SCell)

Note that, the performance gains from using the possible new TDD configurations 10:0:0 and 9:1:0 are expected to be similar.

Performance Evaluation Set 2

The following scenarios are selected to evaluate throughput impact from using the possible new LTE TDD configurations (10:0:0 and 9:1:0) in a scenario with intra-band adjacent LTE TDD operations using different TDD UL/DL configurations.

A.	Scenarios:

i.	Scenario 1:

1.	Operator_A: SCell: small cell (outdoor pico) (PCell is a standalone cell on another frequency band)

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode:

-	Case 1: TDD UL/DL configuration 2 with special configuration 4

-	Case 2: TDD UL/DL configuration 5 with special configuration 4

-	Case 3: New TDD UL/DL configurations (10:0:0)

2.	Operator_B: Standalone Macro cell

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode: TDD UL/DL configuration 2 with special configuration 4

ii.	Scenario 2:

1.	Operator_A: SCell: Macro cell (PCell is a standalone cell on another frequency band)

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode:

-	Case 1: TDD UL/DL configuration 2 with special configuration 4

-	Case 2: TDD UL/DL configuration 5 with special configuration 4

-	Case 3: New TDD UL/DL configurations (10:0:0)

2.	Operator_B: Standalone Macro cell

a.	Carrier frequency: 2.7GHz

b.	Channel bandwidth: 10MHz

c.	Duplex mode: TDD UL/DL configuration 2 with special configuration 4

B.	Evaluation methodology: System level simulation to evaluate the performance of Operator_A (in DL only) and Operator_B (in UL only) for both scenarios.

C.	Traffic model: Non-full buffer model (High traffic load)

D.	PCell is not modelled in the simulations.

Note that, the performance impact from using the possible new TDD configurations 10:0:0 and 9:1:0 is expected to be similar.

6.2	Intra-operator synchronization aspects

For both the existing and new TDD UL-DL configurations, for an operator deploying network on a carrier within a given TDD band, the base-stations of the carrier need to be time synchronized irrespective of whether or not the operator only uses the downlink subframes and DwPTS for downlink traffic on the carrier, i.e., uplink transmission does not occur. If time synchronisation within a single operator is not achieved, the current Cell phase synchronization accuracy requriements are not met. Furthermore, the benefits of advanced feature such as (F)eICIC, NAICS, MBSFN, small cell on/off with discovery cannot be achieved.

6.3	Evaluation results of Performance Evaluation Set 1

The UE throughput gains of a new TDD UL/DL configuration of 10:0 (Case 2) over TDD UL/DL configuration 5 (Case 1) are summarized in Table 6.3-1 and 6.3-2 for 4 and 10 small cells per macro cell, respectively.

Table 6.3-1: UE throughput gains of new TDD UL/DL configuration of 10:0 over TDD UL/DL configuration 5 for 4 small cells per macro cell

| Source | UE throughput gains (%) |  |  |  | Ratio of CA UE and non-CA UEs | Packet arrival rate (files/s/macro) | Packet drop ratio (%) |  | Resource utilizations for macro/small cells (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | Mean |  |  | Case 1 | Case 2 | Case 1 | Case 2 |
| 1 (RP-150607) | 3.70 | 6.87 | 7.33 | 7.30 | 1:0 | 2 | - | - | 65.88/10.59 | 64.28/9.67 |
|  | 3.17 | 7.00 | 8.75 | 7.87 | 1:0 | 3 | - | - | 84.99/19.83 | 84.27/18.27 |
|  | 1.25 | 6.06 | 9.62 | 8.09 | 1:0 | 4 | - | - | 93.80/30.19 | 93.54/29.18 |
| 2 (RP-150952) | -77.28 | -60.87 | 25.58 | - | 1:1 | 1.6 | 0.28 | 12.18 | 68.18/42.69 | 97.75/14.42 |
|  | -43.22 | -22.22 | 41.86 | - | 1:1 | 2.4 | 1.55 | 19.54 | 88.18/63.61 | 99.22/24.12 |
|  | -3.94 | 14.39 | 16.53 | - | 1:0 | 1.8 | - | - | 94.81/27.30 | 96.27/26.04 |
| 3 (RP-151526) | 10.67 | 8.31 | 12.67 | 8.85 | 1:0 | 1 | 2.91 | 3.33 | 50.57/20.83 | 58.80/18.07 |
|  | 12.43 | 16.25 | 13.14 | 13.57 | 1:0 | 1.2 | 4.40 | 5.39 | 70.87/31.52 | 77.33/26.3 |
|  | -85.82 | -78.52 | 12.67 | -25.88 | 1:1 | 1 | 3.18 | 39.80 | 37.14/23.31 | 97.67/6.77 |

Table 6.3-2: UE throughput gains of new TDD UL/DL configuration of 10:0 over TDD UL/DL configuration 5 for 10 small cells per macro cell

| Source | UE throughput gains (%) |  |  |  | Ratio of CA UE and non-CA UEs | Packet arrival rate (files/s/macro) | Packet drop ratio (%) |  | Resource utilizations for macro/small cells (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5% | 50% | 95% | Mean |  |  | Case 1 | Case 2 | Case 1 | Case 2 |
| 4 (RP-150986) | 58.65 | 42.94 | 17.27 | - | 1:0 | 13 | - | - | 53.78/66.97 | 52.14/56.50 |

Note:

- Offset values for RSRQ-based cell association between macro and small cell layers

- 4 dB: source 4

- 0 dB: sources 1, 2, 3

- Interference from CRS

- Modelled: sources 2, 4

- Not modelled: sources 1, 3.

- Mobility aspects are not considered for all the evaluations

It is observed from evaluation results that

- For 4 small cells per macro cell

- One source shows that, when the ratio of CA and non-CA UEs is 1:1 and two cells are available for an operator, a new TDD UL/DL configuration of 10:0 exhibits 50% UE throughput performance losses of -60.87% to -22.22% and increased packet drop ratio over TDD UL/DL configuration 5.

- The performance losses may be alleviated if more than two cells are available for an operator.

- Three sources show that, when the ratio of CA and non-CA UEs is 1:0, a new TDD UL/DL configuration of 10:0 provides 50% UE throughput gains of 6.06% to 16.25% over TDD UL/DL configuration 5.

- For 10 small cells per macro cell

- One source shows that, when the ratio of CA and non-CA UEs is 1:0, a new TDD UL/DL configuration of 10:0 provides 50% UE throughput gains of 42.94% over TDD UL/DL configuration 5.

6.4	Summary of performance set 2 evaluations

6.4.1	Scenario 1

Table 6.4.1-1: Evaluation results from source 1 (RP-150608)

|  |  |  | Operator A (DL, Mbps) |  |  |  |  |  | Operator B(UL, Mbps) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| λDL | λUL |  | Avg. | 5% | 50% | 95% | RU | Packet drop ratio | Avg. | 5% | 50% | 95% | RU | Packet drop ratio |
| 1 | 0.25 | Case 1 | 24.83 (0.00%) | 13.25 (0.00%) | 27.97 (0.00%) | 28.17 (0.00%) | 16% | 0% | 4.49 (0.00%) | 1.97 (0.00%) | 4.46 (0.00%) | 7.28 (0.00%) | 22% | 0% |
|  |  | Case 2 | 29.01 (+16.83%) | 16.26 (+22.72%) | 32.26 (+15.34%) | 32.52 (+15.44%) | 14% | 0% | 3.84 (-14.48%) | 1.62 (-17.77%) | 3.80 (-14.80%) | 6.47 (-11.13%) | 26% | 0% |
|  |  | Case 3 | 33.01 (+32.94%) | 19.05 (+43.77%) | 36.36 (+30.00%) | 36.36 (+29.07%) | 12% | 0% | 3.38 (-24.72%) | 1.34 (-31.98%) | 3.31 (-25.78%) | 5.78 (-20.60%) | 29% | 0% |
| 2 | 0.25 | Case 1 | 20.83 (0.00%) | 6.36 (0.00%) | 23.67 (0.00%) | 28.17 (0.00%) | 36% | 0% | 4.49 (0.00%) | 1.97 (0.00%) | 4.46 (0.00%) | 7.28 (0.00%) | 22% | 0% |
|  |  | Case 2 | 25.46 (+22.23%) | 9.32 (+46.54%) | 29.41 (+24.25%) | 32.52 (+15.44%) | 30% | 0% | 3.43 (-23.61%) | 1.40 (-28.93%) | 3.37 (-24.44%) | 5.81 (-20.19%) | 29% | 0% |
|  |  | Case 3 | 29.83 (+43.21%) | 12.46 (+95.91%) | 35.71 (+50.87%) | 36.36 (+29.07%) | 26% | 0% | 2.67 (-40.53%) | 0.93 (-52.79%) | 2.54 (-43.05%) | 4.77 (-34.48%) | 36% | 0% |
| 3 | 0.25 | Case 1 | 13.21 (0.00%) | 1.98 (0.00%) | 11.03 (0.00%) | 28.17 (0.00%) | 68% | 1% | 4.49 (0.00%) | 1.97 (0.00%) | 4.46 (0.00%) | 7.28 (0.00%) | 22% | 0% |
|  |  | Case 2 | 19.51 (+47.69%) | 3.20 (+61.62%) | 19.05 (+72.71%) | 32.52 (+15.44%) | 54% | 0% | 2.92 (-34.97%) | 1.09 (-44.67%) | 2.84 (-36.32%) | 5.15 (-29.26%) | 33% | 0% |
|  |  | Case 3 | 24.49 (+85.39%) | 5.38 (+171.72%) | 25.97 (+135.45%) | 36.36 (+29.07%) | 45% | 0% | 2.08 (-53.67%) | 0.58 (-70.56%) | 1.90 (-57.40%) | 4.14 (-43.13%) | 44% | 1% |
| 1 | 0.5 | Case 1 | 24.83 (0.00%) | 13.25 (0.00%) | 27.97 (0.00%) | 28.17 (0.00%) | 16% | 0% | 1.88 (0.00%) | 0.51 (0.00%) | 1.53 (0.00%) | 4.28 (0.00%) | 71% | 2% |
|  |  | Case 2 | 29.09 (+17.16%) | 16.39 (+23.70%) | 32.26 (+15.34%) | 32.52 (+15.44%) | 14% | 0% | 1.61 (-14.36%) | 0.47 (-7.84%) | 1.27 (-16.99%) | 3.91 (-8.64%) | 76% | 4% |
|  |  | Case 3 | 33.00 (+32.90%) | 19.05 (+43.77%) | 36.36 (+30.00%) | 36.36 (+29.07%) | 12% | 0% | 1.45 (-22.87%) | 0.45 (-11.76%) | 1.12 (-26.80%) | 3.59 (-16.12%) | 79% | 5% |
| 2 | 0.5 | Case 1 | 20.83 (0.00%) | 6.36 (0.00%) | 23.67 (0.00%) | 28.17 (0.00%) | 36% | 0% | 1.88 (0.00%) | 0.51 (0.00%) | 1.53 (0.00%) | 4.28 (0.00%) | 71% | 2% |
|  |  | Case 2 | 25.48 (+22.32%) | 9.30 (+46.23%) | 29.41 (+24.25%) | 32.52 (+15.44%) | 30% | 0% | 1.46 (-22.34%) | 0.45 (-11.76%) | 1.13 (-26.14%) | 3.60 (-15.89%) | 79% | 5% |
|  |  | Case 3 | 29.79 (+43.01%) | 12.35 (+94.18%) | 35.71 (+50.87%) | 36.36 (+29.07%) | 26% | 0% | 1.19 (-36.70%) | 0.41 (-19.61%) | 0.87 (-43.14%) | 3.06 (-28.50%) | 84% | 10% |
| 3 | 0.5 | Case 1 | 13.21 (0.00%) | 1.98 (0.00%) | 11.03 (0.00%) | 28.17 (0.00%) | 68% | 1% | 1.88 (0.00%) | 0.51 (0.00%) | 1.53 (0.00%) | 4.28 (0.00%) | 71% | 2% |
|  |  | Case 2 | 19.05 (+44.21%) | 3.40 (+71.72%) | 18.52 (+67.91%) | 32.52 (+15.44%) | 55% | 0% | 1.19 (-36.70%) | 0.41 (-19.61%) | 0.87 (-43.14%) | 3.08 (-28.04%) | 85% | 10% |
|  |  | Case 3 | 24.54 (+85.77%) | 5.53 (+179.29%) | 26.14 (+136.99%) | 36.36 (+29.07%) | 45% | 0% | 0.99 (-47.34%) | 0.38 (-25.49%) | 0.69 (-54.90%) | 2.64 (-38.32%) | 88% | 18% |

Note:

- CRS interference is not modeled when a cell does not transmit PDSCH.

- Dropped packets are not included in the packet throughput statistics.

Table 6.4.1-2: Evaluation results from source 2 (RP-150928)

| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 |
| B UL with packet arrival rate 0.25 packets/s/cell | Mean | 2.108 | 0% | 1.098 | -48% | 1.016 | -52% |
|  | 5% | 1.152 | 0% | 0.622 | -46% | 0.544 | -53% |
|  | 50% | 2.034 | 0% | 1.016 | -50% | 0.978 | -52% |
|  | 95% | 3.294 | 0% | 1.890 | -43% | 1.887 | -43% |
|  | Packet drop ratio | 0.95% | N/A | 19.40% | N/A | 28% | N/A |
|  | RU | 42.83% | N/A | 68.46% | N/A | 72.02% | N/A |
| A DL with packet arrival rate 1.5 packets/s/cell | Mean | 14.125 | 0% | 17.694 | 25% | 20.976 | 49% |
|  | 5% | 1.533 | 0% | 2.298 | 50% | 2.693 | 76% |
|  | 50% | 12.690 | 0% | 16.150 | 27% | 19.910 | 57% |
|  | 95% | 26.570 | 0% | 30.620 | 15% | 34.300 | 29% |
|  | Packet drop ratio | 1.99% | N/A | 1.37% | N/A | 0.98% | N/A |
|  | RU | 51.26% | N/A | 44.33% | N/A | 39.46% | N/A |

Note:

- CRS interference is not modeled when a cell does not transmit PDSCH.

- Dropped packets are not included in the packet throughput statistics.

Table 6.4.1-3: Evaluation results from source 3 (RP-150694)

| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 |
| B UL with packet arrival rate 1 packets/s/cell | Mean |  | 0% |  |  |  |  |
|  | 5% | 0.55 | 0% | 0.5 | -9% | 0.48 | -12.73% |
|  | 50% | 2.05 | 0% | 1.49 | -27.32% | 0.91 | -55.61% |
|  | 95% | 3.54 | 0% | 2.65 | -25.14% | 2.2 | -37.85% |
|  | Packet drop ratio |  | N/A |  | N/A |  | N/A |
|  | RU |  | N/A |  | N/A |  | N/A |
| A DL with packet arrival rate 1 packets/s/cell | Mean | 22.87 | 0% | 27.92 | 22.01 | 33.00 | 44.4 |
|  | 5% | 4.2 | 0% | 4.61 | 9.76 | 8.34 | 98.57 |
|  | 50% | 24.4 | 0% | 29.5 | 20.91 | 34.6 | 41.8 |
|  | 95% | 38.8 | 0% | 44.9 | 15.72 | 50.9 | 31.19 |
|  | Packet drop ratio |  | N/A |  | N/A |  | N/A |
|  | RU |  | N/A |  | N/A |  | N/A |

Table 6.4.1-4: Evaluation results from source 4 (RP-150776)

| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 |
| B UL with packet arrival rate 0.25 packets/s/cell | Mean | 3.883 | 0% | 2.422 | -37.62% | 2.193 | -43.54% |
|  | 5% | 1.600 | 0% | 0.902 | -43.62% | 0.783 | -51.07% |
|  | 50% | 3.894 | 0% | 2.301 | -40.90% | 2.077 | -46.65% |
|  | 95% | 6.024 | 0% | 4.411 | -26.77% | 4.039 | -32.94% |
|  | Packet drop ratio | 0.00 | N/A | 0.38 | N/A | 0.59 | N/A |
|  | RU | 26.39 | N/A | 39.15 | N/A | 42.04 | N/A |
| A DL with packet arrival rate 1 packets/s/cell | Mean | 21.232 | 0% | 24.583 | 15.78% | 27.418 | 29.14% |
|  | 5% | 9.776 | 0% | 12.642 | 29.32% | 14.787 | 51.26% |
|  | 50% | 24.094 | 0% | 26.947 | 11.84% | 29.681 | 23.19% |
|  | 95% | 26.089 | 0% | 29.898 | 14.60% | 33.301 | 27.64% |
|  | Packet drop ratio | 0.00 | N/A | 0.00 | N/A | 0.00 | N/A |
|  | RU | 19.21 | N/A | 16.51 | N/A | 14.84 | N/A |
| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 |
| B UL with packet arrival rate 0.375 packets/s/cell | Mean | 2.543 | 0% | 1.410 | -44.55 | 1.265 | -50.26 |
|  | 5% | 0.865 | 0% | 0.555 | -35.84 | 0.544 | -37.11 |
|  | 50% | 2.381 | 0% | 1.152 | -51.60 | 1.027 | -56.85 |
|  | 95% | 4.859 | 0% | 3.081 | -36.60 | 2.711 | -44.21 |
|  | Packet drop ratio | 0.22 | N/A | 14.17 | N/A | 20.08 | N/A |
|  | RU | 50.12 | N/A | 71.77 | N/A | 75.32 | N/A |
| A DL with packet arrival rate 1.5 packets/s/cell | Mean | 18.109 | 0% | 22.259 | 22.92 | 25.012 | 38.12 |
|  | 5% | 5.313 | 0% | 9.022 | 69.82 | 11.011 | 107.26 |
|  | 50% | 21.005 | 0% | 25.441 | 21.12 | 28.248 | 34.48 |
|  | 95% | 25.284 | 0% | 29.468 | 16.55 | 32.508 | 28.57 |
|  | Packet drop ratio | 0.01 | N/A | 0.00 | N/A | 0.00 | N/A |
|  | RU | 33.03 | N/A | 26.69 | N/A | 23.86 | N/A |

Note:

- CRS interference is not modeled when a cell does not transmit PDSCH.

- Dropped packets are not included in the packet throughput statistics.

Table 6.4.1-5: Evaluation results from source 5 (RP-151528)

| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Gain compared to Case 2 |
| B UL with packet arrival rate 0.6 packets/s/cell | Mean | 3.24 | 0 | 2.29 | -29.19% | 1.32 | -59.08% | -42.21% |
|  | 5% | 0.57 | 0 | 0.29 | -48.57% | 0.11 | -81.59% | -64.22% |
|  | 50% | 3.01 | 0 | 2.05 | -31.79% | 0.91 | -69.63% | -55.48% |
|  | 95% | 6.35 | 0 | 5.41 | -14.86% | 3.67 | -42.20% | -32.11% |
|  | Packet drop ratio | 1.41% | N/A | 2.57% | N/A | 5.04% | N/A | N/A |
|  | RU | 30.52% | N/A | 41.20% | N/A | 56.54% | N/A | N/A |
| A DL with packet arrival rate 3.6 packets/s/cell | Mean | 14.73 | 0 | 17.76 | 20.57% | 20.13 | 36.70% | 13.38% |
|  | 5% | 1.92 | 0 | 2.94 | 53.31% | 3.11 | 62.13% | 5.75% |
|  | 50% | 12.50 | 0 | 15.94 | 27.49% | 18.60 | 48.84% | 16.74% |
|  | 95% | 36.04 | 0 | 40.40 | 12.12% | 43.96 | 21.98% | 8.79% |
|  | Packet drop ratio | 3.22% | N/A | 2.32% | N/A | 1.38% | N/A | N/A |
|  | RU | 58.42% | N/A | 60.26% | N/A | 62.86% | N/A | N/A |

Note:

- CRS interference is not modeled when a cell does not transmit PDSCH.

Dropped packets are not included in the packet throughput statistics.

6.4.2	Scenario 2

Table 6.4.2-1: Evaluation results from source 1 (RP-150608)

|  |  |  | Operator A (DL, Mbps) |  |  |  |  |  | Operator B(UL, Mbps) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| λDL | λUL |  | Avg. | 5% | 50% | 95% | RU | Packet drop ratio | Avg. | 5% | 50% | 95% | RU | Packet drop ratio |
| 0.5 | 0.25 | Case 1 | 22.78 (0.00%) | 11.85 (0.00%) | 23.67 (0.00%) | 28.17 (0.00%) | 9% | 0% | 4.52 (0.00%) | 2.00 (0.00%) | 4.50 (0.00%) | 7.28 (0.00%) | 22% | 0% |
|  |  | Case 2 | 26.73 (+17.34%) | 14.65 (+23.63%) | 27.03 (+14.20%) | 32.52 (+15.44%) | 8% | 0% | 1.91 (-57.74%) | 0.67 (-66.50%) | 1.78 (-60.44%) | 3.60 (-50.55%) | 46% | 0% |
|  |  | Case 3 | 30.52 (+33.98%) | 17.47 (+47.43%) | 30.30 (+28.01%) | 36.36 (+29.07%) | 7% | 0% | 0.66 (-85.40%) | 0.32 (-84.00%) | 0.54 (-88.00%) | 1.39 (-80.91%) | 81% | 20% |
| 1 | 0.25 | Case 1 | 18.03 (0.00%) | 6.34 (0.00%) | 18.18 (0.00%) | 28.17 (0.00%) | 23% | 0% | 4.52 (0.00%) | 2.00 (0.00%) | 4.50 (0.00%) | 7.28 (0.00%) | 22% | 0% |
|  |  | Case 2 | 22.57 (+25.18%) | 8.89 (+40.22%) | 23.67 (+30.20%) | 32.52 (+15.44%) | 19% | 0% | 1.44 (-68.14%) | 0.49 (-75.50%) | 1.29 (-71.33%) | 2.79 (-61.68%) | 56% | 1% |
|  |  | Case 3 | 26.83 (+48.81%) | 11.43 (+80.28%) | 28.78 (+58.31%) | 36.36 (+29.07%) | 16% | 0% | 0.32 (-92.92%) | 0.21 (-89.50%) | 0.29 (-93.56%) | 0.52 (-92.86%) | 91% | 58% |
| 1.5 | 0.25 | Case 1 | 10.16 (0.00%) | 2.10 (0.00%) | 7.76 (0.00%) | 25.26 (0.00%) | 55% | 0% | 4.52 (0.00%) | 2.00 (0.00%) | 4.50 (0.00%) | 7.28 (0.00%) | 22% | 0% |
|  |  | Case 2 | 15.49 (+52.46%) | 4.16 (+98.10%) | 13.94 (+79.64%) | 32.26 (+27.71%) | 40% | 0% | 1.37 (-69.69%) | 0.47 (-76.50%) | 1.23 (-72.67%) | 2.68 (-63.19%) | 58% | 2% |
|  |  | Case 3 | 20.62 (+102.95%) | 6.46 (+207.62%) | 19.90 (+156.44%) | 36.36 (+43.94%) | 30% | 0% | 0.24 (-94.69%) | 0.18 (-91.00%) | 0.22 (-95.11%) | 0.30 (-95.88%) | 93% | 70% |
| 0.5 | 0.5 | Case 1 | 22.78 (0.00%) | 11.85 (0.00%) | 23.67 (0.00%) | 28.17 (0.00%) | 9% | 0% | 1.99 (0.00%) | 0.55 (0.00%) | 1.65 (0.00%) | 4.41 (0.00%) | 69% | 1% |
|  |  | Case 2 | 26.64 (+16.94%) | 14.76 (+24.56%) | 27.03 (+14.20%) | 32.52 (+15.44%) | 8% | 0% | 0.82 (-58.79%) | 0.37 (-32.73%) | 0.59 (-64.24%) | 2.07 (-53.06%) | 91% | 21% |
|  |  | Case 3 | 30.54 (+34.06%) | 17.54 (+48.02%) | 30.30 (+28.01%) | 36.36 (+29.07%) | 7% | 0% | 0.47 (-76.38%) | 0.28 (-49.09%) | 0.39 (-76.36%) | 0.93 (-78.91%) | 97% | 53% |
| 1 | 0.5 | Case 1 | 18.03 (0.00%) | 6.34 (0.00%) | 18.18 (0.00%) | 28.17 (0.00%) | 23% | 0% | 1.99 (0.00%) | 0.55 (0.00%) | 1.65 (0.00%) | 4.41 (0.00%) | 69% | 1% |
|  |  | Case 2 | 22.34 (+23.90%) | 8.49 (+33.91%) | 23.39 (+28.66%) | 32.52 (+15.44%) | 19% | 0% | 0.67 (-66.33%) | 0.35 (-36.36%) | 0.50 (-69.70%) | 1.63 (-63.04%) | 94% | 31% |
|  |  | Case 3 | 26.83 (+48.81%) | 11.33 (+78.71%) | 28.99 (+59.46%) | 36.36 (+29.07%) | 16% | 0% | 0.28 (-85.93%) | 0.20 (-63.64%) | 0.26 (-84.24%) | 0.43 (-90.25%) | 98% | 78% |
| 1.5 | 0.5 | Case 1 | 10.16 (0.00%) | 2.10 (0.00%) | 7.76 (0.00%) | 25.26 (0.00%) | 55% | 0% | 1.99 (0.00%) | 0.55 (0.00%) | 1.65 (0.00%) | 4.41 (0.00%) | 69% | 1% |
|  |  | Case 2 | 15.61 (+53.64%) | 4.16 (+98.10%) | 14.04 (+80.93%) | 32.26 (+27.71%) | 39% | 0% | 0.65 (-67.34%) | 0.34 (-38.18%) | 0.49 (-70.30%) | 1.61 (-63.49%) | 94% | 32% |
|  |  | Case 3 | 20.40 (+100.79%) | 6.33 (+201.43%) | 19.61 (+152.71%) | 36.36 (+43.94%) | 31% | 0% | 0.22 (-88.94%) | 0.18 (-67.27%) | 0.20 (-87.88%) | 0.29 (-93.42%) | 98% | 85% |

Table 6.4.2-2: Evaluation results from source 2 (RP-150928)

| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 |
| B UL with packet arrival rate 0.25 packets/s/cell | Mean | 1.964 | 0% | 0.905 | -54% | 1.525 | -22% |
|  | 5% | 1.144 | 0% | 0.567 | -50% | 0.526 | -54% |
|  | 50% | 1.882 | 0% | 0.825 | -56% | 2.010 | 7% |
|  | 95% | 3.135 | 0% | 1.499 | -52% | 3.895 | 24% |
|  | Packet drop ratio | 2.49% | N/A | 49.12% | N/A | 97.11% | N/A |
|  | RU | 38.16% | N/A | 64.07% | N/A | 70.5% | N/A |
| A DL with packet arrival rate 1 packets/s/cell | Mean | 10.311 | 0% | 14.032 | 36% | 17.569 | 70% |
|  | 5% | 3.296 | 0% | 5.557 | 69% | 6.596 | 100% |
|  | 50% | 10.530 | 0% | 14.750 | 40% | 18.560 | 76% |
|  | 95% | 21.710 | 0% | 27.280 | 26% | 30.890 | 42% |
|  | Packet drop ratio | 0.42% | N/A | 0.14% | N/A | 0.01% | N/A |
|  | RU | 38.27% | N/A | 30.07% | N/A | 24.92% | N/A |

Note:

- CRS interference is not modeled when a cell does not transmit PDSCH.

- Dropped packets are not included in the packet throughput statistics.

Table 6.4.2-3: Evaluation results from source 3 (RP-150694)

| Operator | Metric | Case 1 (config 2) |  | Case 2 (config 5) |  | Case 3 (new config 10:0) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 | Packet throughput (Mbits/s) | Gain compared to Case 1 |
| B UL with packet arrival rate 0.25 packets/s/cell | Mean | 0.024 | 0% | 0.0121 | -50.01% | 0 | -100% |
|  | 5% | 0.013 | 0% | 0.0067 | -49.84% | 0 | -100% |
|  | 50% | 0.023 | 0% | 0.0117 | -50.0% | 0 | -100% |
|  | 95% | 0.038 | 0% | 0.0192 | -49.74% | 0 | -100% |
|  | Packet drop ratio |  | N/A |  | N/A |  | N/A |
|  | RU |  | N/A |  | N/A |  | N/A |
| A DL with packet arrival rate 0.5 packets/s/cell | Mean | 0.54 | 0% | 0.67 | 25.19% | 0.84 | 55.56% |
|  | 5% | 0.13 | 0% | 0.14 | 15.2% | 0.16 | 29.6% |
|  | 50% | 0.48 | 0% | 0.56 | 15.08% | 0.63 | 29.96% |
|  | 95% | 1.11 | 0% | 1.31 | 18.01% | 1.49 | 34.23% |
|  | Packet drop ratio |  | N/A |  | N/A |  | N/A |
|  | RU |  | N/A |  | N/A |  | N/A |

6.4.3	Observations from performance set 2

The following is a summary of the results in sections 6.4.1 and 6.4.2:

For UL performance of Operator B:

- For scenario 1 (five sources),

- in case Operator B uses TDD UL/DL configuration 2 and Operator A uses TDD UL/DL configuration 5,

- the 5% packet throughput of Operator B is degraded by 7% -- 49%

- the 50% packet throughput of Operator B is degraded by 14% -- 51%

- the 95% packet throughput of Operator B is degraded by 8% -- 43%

- the mean packet throughput of Operator B is degraded by 14% -- 48%

- the range of packet drop ratio is 0% -- 20%

- in case Operator B uses TDD UL/DL configuration 2 and Operator A uses a new configuration of 10:0,

- the 5% packet throughput of Operator B is degraded by 11% -- 82%

- the 50% packet throughput of Operator B is degraded by 25% -- 70%

- the 95% packet throughput of Operator B is degraded by 16% -- 44%

- the mean packet throughput of Operator B is degraded by 24% -- 59%

- the range of packet drop ratio is 0% -- 28%

- For scenario 2 (from three sources),

- in case Operator B uses TDD UL/DL configuration 2 and Operator A uses TDD UL/DL configuration 5,

- the 5% packet throughput of Operator B is degraded by 33% -- 75%

- the 50% packet throughput of Operator B is degraded by 50% -- 71%

- the 95% packet throughput of Operator B is degraded by 50% -- 63%

- the mean packet throughput of Operator B is degraded by 50% -- 69%

- the range of packet drop ratio is by 0% -- 50%

- in case Operator B uses TDD UL/DL configuration 2 and Operator A uses a new configuration of 10:0,

- the 5% packet throughput of Operator B is degraded by 49% -- 100%

- the 50% packet throughput of Operator B is degraded by 76% -- 100%

- the 95% packet throughput of Operator B is degraded by 79% -- 100%

- the mean packet throughput of Operator B is degraded by 22% -- 100%

- the range of packet drop ratio is 20% -- 97%

For DL performance of Operator A:

- For scenario 1 (from five sources),

- comparing the DL performance of Operator A using TDD UL/DL configuration 5 vs. TDD UL/DL configuration 2,

- the 5% packet throughput of Operator A is improved by 9% -- 72%

- the 50% packet throughput of Operator A is improved by 11% -- 73%

- the 95% packet throughput of Operator A is improved by 12% --17%

- the mean packet throughput of Operator A is improved by 16% -- 47%

- comparing the DL performance of Operator A using a new TDD UL/DL configuration of 10:0 vs. TDD UL/DL configuration 2,

- the 5% packet throughput of Operator A is improved by 43% -- 179%

- the 50% packet throughput of Operator A is improved by 15% -- 72%

- the 95% packet throughput of Operator A is improved by 22% -- 31%

- the mean packet throughput of Operator A is improved by 29% -- 86%

- For scenario 2 (from three sources),

- comparing the DL performance of Operator A using TDD UL/DL configuration 5 vs. TDD UL/DL configuration 2,

- the 5% packet throughput of Operator A is improved by 15% -- 98%

- the 50% packet throughput of Operator A is improved by 14% -- 80%

- the 95% packet throughput of Operator A is improved by 15% -- 27%

- the mean packet throughput of Operator A is improved by 17% -- 54%

- comparing the DL performance of Operator A using a new TDD UL/DL configuration of 10:0 vs. TDD UL/DL configuration 2,

- the 5% packet throughput of Operator A is improved by 29% -- 207%

- the 50% packet throughput of Operator A is improved by 28% -- 156%

- the 95% packet throughput of Operator A is improved by 23% -- 43%

- the mean packet throughput of Operator A is improved by 33% -- 102%

Based on the evaluation results in sections 6.4.1 and 6.4.2, the following observations can be made:

- From five sources, for the macro-pico adjacent channel scenario, the uplink performance of the macro network is degraded if the pico network  uses a downlink heavier UL/DL configuration than the macro network:

- the 50% uplink packet throughput of the macro network is degraded by 14% -- 51% if the macro network uses TDD configuration 2 and the pico network uses TDD configuration 5;

- the 50% uplink packet throughput of the macro network is degraded by 25% -- 70% if the macro network uses TDD configuration 2 and the pico network uses the new TDD configuration of 10:0:0.

- From five sources, for the macro-pico adjacent channel scenario, the downlink performance of the pico network is improved if the pico network  uses a UL/DL configuration with more DL subframes:

- the 50% downlink packet throughput of the pico network is increased by 11% -- 73% if the pico network uses TDD configuration 5 compared to configuration 2;

- the 50% downlink packet throughput of the pico network is increased by 15% -- 72% if the pico network uses the new TDD configuration of 10:0:0 compared to configuration 2.

- From three sources, for the macro-macro adjacent channel scenario, the uplink performance of victim macro network is degraded if the aggressor macro network  uses a downlink heavier UL/DL configuration than the victim macro network:

- the 50% uplink packet throughput of the victim macro network is degraded by 50% -- 71% if the aggressor macro network uses TDD configuration 2 and the victim macro network uses TDD configuration 5;

- the 50% uplink packet throughput of the victim macro network is degraded by 76% -- 100% if the aggressor macro network uses TDD configuration 2 and the victim macro network uses the new TDD configuration of 10:0:0.

- From three sources, for the macro-macro adjacent channel scenario, the downlink performance of the aggressor macro network is improved if the aggressor macro network  uses a UL/DL configuration with more DL subframes:

- the 50% downlink packet throughput of the aggressor macro network is increased by 14% -- 80% if the aggressor macro network uses TDD configuration 5 compared to configuration 2;

- the 50% downlink packet throughput of the aggressor macro network is increased by 28% -- 156% if the aggressor macro network uses the new TDD configuration of 10:0:0 compared to configuration 2.

- From the study results, for the non-full buffer traffic case, it is observed there is a coexistence issue resulting in performance loss of deploying the new TDD UL/DL configuration 10:0:0/9:1:0 in a channel that is adjacent to another channel that uses different TDD UL/DL configuration without adopting any interference mitigation scheme.

6.5	Possible solutions to mitigate potential drawbacks of the additional TDD configuration(s)

This section describes possible solutions to mitigate potential drawbacks of the additional TDD configuration(s) on a high level without detailed study. If specified, the remaining details of the described methods would have to be further clarified.

6.5.1	Co-existence

Operating LTE TDD with both DL and UL transmissions and LTE TDD with DL only transmissions on adjacent carriers of the same frequency band could create co-existence issues and thus, degrade the LTE TDD with DL/UL operations.

6.5.1.1	UE/eNB sensing

One technical solution for avoiding potential co-existence issues between LTE TDD with DL only and LTE TDD with DL/UL operations on the same frequency band without prior deployment knowledge in given region or country, would be a sensing based method, which consists of the following 2 steps of sensing and adapting:

- Sensing of adjacent carriers could be done using one or multiple of the following alternatives:

  - UEs supporting LTE TDD DL only configurations and indicated or configured LTE TDD DL only cell for DL transmission (e.g. used with aggregated LTE FDD PCell) could scan neighboring carrier frequencies of the same frequency band to see if there are other LTE TDD cells deployed, which are using LTE TDD with DL/UL operations. Whether or not such LTE TDD cells exist would be reported to the network by the UE.

  - eNBs intending to use LTE TDD with DL transmission could sense and detect neighbor eNBs on adjacent carriers by measuring, e.g., PSS/SSS/CRS before starting up and occasionally during operation.

- Having sensed adjacent carriers for neighboring LTE TDD cell one or multiple of the following adaptations could be taken:

  - After LTE TDD with DL/UL operations is reported to an eNB, the eNB would automatically stop any LTE TDD DL only operations in the proximity of those frequencies and frequency bands and geographical areas. In addition, the mechanism could include for the UE to automatically stop or reject DL only operations.

  - If an eNB intending to use LTE TDD with DL only operations detects neighbor eNB(s) it could adapt / lower its transmission power, e.g., on all subframes or only on UL subframes of neighbor eNB(s) accordingly.

  - If an eNB intending to use LTE TDD with DL only operations detects very strong neighbor eNB(s) it could adopt the TDD configuration to the same UL/DL configuration of the neighbor eNB. This scheme would result in less specification impact than adapting / lowering its transmission power.

  - If an eNB intending to use LTE TDD with DL only operations detects neighbor eNB(s) it could reduce the impact on the neighbor’s uplink by improving the ACLR limit, at least in those TTIs which are assigned to the uplink in the adjacent channel.

The detailed measurement/reporting frequency configured will impact to how quickly and effectively a new TDD cell is detected on the neighboring frequencies by the UE, and the complexity involved in the detection procedure. Additionally, , the detection time and effectiveness are also dependent on the number of UEs in the cell making measurements. The frequency for the measurements is not concluded as part of the study. When a new TDD cell (not using any of the new configurations) is activated in the victim network, it may take time before being detected after activation. After that the aggressor network is assumed to have knowledge for that location of the interference issue (and thus as one option will not use either of the new configurations).In the eNB/UE sensing method, in the condition that the eNB/UE sensing mechanism detects potential co-existence situation (expressed as detecting existing TDD configuration on the same frequencies and in the same geographical area), then the new LTE TDD configuration, i.e. 10:0:0 and 9:1:0 (DL:Sp:UL), cannot be applied in the above mentioned option and thus the performance benefit of the new LTE TDD configuration is not available.
In some scenarios, like rooftop eNB installations, there can be eNB to eNB interference which may not be always detected by UE sensing but would need to be complemented in such a scenario by eNB sensing.

Complexity analyses of UE based sensing solution:

This neighbor carrier scanning in the UE based sensing could be similar to the current UE scanning used for PLMN selection.

- In the PLMN selection the UE has to scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs and search for the strongest cell on each carrier and read its system information for the PLMN selection.

- In these UE sensing mechanisms for co-existence between LTE TDD DL only and LTE TDD DL/UL configurations the UE does not need to scan all the E-UTRA bands. Instead only part of a given E-UTRA frequency band and potentially also some LTE carriers outside the given frequency band would need to be scanned in order to ensure that there are no LTE TDD DL/UL operations in neighboring carriers to the carrier with LTE TDD DL only configuration is planned to be activated. Carriers outside the given frequency band would only need to be scanned if LTE TDD DL only operations are planned to be activated at the edge of the band.

- For the purposes of the new co-existence solutions the UE is not required to read the system information of these scanned carrier frequencies like in the PLMN selection but instead potentially more optimized detection for detecting LTE TDD DL/UL deployment on adjacent carrier could be sufficient. Naturally the detection could also be done by reading system information if that is seen smaller effort and change from the UE implementation point of view.

- The current L1 hardware and algorithms are the basis of implementation for the resulting new measurement assuming current RRM and ANR functionality could be used in the UE. For UEs in connected mode the assumption is that PCell needs to ensure that measurement gaps are provided.

- As noted earlier the frequency of the measurements/reporting is not concluded in the study (and will have impact on the final implementation complexity)

The existing UEs are also required to be able to search for higher priority PLMNs (or suitable PLMN) at regular time interval.

- Similarly like regular higher priority PLMN search regular sensing and detection of LTE TDD DL/UL operations in the neighboring channels of the planned LTE TDD DL only SCell could be used for avoiding that LTE TDD DL only operations would create co-existence issues to TDD DL/UL operations.

Additionally unlike in case of PLMN searches both the specification and LTE PCell can provide additional information, which carrier frequencies on a given frequency band or on its close proximity should be searched. This will further reduce number of carriers considered for sensing compared to the existing PLMN search support. The regular higher priority PLMN is defined by TS23.122.

The UE reporting of the sensing results to the active PCell can be similar to normal UE reporting mechanisms defined in TS36.331, with one additional measurement report needed to be provided by the UE for eNB.

Ensuring mandatory support and testability of necessary interference mitigation solution(s) is a pre-requisite for specifying the new TDD configurations 10:0:0/9:1:0 for LTE TDD.

6.5.1.2	Antenna downtilt

Base station antenna downtilt is used in network deployments to control inter-cell co-channel interference. Antenna downtilt could further be used to mitigate coexistence issues between LTE TDD cells with both DL and UL transmissions and LTE TDD cells with DL only transmissions on adjacent carriers of the same frequency band. However, using antenna downtilt as the only solution to mitigate coexistence problems will not mitigate coexistence issues.

7	Ecosystem aspects

7.1	Terminal eco-system considerations

From the global eco-system of LTE TDD terminal perspective, 3GPP should ensure that all UEs that support the new TDD configurations (10.0.0 and 9.1.0), if introduced, also support the existing TDD UL/DL configurations. UE support of the new TDD configurations (10.0.0 and 9.1.0), if introduced, will be band agnostic.

8	Summary

Time synchronization and the same UL/DL configuration usage in both inter- and intra-operator cases is essential for LTE TDD operation in the same band. When all operators in the same region and same band always use the same new TDD UL/DL 10:0:0 configuration then interoperator time synchronization may not be required which needs co-ordination between operators, e.g. through regulatory bodies.

The study has shown that there are scenarios where the new TDD UL/DL configurations 10:0:0/9:1:0 provide downlink performance gains for the operator using these new configurations. The amount of gain depends on the percentage of UEs in the network supporting DL inter-band CA and on the number of cells.
It is observed there is a coexistence issue in both full-buffer and non-full-buffer cases resulting in performance loss of deploying the new TDD UL/DL configuration 10:0:0/9:1:0 in a channel that is adjacent to another channel that uses different TDD UL/DL configuration. The degradation was larger for the macro-macro than for the macro-pico scenario.
The study has identified some potential concepts for mitigating the above performance degradation. Several different concepts and solutions were discussed and captured, however, the detailed feasibility has not been assessed for any of the solutions. Ensuring mandatory support and testability of necessary interference mitigation solution(s) is a pre-requisite for specifying the new TDD configurations 10:0:0/9:1:0 for LTE TDD.

The study also concluded that from the global eco-system of LTE TDD terminal perspective, 3GPP should ensure that all UEs that support the new TDD configurations (10.0.0 and 9.1.0), if introduced, also support the existing TDD UL/DL configurations. UE support of the new TDD configurations (10.0.0 and 9.1.0), if introduced, will be band agnostic.

Annex A: Simulation assumptions for co-existence and performance set 2

A.1	ACIR

Table A.1-1: ACIR for the first adjacent channel

| Parameter | Assumption/Value |
| --- | --- |
| ACIR BS-BS | 43dB |
| ACIR BS-UE | 33dB |
| ACIR UE-BS | 30dB |
| ACIR UE-UE | 28dB |
| Note: BS includes Macro eNB and low power nodes. |  |

A.2	Propagation model

# Table A.2-1: Propagation model

| Case | Path loss model |
| --- | --- |
| Macro- outdoor Pico |  |
| Macro-outdoor Pico | PLLOS(R) = 100.7+23.5log10(R) PLNLOS(R) = 125.2+36.3log10(R) For 2GHz, R in km. Case 1: Prob(R)=min(0.018/R,1)*(1-exp(-R/0.072))+exp(-R/0.072) |
| Macro-UE | PLLOS(R)=103.4+24.2log10(R) PLNLOS(R)= 131.1+42.8log10(R) For 2GHz, R in km. Case 1: Prob(R)=min(0.018/R,1)*(1-exp(-R/0.063))+exp(-R/0.063) |
| Outdoor Pico-UE | PLLOS(R)=103.8+20.9log10(R) PLNLOS(R)=145.4+37.5log10(R) For 2GHz, R in km Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) |
| Outdoor UE-outdoor UE | If R<=50m;PL=98.45+20*log10(R),R in km If R>50m;PL=40log(R)+175.78 R in km (Xia model) |
| Macro-Macro |  |
| Macro BS to Macro BS | PL=98.45+20*log10(R),R in km |
| Macro-UE | PLLOS(R)=103.4+24.2log10(R) PLNLOS(R)= 131.1+42.8log10(R) For 2GHz, R in km. Case 1: Prob(R)=min(0.018/R,1)*(1-exp(-R/0.063))+exp(-R/0.063) |

Note: An additional 2.6dB of pathloss is assumed between 2.7GHz and 2.0GHz

A.3	UE parameters

# TableA.3-1: UE parameters used in simulation

| Parameter | Assumption |
| --- | --- |
| UE Antenna gain | 0 dBi |
| UE Noise Figure | 9 dB |
| UE power class | 23 dBm (200 mW) |
| UE antenna configuration | 1 Tx, 2 Rx |
| UL Power control | Macro UE: P0 = -82 dBm; alpha = 0.8 Pico UE: P0 = -76 dBm,alpha = 0.8 |
| Minimum distance between UE and cell | Macro BS-UE >= 35 m* Outdoor Pico-UE >= 10 m* |
| Minimum distance between UE and UE | N/A |

*note: The minimum distance applies between UEs of one operator and macros/picos of another operator.

A.4	Macro parameters

# Table A.4-1: System assumptions for Macro cell

| Parameter | Assumption |
| --- | --- |
| Cellular Layout | Hexagonal grid, 3 sectors per site |
| System bandwidth | 10MHz |
| Carrier frequency | 2.7GHz |
| Inter-site distance | 500 m |
| Number sites | 19sites (=57 cells) with wrap-around. |
| MUE number | 20ues per cell Randomly and uniformly dropped per Macro cell |
| Shadowing standard deviation | 8 dB |
| Penetration Loss | 0 dB |
| BS antenna gain after cable loss | 15 dBi |
| Antenna pattern for Macro eNBs to UEs (horizontal 2D) | = 65 degrees, Am = 20 dB (65 degree horizontal beamwidth) |
| BS noise figure | 5 dB |
| Total BS TX power (Ptotal) | 46 dBm |
| Macro DL power control | Not modeled, i.e. assuming max Macro Tx power |
| Inter-cell Interference Modelling | Explicit modelling (all cells occupied by UEs) |
| Shadowing standard deviation between UE and Macro | 8 dB |
| Shadowing standard deviation between UE and UE | 12dB |

A.5	Pico parameters

# Table A.5-1: system simulation assumptions for outdoor Pico cell

| Parameter |  | Assumption |
| --- | --- | --- |
| Macro deployment |  | The typical 19-cell and 3-sectored hexagon system layout Note that macro cells are deployed but not activated |
| System bandwidth |  | 10MHz |
| Carrier frequency |  | 2.7GHz |
| Pico number |  | 4 Picos/cell |
| LUE per Pico |  | 10UEs/Pico, cluster Uniformly dropped around each of the Pico cells within a radius of 40m |
| Pico type |  | Hotzone |
| Pico TX power (Ptotal) |  | 24dBm |
| Pico antenna pattern |  | Omni-direction |
| Pico antenna gain |  | 5dBi |
| Pico radius |  | 40m |
| Minimum distance between Pico and Pico |  | 40m |
| Minimum distance between Pico and Macro from the same operator |  | 75m |
| Minimum distance between Pico and Macro from different operators |  | 35 |
| Pico deployment |  | random deployment |
| Penetration loss |  | 0dB |
| Shadowing standard deviation | Pico to UE | 10dB |
|  | UE to UE | 12 dB |
|  | Macro to Pico | 6 dB |
| Pico noise figure |  | 13dB |

A.6	Shadowing correlation parameters

# Table A.6-1: shadowing correlation

| Parameters | Assumptions |
| --- | --- |
| Shadowing correlation between UEs | 0 |
| Shadowing correlation between outdoor Picos | 0.5 |
| Shadowing correlation between outdoor Pico and Macro | 0.5 |
| Shadowing correlation between Macro cells | A Shadowing correlation factor of 0.5 for the shadowing between sites (regardless aggressing or victim system) and of 1 between sectors of the same site shall be used[36.942] |

A.7	Other parameters

Table A.7-1: Other evaluation assumptions

| Parameters | Assumptions |
| --- | --- |
| Simulation methodology | DL and UL shall be evaluated in an integrated simulator |
| Pico antenna configuration | Set 1: 2Tx, 2Rx (codebook-based SU-MIMO or fixed rank 1 transmission) Set 2: 1Tx, 2Rx It is up to each company’s choice which set to simulate for. |
| Link adaptation | * MCS selection with 10% BLER If the highest MCS is selected, the BLER may be less than 10%, which shall be modeled * DL based on CQI/PMI/RI reports and UL based on SRS measurement |
| DL CSI feedback | DL CSI modeled as following: -- PUCCH mode 1-1, wideband CQI/PMI reported every 10ms -- CSI reporting based on ideal channel estimation and ideal interference estimation in the reported subframe -- A minimum 5ms CSI feedback delay is modeled -- Error free feedback |
| UL CSI feedback | UL CSI modeled as following --1 symbol SRS per 10ms (Last UL symbol in subframe#1) -- UL CSI based on ideal channel estimation and ideal interference estimation in the SRS subframe -- A minimum 5 ms CSI delay is modeled |
| Channel estimation | Ideal |
| Small scaling fading channel | For set 1: Pico-UE/UE-Pico: TU or ITU; Macro-UE/UE-Macro: TU or ITU; UE-UE: TU or not modeled; Pico-Pico: not modeled. Macro-Macro: not modeled Macro-Pico/Pico-Macro: not modeled For set 2: Not modeled |
| CP length | Normal CP in both downlink and uplink. |
| Special subframe configuration | Special subframe configuration #8 |
| Packet drop time | The packet drop time is model according to 36.814 (i.e. 8s for 0.5MB and 32s for 2MB). |
| Receiver type | MMSE receiver |
| UL modulation order | {QPSK, 16QAM, 64QAM} |
| Traffic model | - FTP model 1 in TR36.814 - Poisson distributed with arrival rate λ, and the arrival rate λ shall be reported by each company, with the focus on high load traffic - Number of UEs according to the simulated scenario - A packet is randomly assigned to a UE with equal probability - Independent traffic modeling for DL and UL per UE, and the ratio of DL/UL packet arrival rate shall be reported by each company. - Fixed size of 0.5Mbytes and 2Mbytes as in TR36.814, with 0,5Mbytes prioritized - Independent traffic generation per cell - Same arriving rate for all the cells |
| HARQ retransmission scheme | CC |
| Control channel and reference signal overhead | DL: • Overhead for CRS according to 36.211; • Overhead for PDCCH: 2 OFDM symbols; UL: • Overhead for SRS defined above; • Overhead for PUCCH: 2 PRBs; • Overhead for UL DMRS: 2 symbols per subframe. |
| Macro site offset between operators | A cell raduis |

A.8	Peformance metrics

- Packet throughput, defined as the packet size over the packet transmission time, including the packet waiting time in the buffer

- UE average packet throughput, defined as the average of packet throughput for the UE

- {5%, 50%, 95%} UE average packet throughput, from the CDF of average packet throughput from all UEs

- Cell average packet throughput, defined as the mean of average packet throughput from all UEs

- Other metrics, e.g.

- Packet drop statistics

- Packet delay statistics

- Frequency resource (PRBs) utilizations

- Time resource (subframes) utilizations

- CDF of packet throughput

- Total number of configured DL/UL subframes

To facilitate the comparison of the results, companies shall at least provide the following metrics. Other metrics can be optionally provided.

- For coexistence evaluation, the following metrics shall be provided:

- CDF of Operator A’s DL geometry, assuming Operator B is performing DL transmission

- CDF of Operator A’s DL geometry, assuming Operator B is performing UL transmission

- CDF of Operator B’s UL geometry, assuming Operator A is performing UL transmission

- CDF of Operator B’s UL geometry, assuming Operator A is performing DL transmission

- For performance evaluation set 2, the following metrics shall be provided:

- {5%, 50%, 95%} UE average packet throughput, from the CDF of average packet throughput from all UEs

- Cell average packet throughput, defined as the mean of average packet throughput from all UEs

- Packet drop ratio

- Time resource (i.e. subframe) utilization ratio

Annex X:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2015-03 | RP-67 | RP-150152 |  |  | TR skeleton for approval | - | 0.0.1 |
| 2015-03 | RP-67 | RP-150487 |  |  | Includes agreed text proposals in RP-150403, RP-150462, and RP-150485. | 0.0.1 | 0.1.0 |
| 2015-03 | RP-67 | RP-150519 |  |  | Editorial clean-up of v0.1.0 | 0.1.0 | 0.1.1 |
| 2015-06 | RP-68 | RP-150984 |  |  | Includes agreed text proposals | 0.1.1 | 0.2.0 |
| 2015-09 | RP-69 | RP-151525 |  |  | Includes agreed pCRs Captures the abbreviations/symbols in clause 3 Deletes the section of “Methods on utilizing the unused TDD spectrum” | 0.2.0 | 0.3.0 |