3GPP TSG RAN meeting #70										RP-151909

Sitges, Spain, Dec. 7 - 10, 2015

## Status Report to TSG

Agenda item:			13.4.08

| Work Item Name |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | - | Perf. part: | - | Testing part: | - |
| Study Item Name | Study on Latency reduction techniques for LTE |  |  |  |  |  |
| Acronym | FS_LTE_LATRED |  |  |  |  |  |
| Unique ID | 670041 |  |  |  |  |  |

Source:

| Leading WG |  | TSG RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | ENBUSKE Henrik |
|  | Company | ERICSSON |
|  | Email | henrik.enbuske@ericsson.com |

## 1	Work plan related evaluation

### 1.1	History

| TSG meeting # | TSG Tdoc number of status report | TSG Tdoc of WI/SI description sheet as approved by TSG (if any) | overall level of completion as decided by TSG for the SI / Core part / Testing part | completion date as decided by TSG for the SI / Core part / Testing part | overall level of completion as decided by TSG for the Perf. part | completion date as decided by TSG for the Perf. part |
| --- | --- | --- | --- | --- | --- | --- |
| 67 | SI started | RP-150465 | 0% | June 2016 | - | - |
| 68 | RP-150780 |  | 10% | June 2016 | - | - |
| 69 | RP-151286 |  | 25% | June 2016 | - | - |

NOTE:	The table covers all TSG meetings from the start of the WI/SI but not the current RAN meeting.
Please indicate the RAN Tdoc numbers for the WI/SI description sheets in the 3rd column above as link to the 3GPP server, i.e. ftp://ftp.3gpp.org/tsg_ran/TSG_RAN/TSGR_xx/Docs/RP-xxnnnn.zip
e.g.: RP-150518

### 1.2	Status at this TSG meeting

NOTE:	This status reflects the conclusion of the leading WG (e.g. achieved by email). In case there was no consensus a corresponding range has to be provided and reason for missing consensus has to be mentioned. If this status report covers Core and Perf. part, then the rapporteur may have to contact 2 WGs (one for the Core and RAN4 for the Perf. part).

#### 1.2.1	Estimated level of completion of the work/study item

overall (mandatory to be provided):		Core part:			XXX %

RAN4 Perf. part:		XXX %

RAN5 Testing part:		XXX %

SI:				50 %

NOTE:	Please leave the XXX for lines that are not applicable for this status report.

per WG (mandatory to be provided) for Core part or SI:	RAN WG1:		15%

RAN WG2:		100%

RAN WG3:		XXX%

RAN WG4:		XXX%

RAN WG5:		XXX%

NOTE:	Please leave the XXX for lines that are not applicable for this status report.

additional comments:

#### 1.2.2	Estimated completion date of the work/study item

This SI is planned to be 100% complete in:				June 2016		which is:	RAN #72

The Core part WI is planned to be 100% complete in:			<e.g. Dec. 2015>	which is:	RAN #XX

The Performance part WI is planned to be 100% complete in:	<e.g. Dec. 2015>	which is:	RAN #XX

The Testing part WI is planned to be 100% complete in:		<e.g. Dec. 2015>	which is:	RAN #XX

NOTE:	Please leave the XX for lines that are not applicable for this status report.

additional comments:

#### 1.2.3	Future time budget situation (not applicable to RAN5 WIs/SIs)

| Any time units modified in this section compared to RP-150518 endorsed by RAN #67 | NO |
| --- | --- |

NOTE:	The last row of the table(s) below have to be filled out (without revision marks) to reflect the status of time units (1 time unit ~ 2h) per session as endorsed by the previous RAN meeting: RP-150518
Then it has to be decided whether any modification is needed and a corresponding Yes or No has to be indicated in the table above.
If any modification is needed, then the table(s) below has to be modified with revision marks and a motivation/explanation of the changes has to be provided below the table(s).
If no time unit is needed for a session, then leave the field empty.
In general: The time units have to be indicated up to the target date of the WI/SI (if necessary add further tables).

| RAN #68 Q3/2015 RAN #69 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 82 | 82 | 91 | 91 | 91 | 89 | 76 | 76 | 76 | 76 |
|  |  | 1 |  |  |  |  |  |  |  |

| RAN #69 Q4/2015 RAN #70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 82bis | 82bis | 91bis | 91bis | 91bis | 89bis | 76bis | 76bis | 76bis | 76bis | 83 | 83 | 92 | 92 | 92 | 90 | 77 | 77 | 77 | 77 |
|  |  | 1 |  |  |  |  |  |  |  | 1 |  | 1 |  |  |  |  |  |  |  |

| RAN #70 Q1/2016 RAN #71 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84 | 84 | 93 | 93 | 93 | 91 | 78 | 78 | 78 | 78 |
| 2 |  |  |  |  |  |  |  |  |  |

| RAN #71 Q2/2016 RAN #72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84bis | 84bis | 93bis | 93bis | 93bis | 91bis | 78bis | 78bis | 78bis | 78bis | 85 | 85 | 94 | 94 | 94 | 92 | 79 | 79 | 79 | 79 |
| 2 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |

L: LTE, U: UMTS, J: Joint, RD: RRM/demodulation

motivation/explanation:-

## 2.	Technical status related evaluation

### 2.1	Detailed progress report since last TSG meeting (for all involved WGs)

NOTE:	A good progress report lists what was done for each open issue in all affected WGs.

#### 2.1.1	Progress of the SI or Core part WI or Testing part WI

TSGR2_90:

In it’s first meeting for the SI, RAN2 discussed use cases and applications that can benefit from improved latencies and agreed to a framework for guidance, [3] – [8]:

1	RAN2 should consider the web application (HTTP/FTP+TCP) use case and analyse the possible gains in the performance metrics of delay and perceived data rate for TCP based data transactions.

2	RAN2 should consider real-time application use case and analyse the possible gains in delay, service coverage and system capacity.

4	Other areas in accordance with the SID may be considered as well

5	Aspects of complexity, energy-consumption, signalling overhead and resource efficiency should be considered.

It was further agreed to capture a summary of related text and observations from company input in the TR.

In relation to [5] – [8], it was further agreed to capture some initial results and observations from reduced latency in the TR.

A skeleton TR 36.881 v0.2.0 was agreed in [2] which will subsequently be updated according to the agreements.

TSGR2_91:

At the RAN2#91 meeting, further evaluation results of latency reduction were discussed. In addition, solutions for lowering latency were discussed for which some first agreements were made.

Due to the large input on evaluations, it was agreed to further progress the inclusion of results with their respective conclusion in an email discussion. The result from this e-mail discussion will be used to update the TR with a more comprehensive analysis and also progress a SI conclusion related to the evaluation part.

The following explicit agreements w.r.t. RAN2 specific study areas were achieved:

1	It is beneficial to allow UEs to skip (most) dynamic and configured uplink transmissions if no data is available for transmission (the UE still sends the regular MAC CE, if any). The eNB may enable this by RRC dedicated signalling.

2	A shorter SPS interval (1 TTI) should be supported.

The SI TR 36.881 was updated accordingly.

TSGR2_91bis:

Solutions details for grant handling with SPS, dynamic and Pre-scheduling were discussed and agreements were subsequently made for resulting text and conclusions in TR 36.881.

More specifically, it was agreed that solution characteristics for SPS activation/deactivation w.r.t acknowledgements will be captured and that at least two possible approaches will be included in the TR.  For each approach, it was agreed that the concerns and advantages would be captured. A resulting text proposal from e-mail discussion was to be agreed at RAN2#92;

For Contention Based schemes (CB-PUSCH), the
meeting decided to capture resource efficiency evaluations of existing solutions and CB-PUSCH solutions and gains. A discussion over e-mail was initiated for a resulting text proposal aiming to capture a summary of the assumptions and how the solution(s) works. Additionally a TR conclusion on the solutions was subject for agreement;

For HO latency enhancements, evaluations with more detailed analysis were presented. An email discussion was started to conclude on
handover evaluation numbers, conclusion details on the main handover steps that contribute to handover delays and Initial assessments on the steps to address. It was also agreed to possibly also capture potential solutions that address enhancements to the different steps identified, and the gains/complexity associated to each solution.

TSGR2_92:

Text proposals for capturing evaluations, descriptions and potential solutions as a result of e-mail discussions initiated in RAN2#91bis were agreed [89]-[91], [93]. These included acknowledgement characteristics for SPS activation/deactivation, Contention based access schemes, and HO latency enhancements. To these text proposals, text to TR 36.881 conclusions was also agreed. To the protocol evaluation parts, additional evaluations on L2 overhead was agreed and added [92].

With the final RAN2 updated TR incorporating all agreed TPs with their corresponding TR conclusions, RAN2 considers their part of the SI completed.

The SI TR 36.881 was updated to v0.5.0 accordingly.

TSGR1_83:

RAN1 discussed the physical layer design options for TTI shortening and agreed on a set of simulation assumptions for both system-level and link-level evaluations.

#### 2.1.2	Progress of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

### 2.2	List of completed elements (compare with open issues of last TSG)

#### 2.2.1	Completed elements of the SI or Core part WI or Testing part WI

RAN2:

- Concluded on gains due to latency improvements on typical applications and use cases based on evaluation results

- Concluded on additional solutions for reduced latency and their resulting gain and protocol impact.

- Concluded on HO latency enhancements and potential solutions

RAN1:

- Agreed on a framework for evaluation of TTI shortening techniques

#### 2.2.2	Completed elements of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

### 2.3	List of open issues

NOTE:	Usually, at the beginning of a WI/SI the list of open issues is copied from the objectives of the WID/SID into this open issues list. Once an open issue is completed it is moved up to section 2.2.
When a WI/SI is 100% complete the list under 2.3 is empty. Otherwise please justify why an open issue is not essential for the WI/SI.

#### 2.3.1	Open issues of the SI or Core part WI or Testing part WI

RAN1:

Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signalling.

Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier).

#### 2.3.2	Open issues of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

## 3.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

R2-152496	Draft Work Plan for Study on latency reduction techniques for LTE; Rapporteur, TSG RAN2 Meeting #90

R2-152918	Skeleton TR for Study on latency reduction techniques for LTE; Rapporteur; TR; 36.881, v0.2.0, TSG RAN2 Meeting #90

R2-152451	Use cases in latency reduction; Ericsson, TSG RAN2 Meeting #90

R2-152455	Use cases and requirements for latency reduction, Huawei, HiSilicon, TSG RAN2 Meeting #90

R2-152326	Latency reductions in LTE, Ericsson, TSG RAN2 Meeting #90

R2-152174	Impact of latency reduction on TCP slow-start behavior; Intel Corporation, TSG RAN2 Meeting #90

R2-152274	Initial analysis on latency reduction; CATT, China Telecom, TSG RAN2 Meeting #90

R2-152456	Evaluation on the gains provided by 0.5ms TTI; Huawei, HiSilicon, TSG RAN2 Meeting #90

R2-153489	Areas for latency reduction; Ericsson; discussion, TSG RAN2 Meeting #91;

R2-153292	Evaluation of TTI reduction gain with additional L1/L2 overhead; Intel Corporation; discussion TSG RAN2 Meeting #91;

R2-153497	Analysis on latency reduction with TTI shortening; CATT, China Telecom; discussion TSG RAN2 Meeting #91;

R2-153332	Skipping uplink transmission with no data to transmit (discussion); Samsung Electronics Co., Ltd; discussion TSG RAN2 Meeting #91;

R2-153490	L2 enhancements to reduce latency; Ericsson; discussion TSG RAN2 Meeting #91;

R2-153185	Latency Reduction during Handover; Nokia Networks; discussion TSG RAN2 Meeting #91;

R2-153374	Uplink latency reduction for synchronized UEs; Huawei, HiSilicon; discussion TSG RAN2 Meeting #91;

R2-153161	Potentail area for latency reduction; LG Electronics Inc.; discussion TSG RAN2 Meeting #91;

R2-153222	Potential protocol enhancement for Fast Uplink Access; Nokia Networks; discussion TSG RAN2 Meeting #91;

R2-153223	Simulation result for fast uplink access and TTI shortening; Nokia Networks; discussion TSG RAN2 Meeting #91;

R2-153810	Effect of UE and eNB processing times on TCP performance; Intel Corporation; discussion TSG RAN2 Meeting #91;

R2-153294	Protocol impact of fast uplink access solution for latency reduction; Intel Corporation; discussion TSG RAN2 Meeting #91;

R2-153331	Discussion on Contention Based Access; Samsung Electronics Co., Ltd; discussion TSG RAN2 Meeting #91;

R2-153333	Skipping uplink transmission with no data to transmit (321 CR); Samsung Electronics Co., Ltd; draftCR; 36.321; 12.6.0, TSG RAN2 Meeting #91;

R2-153334	Skipping uplink transmission with no data to transmit (331 CR); Samsung Electronics Co., Ltd; draftCR; 36.331; 12.6.0, TSG RAN2 Meeting #91;

R2-153373	Downlink latency reduction for unsynchronized UEs; Huawei, HiSilicon; discussion, TSG RAN2 Meeting #91;

R2-153375	Problems of the current LTE system on latency; Huawei, HiSilicon; discussion, TSG RAN2 Meeting #91;

R2-153851	Evaluation on the gains provided by one symbol length TTI; Huawei, HiSilicon; discussion, TSG RAN2 Meeting #91;

R2-153405	Initial consideration of fast uplink access solution; Kyocera; discussion, TSG RAN2 Meeting #91;

R2-153416	Combined SR with BSR for reducing UP latency; III; discussion, TSG RAN2 Meeting #91;

R2-153493	Study of shorter TTI for latency reduction; Ericsson; discussion, TSG RAN2 Meeting #91;

R2-153499	Analysis on VoLTE capacity  with TTI shortening; CATT; discussion, TSG RAN2 Meeting #91;

R2-153500	Analysis on uplink access solutions; CATT; discussion, TSG RAN2 Meeting #91;

R2-153514	Specificaiton impacts of short TTI; ZTE Corporation; discussion, TSG RAN2 Meeting #91;

R2-153548	Consideration on Random Access for Heterogeneous TTIs in a Carrier; ETRI; discussion, TSG RAN2 Meeting #91;

R2-153852	Performance evaluation on short TTI; ZTE Corporation; discussion; revision of R2-153578, TSG RAN2 Meeting #91

R2-153701	Discussion on reduction of handover interruption; Alcatel-Lucent, Alcatel-Lucent Shanghai Bell; discussion, TSG RAN2 Meeting #91;

R2-153702	Discussion on solutions for latency reduction; Alcatel-Lucent, Alcatel-Lucent Shanghai Bell; discussion, TSG RAN2 Meeting #91;

R2-153823	Considerations on latency reduction solutions based on pre-grant procedure for LTE; KDDI Corporation; discussion, TSG RAN2 Meeting #91;

R2-153837	System performance with shorter TTI; QUALCOMM CDMA Technologies; discussion, TSG RAN2 Meeting #91;

R2-154742	Skipping padding in SPS and dynamic grants	Ericsson	discussion

R2-154353	Enhancements on SPS prescheduling 	Samsung Electronics Co., Ltd	discussion

R2-154120	DRX and Short interval SPS	CATT	discussion

R2-154121	PDCCH missing issue with skipping UL transmission	CATT	discussion

R2-154142	Potential issues on enhanced SPS mechanism	China Mobile Com. Corporation	discussion

R2-154192	Further analysis on uplink transmission skipping	Huawei, HiSilicon	discussion

R2-154350	Further discussion on Prescheduling 	Samsung Electronics Co., Ltd	discussion

R2-154354	Enhancements on SPS prescheduling	Samsung Electronics Co., Ltd	pCR							Rel-13	FS_LTE_LATRED

R2-154355	Configuring SPS on SCell	Samsung Electronics Co., Ltd	pCR	36.331						Rel-13	TEI13, FS_LTE_LATRED

R2-154351	Skipping uplink transmission on configured uplink grant with no data to transmit	Samsung Electronics Co., Ltd	pCR	36.321						Rel-13	TEI13, FS_LTE_LATRED

R2-154352	Skipping uplink transmission on configured uplink grant with no data to transmit	Samsung Electronics Co., Ltd	pCR	36.331						Rel-13	TEI13, FS_LTE_LATRED

R2-154533	Supporting MU-MIMO in enhanced uplink SPS transmission	Lenovo (Beijing) Ltd	discussion

R2-154534	The Impact of Latency Reduction on UL SPS	ASUSTEK COMPUTER (SHANGHAI)	discussion

R2-154554	Further Discussion on SPS with Consideration to Resource Efficiency	ETRI	discussion

R2-154386	Further aspects of fast uplink access solutions	Intel Corporation	discussion

R2-154491	Potential protocol enhancement for Fast uplink access	Nokia Networks	discussion

R2-154122	Analysis on resource efficiency of uplink access solutions	CATT, CATR	discussion

R2-154191	Contention based uplink transmission	Huawei, HiSilicon	discussion

R2-154193	Downlink latency reduction for unsynchronized UEs	Huawei, HiSilicon	discussion

R2-154318	Enhanced Scheduling Request for Latency Reduction	LG Electronics Inc.	discussion

R2-154741	Layer 2 solutions for latency reduction	Ericsson	discussion

R2-154322	Long-duration UL grant	LG Electronics Inc.	discussion

R2-154411	Combined SR with BSR for reducing UP latency	III	discussion

R2-154743	Email discussion report on [91#29][LTE/Latency] Evaluation results	Ericsson (Rapporteur)	report

R2-154744	TR Text Proposal capturing outcome of [91#29][LTE/Latency] Evaluation results	Ericsson (Rapporteur)	pCR

R2-154296	Protocol impact of TTI reduction	Intel Corporation	discussion

R2-154172	Utilization of short TTI in higher layer	ZTE Corporation	discussion

R2-154563	Consideration on Heterogeneous TTIs in a Carrier	ETRI	discussion

R2-154123	VoLTE Capacity Analysis with TTI shortening	CATT	discussion

R2-154740	Study of shorter TTI for latency reduction	Ericsson	discussion

R2-154809	TCP Performance with shorter TTI lengths	QUALCOMM CDMA Technologies	discussion

R2-154194	Latency reduction during Handover	Huawei, HiSilicon	discussion

R2-154813	Analysis on reduction of handover interruption	Intel Corporation	discussion

R2-154168	Analysis on the handover latency	ZTE Corporation	discussion

R2-154259	Latency reduction during handover	Nokia Networks	discussion

R2-154716	Discussion on reduction of handover interruption	Alcatel-Lucent Telecom Ltd	discussion

R2-154810	Handover Latency Improvements	QUALCOMM CDMA Technologies	discussion

R2-156658	Email discussion report for [91bis#33][LTE/LATRED] – L2 enhancements	Ericsson	report

R2-156820	Text proposal as result of Email discussion [91bis#33][LTE/LATRED] – L2 enhancements	Ericsson	pCR

R2-156299	Summary of email discussion: [91bis#34][LTE/LATRED] CB-PUSCH	Huawei	report

R2-156926	Updated conclusion on CB-PUSCH 	Huawei, CATT, Nokia Networks, Intel	report							Rel-13	FS_LTE_LATRED

R2-156922	Text Proposal capturing outcome of email discussion: [91bis#34][LTE/LATRED] CB-PUSCH	Huawei	pCR						result of email discussion [91bis#34]	Rel-13	FS_LTE_LATRED

R2-156402	Performance evaluation of CB-PUSCH	Nokia Networks	discussion

R2-156644	Uplink Latency Reduction through Contention-Based PUSCH Transmissions	Motorola Mobility UK Ltd.	Other

R2-156695	Fast UL evaluation for Mission Critical / Real Time applications	Sequans Communications	discussion

R2-156358	Enhanced Scheduling Request for Latency Reduction	LG Electronics Inc.	discussion

R2-156275	Bicasting to ensure minimum-delay transmission on split bearer	Nokia Networks	discussion

R2-156297	Downlink latency reduction for unsynchronized UEs	Huawei, HiSilicon	discussion

R2-156257	DRX and Short Interval SPS	CATT	discussion

R2-156202	Report of email discussion [91bis#35][LTE/LATRED] Handover evaluations and solutions	Intel Corporation	report

R2-156832 Text Proposal capturing outcome of email discussion [91bis#35][LTE/LATRED] Handover evaluations and solutions, Intel Corporation  pCR

R2-156921 Text Proposal for Handover Latency Reduction Solutions, Intel Corporation  pCR

R2-156926 Updated conclusion on CB-PUSCH Huawei, CATT, Nokia Networks, Intel report

R2-156927 TP of evaluation for TTI reduction ZTE, Nokia Net

R2-157117 TP on conclusions for Handover latency reduction, Intel Corporation

R1-157144	Work plan for Study on latency reduction techniques for LTE	Ericsson

R1-157732	WF on evaluation methodology for latency reduction	Ericsson

R1-157806	WF on evaluation methodology for latency reduction	Ericsson, Huawei, HiSilicon, ZTE, CATT, Sharp, Samsung, Intel, Qualcomm, Interdigital, LGE, CMCC, ETRI

R1-157008	On TTI shortening for latency reduction	CMCC

R1-157082	On physical layer aspects of low latency operation	Qualcomm Inc.

R1-157110	Discussion on TTI shortening	ETRI