3GPP TSG-RAN4 Meeting #77	R4-157873

# Anaheim, United States, 16th Nov 2015 - 20th Nov 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3368 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 2 UL and 3 DL mixed intra/inter band carrier aggregation combinations without MSD. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks, LG Electronics, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_3DL_2UL_inter_intra-Core |  |  |  |  |  | Date: |  |  | 2015-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 2 UL CA is introduced to CA_3A-7C, CA_3C-7A and CA_25A-41C |  |  |  |  |  |  |  |  |
| Summary of change: |  | CR introduces new CA configurations with 2 UL and 3 DL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration with 2 UL and 3DL would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.1, 7.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.307 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

******************************* Start of the changes **********************************************************

Table 5.6A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-5A | CA_1A-5A | 1 |  |  |  | Yes |  |  | 20 | 0 |
|  |  | 5 |  |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-7A | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-8A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-11A | - | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-18A | CA_1A-18A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A | CA_1A-19A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-20A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-21A | CA_1A-21A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-26A | CA_1A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-28A | CA_1A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-40A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-41A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-41C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-42A | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-4A | CA_2A-4A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 2 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-4A-4A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-2A-4A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-5A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-5A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-5A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 2 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-12B | - | 2 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2C-12A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-13A | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-2A-13A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-17A | - | 2 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-28A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-29A | - | 2 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-29A | - | 2 | See CA_2C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-5A | CA_3A-5A | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 3 |
|  |  | 5 |  | Yes | Yes | Yes |  |  |  |  |
| CA_3C-5A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A | CA_3A-7A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7B | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-7C | CA_3A-7A CA_7C | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3C-7A | CA_3A-7A CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-8A | CA_3A-8A | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 3 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-3A-8A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-19A | CA_3A-19A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-20A | CA_3A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-26A | CA_3A-26A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 3 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-27A | - | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 27 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-31A | - | 3 |  |  | Yes | Yes | Yes | Yes | 25 | 0 |
|  |  | 31 |  | Yes | Yes |  |  |  |  |  |
| CA_3A-38A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-40A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-40C | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-42C | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-5A | - | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-5A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-7A | CA_4A-7A | 4 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-4A-7A | - | 4 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-12A | CA_4A-12A | 4 | Yes | Yes | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 | Yes | Yes | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 3 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 4 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes |  | 20 | 5 |
|  |  | 12 |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-12B | - | 4 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-13A | CA_4A-13A | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-4A-13A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-17A | CA_4A-17A | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-27A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 27 |  | Yes | Yes | Yes |  |  |  |  |
| CA_4A-28A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-29A | - | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-7A | CA_5A-7A | 5 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  | 30 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_5A-12A | CA_5A-12A | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-13A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_5A-17A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-25A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-29A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-30A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-40A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  | Yes | Yes | Yes |  |  | 30 | 1 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-8A | - | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes | 30 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-12A | - | 7 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-20A | CA_7A-20A | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes | 40 | 1 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-22A | - | 7 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 22 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-28A | CA_7A-28A | 7 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7B-28A | - | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7C-28A | - | 7 | See CA_7C bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-40A | - | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-40C | - | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8A-11A | - | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_8A-20A | - | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  | 20 | 1 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_8A-40A | - | 8 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8A-41A |  | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
| CA_8A-41C | - | 8 | Yes | Yes | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_11A-18A | - | 11 |  |  | Yes | Yes |  |  | 25 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
| CA_12A-25A | - | 12 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_12A-30A | - | 12 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_18A-28A | CA_18A-28A | 18 |  |  | Yes | Yes | Yes |  | 25 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_19A-21A | CA_19A-21A | 19 |  |  | Yes | Yes | Yes |  | 30 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_19A-28A | - | 19 |  |  | Yes | Yes | Yes |  | 25 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_19A-42A | - | 19 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_19A-42C | - | 19 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_20A-31A | - | 20 |  |  | Yes | Yes | Yes | Yes | 25 | 0 |
|  |  | 31 |  | Yes | Yes |  |  |  |  |  |
| CA_20A-32A | - | 20 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_20A-42A | - | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_20A-42A-42A | - | 20 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_21A-42A | - | 21 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_21A-42C | - | 21 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_23A-29A | - | 23 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 23 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
| CA_25A-26A | - | 25 |  | Yes | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 25 |  | Yes | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 25 |  |  | Yes | Yes |  |  | 20 | 2 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_25A-41A | - | 25 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_25A-41C | CA_41C | 25 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-41D | - | 25 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_26A-41A | - | 26 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_26A-41C | - | 26 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_29A-30A | - | 29 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_38A-40A | - | 38 |  |  |  | Yes |  | Yes | 40 | 0 |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  |
| CA_38A-40A-40A | - | 38 |  |  |  | Yes |  | Yes | 60 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_38A-40C | - | 38 |  |  |  | Yes |  | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_39A-41A | CA_39A-41A | 39 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39A-41C | CA_41C CA_39A-41A | 39 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39C-41A | CA_39C CA_39A-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 55 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_41A-42A | - | 41 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_41A-42C | - | 41 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 5: For TDD inter-band Carrier Aggregtion only non-simultaneous Rx/Tx uplink CA configurations can be supported by UE supporting corresponding DL CA configuration without simultaneous Rx/Tx. |  |  |  |  |  |  |  |  |  |  |

*******************************************unchanged section***************************************************

Table 7.3.1A-4: Intra-band non-contiguous CA with two uplinks configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | UL SCC allocation | ΔR2UL_PCC (dB) | ΔR2UL_SCC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-4A | NOTE 2 | NOTE 3 | NOTE 4 | NOTE 5 | 0.0 | 0.0 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 3: All applicable sub-block gap sizes. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. NOTE 5: The SCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. |  |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to four downlink carriers (up to two non-contiguously aggregated carriers per band and up to three contiguously aggregated carriers per band) and up to two contiguously aggregated uplink carriers assigned to one E-UTRA band, the requirement is defined with an uplink configuration in accordance with Table 7.3.1A-3 when the uplink is active in a band supporting two non-contigous component carriers, Table 7.3.1A-1 when the uplink (up to two contiguously aggregated uplink carriers)  is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when an uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to three component carriers. The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band when the uplink is active in band(s) supporting non-contiguous aggregation of up to two component carriers. For these uplink configurations, the UE shall meet the reference sensitivity requirements for intra-band non-contiguous carrier aggregation of two downlink carriers, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.3.1. For the two component carriers within the same band, RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) when the uplink is active in another band. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

*******************************************end of changes***************************************************