3GPP TSG RAN meeting #71										RP-160206

Göteborg, Sweden March 7-10, 2016

## Status Report to TSG

Agenda item:			10.5.2

| Work Item Name |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | - | Perf. part: | - | Testing part: | - |
| Study Item Name | Study on Latency reduction techniques for LTE |  |  |  |  |  |
| Acronym | FS_LTE_LATRED |  |  |  |  |  |
| Unique ID | 670041 |  |  |  |  |  |

Source:

| Leading WG |  | TSG RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | ENBUSKE Henrik |
|  | Company | ERICSSON |
|  | Email | henrik.enbuske@ericsson.com |

## 1	Work plan related evaluation

### 1.1	History

| TSG meeting # | TSG Tdoc number of status report | TSG Tdoc of WI/SI description sheet as approved by TSG (if any) | overall level of completion as decided by TSG for the SI / Core part / Testing part | completion date as decided by TSG for the SI / Core part / Testing part | overall level of completion as decided by TSG for the Perf. part | completion date as decided by TSG for the Perf. part |
| --- | --- | --- | --- | --- | --- | --- |
| 67 | SI started | RP-150465 | 0% | June 2016 | - | - |
| 68 | RP-150780 |  | 10% | June 2016 | - | - |
| 69 | RP-151286 |  | 25% | June 2016 | - | - |
| 70 | RP-151909 |  | 50% | June 2016 | - | - |

NOTE:	The table covers all TSG meetings from the start of the WI/SI but not the current RAN meeting.
Please indicate the RAN Tdoc numbers for the WI/SI description sheets in the 3rd column above as link to the 3GPP server, i.e. ftp://ftp.3gpp.org/tsg_ran/TSG_RAN/TSGR_xx/Docs/RP-xxnnnn.zip
e.g.: RP-150518

### 1.2	Status at this TSG meeting

NOTE:	This status reflects the conclusion of the leading WG (e.g. achieved by email). In case there was no consensus a corresponding range has to be provided and reason for missing consensus has to be mentioned. If this status report covers Core and Perf. part, then the rapporteur may have to contact 2 WGs (one for the Core and RAN4 for the Perf. part).

#### 1.2.1	Estimated level of completion of the work/study item

overall (mandatory to be provided):		Core part:			XXX %

RAN4 Perf. part:		XXX %

RAN5 Testing part:		XXX %

SI:				60 %

NOTE:	Please leave the XXX for lines that are not applicable for this status report.

per WG (mandatory to be provided) for Core part or SI:	RAN WG1:		35%

RAN WG2:		100%

RAN WG3:		XXX%

RAN WG4:		XXX%

RAN WG5:		XXX%

NOTE:	Please leave the XXX for lines that are not applicable for this status report.

additional comments:

#### 1.2.2	Estimated completion date of the work/study item

This SI is planned to be 100% complete in:				June 2016		which is:	RAN #72

The Core part WI is planned to be 100% complete in:			<e.g. Dec. 2015>	which is:	RAN #XX

The Performance part WI is planned to be 100% complete in:	<e.g. Dec. 2015>	which is:	RAN #XX

The Testing part WI is planned to be 100% complete in:		<e.g. Dec. 2015>	which is:	RAN #XX

NOTE:	Please leave the XX for lines that are not applicable for this status report.

additional comments:

#### 1.2.3	Future time budget situation (not applicable to RAN5 WIs/SIs)

| Any time units modified in this section compared to RP-150518 endorsed by RAN #67 | NO |
| --- | --- |

NOTE:	The last row of the table(s) below have to be filled out (without revision marks) to reflect the status of time units (1 time unit ~ 2h) per session as endorsed by the previous RAN meeting: RP-150518
Then it has to be decided whether any modification is needed and a corresponding Yes or No has to be indicated in the table above.
If any modification is needed, then the table(s) below has to be modified with revision marks and a motivation/explanation of the changes has to be provided below the table(s).
If no time unit is needed for a session, then leave the field empty.
In general: The time units have to be indicated up to the target date of the WI/SI (if necessary add further tables).

| RAN #68 Q3/2015 RAN #69 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 82 | 82 | 91 | 91 | 91 | 89 | 76 | 76 | 76 | 76 |
|  |  | 1 |  |  |  |  |  |  |  |

| RAN #69 Q4/2015 RAN #70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 82bis | 82bis | 91bis | 91bis | 91bis | 89bis | 76bis | 76bis | 76bis | 76bis | 83 | 83 | 92 | 92 | 92 | 90 | 77 | 77 | 77 | 77 |
|  |  | 1 |  |  |  |  |  |  |  | 1 |  | 1 |  |  |  |  |  |  |  |

| RAN #70 Q1/2016 RAN #71 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84 | 84 | 93 | 93 | 93 | 91 | 78 | 78 | 78 | 78 |
| 2 |  |  |  |  |  |  |  |  |  |

| RAN #71 Q2/2016 RAN #72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84bis | 84bis | 93bis | 93bis | 93bis | 91bis | 78bis | 78bis | 78bis | 78bis | 85 | 85 | 94 | 94 | 94 | 92 | 79 | 79 | 79 | 79 |
| 2 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |

L: LTE, U: UMTS, J: Joint, RD: RRM/demodulation

motivation/explanation:-

## 2.	Technical status related evaluation

### 2.1	Detailed progress report since last TSG meeting (for all involved WGs)

NOTE:	A good progress report lists what was done for each open issue in all affected WGs.

#### 2.1.1	Progress of the SI or Core part WI or Testing part WI

TSGR1_84:

RAN1 further progressed their study of latency reduction techniques and agreed on the following:

R1-160927	Draft update to 3GPP TR 36.881 V0.5.0	Ericsson

Endorsed by RAN1

R1-161348	[DRAFT] LS on TR update for latency reduction	Ericsson

Agreed in R1-161421

Agreements:

- Following design assumptions are considered:

  - No shortened TTI spans over subframe boundary

  - At least for SIBs and paging, PDCCH and legacy PDSCH are used for scheduling

- The potential specific impacts for the followings are studied:

  - UE is expected to receive a sPDSCH at least for downlink unicast

    - sPDSCH refers PDSCH carrying data in a short TTI

  - UE is expected to receive PDSCH for downlink unicast

    - FFS whether a UE is expected to receive both sPDSCH and PDSCH for downlink unicast simultaneously

  - FFS: The number of supported short TTIs

  - If the number of supported short TTIs is more than one,

    - The length of short TTI can be variable

      - FFS whether a UE is expected to receive sPDSCHs with different lengths simultaneously

      - FFS how to change the length of short TTI for sPDSCH e.g., semi-static vs dynamic, cell-specific vs UE-specific, explicit vs implicit

  - TTI length for DL and UL can be different

R1-161458	WF on Latency Reduction Control Overhead	CATT, Nokia, Ericsson, LGE, Samsung, Panasonic

R1-161422	Way forward on simulation results and methodology	Ericsson

Agreements:

- Following design assumptions are used for the study:

  - From eNB perspective, existing non-sTTI and sTTI can be FDMed in the same subframe in the same carrier

    - FFS: Other multiplexing method(s) with existing non-sTTI for UE supporting latency reduction features

Agreements:

- In this study, following aspects are assumed in RAN1:

  - PSS/SSS, PBCH, PCFICH and PRACH, Random access, SIB and Paging procedures are not modified.

- Following aspects are further studied in the next RAN1 meeting:

  - Note: But the study is not limited to them.

  - Design of sPUSCH DM-RS

    - Alt.1: DM-RS symbol shared by multiple short-TTIs within the same subframe

    - Alt.2: DM-RS contained in each sPUSCH

  - HARQ for sPUSCH

    - Whether/how to realize asynchronous and/or synchronous HARQ

  - sTTI operation for Pcell and/or SCells by (e)CA in addition to non-(e)CA case

TSGR2_92

R2-161869	TR 36.881 v0.5.1 for Study on Latency reduction techniques for LTE	Ericsson	draft TR	36.881	0.5.1				related with RAN1 LS R2-161063	Rel-14	FS_LTE_LATRED

=>	Revision history to be added

=>	Revised in R2-161963 v0.5.2

=>	Agreed in R2-161964 v0.6.0

#### 2.1.2	Progress of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

### 2.2	List of completed elements (compare with open issues of last TSG)

#### 2.2.1	Completed elements of the SI or Core part WI or Testing part WI

#### 2.2.2	Completed elements of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

### 2.3	List of open issues

NOTE:	Usually, at the beginning of a WI/SI the list of open issues is copied from the objectives of the WID/SID into this open issues list. Once an open issue is completed it is moved up to section 2.2.
When a WI/SI is 100% complete the list under 2.3 is empty. Otherwise please justify why an open issue is not essential for the WI/SI.

#### 2.3.1	Open issues of the SI or Core part WI or Testing part WI

RAN1:

Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signalling.

Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier).

#### 2.3.2	Open issues of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

## 3.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-160243	LS on RAN2 Latency agreements and conclusions	RAN2, Ericsson

- R1-160290	Performance evaluation on TTI shortening	Huawei, HiSilicon

- R1-160291	Overview of short TTI	Huawei, HiSilicon

- R1-160292	Short TTI for DL transmissions	Huawei, HiSilicon

- R1-160293	Discussion on DL RS  for short TTI	Huawei, HiSilicon

- R1-160294	Short TTI for UL transmissions	Huawei, HiSilicon

- R1-160371	System Evaluation Results of Slot-based TTI	CATT

- R1-160372	System Evaluation Results of Symbol-based TTI	CATT

- R1-160373	System Analysis of DL TTI Shortening	CATT

- R1-160374	TDD Delay Analysis and Challenges in the TTI shortening	CATT

- R1-160375	System Analysis of UL TTI Shortening	CATT

- R1-160436	Aspects to consider for DL transmission of TTI shortening	Intel Corporation

- R1-160499	Discussion on TTI shortening for latency reduction	CMCC

- R1-160583	System-level performance evaluation of TTI shortening	Samsung

- R1-160584	Study on latency reduction due to TTI shortening	Samsung

- R1-160585	Specification impact for DL due to TTI shortening	Samsung

- R1-160586	Specification impact for UL due to TTI shortening	Samsung

- R1-160596	Overview of TTI shortening	Guangdong OPPO Mobile Telecom.

- R1-160648	System-level simulation results for latency reduction	LG Electronics

- R1-160649	Study on TTI shortening for downlink transmissions	LG Electronics

- R1-160650	Discussion on CRS based PDCCH transmission with TTI shortening	LG Electronics

- R1-160651	Discussion on DMRS based PDCCH transmission with TTI shortening	LG Electronics

- R1-160652	Discussion on TM9 PDSCH transmission with TTI shortening	LG Electronics

- R1-160653	Study on TTI shortening for uplink transmissions	LG Electronics

- R1-160654	Discussion on PUCCH design for Latency reduction	LG Electronics

- R1-160655	Discussion on PUSCH transmission with TTI shortening	LG Electronics

- R1-160681	Discussion on specification impact for DL transmissions	Sony

- R1-160699	Design of UL reference signal for PUSCH with sTTI	Mitsubishi Electric RCE

- R1-160719	Evaluation results on shortened TTIs	Panasonic

- R1-160720	Physical layer aspects of latency reduction for DL	Panasonic

- R1-160721	Physical layer aspects of latency reduction for UL	Panasonic

- R1-160740	Discussion on UL RS  for short TTI	Huawei, HiSilicon

- R1-160754	Discussion on new frame structure for latency reduction in TDD	Huawei, HiSilicon

- R1-160764	Discussion on control signaling design for reduced TTI	HTC

- R1-160765	Uplink latency reduction with prescheduling	HTC

- R1-160779	DMRS for PUSCH with short TTI	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160780	Shorter TTI  enhancements for TDD	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160785	On Shorter TTI for Latency Reduction	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160786	Considerations on required downlink physical layer enhancements for shorter TTI	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160790	System-level Evaluation of TTI Shortening	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160798	Considerations of FDD DL HARQ for Supporting Latency Reduction	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160799	Considerations on UL HARQ for Supporting Latency Reduction	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160800	Views on shortened TTI for DL transmission	China Telecommunications

- R1-160805	PUCCH design for sTTI	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R1-160862	On multiplexing of PDSCH with different TTIs and related enhancements	Intel Corporation

- R1-160863	System level evaluation of shortened TTI	Intel Corporation

- R1-160864	Uplink Reference Signal Design for shortened TTI	Intel Corporation

- R1-160902	System-level evaluation of TTI shortening	Qualcomm Inc.

- R1-160903	Link-level evaluation of TTI shortening in DL	Qualcomm Inc.

- R1-160904	Link-level evaluation of TTI shortening in UL	Qualcomm Inc.

- R1-160905	TTI shortening and reduced processing time for DL transmissions	Qualcomm Inc.

- R1-160906	TTI shortening and reduced processing time for UL transmissions	Qualcomm Inc.

- R1-160917	Support for Short TTIs in DL	INTERDIGITAL COMMUNICATIONS

- R1-160927	Draft update to 3GPP TR 36.881 V0.5.0	Ericsson

- R1-160928	System-level evaluation results for TTI shortening techniques	Ericsson

- R1-160929	Overview of TTI shortening and reduced processing time for DL transmissions	Ericsson

- R1-160930	Processing time reduction in DL	Ericsson

- R1-160931	Design of DL DCI for short TTI	Ericsson

- R1-160932	Link level evaluation of PDCCH for short TTI	Ericsson

- R1-160933	Link level evaluation of PDSCH for short TTI	Ericsson

- R1-160934	Physical layer aspects of short TTI for downlink transmissions	Ericsson

- R1-160935	Link level evaluation of PUCCH for short TTI	Ericsson

- R1-160936	Physical layer aspects of short PUCCH	Ericsson

- R1-160937	Overview of TTI shortening and reduced processing time for UL transmissions	Ericsson

- R1-160938	Design for DCI for short TTI uplink transmissions	Ericsson

- R1-160939	Physical layer aspects of short TTI for uplink transmissions	Ericsson

- R1-160940	Link level evaluation of PUSCH for short TTI	Ericsson

- R1-160941	DCI bit fields for short TTI uplink transmissions	Ericsson

- R1-160942	Processing time reduction in UL	Ericsson

- R1-160964	DL aspects of TTI shortening	NTT DOCOMO INC.

- R1-160965	UL aspects of TTI shortening	NTT DOCOMO INC.

- R1-160966	Discussions on TTI shortening	NTT DOCOMO INC.

- R1-160970	DL system-level performance evaluation for shortened TTI	Motorola Mobility

- R1-160971	On using shortened DL TTI for reduced latency data transmission	Motorola Mobility

- R1-160977	Performance evaluation on PUSCH with short TTIs	Mediatek

- R1-160982	System-level performance evaluation for short TTI	ZTE Corporation

- R1-160983	Downlink control channels for short TTI	ZTE Corporation

- R1-160984	Downlink RS in short TTI	ZTE Corporation

- R1-160985	Latency reduction for TDD	ZTE Corporation

- R1-160986	Design and evaluation for uplink ACK/NACK transmission in short TTI	ZTE Corporation

- R1-160987	Uplink DMRS for PUSCH in short TTI	ZTE Corporation

- R1-161011	Overview on frame structure and downlink design for TTI shortening	ETRI

- R1-161012	Overview on uplink design for TTI shortening	ETRI

- R1-161017	Considerations on TTI shortening for DL	Lenovo (Beijing) Ltd

- R1-161018	Considerations on TTI shortening for UL	Lenovo (Beijing) Ltd

- R1-161066	Discussions on the design of latency reduction	CATR

- R1-161067	Evaluation results for TTI shortening	CATR

- R1-161086	Impact of shortened TTI on uplink transmission	ASUSTEK COMPUTER (SHANGHAI)

- R1-161092	Support for Short TTIs in UL	INTERDIGITAL COMMUNICATIONS

- R1-161093	Support for Short TTIs and Processing Times	INTERDIGITAL COMMUNICATIONS

- R1-161100	System-level evaluation of latency reduction	Fraunhofer HHI

- R1-161115	DL system-level performance evaluation for shortened TTI	Motorola Mobility

- R1-161167	System-level evaluation results for TTI shortening techniques	Ericsson

- R1-161168	Link level evaluation of PUSCH for short TTI	Ericsson

- R1-161169	Link level evaluation of PUCCH for short TTI	Ericsson

- R1-161310	WF on the shortened TTI design	LG Electronics, Ericsson, MTI, ETRI

- R1-161320	Way Forward on control overhead assumption for latency reduction	LG Electronics, Ericsson, Nokia

- R1-161341	WF on sTTI structure	Ericsson, NTT DOCOMO, CATT

- R1-161348	[DRAFT] LS on TR update for latency reduction	Ericsson

- R1-161350	TCP behaviour and latency reduction	MediaTek

- R1-161353	Way forward on simulation results and methodology	Ericsson

- R1-161374	Way Forward on latency reduction for TDD	Huawei, HiSilicon

- R1-161418	WF on the shortened TTI design	LG Electronics

- R1-161419	WF on control channel assumption for latency reduction	LG Electronics

- R1-161420	WF on sTTI structure	Ericsson, NTT DOCOMO, CATT, Mitsubishi Electric

- R1-161421	LS on TR update for latency reduction	RAN1, Ericsson

- R1-161422	Way forward on simulation results and methodology	Ericsson

- R1-161458	WF on Latency Reduction Control Overhead	CATT, Nokia, Ericsson, LGE, Samsung, Panasonic

- R1-161520	Way Forward on latency reduction based on Frame Structure 2	Huawei, HiSilicon, Nokia Networks

- R2-161063	LS on TR update for latency reduction (R1-161421; contact: Ericsson)	RAN1	LS in

- R2-161869	TR 36.881 v0.5.1 for Study on Latency reduction techniques for LTE	Ericsson

- R2-161963	TR 36.881 v0.5.2 for Study on Latency reduction techniques for LTE	Ericsson

- R2-161963	TR 36.881 v0.6.0 for Study on Latency reduction techniques for LTE	Ericsson