3GPP TSG RAN meeting #72										RP-161022

Busan, Korea, June 13 - 16, 2016

## Status Report to TSG

Agenda item:			10.2.10

| Work Item Name |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | X | Perf. part: | - | Testing part: | - |
| Study Item Name | L2 latency reduction techniques for LTE |  |  |  |  |  |
| Acronym | LTE_LATRED_L2-Core |  |  |  |  |  |
| Unique ID | 710180 |  |  |  |  |  |

Source:

| Leading WG |  | TSG RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | ENBUSKE Henrik |
|  | Company | ERICSSON |
|  | Email | henrik.enbuske@ericsson.com |

## 1	Work plan related evaluation

### 1.1	History

| TSG meeting # | TSG Tdoc number of status report | TSG Tdoc of WI/SI description sheet as approved by TSG (if any) | overall level of completion as decided by TSG for the SI / Core part / Testing part | completion date as decided by TSG for the SI / Core part / Testing part | overall level of completion as decided by TSG for the Perf. part | completion date as decided by TSG for the Perf. part |
| --- | --- | --- | --- | --- | --- | --- |
| 71 | WI started | RP-160667 | 0% | September 2016 | - | - |
|  |  |  |  |  | - | - |
|  |  |  |  |  | - | - |
|  |  |  |  |  | - | - |

NOTE:	The table covers all TSG meetings from the start of the WI/SI but not the current RAN meeting.
Please indicate the RAN Tdoc numbers for the WI/SI description sheets in the 3rd column above as link to the 3GPP server, i.e. ftp://ftp.3gpp.org/tsg_ran/TSG_RAN/TSGR_xx/Docs/RP-xxnnnn.zip
e.g.: RP-160617

### 1.2	Status at this TSG meeting

NOTE:	This status reflects the conclusion of the leading WG (e.g. achieved by email). In case there was no consensus a corresponding range has to be provided and reason for missing consensus has to be mentioned. If this status report covers Core and Perf. part, then the rapporteur may have to contact 2 WGs (one for the Core and RAN4 for the Perf. part).

#### 1.2.1	Estimated level of completion of the work/study item

overall (mandatory to be provided):		Core part:			80 %

RAN4 Perf. part:		XXX %

RAN5 Testing part:		XXX %

SI:				XXX %

NOTE:	Please leave the XXX for lines that are not applicable for this status report.

per WG (mandatory to be provided) for Core part or SI:	RAN WG1:		XXX %

RAN WG2:		80 %

RAN WG3:		XXX%

RAN WG4:		XXX%

RAN WG5:		XXX%

NOTE:	Please leave the XXX for lines that are not applicable for this status report.

additional comments:			<if any, otherwise leave it blank>

#### 1.2.2	Estimated completion date of the work/study item

This SI is planned to be 100% complete in:				<e.g. March 17>	which is:	RAN #XX

The Core part WI is planned to be 100% complete in:			September 2016	which is:	RAN #73

The Performance part WI is planned to be 100% complete in:	<e.g. March 17>	which is:	RAN #XX

The Testing part WI is planned to be 100% complete in:		<e.g. March 17>	which is:	RAN #XX

NOTE:	Please leave the XX for lines that are not applicable for this status report.

additional comments:			<if any, otherwise leave it blank>

#### 1.2.3	Future time budget situation (not applicable to RAN5 WIs/SIs)

| Any time units modified in this section compared to RP-160617 endorsed by RAN #71 | No |
| --- | --- |

NOTE:	The last row of the table(s) below have to be filled out (without revision marks) to reflect the status of time units (1 time unit ~ 2h) per session as endorsed by the previous RAN meeting: RP-160617
up to the target date of the WI/SI (if necessary add further tables below).
Then it has to be decided whether any modification is needed and a corresponding Yes or No has to be indicated in the table above.
If any modification is needed, then the table(s) below has to be modified with revision marks and a motivation/explanation of the changes has to be provided below the table(s).
If no time unit is needed for a session, then leave the field empty.
In general: The time units have to be indicated up to the target date of the WI/SI (if necessary add further tables).

For WIs/SIs related to UMTS/LTE:

| RAN #71 Q2/2016 RAN #72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84bis | 84bis | 93bis | 93bis | 89bis | 91bis | 78bis | 78bis | 78bis | 78bis | 85 | 85 | 94 | 94 | 90 | 92 | 79 | 79 | 79 | 79 |
|  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |

| RAN #72 Q3/2016 RAN #73 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 86 | 86 | 95 | 95 | 91 | 93 | 80 | 80 | 80 | 80 |
|  |  | 1 |  |  |  |  |  |  |  |

| RAN #73 Q4/2016 RAN #74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 86bis | 86bis | 95bis | 95bis | 91bis | 93bis | 80bis | 80bis | 80bis | 80bis | 87 | 87 | 96 | 96 | 92 | 94 | 81 | 81 | 81 | 81 |

| RAN #74 Q1/2017 RAN #75 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 88 | 88 | 97 | 97 | 89 | 95 | 82 | 82 | 82 | 82 |

L: LTE, U: UMTS, J: Joint, RD: RRM/demodulation

motivation/explanation:

## 2.	Technical status related evaluation

### 2.1	Detailed progress report since last TSG meeting (for all involved WGs)

NOTE:	A good progress report lists what was done for each open issue in all affected WGs.

#### 2.1.1	Progress of the SI or Core part WI or Testing part WI

RAN2 #93bis

Agreements

- FFS what SPS periodicity values other than 1ms should be introduced

- For retransmission colliding with SPS resources we will not only rely on adaptive retransmissions.   FFS if the UE can retransmit on the SPS resources if no new data is available.

- For TDD, if SPS configuration is below 10ms the existing text in the spec “the UE shall round this parameter down to the nearest integer (of 10 sub-frames)” doesn’t apply.

Discussion on SPS intervals resulted in the following:

=> Discuss what values other than 1 should be introduced in the next meeting

Discussion on DTX detection resulted in the following:

=> We will not consider RAN1 specific solutions.  If companies agrees that there is an error case with the DTX detection probability we can considers RAN2 based solutions.

Discussion on UL skipping resulted in the following:

=> It is assumed that the UE behaviour of muting for SPS or dynamic grants will be the same.

=> Separate configuration of skipping for SPS and dynamic grant will be introduced.

Discussion on SPS activation and deactivation feedback resulted in the following:

=> Feedback for SPS deactivation will be introduced.  FFS whether we have feedback for SPS activation.

Discussion on BSR, PHR and UCI resulted in the following:

=> Common understanding is that we will send the LS at least on the UCI issue to RAN1

=> RAN2 assumption is that the UE will send the BSR/PHR according to legacy trigger (e.g. we will not specify skipping of BSR/PHR when no data is available in the buffer).

=> Padding BSR should not be transmitted.  FFS if anything needs to be specified.

=> If the UE is configured with SPS and UL skipping, then implicit release is not supported.  How this is captured in the specs is FFS

RAN2 #94

Agreements:

- New MAC CE for SPS deactivation feedback when skipping padding feature is configured. No scheduling request is triggered.

- SPS resource is cleared after first transmission of the new MAC CE.   Retransmission of MAC CE can continue after clearing the configured UL grant.

- The new MAC CE will also be used for SPS activation feedback.  The same MAC CE is used (i.e. same LCHID)

- Allow and prioritize non-adaptive retransmissions on SPS resources

- If UL data is skipped RAN2 should not generate a TB.  RAN2 assumes that in this case the UCI should be sent on PUCCH.  Confirm that this is possible according to existing RAN1 specs.

TDD specific agreements

- SPS configuration is not optimized for TDD.  The UL SPS occasion calculation formula is used.

- Two-interval configuration should not be configured for TDD with SPS interval shorter than 10ms.

- The restriction of 10 subframes boundary is only applicable for the SPS interval longer than 10ms

- All the subframes including both UL subframes and DL subframes are considered as potential UL SPS subframes. If one UL SPS subframe calculated according to the SPS occasion calculation formula is a downlink subframe or a special subframe, the UE will skip this UL SPS occasion

Working assumption

- Non-adaptive retransmissions are done based on RV0.  FFS if anything additional is needed

Discussion on BSR, PHR and UCI resulted in addition in the following:

=>	If UL data is skipped RAN2 should not generate a TB, the UCI should be sent on PUCCH

The working assumption including the transmission of UCI is agreed to be confirmed by RAN1 in R2-164542 [1].

#### 2.1.2	Progress of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

### 2.2	List of completed elements (compare with open issues of last TSG)

#### 2.2.1	Completed elements of the SI or Core part WI or Testing part WI

For the following points, RAN2 has agreed on the general functions and related mechanisms, but for where Stage 3 details in specifications remain to be finalized:

- Introduction of short SPS period to allow UL prescheduling

- Reduction of padding in case of dynamic and SPS based UL pre-scheduling to reduce interference and UE power consumption

- Introduction of feedback for SPS activation, reactivation and deactivation command

#### 2.2.2	Completed elements of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

### 2.3	List of open issues

NOTE:	Usually, at the beginning of a WI/SI the list of open issues is copied from the objectives of the WID/SID into this open issues list. Once an open issue is completed it is moved up to section 2.2.
When a WI/SI is 100% complete the list under 2.3 is empty. Otherwise please justify why an open issue is not essential for the WI/SI.

#### 2.3.1	Open issues of the SI or Core part WI or Testing part WI

- Agree on specific values for the introduction of short SPS period to allow UL prescheduling

- Finalize stage 3 details for reduction of padding in case of dynamic and SPS based UL pre-scheduling to reduce interference and UE power consumption

- Finalize stage 3 details for the introduction of feedback for SPS activation, reactivation and deactivation command

#### 2.3.2	Open issues of the Performance part WI

NOTE:	Please leave this section empty if not applicable to this status report.

## 3.	References

[1]	R2-164554	LS to RAN1 on RAN2 agreements on skipping UL transmissions

[2]	R2-162237	Short SPS period for UL prescheduling 	Samsung Electronics Co., Ltd	discussion

[3]	R2-162262	Collision of new transmission and retransmission in short SPS period	Huawei, HiSilicon	discussion

[4]	R2-162263	Short SPS period in TDD	Huawei, HiSilicon	discussion

[5]	R2-162415	Remaining issues related to UL SPS	China Mobile Com. Corporation	discussion

[6]	R2-162423	Impact on DRX with pre-scheduling and short SPS periodicity	Nokia, Alcatel-Lucent Shanghai Bell	discussion

[7]	R2-162466	Further aspects of short SPS interval	Intel Corporation	discussion

[8]	R2-162768	Introduction of short SPS intervals for UL prescheduling	Ericsson	discussion

[9]	R2-162779	Introduction of short SPS intervals	Ericsson	draftCR	36.331	13.1.0	-	-	B		Rel-14	LTE_LATRED_L2

[10]	R2-162235	Issues on Prescheduling and skipping uplink transmission	Samsung Electronics Co., Ltd	discussion

[11]	R2-162236	Discussion on acknowledging SPS command	Samsung Electronics Co., Ltd	discussion

[12]	R2-162264	On UL grants skipping	Huawei, HiSilicon	discussion

[13]	R2-162265	Implicit SPS release under UL grants skipping	Huawei, HiSilicon	discussion

[14]	R2-162266	Necessity of feedback for SPS activation and deactivation	Huawei, HiSilicon	discussion

[15]	R2-162467	Further aspects of UL grant skipping	Intel Corporation	discussion

[16]	R2-162504	Skipping empty BSR and feedback for SPS activation/deactivation 	Nokia, Alcatel-Lucent Shanghai Bell	discussion

[17]	R2-162515	Feedback for SPS PDCCH command	CATT	discussion

[18]	R2-162572	Discussion on the feedback of the SPS activation command	ZTE Corporation	discussion

[19]	R2-162601	Discussion on skipping UL grants	ASUSTEK COMPUTER (SHANGHAI)	discussion

[20]	R2-162690	Details of Short SPS procedure for latency reduction	ETRI	discussion

[21]	R2-162735	Considerations on skipping UL padding transmissions	Sequans Communications	discussion

[22]	R2-162780	Skipping padding transmissions	Ericsson	draftCR	36.321	13.1.0	-	-	B		Rel-14	LTE_LATRED_L2

[23]	R2-162782	Skip padding option for UL grants	Ericsson	discussion

[24]	R2-162901	Need for feedback of SPS command	LG Electronics Inc.	discussion

[25]	R2-162902	SPS feedback transmission	LG Electronics Inc.	discussion

[26]	R2-162909	Open issues for skipping UL grants	QUALCOMM CDMA Technologies	discussion

[27]	R2-162267	PUSCH resource waste in case of short SPS period	Huawei, HiSilicon	discussion

[28]	R2-162468	SPS activation, reactivation and deactivation feedback	Intel Corporation	discussion

[29]	R2-162781	Acknowledgements for SPS commands	Ericsson	discussion

[30]	R2-163671	Feedback for SPS activation/deactivation	Samsung 	discussion

[31]	R2-163698	Feedback for SPS activation/deactivation 	Nokia, Alcatel-Lucent Shanghai Bell	discussion

[32]	R2-163781	Acknowledgements for SPS commands	Ericsson	discussion

[33]	R2-163385	UL SPS command feedback	Fiberhome Technologies Group	discussion

[34]	R2-163475	Feedback for SPS activation and deactivation	CATT	discussion

[35]	R2-163916	Discussion on feedback for SPS activation and deactivation	Huawei, HiSilicon	discussion

[36]	R2-164207	SPS feedback for SPS release	LG Electronics Inc.	discussion

[37]	R2-164217	Need of feedback for SPS activation	LG Electronics Inc.	discussion

[38]	R2-163386	Discussion of retransmission for short SPS period	Fiberhome Technologies Group	discussion

[39]	R2-163785	Non-adaptive retransmissions for SPS	Ericsson	discussion

[40]	R2-163477	Retransmission issue in short SPS interval	CATT	discussion

[41]	R2-163626	HARQ retransmissions for short SPS interval	Intel Corporation	discussion

[42]	R2-163703	Retransmission collision with SPS occasion	Nokia, Alcatel-Lucent Shanghai Bell	discussion

[43]	R2-163913	Collision of new transmission and retransmission in short SPS period	Huawei, HiSilicon

[44]	R2-164299	TDD issue in short SPS  interval	CATT	discussion

[45]	R2-163795	TDD SPS configuration	Ericsson	discussion

[46]	R2-163914	Short SPS period in TDD	Huawei, HiSilicon	discussion

[47]	R2-163692	UCI transmission when UE skips empty BSR	Nokia, Alcatel-Lucent Shanghai Bell	discussion

[48]	R2-163695	UL lost handling when UE configured to skip padding BSR, Nokia, Alcatel-Lucent ShanghaiBell

[49]	R2-163861	Discussion on DTX and HARQ	ZTE Corporation	discussion

[50]	R2-163915	Problem of UL DTX detection	Huawei, HiSilicon	discussion

[51]	R2-164116	On issues with skipping of UL padding transmissions, Sequans Communications	discussion

[52]	R2-163792	Periodic BSR in SPS	Ericsson	discussion

[53]	R2-163699	Impact on DRX with pre-scheduling and short SPS periodicity, Nokia, Alcatel-Lucent ShanghaiBell

[54]	R2-163912	DRX optimization for short SPS period	Huawei, HiSilicon	discussion

[55]	R2-163771	Considerations on the SPS resource efficiency	Fujitsu	discussion

[56]	R2-163793	SPS interval alignment	Ericsson	discussion

[57]	R2-163911	Short SPS periodicity values	Huawei, HiSilicon	discussion

[58]	R2-163917	Implicit SPS release under UL grants skipping	Huawei, HiSilicon	discussion

[59]	R2-163918	PUSCH resource waste in case of short SPS period	Huawei, HiSilicon	discussion

[60]	R2-163838	Introduction of L2 Latency reduction techniques	Ericsson	draftCR	36.300

[61]	R2-163942	Introduction of L2 Latency reduction techniques	Ericsson	draftCR	36.321

[62]	R2-163943	L2 Latency reduction techniques	Ericsson	draftCR	36.321

[63]	R2-163672	Text proposal to capture muting behaviour in 36.321	Samsung 	discussion

[64]	R2-163944	Introduction of L2 Latency reduction techniques	Ericsson 	draftCR	36.331

[65]	R2-164090	TDD configurations with skip padding and short intervals	Ericsson	draftCR	36.321