3GPP TSG RAN WG1 Meeting #84bis                                                           R1-163698

Busan, Korea, April 11th - 15th, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36.211 |  | CR |  |  | 0238 |  | rev |  | - |  |  | Current version: |  |  |  |  |  |  |  | 13.1.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  | UICC apps |  |  |  | ME |  |  | X |  | Radio Access Network |  |  |  |  | X | Core Network |  |  |  |
| Title: |  | Correction to TS 36.211 for eMTC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | R1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | LTE_MTCe2_L1-Core |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 2016-04-13 |  |  |  |  |  |
| Category: |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-13 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |  |  |  |  |  |
| Reason for change: |  |  | We have the following agreement for MPDCCH frequency hopping in RAN1 #84: “Scrambling initialisation and frequency hopping of TDD PDSCH / MPDCCH starts from the absolute subframe #2.” This agreement is not correctly reflected in current TS36.211 for the frequency hopping of MPDCCH with P-RNTI. In the bullet for MPDCCH frequency hopping enabled, the symbol used to represent consecutive downlink subframes including invalid subframes is incorrect. There are also duplicated text and some typographical errors to be revised in TS 36.211. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | The modification includes: - Correct the frequency hopping formula for MPDCCH with P-RNTI according to the above agreement, which is aligned with the frequency hopping formula for other MPDCCH. - Correct the symbol used to represent consecutive downlink subframes including invalid subframes to in the bullet for MPDCCH frequency hopping enabled. - Remove the duplicated text which specifying “When in RRC_CONNECTED, MPDCCH scheduling RAR and PDSCH conveying RAR that are related to MPDCCH order is prioritized over any other unicast DL reception”. - Correct some typographical errors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | The ageement of MPDCCH frequency hopping is not correctly reflected for MPDCCH with P-RNTI. Specification is more difficult to read. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 5.2.4, 5.4.3, 6.2.7, 6.4.1, 6.8B.2, 6.8B.3, 6.8B.5, 6.10.3A.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | x | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |
| affected: |  |  |  | x | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  | x | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 5.2.4	Narrowbands

A narrowband is defined as six non-overlapping consecutive physical resource blocks in the frequency domain. The total number of uplink narrowbands in the uplink transmission bandwidth configured in the cell is given by

The narrowbands are numbered   in order of increasing physical resource-block number where narrowband is composed of physical resource-block indices

where

#### < Unchanged parts are omitted >

### 5.4.3	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in Subclause 5.1.2.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements. PUCCH uses one or more resource block in each of the two slots in a subframe. Within the physical resource block(s) used for transmission, the mapping of  to resource elements  on antenna port  and not used for transmission of reference signals shall be in increasing order of first , then  and finally the slot number, starting with the first slot in the subframe. The relation between the index  and the antenna port number  is given by Table 5.2.1-1.

For non-BL/CE UEs, except for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

For BL/CE UEs, PUCCH is transmitted with repetitions. The PUCCH transmission spans  consecutive subframes, including invalid subframes where the UE postpones the PUCCH transmission. The quantity  is given by the higher-layer parameter pucch-NumRepetitionCE-Msg4 for transmission of Msg4 in the random-access procedure [8], otherwise by pucch-NumRepetitionCE-Format1 for PUCCH format 1/1a and pucch-NumRepetitionCE-Format2 for PUCCH format 2/2a/2b,

The physical resource blocks to be used for transmission of PUCCH in subframe  within the   consecutive subframes are given by

where  is the absolute subframe number of the first uplink subframe intended for PUCCH.

The variable  depends on the PUCCH format.

-	Formats 1, 1a and 1b:

-	Formats 2, 2a and 2b:

-	Format 3:

-	Format 5 (non-BL/CE UEs only):

For non-BL/CE UEs, for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

where  is obtained from [4].

Mapping of modulation symbols for the physical uplink control channel for PUCCH formats 1 – 3 is illustrated in Figure 5.4.3-1.

In case of simultaneous transmission of sounding reference signal and PUCCH format 1, 1a, 1b, 3, 4 or 5 when there is one serving cell configured, a type-0 shortened PUCCH format shall be used where the last SC-FDMA symbol in the second slot of a subframe shall be left empty.

In case of guard period for narrowband retuning for BL/CE UEs,

-	For PUCCH format 1, 1a,

-	if the only SC-FDMA symbol affected by the guard period is the last SC-FDMA symbol in the second slot, a type-0 shortened PUCCH format shall be used.

-	if the first SC-FDMA symbol in the first slot of a subframe is left empty due to guard period, while the last SC-FDMA symbol is available for PUCCH transmission, a type-1shortened PUCCH format shall be used.

-	if the first SC-FDMA symbol in the first slot of a subframe is left empty due to guard period, and the last SC-FDMA symbol in the second slot of a subframe is left empty due to either guard period or sounding reference signal, a type-2shortened PUCCH format shall be used.

-	For PUCCH format 2, 2a, if an SC-FDMA symbol is left empty due to guard period, the SC-FDMA symbol shall be counted in the PUCCH mapping but not used for transmission of the PUCCH. The SC-FDMA symbol affected by the guard period can be the first SC-FDMA symbol in the first slot of a subframe and/or the last SC-FDMA symbol in the second slot of a subframe.

#### < Unchanged parts are omitted >

### 6.2.7	Narrowbands

A narrowband is defined as six non-overlapping consecutive physical resource blocks in the frequency domain. The total number of downlink narrowbands in the downlink transmission bandwidth configured in the cell is given by

The narrowbands are numbered   in order of increasing physical resource-block number where narrowband is composed of physical resource-block indices

where

#### < Unchanged parts are omitted >

### 6.4.1	Physical downlink shared channel for BL/CE UEs

For BL/CE UEs, the following additions and exceptions hold in addition to those in clause 6.4:

-	For BL/CE UEs resource elements occupied by CSI reference signals shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	For BL/CE UEs resource elements within PRBs containing PBCH repetitions shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

#### < Unchanged parts are omitted >

### 6.8B.2	Scrambling

Scrambling shall be perfomed according to Clause 6.8A.2 with EPDCCH replaced by MPDCCH except that the same scrambling sequence is applied per subframe to MPDCCH for a given block of  subframes and  is the MPDCCH set number. For an MPDCCH associated with a 2+4 PRB set as defined in [4],  is used to generate the scrambling sequence for mapping to REs in 6 PRBs.

#### < Unchanged parts are omitted >

### 6.8B.3	Modulation

Modulation shall be performed according to 6.8A.3 with EPDCCH replaced by MPDCCH.

#### < Unchanged parts are omitted >

### 6.8B.5	Mapping to resource elements

Mapping to resource elements shall be done according to Clause 6.8A.5with the following exceptions:

-	The term EPDCCH shall be replaced by MPDCCH.

-	The mapping shall be repeated across each of the  valid downlink subframes.

-	 is the number of ECCEs used for this MPDCCH in the first of the  subframes.

-	For an MPDCCH associated with a 2+4 PRB set as defined in [4], the mapping to resource elements  on antenna port  shall be in increasing order of first the index  and then the index over the 6 PRBs for MPDCCH format 5 and over the 2 or 4 PRBs for the other MPDCCH formats.

-	Resource elements occupied by CSI reference signals shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH. The UE may assume there is no MPDCCH transmission which uses overlapping sets of subframes as PDSCH transmissions to that UE, where the MPDCCH is located at a different narrowband than the PDSCH.

-	In the subframes where an MPDCCH or its associated PDSCH is transmitted in response to a physical random access transmission initiated by a PDCCH order, the UE shall receive the MPDCCH or its associated PDSCH, and assume no other UE-specific reception is needed.

-	For MPDCCH transmission associated with C-RNTI or SPS-RNTI, frequency hopping of the MPDCCH is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig is set.  For MPDCCH transmission associated with RA-RNTI or temporary C-RNTI, frequency hopping of the MPDCCH is enabled when higher layer parameter rar-HoppingConfig is set.

-	The narrowband  for MPDCCH transmission in the first subframe of MPDCCH monitoring instance is provided by higher layers. Starting subframe configuration of UE-specific search space where UE monitors an MPDCCH is also provided by higher layers. The MPDCCH is transmitted with  repetitions, spanning  consecutive subframes, including invalid subframes where the MPDCCH transmission is postponed.

-	If frequency hopping is not enabled for MPDCCH, the repetitions of an MPDCCH candidate are located at the same PRB resources in the same narrowband , and

-	if frequency hopping is enabled for MPDCCH, an MPDCCH candidate shall be transmitted in subframe  within the   consecutive downlink subframes using the same PRB resources within each narrowband

where  is the absolute subframe number of the first downlink subframe of MPDCCH monitoring instance, and ,  and  are a cell-specific higher-layer parameters. The UE shall not expect MPDCCH transmission in subframe  if it is not a valid downlink subframe.

-	The UE may assume the same precoding matrix being used for a PRB across a block of   consecutive subframes for MPDCCH.

The UE may assume that an MPDCCH associated with the P-RNTI is transmitted on the set  of narrowbands where  is defined in section 6.4.1. For a UE monitoring an MPDCCH associated with the P-RNTI, the first MPDCCH narrowband is given by  where ,  is the Paging Narrowband (PN) obtained according to [10], and  is the higher-layer parameter paging-narrowBands.

-	If the higher-layer parameter SIx-HoppingConfigCommon disables frequency hopping for an MPDCCH associated with P-RNTI, each MPDCCH candidate shall be located in the same PRB in narrowband  where .

-	If the higher-layer parameter SIx-HoppingConfigCommon enables frequency hopping for an MPDCCH with P-RNTI, an MPDCCH candidate shall be located in narrowband  in subframe  within the   consecutive downlink subframes using the same PRB resources within each narrowband  where

where  is the absolute subframe number of the first downlink subframe of MPDCCH monitoring instance according to locations of paging opportunity subframes, and ,  and  are a cell-specific higher-layer parameters. The UE shall not expect MPDCCH transmission in subframe  if it is not a valid downlink subframe.

#### < Unchanged parts are omitted >

#### 6.10.3A.1	Sequence generation

For any of the antenna ports , the reference-signal sequence  is defined by

.

For non-BL/CE UEs, the pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe where  and  is configured by higher layers. The EPDCCH set to which the EPDCCH associated with the demodulation reference signal belong is denoted .

For BL/CE UEs, the same scrambling sequence is applied per subframe to the demodulation reference signal associated with MPDCCH for a given block of  subframes. For the block of   subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for MPDCCH. The MPDCCH transmissions span  consecutive subframes, including invalid subframes where the MPDCCH transmission is postponed. For a BL/CE UE configured with CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2. The quantities  and  are configured by higher layers. The MPDCCH set to which the MPDCCH associated with the demodulation reference signal belong is denoted . For an MPDCCH associated with a 2+4 PRB set as defined in [4],  is used to generate the scrambling sequence for the 6 PRBs.

#### < Unchanged parts are omitted >