3GPP TSG RAN WG1 Meeting #84bis                                                           R1-163794

Busan, Korea, April 11-15, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0639 | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the collision between eMTC SIB and MPDCCH/PDSCH in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-04-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In RAN1 #83 meeting, we have the following agreements: - In case of collision between SIB1bis transmissions and M-PDCCH/PDSCH repetitions, - Drop M-PDCCH/PDSCH subframe, the unavailable subframe is counted in the repetition - In case of collision between scheduled SI transmissions and a M-PDCCH/PDSCH repetition in the same narrowband in a subframe - The UE shall assume M-PDCCH/PDSCH in the subframe is dropped, and the unavailable subframe is counted in the repetition The collision described in the agreement between SIB1bis/SI transmission and M-PDCCH/PDSCH repetition is in the same narrowband in a subframe, in which case the UE shall assume M-PDCCH/PDSCH repetition in the subframe is dropped. However, when M-PDCCH/PDSCH is transmitted in the same subframe as SIB1bis/SI transmission but in a different narrowband, the UE can still receive M-PDCCH/PDSCH in the subframe and will not drop it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that the UE shall assume M-PDCCH/PDSCH repetition is dropped in the subframe for SIB1bis/SI transmission only when they are in the same narrowband. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreements are not correctly captured in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.11, 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of the 1st Change >>

### 7.1.11	PDSCH subframe assignment for BL/CE using MPDCCH

A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A/6-1B/6-2 intended for the UE, adjust the corresponding PDSCH transmission in subframe(s) n+ki with i = 0, 1, …, N-1 according to the MPDCCH, where

- subframe n is the last subframe in which the MPDCCH is transmitted and is determined from the starting subframe of MPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

- subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE DL subframe(s) where, 2≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the corresponding DCI, where are given in Table 7.1.11-1 and Table 7.1.11-2 ;

If PDSCH carrying SystemInformationBlockType1-BR or other SI messages is transmitted in one narrowband in subframe n+ki, a BL/CE UE shall assume any other PDSCH in the same narrowband in the subframe n+ki is dropped.

#### < Unchanged parts are omitted >

#### << End of the 1st Change >>

<< Start of the 2nd Change >>

### 9.1.5	MPDCCH assignment procedure

#### < Unchanged parts are omitted >

For MPDCCH UE-specific search space, Type0-common search space, and Type2-common search space locations of starting subframe  are given by where is the th consecutive BL/CE DL subframe from subframe , and , and , and , where

- subframe  is a subframe satisfying the condition , where

- For MPDCCH UE-specific search space, Type0-common search space is given by the higher layer parameter mPDCCH-startSF-UESS,

- For Type2-common search space, is given by the higher layer parameter mPDCCH-startSF-CSS-RA-r13

- is given by higher layer parameter mPDCCH-NumRepetition, and

- ,, , are given in Table 9.1.5-3.

For Type1-common search space, = and is determined from locations of paging opportunity subframes,

If SystemInformationBlockType1-BR or other SI messages are transmitted in one narrowband in subframe , a BL/CE UE shall assume MPDCCH in the same narrowband in the subframe  is dropped. The BL/CE UE is not required to monitor an MPDCCH candidate if any ECCEs corresponding to the MPDCCH candidate occur within a frame other than and also occur within frame.

#### < Unchanged parts are omitted >

<< End of the 2nd Change >>