3GPP TSG RAN WG1 Meeting #85		                                                R1-165627

Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 678 | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRS frequency location in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-05-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | According to the agreement in RAN1 #83, existing R12 SRS configurations can be reused for BL/CE UEs configured with CEModeA. Based on R12 SRS configurations, the 4 PRBs of SRS for BL/CE UEs may contain the remaining PRBs not belonging to any narrowband, or may locate across two narrowbands. In other words, the SRS frequency location for BL/CE UEs may be not confined within a narrowband. The terminology “narrowband of the SRS” should be modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Not to use the terminology “narrowband of the SRS”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether the PRBs location for SRS for BL/CE UEs should be confined within a narrowband. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

## 8.2	UE sounding procedure

#### < Unchanged parts are omitted >

For a BL/CE UE, for a SRS transmission in subframe n and if the UE transmits PUSCH/PUCCH in subframe n and/or n+1, the UE shall not transmit the SRS in subframe n if the SRS transmission bandwidth in subframe n is not completely within the narrowband of PUSCH/PUCCH in subframe n and/or n+1.

A BL/CE UE shall not transmit SRS in UpPTS if SRS frequency location is different from DwPTS reception narrowband in the same special subframe.

#### < Unchanged parts are omitted >