3GPP TSG-RAN Meeting #73       				             RP-161648

New Orleans, U.S.A., September 19 - 22, 2016

Source: 	KT Corp.

Title: 	Text Proposal for TR 38.913: Spectrum Deployment

Agenda Item:	9.2.1

Document for:	Approval

# 1	Introduction

E-mail discussion [RAN#72-06] LTE/NR coexistence requirement for TR 38.913 [1] is being discussed with the aim of defining requirements for flexible resource allocation for co-existence and interworking with legacy 3GPP RATs. However, KT believes that in order to support flexible resource allocation for multiple RATs simple co-existence requirements needs to be defined as clause 8.3 in 3GPP TR 25.913. This contribution provides examples on possible NR and legacy 3GPP RAT operations in existing bands.

2 	Discussion

Widely deployed Band 1 for example, this band is being widely used for UMTS and LTE. Even for today, some operator has been migrated part of Band 1 into LTE and two different RATs are being deployed in the same spectrum band. It is likely that some operator may decide to replace UMTS directly to NR which will eventually have UMTS, LTE, and NR in the same operating Band as shown in Figure 1.

Figure 1. Example of multi-RAT operation in Band 1 with NR migration

TR 25.913 had captured co-existence requirements as below:

8.3	Spectrum deployment

E-UTRA is required to cope with following scenarios:

a)	Co-existence in the same geographical area and co-location with GERAN/3G on adjacent channels.

b)	Co-existence in the same geographical area and co-location between operators on adjacent channels.

c)	Co-existence on overlapping and/or adjacent spectrum at country borders.

d)	E-UTRA shall be possible to operate standalone, i.e. there is no need for any other carrier to be available.

e)	All frequency bands should be allowed following release independent frequency band principles

Note	In case of border co-ordination requirement, other aspects such as possible scheduling solutions should be considered together with physical layer behaviour.

For multi-RAT co-existence requirement in TR 38.913, above requirements should be good enough to resolve possible multi-RAT co-existence operations in the legacy spectrum. KT proposes to inherit above requirements from TR 25.913 and add requirements regarding flexible resource allocation which is being discussed as [RAN#72-06].

3 	Proposal

KT proposes to add below requirements in clause 10.3 of TR 38.913:

NR is required to cope with following scenarios:

a)	Co-existence in the same geographical area and co-location with GERAN/UMTS/LTE on adjacent channels operating in the same band

b)	Co-existence in the same geographical area and co-location between operators on adjacent channels operating in the same band

Text proposal for TR 38.913 is provided in the Annex.

4 	Reference

[1] RP-161334. 3GPP TR 38.913 v0.4.0 Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 14)

[2] 3GPP TR 25.913 v9.0.0 Requirements for Evolved UTRA and Evolved UTRAN (Release 9)

# Annex	Revision to TR 38.913

------------------------------------ BEGIN TEXT PROPOSAL 1 ---------------------------------

## 10.3	Co-existence and interworking with legacy RATs

NR is required to cope with following scenarios:

a)	Co-existence in the same geographical area and co-location with GERAN/UMTS/LTE on adjacent channels operating in the same band

b)	Co-existence in the same geographical area and co-location between operators on adjacent channels operating in the same band

    - Co-existence with LTE

10.3.2	Co-existence with UMTS and GSM/EDGE

------------------------------------ END TEXT PROPOSAL 1 -----------------------------------------