|  | Radiocommunication Study Groups |  |  |
| --- | --- | --- | --- |
| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
|  |  | 19 May 2016 |  |
| Working Party 5A |  |  |  |
| Guide to the use of itu-r texts relating to the land mobile service, Including wireless access in the fixed service |  |  |  |
| (Last update: 19 May 2016) |  |  |  |

Table of Contents

Page

1	Scope		2

2	Land Mobile Service		2

3	Spectrum Sharing		2

4	Interference		4

5	Technology		5

6	Public Protection and Disaster Relief		6

7	Intelligent Transport Systems (ITS)		8

8	Wireless Access, including RLANs		9

9	Trunked Systems		12

10	Cellular Systems		13

11	Cordless Telecommunication Systems		13

12	Personal Radio		14

13	Other Systems (Paging Systems, Shared Private)		14

Contact: José Costa (Canada), Chairman, Radiocommunication Working Party 5A, jose.costa@ericsson.com

1	Scope

This document describes the various topics that are relevant to the Land Mobile Service, including wireless access in the fixed service, and identifies the ITU-R texts (Recommendations, Reports and Handbooks) applicable to these topics. The information included in this document is based on completed work within the ITU-R. The ongoing work on these topics is documented in the latest WP 5A Chairman’s Report.

Useful links:

| Free online access to ITU-R Publications, Software and Databases: | http://www.itu.int/oth/R040200003C/en |
| --- | --- |
| Search ITU Publications: | http://www.itu.int/en/publications/Pages/Search.aspx |

# 2	Land Mobile Service

As defined in Article 1 of the Radio Regulations, Land Mobile Service is a mobile service between base stations and land mobile stations, or between land mobile stations (1.27). Land Mobile Service can be subdivided into a number of different topics. Based on the past and ongoing work within the ITU on land mobile service, the following topics have been established: Spectrum Sharing, Interference, Technology, Public Protection and Disaster Relief (PPDR), Intelligent Transport Systems (ITS), Wireless Access including RLANs, Trunked Systems, Cellular Systems, Cordless Telecommunication Systems, Personal Radio, and Other Systems (Paging Systems, Shared Private). Each of these topics is described below, including relevant definitions and the identification of the applicable ITU-R texts available.

The various definitions related to these topics of the land mobile service have been assembled to create a vocabulary of terms for the terrestrial mobile service (Recommendation ITU-R M.1797).  Other vocabulary Recommendations exist for wireless access (Recommendation ITU-R F.1399) and the fixed service in general (Recommendation ITU-R F.592).

A complete list of relevant Recommendations, Reports and Handbooks is contained in the Annex to this Guide.

# 3	Spectrum Sharing

Since the frequency bands allocated to the Land Mobile Service are shared with other radio-communication services or that some of these bands may be used by different types of systems in the Land Mobile Service, there has been a need to develop ITU-R deliverables on dealing with the issue of sharing in relation with the Land Mobile Service.

These ITU-R texts cover both general methodologies that can apply to various cases and specific sharing scenarios where detailed analysis and means to improve the sharing were required. In particular, Recommendation ITU-R M.1825 provides guidance to perform sharing studies related to systems in the land mobile service. It establishes a list of parameters that characterize a system to assist in sharing studies, provides information on the methodologies that can be used for sharing analyses involving the land mobile service and describes mitigation techniques that can improve spectrum sharing.

For the purpose of sharing studies involving land mobile systems, probabilistic considerations may be required to determine the sensitivity of variations of some key parameters. Recommendation ITU-R M.1634 recommends the use of Monte Carlo simulation analysis methodology, among other suitable methodologies, to calculate interference probability when performing studies of interference involving terrestrial mobile service systems. More details on the Monte Carlo simulation methodology for the use in sharing and compatibility studies between different radio services and systems can be found in Report ITU-R SM.2028.

Since there are several frequency bands, especially below 1 GHz, in the Radio Regulations that are allocated to both the Mobile and the Mobile-Satellite services, ITU-R produced Recommendation ITU-R M.1039 dealing with co-frequency sharing between stations in the mobile service below 1 GHz and mobile earth stations of non-geostationary mobile-satellite systems (Earth-space) using frequency division multiple access.

Due to the fact that many bands below 1 GHz are shared between the land mobile service and the broadcasting service (television), Report ITU-R M.1023 was published in 1990, providing guidance on the possibility of frequency sharing between the land mobile service and the broadcasting service, in particular analogue television below 1 GHz.  In addition, Recommendation ITU-R M.1767 gives protection criterion of land mobile systems from terrestrial digital video and audio broadcasting systems in the VHF (174-230 MHz) and UHF (470-862 MHz) shared bands allocated on a primary basis, where appropriate.

The band 1452-1492 MHz, allocated to the mobile service on a primary basis, is also allocated to the broadcasting-satellite service (sound) subject to the provisions of the Radio Regulations (RR) Nos. 5.345, 5.347 and 5.347A. For that reason, Recommendation ITU-R M.1388  deals with threshold levels to determine the need to coordinate between space stations in the broadcasting-satellite service (sound) and particular systems in the land mobile service in the band 1 452-1 492 MHz.

In addition, it should be noted that several ITU-R texts have been developed on sharing issues related with Wireless Access including RLANs (see Section 8 for further details).

Characteristics of systems and specific compatibility studies: A number of Recommendations and Reports are being developed which contain characteristics of systems for sharing studies.  The following ones are in force:

•	Recommendation ITU-R M.1795  provides technical and operational characteristics of land mobile service MF/HF systems for use in sharing studies.

•	Recommendation ITU-R M.1808 provides technical and operational characteristics of conventional and trunked land mobile systems to be used in sharing studies. Given the variety of those systems within the mobile service below 869 MHz, a range of parameters and typical values are provided for different analogue as well as digital systems.

•	Recommendation ITU-R M.1823 provides technical and operational characteristics of digital cellular land mobile systems for use in sharing studies. It provides the relevant characteristics for some specific systems used for digital cellular land mobile communications, and complements the information available in Recommendation ITU-R M.1073.

•	Recommendation ITU-R M.1824 provides system characteristics of television outside broadcast (TVOB), electronic news gathering (ENG) and electronic field production (EFP) in the mobile service to assist sharing studies, contains the typical operational and technical characteristics of broadcast auxiliary services (BAS), which are required for sharing studies between the BAS in the mobile service and other radiocommunication services.

•	Report ITU-R M.2116 provides characteristics for a number of terrestrial broadband wireless access (BWA) systems, including mobile and nomadic applications, operating, in the mobile service for use in sharing studies between these terrestrial BWA systems and other fixed or mobile systems.

•	Recommendation ITU-R M.2068 provides characteristics of and protection criteria for systems operating in the mobile service in the frequency range 14.5-15.35 GHz.

•	Report ITU-R M.2335 contains a sharing and compatibility analysis of possible amateur service stations with fixed, land mobile, and radiolocation services in the frequency band 5 250-5 450 kHz and the aeronautical mobile service in an adjacent band

•	Report ITU-R S.2199 examines the possibility of compatibility between broadband wireless access (BWA) systems and FSS networks in the range 3 400-4 200 MHz for both co-channel and adjacent channel operations.

4	Interference

The term “interference” is defined in article 1.166 of the Radio Regulations as “The effect of unwanted energy due to one or a combination of emissions, radiations, or inductions upon reception in a radiocommunication system, manifested by any performance degradation, misinterpretation, or loss of information which could be extracted in the absence of such unwanted energy.”

Within the framework of the ITU-R, a number of studies have been carried out on the concept of harmful interference (RR No. 1.169) and the ways to address it.

The main effects that characterise interference are:

–	unwanted emissions (RR No. 1.146) that consist of spurious emissions and out-of-band emissions,

–	blocking of the receiver,

–	effect of intermodulation products.

These effects are not necessarily specific to the Land Mobile Service and are described in some essential ITU-R texts that have been developed within Study Group 1 and that are applicable, inter alia, to the Land Mobile Service, such as Recommendation ITU-R SM.329 on “unwanted emissions in the spurious domain”, Recommendation ITU-R SM.1541 on “unwanted emissions in the Out of Band domain” (where, in particular, Annex 10 provides the out-of band emission limits for Land Mobile systems). Other texts from the SM series deal more specifically with intermodulation effects, in particular in relation with Land Mobile Service, such as Recommendation ITU-R SM.1134 and Report ITU-R SM.2021.

In addition to those general texts, ITU-R has developed deliverables on issues related to interference that are more specific to the Land Mobile Service.

On the complex issue of intermodulation products, Report ITU-R M.739 contains detailed information on the production of intermodulation products in transmitters and the intermodulation response at the output of a receiver for the Land Mobile service between 25 and 1000 MHz. Recommendation ITU-R M.1072 provides possible means to reduce interference due to intermodulation products in the land mobile service between 25 and 3000 MHz.

Report ITU-R M.2141 provides information on the determination of isolation for mobile radio antennas separated horizontally less than a few wavelengths in the 30 to 108 MHz bands. The study presents analytical and experimental isolation results for two antennas.

•	Propagation:  The ITU-R Handbook “Terrestrial land mobile radiowave propagation in the VHF/UHF bands” (2002) gives the technical basis for predicting the radio propagation in point-to-point, point-to-area, and point-to-multipoint mobile networks.

# 5	Technology

Technology is the key enabler of all the land mobile systems and applications and over the years, a number of ITU-R Recommendations and Reports have been developed to address specific technology issues at the time.  However, technology advances very quickly and in many cases it relates to implementation issues that are not necessarily in the scope of the work of ITU-R.  Therefore, the reader should exert caution when considering the Recommendations and Reports outlined in this section; they all contain valuable information but in some cases, the technologies may have been surpassed and due to priorities, it has not been possible to update or suppress obsolete information.  This overview organizes the existing texts about technology in the following applications: channel assignments, impaired hearing solutions, transmission, PSTN in aircrafts, antennas, and implementation technologies.

Channel assignments:  Recommendation ITU-R M.478 gives the preferred technical characteristics for VHF and UHF land mobile equipment using F3E class of emission, including the considerations of the necessary bandwidth, the transmitter, the receiver and the station characteristics.  Rep. ITU-R M.319 covers the characteristics of equipment and principles governing the allocation of frequency channels in terms of assignment methods, components and engineering techniques, and national practices.

Impaired hearing solutions: Recommendation ITU-R M.1076 recommends the technical parameters for radiocommunication systems for persons with impaired hearing and the practical application of infrared systems and audio frequency induction loops to communicating with persons with impaired hearing that should be considered for some applications. Report ITU-R M.778 covers techniques to transfer speech signals from a microphone to the listener’s hearing device, including infra-red radiation, the magnetic induction field internal to current loops, VHF radio and the external induction field of a radiating antenna.

Transmission:

Report ITU-R M.2026 introduces in detail real-zero single sideband technology (RZ SSB), which is a combination of two technologies employing digital signal processing techniques; SSB modulation and phase modulation (PM) reception.

Report ITU-R M.1021 (1986) describes major characteristics of land mobile digital modulation equipment: BER performance (sensitivity), adjacent signal selectivity, and acceptable bandwidth.

Report ITU-R M.2117 addresses the application and implications of software-defined radio (SDR) to land mobile systems, including issues on the efficient use of spectrum using SDR techniques and adaptive control mechanisms, frequency-sharing issues relating to SDR and general technical issues.

PSTN in aircrafts: Report ITU-R M.2282 deals with the general principles, technical characteristics and operational features of terrestrial systems for public mobile communications with aircraft, including descriptions of systems for public communications with aircraft in some countries in Region 1, Region 2, and Region 3.Antennas:  Rec. ITU-R M.1678 recommends aspects on the use of adaptive antenna systems.  Technical details of adaptive antennas are in Rep. ITU-R M.2040.  Rec. ITU-R M.1075 and Rep. ITU-R M.902 cover a special case of antennas known as leaky feeders, which are used wherever mobile communication is required in any underground or confined environment where conventional methods of propagation are known to be inadequate.

Implementation technologies:  Report ITU-R M.2114 defines the essential technical and operational characteristics needed to support IP applications over mobile systems. Report ITU-R M.2117 addresses the application and implications of software defined radio to mobile systems, including, but not limited to, IMT-2000 and systems beyond, dispatch systems, intelligent transport systems (ITS), public mobile systems including public protection and disaster relief (PPDR), and first and second generation cellular systems including their enhancements. It addresses issues on the efficient use of spectrum using SDR techniques and adaptive control mechanisms, frequency sharing issues relating to SDR and general technical issues.

Cognitive Radio Systems (CRS): Report ITU-R M.2225 addresses the cognitive radio systems in the land mobile service (LMS) above 30 MHz (excluding international mobile telecommunications (IMT)). It provides a general description of cognitive radio systems addressing technical features and capabilities, potential benefits and technical challenges. It also describes a set of deployment scenarios.  Report ITU-R M.2330 presents the existing, emerging and potential applications employing CRS capabilities and the related enabling technologies, including the impacts of CRS technology on the use of spectrum from a technical perspective. The description of such technologies, operational elements and their challenges are also presented. Furthermore this Report provides high level characteristics, operational and technical requirements related to CRS technology, their performances and potential benefits. Finally, factors related to the introduction of CRS technologies and corresponding migration issues are discussed.

# 6	Public Protection and Disaster Relief

Radiocommunication services have become extremely important to Public Protection and Disaster Relief (PPDR) organizations to the extent that PPDR communications are highly dependent upon a range of radiocommunication services, particularly the mobile service. Experience from recent major disaster events has shown that at times, these agencies are solely dependent on radiocommunication services as the only form of communications available. Trunked systems (see Section 9) are often used for PPDR applications.

In order to provide effective communications, PPDR agencies and organizations have a set of objectives and requirements that include interoperability between agencies and staff in the field, reliability, functionality, security in operations and fast call set-up for rapid access to wider communication networks. Considering that the functional needs of PPDR agencies and organizations are growing, future advanced solutions used in PPDR applications will require higher data rates than narrow-band solutions predominantly in use today, along with video and multimedia capabilities.

Prior to WRC-07, studies were undertaken within the ITU-R in order to identify from a spectrum management perspective technical and operational requirements of future advanced solutions to meet the needs of public protection agencies, including those dealing with emergency situations and disaster relief.

At RA-07, the Radiocommunication Assembly adopted two Resolutions.  Resolution ITU-R 53 covers “the use of radiocommunications in disaster and relief response” while Resolution ITU-R 55 provides a detail list of “ITU studies of disaster prediction, detection, mitigation and relief.”

The decisions taken by WRC-03 on PPDR are contained in Resolution ITU-R 646 (Rev.WRC-12) “Public protection and disaster relief”. This resolution provides recommendations on the use of harmonized frequency bands for PPDR applications, cross-border circulation of equipment, interoperability between agencies, the use of advanced technologies as well as encouragement to manufacturers of equipment used in PPDR to take into account the needs of administrations and PPDR agencies.

At WRC-07 two Resolutions were developed addressing PPDR. In Resolution 644 (Rev. WRC-12), the ITU-R was asked to study those aspects of Radiocommunications/ICT that are relevant to early warning, disaster mitigation and relief operations; while the ITU- R Study Groups were asked to accelerate their work, particularly in the areas of disaster prediction, detection, mitigation and relief. In Resolution 647 (Rev.WRC-12), the ITU-R was invited to conduct studies in support of the establishment of appropriate spectrum management guidelines applicable in emergency and disaster relief operations.

WRC-12 agreed upon Agenda Item 1.3 for WRC-15: “to review and revise Resolution 646 (Rev.WRC 12) for broadband public protection and disaster relief (PPDR), in accordance with Resolution 648 (WRC-12)”. In addition, WRC-12 revised Resolutions 644, 646, and 647.

Recommendation ITU-R M.1042 “Disaster communications in the amateur and amateur-satellite services” provides guidance on the roles of the amateur and amateur satellite services in providing communications during disaster situations.

Report ITU-R M.2085 “Role of the amateur and amateur-satellite services in support of disaster mitigation and relief” is intended to document the role of the amateur and amateur-satellite services in provision of radiocommunications in support of disaster mitigation and relief. It includes information developed after the South-East Asia tsunami in December 2004.

Recommendation ITU-R M.1637 “Global cross-border circulation of radiocommunication equipment in emergency and disaster relief situations” offers guidance to facilitate the global circulation of radiocommunication equipment in emergency and disaster relief situations including the need for plans and procedures to be in place before a possible disaster event in order to facilitate the speedy authorization of the use of such equipment.

Report ITU-R M.2377 “Radiocommunication objectives and requirements for Public Protection and Disaster Relief (PPDR)” defines the PPDR objectives and requirements for the implementation of future advanced solutions to satisfy the operational needs of PPDR organizations. Specifically, it addresses:

–	the categorization of operational, technical and functional objectives and requirements relating to PPDR systems;

–	the use of PPDR systems, not only in terms of generic capabilities, but also as they vary according to narrowband, wideband and broadband capabilities;

–	the development of mobile broadband PPDR services and applications enabled by the evolution of advanced broadband technologies;

–	the efficient and economical use of the radio spectrum; and

–	the needs of developing countries.

Recommendation ITU-R M.2015 “Frequency arrangements for public protection and disaster relief radiocommunication systems in UHF bands in accordance with Resolution 646 (Rev.WRC-12)”  provides guidance on frequency arrangements for public protection and disaster relief radiocommunications in certain regions in some of the bands below 1 GHz identified in Resolution 646 (Rev.WRC-12).

Recommendation ITU-R M.2009 “Radio interface standards for use by public protection and disaster relief operations in some parts of the UHF band in accordance with Resolution 646 (WRC-03)” identifies radio interface standards applicable for public protection and disaster relief (PPDR) operations in some parts of the UHF band.

Recommendation ITU-R M.1826 addresses harmonized frequency channel plans in the band 4 940-4 990 MHz for broadband public protection and disaster relief radiocommunications in Regions 2 and 3.

Recommendation ITU-R M.1746 addresses system interoperability and harmonized frequency channel plans for the protection of property using data communication.

# 7	Intelligent Transport Systems (ITS)

Intelligent Transport Systems (ITS) are defined as systems utilizing the combination of computers, communications, positioning, and automotion technologies to improve the safety, management, and efficiency of terrestrial transportation. Work on ITS within the ITU was initiated in 1995, due to a significant increase in traffic on the roads, along with the growing need to integrate new technologies into land transport systems. At that time, ITS was referred to as Transport Information and Control Systems (TICS). In 2003, the term ‘TICS’ was replaced by ‘ITS’, due to the more prevalent use of the term ‘ITS’ within the industry; however, the definition remains the same for both terms.

Recommendation ITU-R M.1890 provides the objectives and radio requirements of ITS. This Recommendation establishes a framework or foundation for subsequent studies and output that are technology (such as DSRC, collision avoidance radar, or ITS millimetre wave radiocommunication) or application specific.

Dedicated Short Range Communications (DSRC), a fundamentally unique application to ITS, uses non-voice radiocommunication techniques to transfer data over short distances between a roadside infrastructure and mobile units. Recommendation ITU-R M.1453 describes the technical and operational characteristics of DSRC for ITS applications in the 5 725-5 875 MHz 
(centre frequency 5.8 GHz) Industrial, Scientific and Medical (ISM) band.

Another important application of ITS is the use sensor technologies for the monitoring and identifying of objects near vehicles. It was determined that radar using radio waves is suitable for this application. Subsequently, Recommendation ITU-R M.1452 was developed to identify the system requirements for low power, short-range vehicular radar operating in the 60-61 GHz and 
76-77 GHz bands.

Recommendation ITU-R M.2057 provides system characteristics of automotive radars operating in the frequency band 76-81 GHz for intelligent transport systems applications.

Recommendation ITU-R M.2084 identifies specific radio interface standards of vehicle-to-vehicle and vehicle-to-infrastructure communications for ITS applications. The technical and operational characteristics described in this Recommendation are based on current and existing frequency bands already in use for ITS and the applications in the mobile service.

Report ITU-R M.2228 provides characteristics, requirements and status of advanced ITS radiocommunications in various countries.

Finally, Volume 4 of the Land Mobile Handbook (including Wireless Access), provides a tutorial on Intelligent Transport Systems (ITS), with specific examples of ITS applications. This Volume also complements the Recommendations completed to date, and provides examples of ITS applications and on-going work.

# 8	Wireless Access, including RLANs

Wireless access has been defined as end-user radio connection(s) to core networks. In the 1990s, cellular mobile technologies became attractive to support fixed telephone access in many areas.  ITU-R has recommended a vocabulary for Wireless Access (Recommendation ITU-R F.1399).

8.1	Fixed Wireless Access (FWA)

One of the origins of FWA is perhaps in the use of mobile-derived cellular technologies to provide wireline access replacement to the PSTN, which became very attractive in the 1990s. A recommendation established the basic system requirements and performance objectives for fixed wireless access using mobile-derived technologies offering telephony and data communication services (Recommendation ITU-R F.757).

Two important considerations for the successful deployment of FWA systems are the frequency bands that can be used for such an application and the quality of service (QoS) requirements, particularly if FWA is to be considered as a competitive alternative to wireline access.

Recommendation ITU-R F.1400 specifies performance and availability requirements and objectives for fixed wireless access to public switched telephone network.  These requirements are based on ITU-T Recommendations and are given for three classes of telecommunication services in terms of the supported bit rates.  Recommendation ITU-R F.1490 provides generic requirements for FWA systems mainly from the point of view of integration with the core network.

Recommendation ITU-R F.1401 describes general characteristics of the bands in the frequency spectrum that may be used for FWA systems and it also provides considerations for the identification of possible frequency bands, including examples of Recommendations dealing with spectrum sharing studies. This Recommendation includes the identification of mobile service bands for fixed wireless access.

To facilitate the deployment of FWA systems from a regulatory perspective, it is desirable to specify the frequency arrangements in terms of blocks rather that by carrier, which was the traditional way previously.  Recommendation ITU-R F.1488 recommends the frequency block arrangements for FWA systems in the range 3 400-3 800 MHz, and it is perhaps the first Recommendation specifying frequency arrangements in terms of blocks. Other examples of frequency block arrangements are reflected also in Recommendations ITU-R F.748 and 
ITU-R F.749 which handle RF arrangements in the frequency ranges of 25-28 GHz and 38 GHz, respectively. Furthermore, Recommendation ITU-R F.1519 specifies general principle of the block arrangements.

Specific frequency sharing considerations for FWA systems are addressed in a number of recommendations, including:

•	Recommendation ITU-R F.1402, which recommends frequency sharing criteria between a land mobile wireless access system and a fixed wireless access system using the same equipment type as the mobile wireless access system.

•	Recommendation ITU-R F.1489, which recommends a methodology for assessing the level of operational compatibility between FWA and radiolocation systems when sharing the band 3.4-3.7 GHz.

•	Recommendation ITU-R F.1518, which recommends a spectrum requirement methodology when a FWA system and a mobile wireless access (MWA) system using the same type of equipment coexist in the same frequency band.

•	Recommendation ITU-R F.1613, which recommends operational and deployment restrictions for FWA systems in Region 3 to ensure the protection of systems in the EES service (active) in the band 5 250-5 350 MHz.

There is also a standard for fixed broadband wireless access (BWA) radio transmission systems based on cable modem standards, given in Recommendation ITU-R F.1499.

Recommendation ITU-R F.1763 specifies radio interface standards for broadband wireless access (BWA) systems in the fixed service operating below 66 GHz.

There are a number of other Recommendations and Reports that contain elements applicable to both wireless access in the fixed service and fixed systems.  These are included in the Annex but they are not described in this guide unless specific aspects need to be highlighted.

From a practical perspective many aspects of the deployment of FWA systems are covered in Volume 1 of the Land Mobile Handbook (including Wireless Access).

  - Nomadic Wireless Access

The major usage within the category Nomadic Wireless Access is for radio local area networks (RLANs). Elimination of the cables in a conventional wired network has obvious attractions: for example, in a commercial environment there may be considerable savings in both cost and time to install a network. However, a number of factors have driven the demand for RLANs further:

•	Domestic broadband installations are particularly suited to using RLANs as a means of providing cheap, flexible, connectivity to multiple computers used in the home.

•	The increasing use of portable computers, for both business and domestic use and the increase in flexible working practices, e.g. nomadic workers using laptop PC’s not just in the office and home, but in hotels, conference centres, airports, trains, and planes.

More information on RLANs can be found in Recommendation ITU-R M.1450, which recommends characteristics of broadband radio local area networks

In anticipation of greater demand as usage increases in terms of both applications and users, the ITU-R conducted studies supporting the primary allocation of spectrum in the 5 GHz bands at WRC-03. This required the use of innovative interference mitigation techniques to facilitate sharing with existing users of these bands. Results from this work are detailed in relevant recommendations listed below:

Recommendation ITU-R M.1454, which recommends e.i.r.p. density limit and operational restrictions for RLANS or other wireless access transmitters in order to ensure the protection of feeder links of non-geostationary systems in the mobile-satellite service in the frequency band 5 150-5 250 MHz.

Recommendation ITU-R M.1651, which recommends a method for assessing the required spectrum for broadband nomadic wireless access systems including radio local area networks using the 5 GHz band.

Recommendation ITU-R M.1652-1, which recommends a dynamic frequency selection (DFS) in wireless access systems including radio local area networks for the purpose of protecting the radiodetermination service in the 5 GHz band.

Report ITU-R M.2115-1 consolidates the DFS test methodology used and findings across several administrations, as shown in several annexes to the report.  Information is provided on the test methodologies in place in various administrations and/or regional groups to test compliance with DFS requirements.  These procedures may be updated over time, and as technology evolves.  As a result, web links are provided (in some cases) to the test methodologies themselves, so that the most up-to-date information may be obtained. Recommendation ITU-R M.1653, which recommends operational and deployment requirements for wireless access systems including radio local area networks in the mobile service to facilitate sharing between these systems and systems in the Earth exploration-satellite service (active) and the space research service (active) in the band 5 470-5 570 MHz within the 5 460-5 725 MHz range.

Report ITU-R M.2034, which addresses the impact of radar detection requirements of dynamic frequency selection on 5 GHz wireless access system receivers.

Recommendation ITU-R M.1739 provides protection criteria for wireless access systems, including radio local area networks (WAS/RLAN), operating in the mobile service in accordance with Resolution 229 (Rev.WRC-12), for the purposes of carrying out compatibility studies with services or applications from which WAS/RLAN systems are to be protected.

  - Mobile Wireless Access

Mobile wireless access systems have the capabilities to support not only mobile, but also fixed and nomadic applications.  These systems support a wide range of applications in urban, suburban and rural areas for both generic broadband internet data and real-time data, including applications such as voice and videoconferencing.

Recommendation ITU-R M.1801 “Radio interface standards for broadband wireless access systems, including mobile and nomadic applications, in the mobile service operating below 6 GHz” identifies specific radio interface standards for BWA systems in the mobile service operating below 6 GHz. The standards included in this Recommendation are capable of supporting users at broadband data rates, taking into account the ITU-R definitions of “wireless access” and “broadband wireless access” found in Recommendation ITU-R F.1399. Using this Recommendation, manufacturers and operators should be able to determine the most suitable standards for their needs.

Report ITU-R M.2378 “Operational guidelines for the deployment of broadband wireless access systems for local coverage operating below 6 GHz”deals with a specific operation concept of local coverage for Broadband Wireless Access (BWA) systems in frequency bands below 6 GHz. From a technical point of view, this Report is intended to provide some operational guidelines for the deployment of BWA systems for local coverage in the frequency bands above those frequencies typically used for macro cell deployments. The study in this Report addresses technical analysis of local coverage, modelling of categorized operational scenarios and analysis to investigate possibilities of enhancing operational flexibilities,  Taking advantage of properties of local coverage, the study focuses on flexible and efficient operation of BWA systems such as multi-standard radio operation and increasing degree of freedom in operational conditions. Possibilities are presented for flexible operation of BWA systems under certain operational conditions of local coverage.

8.4	Multiple Gigabit Wireless Systems

# Multiple Gigabit Wireless Systems (MGWS) radiocommunication networks can be used in short-range, line-of-sight and non-line-of-sight circumstances. Total communication range and performance will vary depending on the environment, but multiple gigabit performance is typically expected at ranges around 10 m for in-room use. These networks can be deployed with an access point as in existing WLAN deployments or without such an infrastructure such as in both WLAN in ad hoc mode and wireless personal area network (WPAN).

# When access points are used, they are mounted indoor with service covering home or an office space with a nomadic user terminal typically also used indoor, i.e. the entire WLAN system would be used in indoor environment.

# When access points are not used, MGWS devices are allowed to communicate by setting up direct links for data exchange between the devices/equipment. Typical applications include equipment to equipment (e.g. laptop to projector) and a consumer electronics (CE) device to a kiosk , and it may be assumed that usage would predominantly be indoors.

Recommendation ITU-R M.2003 provides general characteristics and radio interface standards for Multiple Gigabit Wireless Systems in frequencies around 60 GHz.

Report ITU-R M.2227 describes applications, deployment scenarios, technical characteristics, and specifications of Multiple Gigabit Wireless Systems.

8.5	Wide-area Sensor and Actuator Network (WASN) Systems

Wide-area sensor and/or actuator network (WASN) systems support machine-to-machine communications to a large number of sensors and/or actuators.

Recommendation ITU-R M.2002 provides the objectives, system characteristics, functional requirements, service applications and fundamental network functionalities for mobile wireless access systems (WAS) providing communications to a large number of ubiquitous sensors and/or actuators scattered over wide areas in the land mobile service. The key objective of wide area sensor and/or actuator network (WASN) systems is to support machine-to-machine service applications irrespective of machine location.

Report ITU-R M.2224 provides detailed information for system design policy, the wireless applications and examples of wide area sensors and/or actuators network (WASN) systems for information sharing.

# 9	Trunked Systems

Analogue trunked systems were developed in the 1980s as a way of achieving more spectrally efficient dispatch land mobile systems that could also be interconnected with the PSTN.  Analogue trunked systems are no longer being deployed.

Digital trunked systems of the type that were developed in the 1990s are described in Report ITU-R M.2014 revised in 2012, entitled “Digital land mobile systems for dispatch traffic”.

Descriptions of the technical and operational characteristics of trunked systems are summarised in Volume 3 “Dispatch and Advanced Messaging Systems” of the Land Mobile Handbook published by the ITU-R in 2005.

# 10	Cellular Systems

Cellular systems started operations in the early 1980s to support public communications for mobile users.  The first-generation systems used bulky analogue radio equipment and could only be used in vehicles.  The services provided by cellular systems became very successful and as technology advanced, including the use of digital techniques, better systems were deployed that enable the use of hand-held mobile stations.  These second-generation systems using digital transmission techniques started to be deployed in the early 1990s. Their integration is covered in Recommendation ITU-R M.1074 and Report ITU-R M.1157. Report ITU-R M.1156 covers various generic and specific aspects of these public land mobile cellular telephone systems.  The adaptation of mobile Radiocommunication technology to the needs of developing countries is addressed in Report ITU-R M.1155.  The characteristics of digital cellular land mobile telecommunication systems are collected in Recommendation ITU-R M.1073, which has undergone revisions over the years to reflect the latest standards (the most recent one is dated 2012).

IMT-2000 are third-generation systems that started to be deployed in the year 2000, and recommendations and guidance for the evolution of pre-IMT-2000 systems towards IMT-2000 are provided in Recommendation ITU-R M.1308 and Volume 2 of the Land Mobile Handbook.

Recommendation ITU-R M.1767 establishes a protection criterion of land mobile systems from terrestrial digital video and audio broadcasting systems in the VHF (174-230 MHz) and UHF (470-862 MHz) shared bands allocated on a primary basis, where appropriate. It provides the methodology and formulas to assess the maximum allowable field strength of digital terrestrial broadcasting signals into the land mobile system bandwidth, also taking into account the case of potential partial overlap in frequencies between both systems. Some examples are provided to illustrate the use of this methodology. In addition, for some specific types of land mobile systems and specific types of interfering digital television signals, measured values of protection ratios are included.

Report ITU-R M.2264 provides guidance on the development of frequency arrangements for systems operating in large contiguous bandwidths in the mobile service, with a view to assisting administrations on spectrum planning issues.

# 11	Cordless Telecommunication Systems

Cordless telecommunication systems, including cordless telephones, provide the ability to communicate in areas of up to a few hundred meters and can be connected to the public switched telephone network.  A Recommendation and a Report cover the characteristics of such systems:

•	Recommendation ITU-R M.1033, Technical and operational characteristics of cordless telephones and cordless telecommunication systems.

•	Report ITU-R M.1025, Technical and operating characteristics of cordless telephones.

# 12	Personal Radio

Personal radio refers to the use of personal radio devices for direct communications without a central controller or infrastructure network.  These systems are sometimes referred to as wireless ad-hoc networks or wireless peer-to-peer networks.  Currently, there are no Recommendations or Reports in force on this topic.

# 13	Other Systems (Paging Systems, Shared Private)

Paging systems: In the 1980s and 1990s, a solution emerged to reach people but with the advances in and widespread deployment of cellular systems, dedicated paging systems have fallen in disuse.  However, advanced paging functions are integrated with other communication systems.   Recommendation ITU-R M.584 describes the operational characteristics, including codes and formats, of the prevailing systems in the 1980s and 1990s.  A more recent handbook, Volume 3 of the Land Mobile handbook, describes some advanced messaging systems.

Annex A

All documents sorted by topic

| Number | Rev. | Title | Year | Relevant Topic(s) |
| --- | --- | --- | --- | --- |
| M.1073 | 3 | Digital cellular land mobile telecommunication systems | 2012 | Cellular Systems |
| M.1074 | 0 | Integration of public mobile radiocommunication systems | 1993 | Cellular Systems |
| M.1308 | 0 | Evolution of land mobile systems towards IMT-2000 | 1997 | Cellular Systems |
| M.1155 | Rep | Adaptation of mobile radiocommunication technology to the needs of developing countries (Note: with WP 5D) | 1990 | Cellular Systems |
| M.1156 | Rep | Digital cellular public land mobile telecommunication systems (DCPLMTS) | 1990 | Cellular Systems |
| Volume 2 | 0 | Land Mobile Handbook (including wireless access), Volume 2: Principles and approaches on evolution to IMT-2000 | 1997 | Cellular Systems |
| M.1157 | Rep | Integration of public mobile radiocommunication systems | 1990 | Cellular Systems |
| M.1767 | 0 | Protection of land mobile systems from terrestrial digital video and audio broadcasting systems in the VHF and UHF shared bands allocated on a primary basis | 2006 | Cellular Systems |
| M.1823 | 0 | Technical and operational characteristics of digital cellular land mobile systems to be used in sharing studies | 2007 | Cellular Systems Spectrum Sharing |
| M.1825 | 0 | Guidance on technical parameters and methodologies for sharing studies related to systems in the land mobile service | 2007 | Cellular Systems Spectrum Sharing |
| M.1033 | 1 | Technical and operational characteristics of cordless telephones and cordless telecommunication systems | 1997 | Cordless Telecommunication Systems |
| M.1025-1 | Rep | Technical and operating characteristics of cordless telephones | 1990 | Cordless Telecommunication Systems |
| M.1890 | 0 | Intelligent transport systems - Guidelines and objectives | 2011 | Intelligent Transport Systems |
| M.1452 | 2 | Transport information and control systems – Low power short-range vehicular radar equipment at 60 GHz and 76 GHz | 2012 | Intelligent Transport Systems |
| M.1453 | 2 | Intelligent transport systems - Dedicated short range communications at 5.8 GHz | 2005 | Intelligent Transport Systems |
| M.2084 | 0 | Radio interface standards of vehicle-to-vehicle and vehicle-to-infrastructure communications for Intelligent Transport System applications | 2015 | Intelligent Transport Systems |
| M.2228-1 | Rep | Advanced intelligent transport systems (ITS) radiocommunications | 2015 | Intelligent Transport Systems |
| Volume 4 |  | Land Mobile Handbook (including wireless access), Volume 4: Intelligent Transport Systems (ITS) | 2006 | Intelligent Transport Systems |
| M.1072 | 0 | Interference due to intermodulation products in the land mobile service between 25 and 3 000 MHz | 1994 | Interference |
| M.739-1 | Rep | Interference due to intermodulation products in the land mobile service between 25 and 100 MHz | 1986 | Interference |
| M.2141 | Rep | Study of the isolation between VHF land mobile radio antennas in close proximity | 2009 | Interference |
| ITU-R Handbook | 0 | Terrestrial land mobile radiowave propagation in the VHF/UHF bands | 2002 | Interference |
| M.1797 | 0 | Vocabulary of terms for the land mobile service | 2007 | Land Mobile |
| M.584 | 2 | Codes and formats for radio paging | 1997 | Paging Systems |
| M.1746 | 0 | Harmonized frequency channel plans for the protection of property using data communication | 2006 | PPDR |
| M.1307 | 0 | Automatic determination of location and guidance in the land mobile services | 1997 | Public Protection Trunked systems Intelligent Transport Systems |
| M.904-2 | Rep | Automatic determination of location and guidance in the land mobile service | 1990 | Public Protection Trunked systems Intelligent Transport Systems |
| M.1042 | 3 | Disaster communications in the amateur and amateur-satellite services | 2007 | Public Protection and Disaster Relief |
| M.1637 | 0 | Global cross-border circulation of radiocommunication equipment in emergency and disaster relief situations | 2003 | Public Protection and Disaster Relief |
| M.2377 | Rep | Radiocommunication objectives and requirements for public protection and disaster relief | 2015 | Public Protection and Disaster Relief |
| M.2085-1 | Rep | Role of the amateur and amateur-satellite services in support of disaster mitigation and relief | 2011 | Public Protection and Disaster Relief |
| M.2009 | 1 | Radio interface standards for use by public protection and disaster relief operations in some parts of the UHF band in accordance with Resolution 646 (Rev.WRC-12) | 2015 | Public Protection and Disaster Relief |
| M.2015 | 1 | Frequency arrangements for public protection and disaster relief radiocommunication systems in UHF bands in accordance with Resolution 646 (Rev.WRC-12) | 2015 | Public Protection and Disaster Relief |
| M.1826 | 0 | Harmonized frequency channel plan for broadband public protection and disaster relief operations at 4940-4990 MHz in Regions 2 and 3 | 2007 | Public Protection and Disaster Relief |
| M.1634 | 0 | Interference protection of terrestrial mobile service systems using Monte Carlo simulation with application to frequency sharing | 2003 | Spectrum Sharing Interference |
| M.1388 | 0 | Threshold levels to determine the need to coordinate between space stations in the broadcasting-satellite service (sound) and particular systems in the land mobile service in the band 1 452-1 492 MHz | 1999 | Spectrum sharing |
| M.1023-1 | Rep | Frequency sharing between the land mobile service and the broadcasting service (television) below 1 GHz | 1990 | Spectrum sharing |
| M.1039 | 3 | Co-frequency sharing between stations in the mobile service below 1 GHz and mobile earth stations of non-geostationary mobile- satellite systems (Earth-space) using frequency division multiple access | 2006 | Spectrum sharing |
| M.1795 | 0 | Technical and operational characteristics of land mobile MF/HF systems | 2007 | Spectrum sharing |
| M.1808 | 0 | Technical and operational characteristics of conventional and trunked land mobile systems operating in the mobile service allocations below 960 MHz to be used in sharing studies | 2007 | Spectrum sharing |
| M.1824 | 1 | System characteristics of television outside broadcast (TVOB), electronic news gathering (ENG) and electronic field production (EFP) in the mobile service for use in sharing studies | 2015 | Spectrum Sharing |
| M.2068 | 0 | Characteristics of and protection criteria for systems operating in the mobile service in the frequency range 14.5-15.35 GHz | 2015 | Spectrum Sharing |
| M.2116-2 | Rep | Characteristics of broadband wireless access systems operating in the land mobile service to be used in sharing studies | 2013 | Spectrum Sharing Wireless Access, including RLANs |
| M.2335 | Rep | Sharing and compatibility analysis of possible amateur service stations with fixed, land mobile, and radiolocation services in the frequency band 5 250-5 450 kHz and the aeronautical mobile service in an adjacent band | 2014 | Spectrum Sharing |
| M.478 | 5 | Technical characteristics of equipment and principles governing the allocation of frequency channels between 25 and 3 000 MHz for the FM land mobile service | 1995 | Technology |
| M.1076 | 1 | Wireless communication systems for persons with impaired hearing | 2015 | Technology |
| M.1678 | 0 | Adaptive antennas for mobile systems | 2004 | Technology |
| M.1075 | 0 | Leaky feeder systems in the land mobile services | 1993 | Technology |
| M.319-7 | Rep. | Characteristics of equipment and principles governing the assignment of frequency channels between 25 and 100 MHz for land mobile services | 1990 | Technology |
| M.778-2 | Rep | Wireless communication systems for persons with impaired hearing | 1990 | Technology |
| M.902-1 | Rep | Leaky-feeder systems in the land mobile service | 1990 | Technology |
| M.1021 | Rep | Equipment characteristics for digital transmission in the land mobile services | 1986 | Technology |
| M.2282 | Rep | Systems for public mobile communications with aircraft | 2013 | Technology |
| M.2026 | Rep | Adaptability of real zero single sideband technology to HF data communications (Note: Responsibility of WP 5C) | 2001 | Technology |
| M.2040 | Rep | Adaptive antennas concepts and key technical aspects | 2004 | Technology |
| M.2114 | Rep | Key technical and operational characteristics for access technologies to support IP applications over land mobile systems | 2007 | Technology |
| M.2117-1 | Rep | Software defined radio in the land mobile, amateur and amateur satellite services | 2012 | Technology |
| M.2225 | Rep | Introduction to cognitive radio systems in the land mobile service | 2011 | Technology |
| M.2330 | Rep | Cognitive radio systems in the land mobile service | 2014 | Technology |
| M.2014-2 | Rep | Digital land mobile systems for dispatch traffic | 2012 | Trunked Systems |
| Volume 3 | 0 | Land Mobile Handbook (including wireless access), Volume 3: Dispatch and Advanced Messaging Systems | 2005 | Trunked Systems Paging Systems |
| M.1450 | 5 | Characteristics of broadband radio local area networks | 2014 | Wireless Access, including RLANs |
| M.1454 | 0 | E.i.r.p. density limit and operational restrictions for RLANS or other wireless access transmitters in order to ensure the protection of feeder links of non-geostationary systems in the mobile-satellite service in the frequency band 5 150-5 250 MHz | 2000 | Wireless Access, including RLANs |
| M.1651 | 0 | A method for assessing the required spectrum for broadband nomadic wireless access systems including radio local area networks using the 5 GHz band | 2003 | Wireless Access, including RLANs |
| M.1652 | 1 | Dynamic frequency selection in wireless access systems including radio local area networks for the purpose of protecting the radiodetermination service in the 5 GHz band | 2005 | Wireless Access, including RLANs |
| M.1653 | 0 | Operational and deployment requirements for wireless access systems including radio local area networks in the mobile service to facilitate sharing between these systems and systems in the Earth exploration-satellite service (active) and the space research service (active) in the band 5 470-5 570 MHz within the 5 460-5 725 MHz range | 2003 | Wireless Access, including RLANs |
| M.1739 | 0 | Protection criteria for wireless access systems, including radio local area networks, operating in the mobile service in accordance with Resolution 229 (WRC-03) in the bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz | 2006 | Wireless Access, including RLANs |
| M.1801 | 2 | Radio interface standards for broadband wireless access systems, including mobile and nomadic applications, in the mobile service operating below 6 GHz | 2013 | Wireless Access, including RLANs |
| M.2378 | Rep | Operational guidelines for the deployment of broadband wireless access systems for local coverage operating below 6 GHz | 2015 | Wireless Access, including RLANs |
| M.2002 | 0 | Objectives, characteristics and functional requirements of wide-area sensor and/or actuator network (WASN) systems | 2012 | Wireless Access, including RLANs |
| M.2224 | Rep | System design guidelines for wide area sensor and/or actuator network (WASN) systems | 2011 | Wireless Access, including RLANs |
| M.2003 | 1 | Multiple gigabit wireless systems in frequencies around 60 GHz | 2015 | Wireless Access, including RLANs |
| M.2227-1 | Rep | Multiple Gigabit Wireless Systems in frequencies around 60 GHz | 2014 | Wireless Access, including RLANs |
| F.748 | 4 | Radio-frequency arrangements for systems of the fixed service operating in the 25, 26 and 28 GHz bands | 2001 | Wireless Access, including RLANs |
| F.749 | 2 | Radio-frequency channel arrangements for radio-relay systems in the 38 GHz band | 2001 | Wireless Access, including RLANs |
| F.1489 | 0 | A methodology for assessing the level of operational compatibility between fixed wireless access (FWA) and radiolocation systems when sharing the band 3.4-3.7 GHz | 2000 | Wireless Access, including RLANs Spectrum Sharing |
| F.1490 | 0 | Generic requirements for fixed wireless access (FWA) systems | 2000 | Wireless Access, including RLANs |
| F.1499 | 0 | Radio transmission systems for fixed broadband wireless access (BWA) based on cable modem standards | 2000 | Wireless Access, including RLANs |
| F.1518 | 0 | Spectrum requirement methodology when a fixed wireless access (FWA) system and a mobile wireless access (MWA) system using the same type of equipment coexist in the same frequency band | 2001 | Wireless Access, including RLANs |
| F.1519 | 0 | Guidance on frequency arrangements based on frequency blocks for systems in the fixed service | 2001 | Wireless Access, including RLANs |
| F.1613 | 0 | Operational and deployment restrictions for FWA systems in Region 3 to ensure the protection of systems in the EES service (active) in the band 5 250-5 350 MHz | 2003 | Wireless Access, including RLANs |
| F.1763 | 1 | Radio interface standards for broadband wireless access systems in the fixed service operating below 66 GHz | 2014 | Wireless Access, including RLANs |
| M.2115 | Rep | Testing procedures for implementation of dynamic frequency selection | 2007 | Wireless Access, including RLANs |
| Volume 1 | 1 | Land Mobile Handbook (including wireless access), Volume 1: Fixed Wireless Access | 2001 | Wireless Access, including RLANs |
| S.2199 | Rep | Studies on compatibility of broadband wireless access (BWA) systems and fixed-satellite service (FSS) networks in the 3 400 4 200 MHz band | 2010 | Wireless Access, including RLANs |
| Volume 5 | 0 | Land Mobile Handbook (including wireless access), Volume 5: Deployment of Broadband Wireless Access systems | 2011 | Wireless Access, including RLANs |
| M.2264 | Rep | Guidance for the development of band plans with contiguous bandwidths for mobile broadband applications for use in spectrum planning | 2012 | Cellular systems Wireless Access, including RLANs |
| M.2057 | Rec | Systems characteristics of automotive radars operating in the frequency band 76-81 GHz for intelligent transport systems applications | 2014 | Intelligent Transport Systems |

_______________

____________________

http://www.itu.int/ITU-R/go/rwp5a.

The term “BAS”, also known as services ancillary to broadcasting (SAB), is defined in Report ITU-R BT.2069.

The terrestrial component of IMT is also part of the Land Mobile service, but it is outside the scope of this document; for further information see for example the ITU Handbook on “Deployment of IMT-2000 Systems” 2003 and its ITU-R Supplement No. 1, 2005. The links are:

Handbook: http://www.itu.int/itudoc/gs/imt2000/84207.html

Supplement: http://www.itu.int/pub/R-HDB-46-2005/en.

ITU-R Working Party 5D: http://www.itu.int/ITU-R/index.asp?category=study-groups&link=rwp5d&lang=en.

The electronic version of this table can be sorted by columns as required.