3GPP TSG RAN WG1 Meeting #86bis	R1-1609605

# Lisbon, Portugal, 10th - 14th October 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0289 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DMRS for MPDCCH associated with P-RNTI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Virtuosys Ltd, Qualcomm Incorporated, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-09-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In section 6.8B.2 the scrambling for the MPDCCH is specified. It is stated that for MPDCCH with P-RNTI that Nacc=4 for frame structure type 1 and for frame structure type 2. For other MPDCCH the Nacc=1 for CEMode A and either Nacc=4 or Nacc=10 for CEMode B depending on the frame structure. However, in section 6.10.3A.1 where the DMRS is defined the case for MPDCCH with P-RNTI is missing from the specification. Thus for CEMode A in the case of MPDCCH with P-RNTI the scrambling and DMRS will use different values of Nacc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the case specifying the DMRS for MPDCCH associated with P_RNTI in order to align Nacc with the value used for scrambling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For MPDCCH associated with P-RNTI for CEMode A (frame structure type 1) then Nacc=4 for the scrambling but Nacc=1 for the DMRS. Therefore it will not be possible to use cross-subframe channel estimation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact. |  |  |  |  |  |  |  |  |

### 6.8B.2	Scrambling

Scrambling shall be perfomed according to Clause 6.8A.2 with EPDCCH replaced by MPDCCH except that the same scrambling sequence is applied per subframe to MPDCCH for a given block of  subframes and  is the MPDCCH set number. For MPDCCH format 5,  is used to generate the scrambling sequence for mapping to REs in 6 PRBs.

For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for the MPDCCH. The MPDCCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the MPDCCH transmission is postponed.

For BL/CE UEs,

-	if the MPDCCH transmission is associated with P-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	 for UEs assuming CEModeA (according to the definition in section 12 of [4]) or configured with CEModeA:

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in section 12 of [4]) or configured with CEModeB.

#### < Unchanged parts are omitted >

#### 6.10.3A.1	Sequence generation

For any of the antenna ports , the reference-signal sequence  is defined by

.

For non-BL/CE UEs, the pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe where  and  is configured by higher layers. The EPDCCH set to which the EPDCCH associated with the demodulation reference signal belong is denoted .

For BL/CE UEs, the same scrambling sequence is applied per subframe to the demodulation reference signal associated with MPDCCH for a given block of  subframes. For the block of   subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for MPDCCH. The MPDCCH transmissions span  consecutive subframes, including non-BL/CE UL subframes where the MPDCCH transmission is postponed.

For BL/CE UEs,

-	if the MPDCCH transmission is associated with P-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	 for UEs assuming CEModeA (according to the definition in section 12 of [4]) or configured with CEModeA.

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in section 12 of [4]) or configured with CEModeB.

The quantities  and  are configured by higher layers. The MPDCCH set to which the MPDCCH associated with the demodulation reference signal belong is denoted . For an MPDCCH associated with a 2+4 PRB set as defined in [4],  is used to generate the scrambling sequence for the 6 PRBs as well as for the 2 PRBs and 4 PRBs.

< Unchanged parts are omitted >