3GPP TSG-RAN Meeting #75       				             RP-170249

Dubrovnik, Croatia, March 6 - 9, 2017

Source: 	KT Corp.

Title: 	Text Proposal for TR 38.805: Regulations for 60GHz in Korea

Agenda Item:	9.4

Document for:	Approval

# 1	Introduction

At RAN#73 it was agreed to start e-mail discussion until RAN#75 on regulatory landscape of the 60GHz Band. KT volunteers to provide information of the regulations for 60GHz in Korea according to [1]. Text proposal is provided in the Annex.

# 2. Reference

[1] (MSIP Notification 2016-52) Regulations for Radio Equipment. Ministry of Science, ICT and Future Planning. (http://www.msip.go.kr/cms/www/news/law/law_2/__icsFiles/afieldfile/2016/05/30/(%EB%AF%B8%EB%9E%98%EB%B6%80%EA%B3%A0%EC%8B%9C%20%EC%A0%9C2016-52%ED%98%B8)%EB%AC%B4%EC%84%A0%EC%84%A4%EB%B9%84%EA%B7%9C%EC%B9%99%20%EC%A0%84%EB%AC%B8.hwp) (Document in Korean)

# Annex	Revision to TR 38.805

------------------------------------ BEGIN TEXT PROPOSAL 1 ---------------------------------

### 4.3.3	Korea

Unlicensed spectrum range 57 to 66GHz needs to meet following regulations:

- Antenna power shall be less than 500mW. For omni-directional antenna, power shall be less than 100mW

- Power Density shall be less than 13dBm/MHz

- EIRP shall be less than 43dBm. Except for point to point communication purpose where EIRP shall be less than 57dBm

- Antenna absolute gain shall be less than 16dBi. However, when antenna absolute gain is over the regulation limit, equivalent amount of antenna power shall be decreased

- Occupied bandwidth shall be within the range of 57 to 66GHz

- Unwanted emission for 57 – 66GHz spectrum shall satisfy below table:

| Spectrum Range | Unwanted Emission Limit | Measurement Bandwidth |
| --- | --- | --- |
| Less than 1GHz | -36dBm | 100kHz |
| 1GHz to 40GHz | -30dBm | 1MHz |
| More than 40GHz | -10dBm | 1MHz |

- Standby status of transmit/receive secondary emissions shall follow below limits:

| Frequency | Reference Value (Average) | Reference Bandwidth |
| --- | --- | --- |
| Less than 1GHz | -54dBm | 100kHz |
| 1GHz and above | -47dBm | 1MHz |

- To avoid malfunction of/from other equipment, devices shall use unique identification code. However this regulation does not apply to fixed point to point communication devices.

- For spectrum range of 57 to 58GHz, EIRP of more than 27dBm shall indicate following text in the manual: “When device is needed to be installed within 300m of Radio Astronomical Antenna, prearranged mutual agreement with Astronomical Observatory shall be made in advance”.

------------------------------------ END TEXT PROPOSAL 1 -----------------------------------------