3GPP TSG RAN ad hoc	RPA160021

Barcelona, Spain, 28th - 29th January, 2016

Agenda item:	5

Source:	ZTE

Title:	Deployment Scenarios for eMBB

Document for:		Discussion

Introduction

In 3GPP RAN#70 meeting, the “SI Scenarios and Requirements for Next Generation Access Technologies” was approved with the aim to identify typical usage scenarios for next generation access technologies and the required capabilities in each corresponding usage scenarios, and to provide guidance to the technical work to be performed in RAN WGs. One of the objectives in the SID[1] regarding scenarios is to:

- Identify the typical deployment scenarios associated with attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, etc.

This contribution presents our views on the indoor hotspot and dense urban deployment scenarios for eMBB.

Deployment scenarios for eMBB

The requirements of the future communications systems, such as higher data rate, higher connection density, better coverage and more consistent user experience, [2] have been widely discussed in 3GPP SA and organizations such as ITU-R, NGMN and IMT-2020(PG) in China[3][4][5].  It is a common understanding that different deployment scenarios can be used to satisfy different requirements.  Hotspot is the major type of scenarios for achieving higher user data rate.  In this section we discuss about our consideration on two scenarios - indoor hotspot and dense urban for hotspot environment.

- 2.1	Indoor hotspot

Indoor hotspot, one of the typical eMBB deployment scenarios [6], can be considered as a scenario with isolated cells in a building/hotspot serving stationary and pedestrian users. Moreover, indoor hotspot scenario focuses on achieving higher user throughput by deploying denser sites and hence the smallest cells.  Indoor hotspot scenario was considered when 4G scenarios [7] were defined and evaluated. In order to meet new challenges in designing new radio access system, the scenario needs to be re-considered with some new elements added.

Table 1 Parameters for indoor hotspot scenario

| Attribute | Value |
| --- | --- |
| Deployment scenarios | Indoor Hotspot |
| Carrier Frequency | When carrier frequency is lower than 6GHz, carrier frequency is 3.5GHz; |
|  | When carrier frequency is higher than 6GHz, carrier frequency is is FFS |
| Total system bandwidth | 100MHz for LF |
|  | FFS for HF |
| Network layout | Small cell only Indoor floor |
| Inter-Site Distance (ISD) | 10-20m |
| Antenna configuration | up to 32 antenna elements with XPOL for LF |
|  | FFS for HF |
| Maximum mobility speed | 3km/h |
| Backhaul/Fronthaul | Ideal or very fast |
| Device distribution | 100% indoor |

Table 1 shows the parameters for the indoor hotspot scenario.  Consideration on these parameters is discussed below.

Network Layout and Inter-Site Distance

Figure 1(a) Indoor hotspot scenarios with ISD=10m

Figure 1(b) Indoor hotspot scenarios with ISD=15m

As to the network layout, we propose to follow existing indoor hotspot environment in [8] which consists of one of floor of a building. The height of the floor is 6m. The floor contains 16 room of 15m X 15m and a long hall of 120m× 20m as shown in figures 1(a) and 1(b) with ISD=10m and 15m respectively.  Figure 2 shows the spectral efficiency of indoor scenario with ISD=10m and 15m with 2Tx and 4Tx equipped at each small cell.  It can be observed that 15m ISD provides better cell average spectral efficiency while 10m ISD provides better cell edge spectral efficiency.  Therefore, we propose to allow the smallest ISD=10m.

Figure 2 Spectral efficiency of indoor scenario with different ISDs

Carrier Frequency

As to carrier frequency, both below 6GHz (LF) and above 6GHz (HF) should be considered as the candidate for this scenario, and priority is the same between LF and HF in current phase. The total system bandwidth is 100MHz-300MHz for LF.  Decision on the carrier frequency and bandwidth will depend on the SI of “channel model for frequency spectrum above 6 GHz” [9].

Antenna Configuration

Regarding antenna configuration, the number of antenna elements depends on the carrier frequency as the antenna spacing depends on the wavelength.  If carrier frequency is LF, the number of antenna elements can be up to 32 with dual polarized antennas.  If the frequency is HF, the number of antenna elements could be higher than LF but the number of antenna elements depends on the frequency band.

Backhaul/Fronthaul

In this scenario, ideal or very fast backhaul/fronthaul is considered as it is relatively easier to support very fast wireline access such as fiber or VDSL in indoor hotspot such as in office.  Different partition schemes of NGFI (Next Generation Fronthaul Interface) can be considered in this scenario.  The partition scheme would affect the latency and throughput of the fronthaul interface.

- 2.2	Dense Urban

Dense urban is another eMBB scenarios considered for denser network deployment scheme, higher user densities and higher traffic loads than before.  Moreover, dense urban focuses on achieving higher user throughput by deploying denser sites.  Unlike indoor hotspot, dense urban scenario considers multi-tier of outdoor cells.  Table 2 shows the parameters we considered for the dense urban scenario.

Table 2 Parameters for dense urban scenario

| Attribute | Value |  |
| --- | --- | --- |
|  | Macro/micro cells | small cells |
| Network layout | macro cells with outdoor small cells |  |
| Inter-Site Distance (ISD) | 200-500m | More than 10 small cells in a cluster i.e. around 10-50m |
| Carrier Frequency | 2G / 3.5GHz | When carrier frequency is lower than 6GHz, carrier frequency is 3.5GHz; |
|  | When carrier frequency is higher than 6GHz, carrier frequency is is FFS |  |
| The total system Bandwidth | 100MHz for LF | 100MHz for LF |
|  | FFS for HF | HF is FFS |
| Antenna Configuration | Up to 128 antenna elements for LF | Up to 64 antenna elements for LF |
|  | FFS for HF |  |
| Backhaul | Non-Ideal backhaul/ Self backhaul |  |
| Maximum mobility speed | 3km/h for indoor UEs 30km/h for outdoor UEs |  |
| Device distribution | 50% indoor, 50% outdoor More Option is FFS |  |
| Maximum mobility speed | 3km/h indoor UE 30km/h outdoor UE |  |

Network Layout and Inter-Site Distance

Figure 3 Dense urban scenario with 3 layers

The network topology of Dense Urban deployment scenario is multi-tier network layout as shown in figure 3.  It can be up to 3 layers which consists of macro, micro and outdoor small cell layers. Comparing with the heterogeneous network considered in [7] , Dense Urban should consider denser deployment.

Carrier Frequency

Multi-layers may operate in different frequency bands.  Both macro cells and micro cells operate in bands below 6GHz i.e. LF bands.  Macro cells typically operate in 2GHz which provides better coverage but 3.5GHz can be considered at as well.  Micro cells operate in 3.5GHz typically but can consider high frequency in this stage.   For small cells, both below 6GHz (LF) and above 6GHz (HF) should be considered as the candidate for this scenario, and priority is the same between LF and HF in current phase.  The total system bandwidth is 100MHz-300MHz for LF.  Decision on the carrier frequency and bandwidth will depend on the SI of “channel model for frequency spectrum above 6 GHz” [9].

Antenna Configuration

Regarding antenna configuration, the number of antenna elements depends on the carrier frequency as the antenna spacing depends on the wavelength.  If carrier frequency is LF, the number of antenna elements can be up to 128 for macro/micro cells and up to 64 for small cells.  If the frequency is HF, the number of antenna elements could be higher than LF but needs FFS.

Backhaul

In this scenario, non-ideal backhaul should be considered as it is relatively difficult to deploy very fast wireline backhaul access especially between multiple layers.  Both wireline and wireless backhaul can be considered in this scenario.  Self backhaul can be considered between micro and small cells or between small cells.  The latency and throughput should be derived from explicit model which takes into account different backhaul schemes e.g. access and backhaul integration.

Conclusions

In this contribution, we discuss about the indoor hotspot and dense urban deployment scenarios for eMBB.  Based on the discussion, we have the following proposals:

Proposal 1: For eMBB scenarios, both below 6GHz (LF) and above 6GHz (HF) should be considered as the candidate of carrier frequency for small cells, and priority is the same between LF and HF in current phase. The total system bandwidth is 100MHz-300MHz for LF.

Proposal 2: Ultra dense network should be considered in eMBB scenarios. The shortest ISD is 10m in indoor hotspot. In dense urban, more than 10 outdoor small cells should be considered in a small cell cluster.

Proposal 3: The number of antenna elements for indoor hotspot can be up to 32 for LF. In dense urban, the number of antenna elements can be up to 128 for macro/micro cells and up to 64 for small cells for LF.

Proposal 4:  Backhaul/fronthaul technologies should be considered in eMBB scenarios.  Ideal or very fast backhaul/fronthaul and different partition schemes of NGFI can be considered in this indoor hotspot.  Non-ideal backhaul should be considered in dense urban scenario.  Self backhaul can be considered between micro and small cells or between small cells.

References

[1] 3GPP RAN #70, RP-151760 “New SI proposal-Scenarios and Requirements for Next Generation Access Technologies” CMCC

[2] 3GPP SA1, S1-154454, “Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1”

[3] ITU-R IMT Vision – “Framework and overall objectives of the future development of IMT for 2020 and beyond” 2015

[4] NGMN Alliance, "5G White Paper," 2015.

[5] IMT-2020(PG), “White Paper on 5G Concept”, 2015

[6] ITU-R Recommendation M.2083,"IMT Vision - Framework and overall objectives of the future development of IMT for 2020 and beyond, " 2015

[7] 3GPP TR 36.872: “Small cell enhancement for E-UTRA and E-UTRAN physical layer aspects”

[8] ITU-R Report M.2135“ Guidelines for evaluation of radio interface technologies for IMT-Advanced”, 2009

[9] SID in RP-151854 Study Item “Ichannel model for frequency spectrum above 6 GHz”